3GPP TSG-RAN WG3 Meeting #128                                                                    R3-253303

Malta, MT, 19th – 23rd, May 2025

Source:	CATT

Title:	(TP for BLCRs to 38.423, 38.473) On resource coordination and Xn management for WAB

Agenda Item:	12.2

Document for:	Approval

1 Introduction

In RAN3#127bis meeting [1], additional topological enhancements for NR are studied. And a part of agreements of resource coordination have been provided.

In this contribution, we will discuss remain issue of resource coordination and Xn management to WAB and provide the TPs to TS 38.423 and TS 38.473.

2 Discussion

2.1 Resource coordination

2.1.1 XnAP signalling design

In the last meeting, the agreements of resource coordination are as follow:

When access and backhaul links of a WAB node are operated out-of-band, there is no need for WAB resource coordination. RAN3 assumes that WAB deployments use out-of-band operation in case access and backhaul use different PLMNs. In this release, In-band WAB operation is only considered for intra PLMN scenarios where BH gNB is upgraded with WAB-specific enhancements. For WAB resource coordination, introduce a new class-1 Xn procedure between WAB-gNB and BH gNB. The message content can be based on relevant parts of the XnAP IEs defined in clauses 9.2.2.94-97 of TS 38.423. FFS whether to reuse the F1AP NB-DU RESOURCE CONFIGURATION procedure between BH gNB-CU and BH gNB-DU. After RAN3 has finished specifying WAB resource coordination, RAN3 to send an LS to inform RAN1 and RAN2 about conclusions.

In IAB, an IAB-node is served as a UE by both F1-terminating donor and non-F1-terminating donor, and the resource coordination information for the IAB-node is exchanged via the F1-terminating donor and non-F1-terminatig donor over Xn, so the IAB Resource Coordination procedure is a UE-associated Xn procedure. Different to IAB, where the resource coordination information for the IAB-node must be transferred via its donor-CUs, the WAB-node can exchange the resource coordination information with the BH-RAN node by itself. Thus, the resource coordination procedure of WAB should be non-UE associated.

Proposal 1-1: The new Class 1 XnAP procedure is non-UE associated.

The WAB-gNB can perform resource coordination with a gNB only if the WAB-gNB recognizes it is BH-gNB. Normally, when accessing the BH-gNB, a WAB-MT can read the gNB ID in SIB1 broadcasted by the serving cell. Then the co-located WAB-gNB can recognize the BH-RAN-node, set up Xn connection with the BH-gNB and send the WAB-MT identifier to the BH-gNB for the co-location discovery of WAB-MT and WAB-gNB, after which the resource coordination between the WAB-gNB and BH-gNB happens.

Thus, for resource coordination, WAB-MT should be capable of decoding the gNB ID in SIB1. RAN3 should send LS to RAN2 for introducing this WAB-MT capability.

Proposal 1-2: WAB-MT should optionally support the capability of decoding gNB ID in SIB1. RAN3 can send LS to RAN2 for specifying this capability of WAB-MT.

According to the agreement above, the cell resource configuration exchanged between the WAB-gNB and the BH-RAN node via the new Xn procedure can be based on clauses 9.2.2.94-97 of TS 38.423 for the resource configuration of IAB cell.

- Clause 9.2.2.94 (IAB Cell Information)

This IE contains IAB cell resource configuration, including NR CGI, resource configuration, cell specific signal configuration and multiplexing info of IAB cell. The resource configuration includes the frequency, bandwidth and resource parameters (clause 9.2.2.95) of IAB cell. The multiplexing information indicates whether simultaneous operation of IAB-MT Rx / IAB-gNB Rx, IAB-MT Tx / IAB-gNB Tx, IAB-MT Rx / IAB-gNB Tx, IAB-MT Tx / IAB-gNB Rx is supported. The cell specific signal configuration for the IAB-DU cell is considered as hard resources of IAB-DU cell. The relevant configurations are sent to the IAB-donor CU, including STC info, RACH Config Common, CSI-RS Configuration, SR Configuration and PDCCH Configuration SIB1. All the content in this IE can be used in WAB cell equivalently.

- Clause 9.2.2.95 (gNB-DU Cell Resource Configuration)

This IE contains the TDD/FDD resource parameters for an activated IAB cell, including subcarrier spacing, DUF transmission periodicity, DUF slot configuration List, HSNA transmission periodicity, HSNA slot configuration list, RB set configuration, frequency-domain HSNA configuration list, NA cell resource configuration list. The DUF slot configuration list includes DUF slot configuration of every slot. The HSNA slot configuration list includes HSNA slot configuration of every slot. RB set configuration indicates the configuration for non-overlapping RB sets for a given DU cell, used for frequency domain resource allocation. The frequency-domain HSNA configuration list includes frequency-domain HSNA slot configuration of every slot. NA cell resource configuration List includes the list of unavailable resources of the cell for the dual-connected boundary IAB-node.

Subcarrier spacing, DUF transmission periodicity, DUF slot configuration List, HSNA transmission periodicity, HSNA slot configuration list, RB set configuration, frequency-domain HSNA configuration list in this IE can be used for WAB cell equivalently. But, NA cell resource configuration list is unnecessary for WAB.

- Clause 9.2.2.96 (IAB STC Info)

This IE contains cell SSB Transmission Configuration (STC) information of an IAB-DU or an IAB-donor-DU, including IAB STC-Info List. All the content in this IE can be used in WAB cell equivalently.

- Clause 9.2.2.97 (RB Set Configuration)

This IE contains the configuration for non-overlapping RB sets for a given gNB-DU cell, used for frequency domain resource allocation, including subcarrier spacing, RB set size, number of RB sets. All the content in this IE can be used in WAB cell equivalently.

WAB cell resource configuration can include all the equivalent content in clauses 9.2.2.94-97 for IAB cell resource configuration, except the NA cell resource configuration in clause 9.2.2.95.

Proposal 1-3: WAB cell resource configuration can include all the equivalent content in clauses 9.2.2.94-97 for IAB cell resource configuration, except the NA cell resource configuration in clause 9.2.2.95.

Similar to IAB, there are three symbol types configured for each slot for WAB cell. The hard symbols can be used by WAB cell, the unavailable symbols cannot be used by WAB cell. The soft symbols can be conditionally used by WAB cell conditionally. For determining the availability of soft symbols, 3 alternatives are possible:

- Alt.1: New L1 signalling is introduced for the BH-node explicitly indicating availability of soft symbols

- Alt.2: DCI format 2_5 is reused for the BH-node explicitly indicating availability of soft symbols

- Alt.3: WAB-node determines availability of soft symbols up to implementation

Alt.1 has more impact than Alt.2 and Alt.3 to RAN1 spec. As no TU is allocated to RAN1, the impact to RAN1’s specs should be as little as possible.

For this issue is in RAN1’s scope, RAN1 should discuss the down select. After RAN3 has finished specifying WAB resource coordination, RAN3 can send an LS to RAN1 for information and RAN1 can discuss how to determine the availability of slot symbols.

Since BH-RAN-node is legacy NG-RAN node, the resource configuration of BH-RAN-node’s cell serving the WAB-MT only needs to include the cell specific signal configuration including STC info, RACH Config Common, CSI-RS Configuration, SR Configuration and PDCCH Configuration SIB1, and DUF slot configuration. The relevant IEs in clause 9.2.2.95/96 can be reused.

Proposal 1-4: Resource configuration of BH-gNB cell serving the WAB-MT can include cell specific DUF slot configuration.

For resource coordination, the BH-gNB can send the resource configuration of the BH-gNB cells and the (recommended) resource configuration of the WAB-gNB cells to the WAB-gNB, and WAB-gNB can send the resource configuration of the WAB-gNB cells to the BH-gNB. If WAB-gNB can also send the (recommended) resource configuration of the BH-gNB to the BH-gNB is FFS.

Proposal 1-5: The BH-gNB can send the resource configuration of the BH-gNB cells and the (recommended) resource configuration of the WAB-gNB cells to the WAB-gNB, and WAB-gNB can send the resource configuration of the WAB-gNB cells to the BH-gNB. If WAB-gNB can also send the (recommended) resource configuration of the BH-gNB cells to the BH-gNB is FFS.

In the last meeting [1], the MT identifier transferring via XnAP messages for resource multiplexing was agreed:

Include a WAB-MT Identifier in the XN SETUP REQUEST, XN SETUP RESPONSE, NG-RAN NODE CONFIGURATION UPDATE and NG-RAN NODE CONFIGURATION UPDATE ACK e.g. for colocation discovery for resource multiplexing or for WAB node indication. It is understood that reception of the WAB-MT Identifier indicates that the sender is a WAB node. The receiver will behave accordingly.

According above method, WAB-gNB can report the identity of WAB-MT assigned by the BH-RAN node over Xn to the BH-RAN-node, so that BH-RAN-node can discover co-location of the WAB-MT and the WAB-gNB by the identity of WAB-MT.

Xn setup of the WAB-node is after the WAB-MT accessing to BH-RAN node during integration, thus the BH-RAN-node can always allocate C-RNTI to WAB-MT before the Xn setup of the WAB-node.

The WAB-MT’s C-RNTI assigned can be used as the identifier. As C-RNTI is a cell unique identity, the identifier should also include identity of the cell serving the WAB-MT, so that the WAB-MT can be uniquely identified by the BH-RAN node.

Proposal 1-6: WAB-MT identifier sent via XnAP messages includes the WAB-MT’s C-RNTI allocated by the BH-RAN node and the identity of the cell serving the WAB-MT.

With the WAB-MT identifier sent via Xn Setup messages and NG-RAN Node Configuration Update messages, the BH-RAN node can discover the co-location of the WAB-MT with a WAB-gNB and initiate resource coordination towards the WAB-gNB over Xn.

The WAB-MT identifier can also be included in messages of the new Xn procedure, to facilitate, for example, the WAB-gNB initiating the resource coordination to the BH-RAN node, in which case WAB-gNB can just include the WAB-MT identifier in the resource coordination request message.

Proposal 1-7: The WAB-MT identity information can be included in the request message of the new class-1 Xn procedure.

2.1.2 F1AP signaling design

For IAB resource coordination, the gNB-DU Resource Configuration procedure was introduced in F1AP. This procedure is used for IAB-donor-CU to configure the resource usage for an IAB-DU.

In WAB resource coordination, if the BH-gNB is split architecture, the BH-gNB-CU should also configure the resource usage for the BH-gNB-DU. The BH-gNB-CU can send the resource configuration for the BH-gNB-DU and the resource configuration of WAB-gNB whose co-located WAB-MT is served by the BH-gNB-DU.

The message sent by a BH-gNB-CU can be based on the GNB-DU RESOURCE CONFIGURATION message in TS 38.473 with BH-gNB-CU corresponding to IAB-donor-CU and BH-gNB-DU corresponding to the IAB-DU.

The GNB-DU RESOURCE CONFIGURATION message includes information:

- Resource configuration of IAB-DU

- Resource configuration of child IAB-DU

- Resource configuration of neighbour IAB-DU

- NA resource configuration of parent IAB-DU

The last two are unnecessary for WAB. “Resource configuration of IAB-DU” can correspond to resource configuration of BH-gNB-DU, but it’s FFS whether resource of BH-gNB-DU can be reconfigured by BH-gNB-CU. “Resource configuration of child IAB-DU” can correspond to resource configuration of WAB-gNB.

“Resource configuration of child IAB-DU” contains the child IAB-MT ID, cell specific signal configuration and resource configuration for child IAB-DU’s cell (clause 9.3.1.279) which further includes frequency, bandwidth and resource parameters (clause 9.2.1.107). So, resource configuration of WAB-gNB can contain the WAB-MT identifier, cell specific signal configuration and resource configuration of WAB-gNB cell which further includes frequency, bandwidth and resource parameters.

Resource parameters of WAB-gNB cell can be based on clause 9.2.1.107. All the IEs in clause 9.2.1.107 except Child IAB-Nodes NA Resource and Parent IAB Nodes NA Resource Configuration IEs can be used for WAB cell equivalently.

Proposal 1-8: WAB-gNB resource configuration can be sent to the BH-gNB-DU including the equivalent content of “Resource configuration of child IAB-DU” in the existing GNB-DU RESOURCE CONFIGURATION message, except Child IAB-Nodes NA Resource and Parent IAB Nodes NA Resource Configuration IEs. It’s FFS whether the resource configuration of BH-gNB-DU can be reconfigured by BH-gNB-CU.

Since the gNB-DU Resource Configuration procedure belongs to IAB procedures, and the IEs in the messages of this procedure are highly relevant to IAB node, thus, it’s better not to reuse this procedure for WAB between BH-gNB-CU and BH-gNB-DU. A new F1 procedure for BH-gNB-CU to configure the resource usage of WAB-gNB should be introduced instead.

Proposal 1-9: A new F1 procedure should be introduced for BH-gNB-CU to send the resource configuration of WAB-gNB to BH-gNB-DU.

2.2 Xn connection management

2.2.1 Dynamic Xn setup

Based on the WID approved during RAN#105 [2], Xn interface setup for the WAB-gNB with BH-RAN-node or surrounding NG-RAN nodes should be specified, which has not been captured in [3]. Different to normal gNB, Xn setup is dynamic for WAB-gNB, due to the neighbour relationship may change with the movement of WAB-node.

As a legacy mechanism, UEs served by WAB-node can detect the PCI of the neighbour cell of WAB-gNB, and UE’s CGI reporting procedure can be used to derive the RAN-node ID of the neighbour nodes. However, the CGI acquisition of UE is not reliable due to the WAB-node’s movement. Other methods should be studied.

As the RAN-node ID of the BH-RAN-node is broadcasted in SIB1, the WAB-gNB can recognize the BH-RAN-node after the co-located WAB-MT obtaining SIB1 of the serving cell. Then, the WAB-gNB should obtain IP address of the BH-RAN-node. IP address of the BH-RAN-node can be obtained by the legacy Xn-C TNL address discovery mechanism.

Proposal 2-1: WAB-gNB can recognize BH-RAN-node through co-located WAB-MT reading SIB1 of the serving cell.

Proposal 2-2: WAB-gNB obtains IP address of the BH-RAN-node via legacy Xn-C TNL address discovery mechanism to set up Xn connection with the BH-RAN node.

If setting up Xn with the BH-RAN-node, the WAB-gNB may discover some neighbour cells’ CGI from the serving cell information delivered by the BH-RAN-node over Xn interface.

There may be other neighbour cells of WAB-gNB, which are supposed in scope of the neighbour cells of the BH-RAN-node. The WAB-gNB can also discover neighbour cells’ CGI from the neighbor cells of the BH-RAN-node. The information of BH-RAN-node’s neighbour cells is sent from BH-RAN-node via Neighbour Information NR IE or Neighbour Information E-UTRA IE in some Xn messages, including Xn SETUP REQUEST/RESPONSE message, NG-RAN NODE CONFIGURATION UPDATE message, and NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

After discovering the neighbour cells’ CGI, the WAB-gNB can add the neighbour cells to the NCRT and look up the TNL address of the neighbour nodes to set up Xn interface. The WAB-gNB can get the IP addresses of the neighbour RAN-nodes using the legacy Xn-C TNL address discovery mechanism. This procedure impacts only stage-2.

Proposal 2-3: If WAB-gNB establishes Xn connection with BH-RAN-node, it discovers neighbour cells through Xn messages sent from the BH-RAN-node.

Proposal 2-4: WAB-gNB looks up the IP address(es) of the neighbour nodes using legacy Xn-C TNL address discovery mechanism to set up Xn connections with those RAN nodes.

2.2.2 Dynamic Xn removal

Due to the mobility of WAB-node, the NCRT in the WAB-gNB may change more frequently than legacy gNB. The WAB-node would need to set up Xn with new neighbour nodes and release Xn with the original neighbour nodes during its movement. The WAB-node can discover PCI of the new neighbour cell through the UE’s measurement result.

If the WAB-MT connects to a new BH-RAN-node and WAB-gNB establishes Xn with the new BH-RAN-node, the WAB-gNB can derive CGI of the current neighbour cells through serving cell information or neighbour cell information sent by the new BH-RAN-node over Xn. The WAB-gNB may add the new neighbour cells to the neighbour relationship, and set up Xn connection with the new neighbour nodes. Accordingly, the WAB-gNB would have to remove cells from the NCRT dynamically.

The WAB-gNB will consider the cell(s), which is not included in the serving/neighbour cell information of current BH-RAN-node, no longer acting as the neighbouring cells and remove the cell(s) from the NCRT. If there is any NG-RAN node which has already set up Xn connection with the WAB-gNB no longer serves the neighbour cell of the WAB-gNB, the WAB-gNB can initiate Xn removal to the NG-RAN node.

Proposal 2-5: WAB-gNB considers the cell not included in the serving/neighbour cell information of current BH-RAN-node as no longer being the neighbour cell.

Proposal 2-6: WAB-gNB removes Xn with the NG-RAN nodes which do not serve any neighbour cell of the WAB-gNB.

3 Conclusion

In the previous sections we made the following proposals:

Resource coordination

Proposal 1-1: The new Class 1 XnAP procedure is non-UE associated.

Proposal 1-2: WAB-MT should optionally support the capability of decoding gNB ID in SIB1. RAN3 can send a LS to RAN2 for specifying this capability of WAB-MT.

Proposal 1-3: WAB cell resource configuration can include all the equivalent content in clauses 9.2.2.94-97 for IAB cell resource configuration, except the NA cell resource configuration in clause 9.2.2.95.

Proposal 1-4: Resource configuration of BH-gNB cell serving the WAB-MT can include cell specific DUF slot configuration.

Proposal 1-5: The BH-gNB can send the resource configuration of the BH-gNB cells and the (recommended) resource configuration of the WAB-gNB cells to the WAB-gNB, and WAB-gNB can send the resource configuration of the WAB-gNB cells to the BH-gNB. If WAB-gNB can also send the (recommended) resource configuration of the BH-gNB cells to the BH-gNB is FFS.

Proposal 1-6: WAB-MT identifier sent via XnAP messages includes the WAB-MT’s C-RNTI allocated by the BH-RAN node and the identity of the cell serving the WAB-MT.

Proposal 1-7: The WAB-MT identity information can be included in the request message of the new class-1 Xn procedure.

Proposal 1-8: WAB-gNB resource configuration can be sent to the BH-gNB-DU including the equivalent content of “Resource configuration of child IAB-DU” in the existing GNB-DU RESOURCE CONFIGURATION message, except Child IAB-Nodes NA Resource and Parent IAB Nodes NA Resource Configuration IEs. It’s FFS whether the resource configuration of BH-gNB-DU can be reconfigured by BH-gNB-CU.

Proposal 1-9: A new F1 procedure should be introduced for BH-gNB-CU to send the resource configuration of WAB-gNB to BH-gNB-DU.

Xn connection management

Proposal 2-1: WAB-gNB can recognize BH-RAN-node through co-located WAB-MT reading SIB1 of the serving cell.

Proposal 2-2: WAB-gNB obtains IP address of the BH-RAN-node via legacy Xn-C TNL address discovery mechanism to set up Xn connection with the BH-RAN node.

Proposal 2-3: If WAB-gNB establishes Xn connection with BH-RAN-node, it discovers neighbour cells through Xn messages sent from the BH-RAN-node.

Proposal 2-4: WAB-gNB looks up the IP address(es) of the neighbour nodes using legacy Xn-C TNL address discovery mechanism to set up Xn connections with those RAN nodes.

Proposal 2-5: WAB-gNB considers the cell not included in the serving/neighbour cell information of current BH-RAN-node as no longer being the neighbour cell.

Proposal 2-6: WAB-gNB removes Xn with the NG-RAN nodes which do not serve any neighbour cell of the WAB-gNB.

4 Reference

- RAN3#127bis agenda, RAN3 Chair

- RP-242359, New WID on additional topological enhancements for NR, NTT DOCOMO, INC., AT&T, RAN#105

- R3-244849, 3GPP TR 38.799, Study on additional topological enhancements for NR, v1.2.0 (2024-08)

5 Annex A (TP to BLCR for TS 38.423)

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Start of TP<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

# 8	XnAP procedures

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | XN REMOVAL REQUEST | XN REMOVAL RESPONSE | XN REMOVAL FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| IAB Transport Migration Management | IAB TRANSPORT MIGRATION MANAGEMENT REQUEST | IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE | IAB TRANSPORT MIGRATION MANAGEMENT REJECT |
| IAB Transport Migration Modification | IAB TRANSPORT MIGRATION MODIFICATION REQUEST | IAB TRANSPORT MIGRATION MODIFICATION RESPONSE |  |
| IAB Resource Coordination | IAB RESOURCE COORDINATION REQUEST | IAB RESOURCE COORDINATION RESPONSE |  |
| Partial UE Context Transfer | PARTIAL UE CONTEXT TRANSFER | PARTIAL UE CONTEXT TRANSFER ACKNOWLEDGE | PARTIAL UE CONTEXT TRANSFER FAILURE |
| Data Collection Reporting Initiation | DATA COLLECTION REQUEST | DATA COLLECTION RESPONSE | DATA COLLECTION FAILURE |
| WAB Resource Coordination | WAB RESOURCE COORDINATION REQUEST | WAB RESOURCE COORDINATION RESPONSE |  |

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Next Change<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

## 8.6	WAB Procedures

### 8.6.1	WAB Resource Coordination

#### 8.6.1.1	General

The purpose of the WAB Resource Coordination procedure is to exchange the semi-static radio resource configuration pertaining to a WAB-node and its BH-gNB, for the purpose of resource multiplexing between the WAB-MT and the WAB-gNB of the WAB-node. The procedure can be initiated by the WAB-node or the BH-gNB. The procedure can be used for WAB-node.

The procedure uses non-UE-associated signalling.

#### 8.5.4.2	Successful Operation

Figure 8.5.4.2-1: WAB Resource Coordination triggered by BH-gNB/WAB-node, successful operation

The BH-gNB/WAB-node initiates the procedure by sending the WAB RESOURCE COORDINATION REQUEST message to the WAB-node/BH-gNB. The WAB-node/BH-gNB shall respond with the WAB RESOURCE COORDINATION RESPONSE message to the BH-gNB/WAB-node.

If the WAB-MT Identity Information IE is included in the WAB RESOURCE COORDINATION REQUEST, the receiving BH-gNB shall consider the information therein for discovering the co-location of a WAB-gNB and a WAB-MT.

If the WAB-gNB Cells List IE and/or BH-RAN Node Cells List IE is included in the WAB RESOURCE COORDINATION REQUEST or in the WAB RESOURCE COORDINATION RESPONSE message, the receiving WAB-node/BH-gNB shall take this information into account for resource coordination with the sending BH-gNB/WAB-node.

#### 8.5.4.3	Unsuccessful Operation

Not applicable.

#### 8.5.4.4	Abnormal Conditions

Not applicable.

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Next Change<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

### 9.1.5	Messages for WAB Procedures

#### 9.1.5.1	WAB RESOURCE COORDINATION REQUEST

This message is sent by a BH-gNB/WAB-node to a WAB-node/BH-gNB, for the purpose of coordination of the semi-static resources of WAB-node.

Direction: BH-gNB  WAB-node, WAB-node  BH-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| WAB-MT Identity Information |  | 0..1 |  | This IE contains the identity information of the co-located WAB-MT in case this message is sent from the WAB-gNB to the BH-gNB. | YES | ignore |
| >C-RNTI | M |  | BIT STRING (SIZE (16)) |  | - |  |
| >NR CGI | M |  | 9.2.2.7 | Cell identity of the cell serving the WAB-MT. | - |  |
| WAB-gNB Cells List |  | 0..1 |  | List of cells served by the WAB-gNB. | YES | reject |
| > WAB-gNB Cells List Item |  | 1 .. <maxnoofServedCellsWAB > |  |  | – |  |
| >>WAB Node cell Information | M |  | 9.2.2.xx |  | – |  |
| BH-gNB Cells List |  | 0..1 |  | List of cells served by the BH-gNB. [FFS whether this IE is present only when this message is sent from BH-gNB to WAB-node.] | YES | reject |
| > BH-gNB Cells List Item |  | 1 .. < maxnoofServingCells > |  |  | – |  |
| >>BH-RAN Node Cell Information | M |  | 9.2.2.xx |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofServedCellsWAB | Maximum number of cells served by a WAB-gNB. Value is 512. |
| maxnoofServingCells | Maximum no. of serving cells for a WAB-MT. Value is 32, as defined by the maxNrofServingCells in TS 38.331 [10]. |

#### 9.1.5.2	WAB RESOURCE COORDINATION RESPONSE

This message is sent by a WAB-node/BH-gNB to a BH-gNB/WAB-node, in response to an WAB RESOURCE COORDINATION REQUEST message.

Direction: WAB-node  BH-gNB, BH-gNB  WAB-node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| WAB-gNB Cells List |  | 0..1 |  | List of cells served by the WAB-gNB. | YES | reject |
| >WAB-gNB Cells List Item |  | 1 .. <maxnoofServedCellsWAB > |  |  | – |  |
| >>WAB Node cell Information | M |  | 9.2.2.xx |  | – |  |
| BH-gNB Cells List |  | 0..1 |  | List of cells served by the BH-RAN node. [FFS whether this IE is present only when this message is sent from BH-gNB to WAB-node.] | YES | reject |
| > BH-gNB Cells List Item |  | 1 .. < maxnoofServingCells > |  |  | – |  |
| >>BH-gNB Cell Information | M |  | 9.2.2.xx |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofServedCellsWAB | Maximum number of cells served by a WAB-gNB. Value is 512. |
| maxnoofServingCells | Maximum no. of serving cells for a WAB-MT. Value is 32, as defined by the maxNrofServingCells in TS 38.331 [10]. |

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Next Change<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

#### 9.2.2.97	RB Set Configuration

This IE contains the configuration for up to M non-overlapping RB sets for a given gNB-DU cell or WAB-gNB cell, used for frequency domain resource allocation. The maximum value of M is 8.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Subcarrier Spacing | M |  | ENUMERATED (kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1, …) | Subcarrier spacing used as reference for the RB set configuration. |
| RB Set Size | M |  | ENUMERATED (rb2, rb4, rb8, rb16, rb32, rb64) | Number of PRBs in each RB set. If the RB sets of IAB-DU H/S/NA resource configuration do not cover the entire carrier bandwidth, the remaining RBs not part of an RB set configuration are considered as included in the last RB set. |
| Number of RB Sets | M |  | INTEGER(1.. maxnoofRBsetsPerCell) | Number of configured RB sets. The RB sets are contiguous and non-overlapping. If the NR Carrier List IE(9.2.2.63) is provided, the start RB index of the first RB set is the RB index of the lowest common RB with the SCS provided by RB Set Configuration IE, which overlaps with the lowest usable RB across all SCS-specific carriers provided by the NR Carrier List IE for the IAB-DU cell. Otherwise, the lowest subcarrier of the start RB of the first RB set is aligned with point A for the IAB-DU cell or the WAB-gNB cell. |

| Range bound | Explanation |
| --- | --- |
| maxnoofRBsetsPerCell | Maximum no. of RB sets per IAB-DU cell or WAB-gNB cell. Value is 8. |

#### >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Next Change<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

#### 9.2.2.xx	Cell Information for WAB

This IE contains cell resource configuration, cell specific signal/channel configuration and multiplexing info of the cell of a WAB-node or BH-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.2.2.7 |  |
| CHOICE Cell Resource Configuration-Mode-Info for WAB | O |  |  |  |
| >TDD |  |  |  |  |
| >>TDD Info |  | 1 |  |  |
| >>>Cell Resource Configuration-TDD | M |  | 9.2.2.xx+1 | Contains TDD resource configuration of WAB-gNB’s cell or BH-gNB’s cell which serves the WAB-MT co-located with the WAB-gNB. |
| >>>Frequency Info | O |  | 9.2.2.19 |  |
| >>>Transmission Bandwidth | O |  | 9.2.2.20 |  |
| >>>Carrier List | O |  | 9.2.2.63 | If included, the Transmission Bandwidth IE shall be ignored. |
| >FDD |  |  |  |  |
| >>FDD Info |  | 1 |  |  |
| >>> Cell Resource Configuration-FDD-UL | M |  | 9.2.2.xx+1 | Contains FDD UL resource configuration of WAB-gNB’s cell or BH-gNB’s cell which serves the WAB-MT co-located with the WAB-gNB.. |
| >>> Cell Resource Configuration-FDD-DL | M |  | 9.2.2.xx+1 | Contains FDD DL resource configuration of WAB-gNB’s cell or BH-gNB’s cell which serves the WAB-MT co-located with the WAB-gNB. |
| >>>UL Frequency Info | O |  | 9.2.2.19 |  |
| >>>DL Frequency Info | O |  | 9.2.2.19 |  |
| >>>UL Transmission Bandwidth | O |  | 9.2.2.20 |  |
| >>>DL Transmission Bandwidth | O |  | 9.2.2.20 |  |
| >>>UL Carrier List | O |  | 9.2.2.63 | If included, the UL Transmission Bandwidth IE shall be ignored. |
| >>>DL Carrier List | O |  | 9.2.2.63 | If included, the DL Transmission Bandwidth IE shall be ignored. |
| STC Info | O |  | 9.2.2.xx+2 | STC configuration of this WAB-gNB’s or BH-gNB’s cell. |
| RACH Config Common | O |  | OCTET STRING | Includes the rach-ConfigCommon as defined in subclause 6.3.2 of TS 38.331 [10]. |
| CSI-RS Configuration | O |  | OCTET STRING | Includes the NZP-CSI-RS-Resource IE as defined in subclause 6.3.2 of TS 38.331 [10]. |
| SR Configuration | O |  | OCTET STRING | Includes the SchedulingRequestResourceConfig IE as defined in subclause 6.3.2 of TS 38.331 [10]. |
| PDCCH Configuration SIB1 | O |  | OCTET STRING | Includes the PDCCH-ConfigSIB1 IE as defined in subclause 6.3.2 of TS 38.331 [10]. |
| SCS Common | O |  | OCTET STRING | Includes the subCarrierSpacingCommon as defined in subclause 6.2.2 of TS 38.331 [10]. |
| WAB Multiplexing Info | O |  | 9.2.2.xx+3 | Contains information on multiplexing with cells configured for collocated WAB-MT, if applicable. |

#### 9.2.2.xx+1	Cell Resource Configuration for WAB

This IE contains the resource configuration of the cells served by a WAB-gNB or BH-gNB, i.e. the TDD/FDD resource parameters for each activated cell (TS 38.213 [40], clause 11.1.1).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Subcarrier Spacing | M |  | ENUMERATED (kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1, …) | Subcarrier spacing used as reference for the TDD/FDD slot configuration. |
| DUF Transmission Periodicity | O |  | ENUMERATED (ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, …) |  |
| DUF Slot Configuration List |  | 0..1 |  |  |
| >DUF Slot Configuration Item |  | 1..<maxnoofDUFSlots> |  |  |
| >>CHOICE DUF Slot Configuration | M |  |  |  |
| >>>Explicit Format |  |  |  |  |
| >>>>Permutation | M |  | ENUMERATED (DFU, UFD, …) |  |
| >>>>Number of Downlink Symbols | O |  | INTEGER (0..14) |  |
| >>>>Number of Uplink Symbols | O |  | INTEGER (0..14) |  |
| >>>Implicit Format |  |  |  |  |
| >>>>DUF Slot Format Index | M |  | INTEGER (0..254) | Index into Table 11.1.1-1 and Table 14-2 in TS 38.213 [40], excluding the last row in Table 14-2. |
| HSNA Transmission Periodicity | O |  | ENUMERATED (ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, ms20, ms40, ms80, ms160, …) |  |
| HSNA Slot Configuration List |  | 0..1 |  |  |
| >HSNA Slot Configuration Item |  | 1..<maxnoofHSNASlots> |  |  |
| >>HSNA Downlink | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for downlink symbols in a slot. |
| >>HSNA Uplink | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for uplink symbols in a slot. |
| >>HSNA Flexible | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for flexible symbols in a slot. |
| RB Set Configuration | O |  | 9.2.2.97 | Indicates the configuration for up to M non-overlapping RB sets for a given WAB-gNB cell, used for frequency domain resource allocation. The maximum value of M is 8. |
| Frequency-domain HSNA Configuration List |  | 0..1 |  |  |
| >Frequency-domain HSNA Configuration Item |  | 1..<maxnoofRBsetsPerCell> |  |  |
| >>RB Set Index | M |  | INTEGER (0.. maxnoofRBsetsPerCell1, …) | Refers to an RB set defined by RB Set Configuration. The RB set indexes are consecutive (and increasing) starting at 0. |
| >>Frequency-domain HSNA Slot Configuration List |  | 1 |  |  |
| >>>Frequency-domain HSNA Slot Configuration item |  | 1..<maxnoofHSNASlots> |  |  |
| >>>>Slot Index | M |  | INTEGER (.. maxnoofHSNASlots) | Index to a slot within the HSNA Transmission Periodicity. |
| >>>>HSNA Downlink | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for downlink symbols in a slot, for an RB set. |
| >>>>HSNA Uplink | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for uplink symbols in a slot, for an RB set. |
| >>>>HSNA Flexible | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for flexible symbols in a slot, for an RB set. |

| Range bound | Explanation |
| --- | --- |
| maxnoofDUFSlots | Maximum no. of slots in 10ms. Value is 320. Corresponds to the maxNrofSlots defined in TS 38.331 [10]. |
| maxnoofHSNASlots | Maximum no of "Hard", "Soft" or "Not available" slots in 160ms. Value is 5120. |
| maxnoofRBsetsPerCell | Maximum no. of RB sets per WAB-gNB cell. Value is 8. |
| maxnoofRBsetsPerCell1 | Maximum no. of RB sets per WAB-gNB cell minus 1. Value is 7. |

#### 9.2.2.xx+2	STC Info

This IE contains cell SSB Transmission Configuration (STC) information of a WAB node or a BH-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| STC-Info List |  | 1 |  |  |
| >STC-Info Item |  | 1 ..<maxnoofWABSTCInfo> |  |  |
| >>SSB Frequency Info | M |  | INTEGER (0.. maxNRARFCN) | The SSB central frequency. |
| >>SSB Subcarrier Spacing | M |  | ENUMERATED (kHz15, kHz30, kHz120, kHz240, spare3, spare2, spare1, …) | The SSB subcarrier spacing. |
| >>SSB Transmission Periodicity | M |  | ENUMERATED (sf10, sf20, sf40, sf80, sf160, sf320, sf640, …, sf5) |  |
| >>SSB Transmission Timing Offset | M |  | INTEGER (0.. 127, …) | SSB transmission timing offset in number of half-frames. |
| >>CHOICE SSB Transmission Bitmap | M |  |  | Corresponds to information provided in the SSB-ToMeasure IE defined in TS 38.331 [10]. |
| >>>short bitmap |  |  |  |  |
| >>>>Short Bitmap | M |  | BIT STRING (SIZE (4)) |  |
| >>>medium bitmap |  |  |  |  |
| >>>>Medium Bitmap | M |  | BIT STRING (SIZE (8)) |  |
| >>>long bitmap |  |  |  |  |
| >>>>Long Bitmap | M |  | BIT STRING (SIZE (64)) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofWABSTCInfo | Maximum no. of STC configurations. Value is 5. This includes 1 STC configuration for access and 4 STC configurations for backhaul. |
| maxNRARFCN | Maximum value of NR ARFCNs. Value is 3279165. |

#### 9.2.2.xx+3	WAB Multiplexing Info

This IE contains information about the multiplexing capabilities between the WAB-gNB’s cell and the cells configured on the co-located WAB-MT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| WAB-MT Cell List |  | 1 |  |  |
| >WAB-MT Cell Item |  | 1 .. <maxnoofServingCells> |  |  |
| >>NR Cell Identity | M |  | BIT STRING (SIZE(36)) | Cell identity of a serving cell configured for a co-located WAB-MT. |
| >>WAB-gNB_RX/WAB-MT_RX | M |  | ENUMERATED (supported, not supported , …) | An indication of whether the WAB-node supports simultaneous reception at its gNB and MT side. |
| >> WAB-gNB _TX/ WAB-MT _TX | M |  | ENUMERATED (supported, not supported, …) | An indication of whether the WAB-node supports simultaneous transmission at its gNB and MT side. |
| >> WAB-gNB _TX/ WAB-MT _RX | M |  | ENUMERATED (supported, not supported, …) | An indication of whether the WAB-node supports simultaneous transmission at its gNB and reception at its MT side. |
| >> WAB-gNB _RX/ WAB-MT _TX | M |  | ENUMERATED (supported, not supported, …) | An indication of whether the WAB-node supports simultaneous reception at its gNB and transmission at its MT side. |

| Range bound | Explanation |
| --- | --- |
| maxnoofServingCells | Maximum no. of serving cells for an WAB-MT. Value is 32, as defined by the maxNrofServingCells in TS 38.331 [10]. |

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>End of TP<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

6 Annex B (TP to BLCR for TS 38.473)

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Start of TP<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

## 8.11	WAB Procedures

### 8.11.1	BH-gNB-DU Resource Configuration [Name is FFS]

#### 8.11.1.1	General

The BH-gNB-DU Resource Configuration procedure is initiated by the BH-gNB-CU in order to send [FFS][the resource configuration of the WAB-gNB whose co-located WAB-MT is connecting with the BH-gNB-DU]. The procedure uses non-UE associated signalling.

In this version of the specification, this procedure is used to configure WAB resources.

#### 8.11.1.2	Successful Operation

Figure 8.10.2.2-1: BH-gNB-DU Resource Configuration procedure: Successful Operation

The BH-gNB-CU initiates the procedure by sending the BH-GNB-DU RESOURCE CONFIGURATION [FFS]message to BH-gNB-DU. The BH-gNB-DU replies to the BH-gNB-CU with the BH-GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE [FFS] message.

If the WAB-gNB List IE is included in the BH-GNB-DU RESOURCE CONFIGURATION message, the BH-gNB-DU shall store the received information and use this information for scheduling.

#### 8.11.1.B	Unsuccessful Operation

Figure 8.10.2.3-1: gNB-DU Resource Configuration procedure: Unsuccessful Operation

If the BH-gNB-DU cannot accept the configuration, it shall respond with a BH-GNB-DU RESOURCE CONFIGURATION FAILURE and appropriate cause value.

#### 8.11.1.3	Abnormal Conditions

Not applicable.

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Next Change<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

### 9.2.18	WAB messages

#### 9.2.18.1	BH-GNB-DU RESOURCE CONFIGURATION

This message is sent by the BH-gNB-CU to provide [the resource configuration of WAB-gNB][FFS] to a BH-gNB-DU.

Direction: BH-gNB-CU  BH-gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| WAB-gNB List |  | 0..1 |  | List of WAB-node served by the BH-gNB. | YES | reject |
| >WAB-gNB List Item |  | 1 .. <maxnoofWABNodes> |  |  | EACH | reject |
| >>gNB-CU UE F1AP ID | M |  | 9.3.1.4 | Identifier of WAB-MT at the BH-gNB-CU. | YES | reject |
| >>gNB-DU UE F1AP ID | M |  | 9.3.1.5 | Identifier of WAB-MT at the BH-gNB-DU. | YES | reject |
| >>WAB-gNB Cells List |  | 0..1 |  | List of cells served by the WAB-gNB whose resource configuration is updated. | YES | reject |
| >>>WAB-gNB Cells List Item |  | 1 .. <maxnoofServedCellsWAB > |  |  | EACH | reject |
| >>>>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>>>WAB-gNB Cell Resource Configuration-Mode-Info | O |  | 9.3.1.xxx |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofWABNodes | Maximum number of WAB-nodes served by an BH-RAN-DU. Value is 1024. |
| maxnoofServedCellsWAB | Maximum number of cells served by an WAB-gNB. Value is 512. |

#### 9.2.18.2	BH-GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE

This message is sent by the BH-gNB-DU to acknowledge the reception of a BH-GNB-DU RESOURCE CONFIGURATION message.

Direction: BH-gNB-DU  BH-gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.18.2A	BH-GNB-DU RESOURCE CONFIGURATION FAILURE

This message is sent by the BH-gNB-DU to indicate a BH-gNB-DU Resource Configuration Update failure.

Direction: BH-gNB-DU  BH-gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Next Change<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

#### 9.3.1.230	RB Set Configuration

This IE contains the RB Set Configuration. The IE is only applicable if the gNB-DU is an IAB-DU or WAB-gNB cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Subcarrier Spacing | M |  | ENUMERATED (kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1, …) | Subcarrier spacing used as reference for the RB set configuration. |
| RB Set Size | M |  | ENUMERATED (rb2, rb4, rb8, rb16, rb32, rb64) | Number of PRBs in each RB set. If the RB sets of IAB-DU H/S/NA resource configuration do not cover the entire carrier bandwidth, the remaining RBs not part of an RB set configuration are considered as included in the last RB set. |
| Number of RB Sets | M |  | INTEGER(1.. maxnoofRBsetsPerCell) | Number of configured RB sets. The RB sets are contiguous and non-overlapping. If NR Carrier List IE(9.3.1.137) is provided, the start RB index of the first RB set is the RB index of the lowest common RB with the SCS provided by RB Set Configuration IE, which overlaps with the lowest usable RB across all SCS-specific carriers provided by the NR Carrier List IE for the IAB-DU cell. Otherwise, the start RB of the first RB set is aligned with point A for the IAB-DU cell or the WAB-gNB cell. |

| Range bound | Explanation |
| --- | --- |
| maxnoofRBsetsPerCell | Maximum no. of RB sets per IAB-DU cell or WAB-gNB cell. Value is 8. |

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Next Change<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

#### 9.3.1.xxx	WAB-gNB Cell Resource Configuration-Mode-Info

This IE contains the resource configuration of the cells served by a WAB-gNB, i.e. the TDD/FDD resource parameters for each activated cell (TS 38.213 [31], clause 11.1.1).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Subcarrier Spacing | M |  | ENUMERATED (kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1, …) | Subcarrier spacing used as reference for the TDD/FDD slot configuration. | - |  |
| DUF Transmission Periodicity | O |  | ENUMERATED (ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, …) |  | - |  |
| DUF Slot Configuration List |  | 0..1 |  |  | - |  |
| >DUF Slot Configuration Item |  | 1..<maxnoofDUFSlots> |  | The maxNrofSlots in TS 38.331 [8]. | - |  |
| >>CHOICE DUF Slot Configuration | M |  |  |  | - |  |
| >>>Explicit Format |  |  |  |  |  |  |
| >>>>Permutation | M |  | ENUMERATED (DFU, UFD, …) |  | - |  |
| >>>>Number of Downlink Symbols | O |  | INTEGER (0..14) |  | - |  |
| >>>>Number of Uplink Symbols | O |  | INTEGER (0..14) |  | - |  |
| >>>Implicit Format |  |  |  |  |  |  |
| >>>>DUF Slot Format Index | M |  | INTEGER (0..254) | Index into Table 11.1.1-1 and Table 14-2 in TS 38.213 [31], excluding the last row in Table 14-2. | - |  |
| HSNA Transmission Periodicity | M |  | ENUMERATED (ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, ms20, ms40, ms80, ms160, …) |  | - |  |
| HSNA Slot Configuration List |  | 0..1 |  |  | - |  |
| >HSNA Slot Configuration Item |  | 1..<maxnoofHSNASlots> |  |  | - |  |
| >>HSNA Downlink | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for downlink symbols in a slot. | - |  |
| >>HSNA Uplink | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for uplink symbols in a slot. | - |  |
| >>HSNA Flexible | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for flexible symbols in a slot. | - |  |
| RB Set Configuration | O |  | 9.3.1.230 |  | YES | reject |
| Frequency-Domain HSNA Configuration List |  | 0..1 |  |  | YES | reject |
| >Frequency-Domain HSNA Configuration Item |  | 1..<maxnoofRBsetsPerCell> |  |  | EACH | reject |
| >>RB Set Index | M |  | INTEGER (0.. maxnoofRBsetsPerCell-1, …) | Refers to an RB set defined by RB Set Configuration. The RB set indices are consecutive (and increasing) starting at 0. | - |  |
| >>Frequency-Domain HSNA Slot Configuration List |  | 1 |  |  | - |  |
| >>>Frequency-Domain HSNA Slot Configuration Item |  | 1..<maxnoofHSNASlots> |  |  | - |  |
| >>>>Slot Index | O |  | INTEGER (0..5119) | Indicates an index to a slot within the HSNA Transmission Periodicity. | - |  |
| >>>>HSNA Downlink | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for downlink symbols in a slot, for an RB set. | - |  |
| >>>>HSNA Uplink | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for uplink symbols in a slot, for an RB set. | - |  |
| >>>>HSNA Flexible | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for flexible symbols in a slot, for an RB set. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDUFSlots | Maximum no. of slots in 10ms. Value is 320. |
| maxnoofSymbols | Maximum no. of symbols in a slot. Value is 14. |
| maxnoofHSNASlots | Maximum no of "Hard", "Soft" or "Not available" slots in 160ms. Value is 5120. |
| maxnoofRBsetsPerCell | Maximum no. of RB sets per WAB-gNB cell. Value is 8 |
| maxnoofRBsetsPerCell-1 | Maximum no. of RB sets per WAB-gNB cell minus 1. Value is 7 |

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>End of TP<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<