3GPP TSG-RAN WG3 Meeting #128	R3-253314

Malta, MT, May 19 – 23, 2025

Agenda item:	 10.4

Source:		Lenovo

Title:		Discussion on UHI enhancement for SCG Deactivation or Activation

Document for:		Discussion and Decision

1	Introduction

The following agreements have been achieved until now:

MRO for MR-DC SCG failure:

Keep the following R18 agreement in R19:

Support MRO for SCG failure in EN-DC, NGEN-DC and NE-DC scenarios.

Take Stage 2 descriptions of PSCell change failure in TS37.340 as baseline for EN-DC and NGEN-DC SCG failure, and necessary updates can be added on top of it if needed.

Introduce new SCG FAILURE INFORMATION REPORT message and SCG FAILURE TRANSFER message over X2 interface.

No need to add failureType and perRA-InfoList in SCG FAILURE INFORMATION REPORT message in X2/Xn interface.

RACH optimization for SDT:

Work on scenario of RACH optimization for SDT first.

There is no need for UE to report SDT configurations, e.g., RSRP threshold, data volume threshold, T319a, and other configurations.

The information that stored and reported in the RA report for SDT:

c. the time from the start of RA-SDT to the reporting of RA report

We will send an LS to RAN2 to inform about the previous agreement to report the time from the start of RA-SDT to the reporting of RA report.

No need to report RSRP and data volume when triggered RA-SDT fails in RA report.

MHI Enhancement for SCG Deactivation/Activation:

Wait for RAN2 progress on whether to use absolute time, relative time or another form to express SCG activated time

In this paper, we would further discuss the left issues of UHI enhancement for SCG Deactivation/Activation.

2	Discussion

RAN2#129bis meeting agreed that:

UE reports the absolute time it has spent in PSCell with SCG activated in MHI.

Based on RAN2 progress, RAN3 should align the design to include absolute time of SCG activated state in UHI. Based on the collected absolute time duration of SCG activation, network can directly understand the time duration when SCG is activated or deactivated, in order to know whether or how to optimize the DC related configuration.

Proposal 1: Absolute time of SCG activated state is collected in UHI.

For the FFS on whether additional information e.g. throughput threshold used for activating, average throughput, or number of activations is needed, since network can perform optimization if needed based on the enhanced UHI, and the usage of these additional information at network side is unclear, it is not needed to include the additional information e.g. throughput threshold used for activating, average throughput, or number of activations in UHI.

Proposal 2: Additional information e.g. throughput threshold used for activating, average throughput, or number of activations is not needed in UHI.

3	Conclusion

In this paper, UHI enhancement for SCG deactivation/activation are discussed. We have the following proposals:

Proposal 1: Absolute time of SCG activated state is collected in UHI.

Proposal 2: Additional information e.g. throughput threshold used for activating, average throughput, or number of activations is not needed in UHI.