3GPP TSG-RAN WG3 Meeting #128	R3-253315

Malta, MT, May 19 – 23, 2025

Agenda item:	 11.3

Source:		Lenovo

Title:		Discussion on AI/ML based CCO

Document for:		Discussion and Decision

1	Introduction

RAN3#127bis meeting agreed that:

The maximum value of the Time interval for predicted CCO issue and future CCO state is 60s.

Legacy UE performance measurement metrics can be reused for CCO.

For the receiving side, whether gNB-CU can also provide to gNB-DU a future CCO state as assistance information to derive a matching future CCO state?

Option1: no need

Opton2: gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU.

Option3: gNB-CU generates a recommend further CCO state and send to gNB-DU.

Whether/what additional UE performance measurement metrics is needed?

Timing information for predicted CCO issue is NOT needed to exchange over Xn? Whether the predicted CCO issue and /or future CCO state can be updated over Xn?

In this contribution, we would further discuss the left issues related to AI/ML based CCO.

2	Discussion

  - 2.1 AI/ML based CCO over Xn interface

In non-split architecture, it has agreed that:

- Step 0: gNB1 predicts the CCO issue/Coverage Modification Cause.

- Step 1: gNB1 generates the future coverage status based on the predicted CCO issue/Coverage Modification Cause and other information.

- Step 2: gNB1 sends the predicted CCO issue/Coverage Modification Cause and future coverage status to neighbour gNB2.

As captured in [1], gNB1 sends the Time for Future Coverage State to neighbour gNBs, which indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the gNB1 relative to the time of receiving the NG-RAN NODE CONFIGURATION UPDATE message. It is FFS on whether to send time information related to the predicted coverage modification cause over Xn. Regarding CU-DU split architecture, time information for Predicted CCO Issue/Coverage Modification Cause is a relative time from the time of receiving the gNB-CU Configuration Update message over F1, and time information for future CCO state is a relative time from the time of receiving the gNB-DU Configuration Update message over F1, the two time information are totally different. To make neighbour gNB2 clearly understand and use the information for deducing the CCO issue predicted in gNB1 and for configuring coverage state of its served cell(s), as well as to enable gNB2 to further predict its own CCO issue2 based on the predicted CCO issue1 and future coverage status received from gNB1, it is beneficial to introduce time information for predicted coverage modification cause over Xn.

Proposal 1: Introduce time information for predicted coverage modification cause over Xn.

Since the future CCO state is a prediction or based on a predicted CCO issue, the prediction might be updated or might not be correct. For example, NG-RAN node may predict a future CCO state or CCO issue for 10 seconds later and transfer the related information to a neighbour NG-RAN node, such that the neighbour NG-RAN node would expect a CCO state change after 10 seconds. However, the NG-RAN node may later detects/predicts the CCO issue to happen at 5 seconds sooner than previously predicted. In such case, it is important for the neighbour NG-RAN node to understand the newly detected or predicted CCO issue with corresponding CCO state and the neighbour NG-RAN node should consider the previously received future CCO state and/or predicted CCO issue to be no longer valid.

An example is depicted below to illustrate the point, wherein the NG-RAN node 1 first informs NG-RAN node 2 about a predicted CCO state/issue after time T, but then trigger a CCO state modification before time T due to detected CCO issue. The NG-RAN node 2 should consider the previously received CCO state/issue prediction no longer valid.

Proposal 2: NG-RAN node can update a previously transferred future CCO state and/or CCO issue (e.g., due to newly detected/predicted CCO issue).

  - 2.2 AI/ML based CCO over F1 interface

In split-architecture, it has agreed that:

- Step 0: gNB-CU predicts the CCO issue.

- Step 1: gNB-CU sends the predicted CCO issue to gNB-DU.

- Step 2: gNB-DU generates the future coverage status based on the predicted CCO issue and other local information.

- Step 3: gNB-DU sends the future coverage status to gNB-CU.

- Step 4: gNB-CU sends the predicted CCO issue and future coverage status to neighbour gNB-CU.

Regarding the predicted CCO issue from gNB-CU to gNB-DU, the following two scenarios are agreed:

- gNB-CU1 predicts the CCO issue1 for gNB-DU1 and sends to its gNB-DU1;

- Based on the received predicted CCO issue1 and future CCO state from gNB-CU1, gNB-CU2 will deduce for gNB-DU2 the predicted CCO issue2 as well as affected cells and beams, and send it to its gNB-DU2.

For the issue on whether gNB-CU2 would send the received CCO issue1 predicted by gNB-CU1 to gNB-DU2, regarding gNB-CU2 would predict CCO issue2 for gNB-DU2 based on the predicted CCO issue1 and future coverage status received from gNB-CU1, it makes no sense of gNB-CU2 to redundantly send the CCO issue1 predicted by gNB-CU1 for gNB-DU1 to gNB-DU2.

Proposal 3: gNB-CU2 may not send the CCO issue predicted by gNB-CU1 to its gNB-DU2.

RAN3#126 meeting discussed whether gNB-CU2 sends the received future coverage status from gNB-CU1 to gNB-DU2 but got no consensus. Considering gNB-CU2 can predict its own CCO issue2 for gNB-DU2 based on the predicted CCO issue1 and future coverage state received from gNB-CU1, gNB-DU2 would generate future coverage state to solve the predicted CCO issue2 for gNB-DU2 after receiving the predicted CCO issue2 from gNB-CU2, it is not essential for gNB-CU2 to directly send the received future coverage state for solving the predicted CCO issue1 for gNB-DU1 from gNB-CU1 to gNB-DU2.

Proposal 4: gNB-CU2 may not send the received future coverage state for solving the predicted CCO issue for gNB-DU1 from gNB-CU1 to its gNB-DU2.

With regarding to whether gNB-CU generates a recommend future CCO state and sends it to gNB-DU as assistance information, similar as legacy CCO mechanism, the gNB-DU may resolve the predicted CCO issue concerning own served cell by local action within the OAM configured limits, and the gNB-DU may also take into account information received for other cells when generating future CCO state. Therefore, it is unnecessary for the gNB-CU to generate a recommend future coverage state and send it to gNB-DU as assistance information.

Proposal 5: gNB-CU does not generate a recommend future coverage state and send it to gNB-DU as assistance information.

3	Conclusion

In this contribution, AI/ML based CCO are discussed. We have the following proposals:

Proposal 1: Introduce time information for predicted coverage modification cause over Xn.

Proposal 2: NG-RAN node can update a previously transferred future CCO state and/or CCO issue (e.g., due to newly detected/predicted CCO issue).

Proposal 3: gNB-CU2 may not send the CCO issue predicted by gNB-CU1 to its gNB-DU2.

Proposal 4: gNB-CU2 may not send the received future coverage state for solving the predicted CCO issue for gNB-DU1 from gNB-CU1 to its gNB-DU2.

Proposal 5: gNB-CU does not generate a recommend future coverage state and send it to gNB-DU as assistance information.

Reference

[1] R3-247915, (TP for BLCR of TS 38.423) AI/ML enabled CCO