3GPP TSG-RAN WG3 Meeting #128	R3-253321

Malta, MT, 19th – 23th May, 2025

Agenda Item:	12.2

Source: 	Lenovo

Title: 	(TP to BL CR 38.423) Radio resource configuration for WAB-node

Document for:	Discussion and decision

- Introduction

A new WID for additional topological enhancements [1] has been approved to be discussed in Rel-19, and following detail objectives are identified for Wireless Access Backhaul (WAB) focus on RAN3.

The objectives of the Wireless Access Backhaul (WAB) work are as follows: - Specifications for the support of WAB including [RAN3]: - … - The handling of: - PCI collision avoidance. - Reconfiguration of TAC and RANAC on WAB-gNBs. - Mechanisms to avoid multi-hop WAB topology. - Radio-resource coordination between access and backhaul links. - NG connection management.

In this contribution, the radio resource configuration and coordination for WAB are discussed.

- Discussion

RAN3 127bis: For WAB resource coordination, introduce a new class-1 Xn procedure between WAB-gNB and BH gNB. The message content can be based on relevant parts of the XnAP IEs defined in clauses 9.2.2.94-97 of TS 38.423. FFS whether to reuse the F1AP NB-DU RESOURCE CONFIGURATION procedure between BH gNB-CU and BH gNB-DU. After RAN3 has finished specifying WAB resource coordination, RAN3 to send an LS to inform RAN1 and RAN2 about conclusions.

For IAB, an F1AP gNB-DU Resource Configuration procedure had been defined to achieve resource configuration on IAB-DU/IAB-donor-DU and to achieve resource coordination based on the resource configuration of child links and neighbour nodes. In addition, an Xn IAB Resource Coordination procedure had been defined to achieve resource coordination between two IAB-donor-CUs of the IAB-node.

When it comes to WAB, because WAB-node has a full CU function at the WAB-gNB, and then the resource configuration on the WAB-gNB can be based on its implementation and there is no need for the BH-gNB to configure the semi-static resource configuration for WAB-gNB.

For resource coordination between WAB-gNB and BH-gNB, it’s beneficial for each of the two nodes to be aware of the resource configuration of the peer node. From the WAB-gNB side, it can get the resource configuration of BH-gNB from the co-located WAB-MT via Uu interface. While from the BH-gNB side, the WAB-gNB needs to report its resource configuration to the BH-gNB via Xn.

Proposal 1: WAB-gNB reports its resource configuration to BH-gNB over Xn.

For resource coordination between WAB-gNB and neighbour node, the solution introduced by IAB can be reused, where the BH-gNB can send the resource configuration of neighbour node to the WAB-gNB. And then the WAB-gNB can use it for cross-link interference management and semi-static resource coordination.

Proposal 2: BH-gNB sends resource configuration of neighbour nodes to WAB-gNB over Xn.

As for the resource configuration on DU of the BH-gNB, legacy F1AP GNB-DU RESOURCE CONFIGURATION can be reused, which includes the resource configuration (D/U/F/NA/H/S) on the gNB-DU of the BH-gNB, and it further includes the resource configuration and the multiplexing info of the WAB-gNB, which is helpful for the gNB-DU of the BH-gNB to avoid scheduling the WAB-MT in the slot/symbol with hard resources in the co-located WAB-gNB.

Proposal 3: Legacy F1AP GNB-DU RESOURCE CONFIGURATION can be reused for resource configuration on gNB-DU of the BH-gNB.

# Conclusion

This contribution aims to discuss radio resource configuration and coordination for WAB. And following observations and proposals are concluded.

Proposal 1: WAB-gNB reports its resource configuration to BH-gNB over Xn.

Proposal 2: BH-gNB sends resource configuration of neighbour nodes to WAB-gNB over Xn.

Proposal 3: Legacy F1AP GNB-DU RESOURCE CONFIGURATION can be reused for resource configuration on gNB-DU of the BH-gNB.

And a TP is drafted to reflect radio resource configuration of WAB in the Appendix.

# Reference

- RP-242395. New WID on additional topological enhancements for NR.

- Chairman notes of RAN3 #127bis.

# TP to BL CR 38.423

Start of Change

### 8.x.y	WAB Resource Coordination

#### 8.x.y.1	General

The purpose of the WAB Resource Coordination procedure is to exchange the semi-static radio resource configuration between WAB-node and its BH-gNB, for the purpose of resource multiplexing between the WAB-MT and the WAB-gNB of the WAB-node, and for the purpose of resource coordination between the WAB-gNB and neighbour gNBs.

The procedure uses non UE-associated signalling.

#### 8.x.y.2	Successful Operation

Figure 8.5.4.2-1: WAB Resource Coordination, successful operation

The BH-gNB initiates the procedure by sending the WAB RESOURCE COORDINATION message to the WAB-gNB. The WAB-gNB shall respond with the WAB RESOURCE COORDINATION ACKNOWLEDGE message to the BH-gNB.

If the Neighbour-Node Cells List IE is included in the WAB RESOURCE COORDINATION message, the WAB-gNB shall store the received information and use this information for cross-link interference management and/or semi-static resource coordination.

If the WAB-Node Cell List IE is included in the WAB RESOURCE COORDINATION ACKNOWLEDGE message, the BH-gNB shall store the received information and use this information for resource configuration at the co-located WAB-MT of the WAB-node.

#### 8.x.y.3	Unsuccessful Operation

Not applicable.

#### 8.x.y.4	Abnormal Conditions

Not applicable.

#### 9.xx.yy.zz1	WAB RESOURCE COORDINATION

This message is sent by a BH-gNB of a WAB-gNB to the WAB-gNB, for the purpose of coordination of the semi-static resources of the WAB-gNB.

Direction: BH-gNB  WAB-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Global NG-RAN Node ID | M |  | 9.2.2.3 |  | YES | reject |
| Neighbour-Node Cells List |  | 0..1 |  | List of neighbor node cells. | YES | reject |
| >Neighbour-Node Cells List Item |  | 1 .. < maxnoofNeighbourNodeCellsWAB> |  |  |  |  |
| >>Neighbour-Node Cell Information | M |  | IAB Cell Information 9.2.2.94 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofNeighbourNodeCellsWAB | Maximum no. of neighbour cells. Value is 1024. |

#### 9.xx.yy.zz2	WAB RESOURCE COORDINATION ACKNOWLEDGE

This message is sent by a WAB-gNB to a BH-gNB of the WAB-gNB, in response to a WAB RESOURCE COORDINATION message.

Direction: WAB-gNB  BH-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Global NG-RAN Node ID | M |  | 9.2.2.3 |  | YES | reject |
| WAB-Node Cells List |  | 0..1 |  | List of WAB-node cells. | YES | reject |
| >WAB-Node Cells List Item |  | 1 .. < maxnoofServedCellsWAB> |  |  |  |  |
| >>WAB-Node Cell Information | M |  | IAB Cell Information 9.2.2.94 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofServedCellsWAB | Maximum number of cells served by an WAB-gNB. Value is 512. |

End of Change