3GPP TSG RAN WG3 #128	R3-253329

St. Julian, Malta, 19th – 23rd May 2025

Agenda item:	17.3

Source: 	Lenovo

Title: 	(TP to BL CR 38.423) Discussion on On-demand SIB1 for Idle/Inactive UE

Document for:	Discussion and Decision

Introduction

RAN3#127bis has discussed the on demand SIB1 and reached the agreements：

In the new Class 1 message (Direction NES gNB to Cell A gNB), RAN3 has agreed that: • One Choice “Start with UL WUS Configuration” • One Choice “Stop” meaning the Cell A gNB will remove/release/discard the UL WUS Configuration and stop the broadcasting, it also means that next time the NES gNB should use “Start with UL WUS Configuration”.

In this contribution, we further analyse potential RAN3 impacts.

Discussion

UL WUS Configuration Provision over Xn：

Figure 1

NES cell gNB transmits the UL WUS configuration and one or more selected Cell A to Cell A gNB in UL WUS CONFIGURATION PROVISION REQUEST message. If the Cell A gNB accepts the UL WUS configuration and transmits UL WUS CONFIGURATION PROVISION RESPONSE to NES cell gNB, Cell A can start to provide UL WUS config for the NES cells, i.e., no need further instruction from NES cell. In the response message, the Cell A gNB can notify the NES cell gNB a list of accepted NES cell and a list of Cell A accepting the request. Otherwise, the Cell A gNB transmits a failure cause to the NES gNB and a list of Cell A rejecting the request. Also, the Cell A gNB may indicate a list of rejected NES cell. This helps the NES cell gNB select other Cell A candidates for request for providing the ULWUS config.

Proposal 1:  NES cell gNB includes Cell A requested for providing with the UL WUS configuration in UL WUS CONFIGURATION PROVISION REQUEST message.

Proposal 2:  Cell A gNB includes Cell A accepting or rejecting for providing with the UL WUS configuration in UL WUS CONFIGURATION PROVISION RESPONSE message.

Proposal 3:  Cell A gNB includes a list of accepted or rejected NES cell in UL WUS CONFIGURATION PROVISION RESPONSE message.

If the OD-SIB1 for an NES cell is not needed, the NES cell can request Cell A to stop and remove the UL WUS configuration for the NES cell by UL WUS CONFIGURATION PROVISION REQUEST message. From our view, the UL WUS CONFIGURATION PROVISION REQUEST can indicate CELL A to remove the UL WUS configuration for specified NES cells by including a list of NES cells in the UL WUS CONFIGURATION PROVISION REQUEST.  For example,

Figure 2

- 9.x.x.x	UL WUS CONFIGURATION PROVISION REQUEST

This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to request or stop provisioning of an UL WUS configuration in one or more cells.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE Request Type | M |  |  |  | YES | reject |
| >Start |  |  |  |  |  |  |
| >> UL WUS Configuration | M |  | OCTET STRING | Includes the xyz message as defined in subclause a.b.c of TS 38.331 [10]. |  |  |
| >Stop |  |  |  |  |  |  |
| >> Cell list |  |  |  |  |  |  |
| >>>Cell List Item |  | 1 .. < FFS> |  |  |  |  |
| >>>>NG-RAN Cell Identity | M |  | 9.2.2.9 |  |  |  |

Proposal 4: NES cell gNB includes a list of NES cell to remove the UL WUS configuration in UL WUS CONFIGURATION PROVISION REQUEST message including Stop IE.

UL WUS Configuration Transmission Status Update over Xn：

If Cell A gNB decides to stop providing UL WUS config for an NES cell, the Cell A gNB can directly notify the NES cell gNB with the NES cell ID, so that the NES cell can decide to stop the OD-SIB1 operation.  And the Cell A gNB can notify the NES cell gNB a list of Cell A stopping providing UL WUS config, so that the NES cell gNB can request another Cell A for providing the UL WUS configuration.

Proposal 5: Cell A gNB includes a list of NES cells stopped for providing with the UL WUS configuration and the Cell A stopping providing the UL WUS configuration in UL WUS CONFIGURATION TRANSMISSION STATUS UPDATE message.

In addition, RAN3 has agreed that a Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast for NES cells in its SIB. RAN3 needs to discuss whether to maintain the stored WUS configuration.

- Option 1 is that Cell A gNB releases the UL WUS configuration for the NES cells after signalling the stopping the UL WUS configuration broadcast. It can avoid the additional release signalling from NES gNB. However, NES gNB needs to transmit the complete UL WUS configuration to Cell A gNB again when the UL WUS configuration request is triggered.

- Option 2 is Cell A gNB maintains the UL WUS configuration after signalling the stopping the UL WUS configuration broadcast. It can avoid the complete UL WUS configuration signalling from NES gNB. However, the NES gNB needs to transmit an explicit release request to Cell A if NES cell determines to release the UL WUS configuration.

We observe each option has different benefits, and slightly prefer option 2. In option 2, the UL WUS configuration in cell A is managed in a unified way by the NES gNB.

Proposal 6: If Cell A gNB transmits stop indication to NES cell gNB, Cell A gNB maintains the UL WUS configuration for the NES cell till receiving release indication(e.g., stop IE) from the NES cell gNB.

On-demand SIB1 allowed and disable indication over F1：

After the NES cell gNB-CU transmits UL WUS configuration of a list of NES cells to Cell A gNB and receives accept response with a list of accepted NES cells, the NES cell gNB-CU transmits on-demand SIB1 allowed indication for the NES cells to NES cell gNB-DU, then the NES cell gNB-DU considers that the indicated NES cells are allowed to enter the on-demand SIB1 operation. For example, the NES cell can stop periodical SIB1 broadcast and enter OD-SIB1 operation.

Proposal 7:  NES cell gNB-CU includes On-demand SIB1 allowed indication for a list of NES cell to DU in GNB-CU CONFIGURATION UPDATE message.

In agreed CR R3-252386,  the NES cell gNB-CU stops the coordination of on-demand SIB1 transmission if receiving “Stop provision” in the Served Cell Information in GNB-DU CONFIGURATION UPDATE message. In our view, the current text is not complete. If NES cell gNB-DU determines not to perform On-demand SIB1 operation anymore, it transmits an indication e.g., OD-SIB1 stop or OD-SIB1 disable indication to NES cell gNB-CU. Based on the indication, NES cell gNB-CU considers the OD-SIB1 is disabled and transmits UL WUS CONFIGURATION PROVISION REQUEST message including Stop IE and a list of NES cell to the Cell A gNB if the Cell A gNB has accepted for providing the UL WUS configuration for the NES cells.

Proposal 8:  If NES cell gNB-CU receives On-demand SIB1 stop indication for a list of cells in the GNB-DU CONFIGURATION UPDATE message, NES cell gNB-CU transmits UL WUS CONFIGURATION PROVISION REQUEST message including Stop IE and a list of NES cell to the Cell A gNB.

Transfer UL WUS configuration in new SIB over F1：

Based on RAN2 agreement, a NES cell can include neighbouring NES cell’s UL WUS configuration, and the NES cell’s UL WUS configuration is included in a new SIB (including its own UL WUS configuration). Therefore, the UL WUS configuration needs to be exchanged between NES cells as that between Cell A and NES cell. The agreed class 1 procedure about UL WUS configuration transmission should be extended to the Xn between the NES cells.

In case of CU-DU split case, after the CU of Cell A or NES cell accepting the UL WUS configuration, it includes the received UL WUS configuration in a new SIB and transfers it to the DU of Cell A or NES cell. As for the IE design, RAN3 can reuse gNB-CU System Information IE.

Proposal 9:  gNB-CU includes the UL WUS configuration in a new SIB and transfers it to gNB-DU in gNB-CU System Information IE.

Conclusion

In summary, we have following observations and proposals for on-demand SIB1 for Idle/Inactive UE:

Proposal 1:  NES cell gNB includes Cell A requested for providing with the UL WUS configuration in UL WUS CONFIGURATION PROVISION REQUEST message.

Proposal 2:  Cell A gNB includes Cell A accepting or rejecting for providing with the UL WUS configuration in UL WUS CONFIGURATION PROVISION RESPONSE message.

Proposal 3:  Cell A gNB includes a list of accepted or rejected NES cell in UL WUS CONFIGURATION PROVISION RESPONSE message.

Proposal 4: NES cell gNB includes a list of NES cell to remove the UL WUS configuration in UL WUS CONFIGURATION PROVISION REQUEST message including Stop IE.

Proposal 5: Cell A gNB includes a list of NES cells stopped for providing with the UL WUS configuration and the Cell A stopping providing the UL WUS configuration in UL WUS CONFIGURATION TRANSMISSION STATUS UPDATE message.

Proposal 6: If Cell A gNB transmits stop indication to NES cell gNB, Cell A gNB maintains the UL WUS configuration for the NES cell till receiving release indication(e.g., stop IE) from the NES cell gNB.

Proposal 7:  NES cell gNB-CU includes On-demand SIB1 allowed indication for a list of NES cell to DU in GNB-CU CONFIGURATION UPDATE message.

Proposal 8:  If NES cell gNB-CU receives On-demand SIB1 stop indication for a list of cells in the GNB-DU CONFIGURATION UPDATE message, NES cell gNB-CU transmits UL WUS CONFIGURATION PROVISION REQUEST message including Stop IE and a list of NES cell to the Cell A gNB.

Proposal 9:  gNB-CU includes the UL WUS configuration in a new SIB and transfers it to gNB-DU in gNB-CU System Information IE.

Text Proposal for BL CR for TS 38.423 – on top of 252385

*************** Start of TP ***************

Editor’s Note: The procedure, message and the IE names are FFS

Editor’s Note: How to address the role of the NG-RAN node1 and NG-RAN node2 to be further discussed.

- 8.x.x	UL WUS Configuration Provision (FFS)

- 8.x.x.1	General

The purpose of the UL WUS Configuration Provision procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2 “and request NG-RAN node2 to transmit UL WUS configuration information (FFS)”.

The procedure is also used to enable an NG-RAN node1 to request NG-RAN node2 to stop the transmission of UL WUS configuration.  (FFS)”

The procedure uses non UE-associated signaling.

- 8.x.x.2	Successful Operation

Figure 8.x.x.2-1: UL WUS Configuration Provision, successful operation

If the “start with UL WUS Configuration” (details on this codepoint/choice of “start with UL WUS Configuration” are FFS) is included in the UL WUS CONFIGURATION PROVISION REQUEST (Naming FFS) message, the Cell A gNB (Naming FFS) shall, if supported, broadcast UL WUS Configuration (Naming FFS) in SIBx (SIB naming FFS) and reply with the UL WUS CONFIGURATION PROVISION RESPONSE (Naming FFS) message. The Cell A gNB (Naming FFS) stores the UL WUS configuration information (Naming FFS).

If UL WUS config providing cell List IE is included in the UL WUS CONFIGURATION PROVISION REQUEST message, the Cell A gNB shall, consider the list of Cell A requested for providing UL WUS Configuration.

If UL WUS config Stopped cell List IE is included in the UL WUS CONFIGURATION PROVISION REQUEST message, the Cell A gNB shall, stop for broadcasting UL WUS Configuration and remove the UL WUS Configuration for the indicated cells.

If the “stop” is included in the UL WUS CONFIGURATION PROVISION REQUEST (Naming FFS) message, the Cell A gNB (Naming FFS) shall, if supported, stop broadcasting UL WUS Configuration (Naming FFS) in SIBx (SIB naming FFS). Cell A gNB (Naming FFS) removes the 0received UL WUS Configuration.

If UL WUS config Stopping providing cell List IE is included in the UL WUS CONFIGURATION PROVISION RESPONSE message, the NES cell gNB shall, consider the list of Cell A has stopped for providing UL WUS Configuration.

“Cell A stores the UL WUS configuration information after it has been requested to be discontinued (FFS)”

“Cell A removes the UL WUS configuration information after it has been requested to be stopped (FFS)”

*************** Next change ***************

- 9.x.x.x	UL WUS CONFIGURATION PROVISION REQUEST

This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to request or stop provisioning of an UL WUS configuration in one or more cells.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE Request Type | M |  |  |  | YES | reject |
| >Start |  |  |  |  |  |  |
| >> UL WUS Configuration | M |  | OCTET STRING | Includes the xyz message as defined in subclause a.b.c of TS 38.331 [10]. |  |  |
| >> Cell A | M |  | 9.2.2.9 |  |  |  |
| >Stop |  |  |  |  |  |  |
| >> Cell list |  |  |  |  |  |  |
| >>>Cell List Item |  | 1 .. < FFS> |  |  |  |  |
| >>>>NG-RAN Cell Identity | M |  | 9.2.2.9 |  |  |  |

*************** End of TP ***************