3GPP TSG-RAN WG3 #128	R3-253330

Malta, MT, 19th – 23rd May 2025

Agenda Item:	20.1

Source:	Lenovo

Title: 		Discussion on OAM centric data collection and non-OTA based dataset/parameter sharing

Document for:	Discussion and Approval

# Introduction

In this paper, we discuss issues related to OAM centric data collection in AI for PHY as indicated in the LS from RAN2 R3-253011, and R3-253010.

# Discussion

## 2.1	OAM centric data collection

RAN3 is informed by RAN2 regarding the following agreements:

R3-253011 LS on OAM-centric solution for NW-side data collection For OAM-centric solution for NW-side data collection, RAN2 has made the following agreements: RAN2 assumes to only adopt management based immediate MDT for NW-side data collection. For NR-DC, the configuration and reporting of NW sided data collection can only be via MN.

First of all, we would like to clarify that the OAM-centric solution for NW-side data collection is for the AI based beam management and AI based CSI prediction use cases, wherein training data is collected by the OAM to train an gNB-sided AIML model.

RAN3 clarifies/confirms that the OAM-centric solution for NW-side data collection is for the AI based beam management and AI based CSI prediction use cases, wherein training data is collected by the OAM to train an gNB-sided AIML model.

Besides, it is still not very clear from the second agreement whether SN can configure the UE to perform training data collection. There are two possible interpretations:

- A: Only MN can configure the UE to perform training data collection and reporting.

- B: SN can configure the UE to perform training data collection and reporting via MN (e.g., via SRB1).

For NR-DC, the configuration and reporting of NW sided data collection can only be via MN.

In our understanding, according to the RAN2 agreements, the configuration related to the NW sided data collection is configured via immediate MDT framework in RRC connected state, while the design of data logging and reporting follows the principle of logged MDT:

- UE logs the L1 measurements according to the NW-sided data collection configuration

- UE indicates to NW if UE has data available

- NW fetches the logged L1 measurements via the UEInformationRequest/UEInformationResponse procedures

RAN2 Agreements: For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature. Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model Enhance the immediate MDT framework to support periodical reporting. UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection. The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest. As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT.

the configuration related to the NW sided data collection is configured via immediate MDT framework in RRC connected state, while the design of data logging and reporting follows the principle of logged MDT:

  - UE logs the L1 measurements according to the NW-sided data collection configuration

  - UE indicates to NW if UE has data available

  - NW fetches the logged L1 measurements via the UEInformationRequest/UEInformationResponse procedures

If SN can configure the UE to perform NW-sided data collection as described above via MN SRB1, then there will be several specification impacts considering:

- SN needs to send the SN data collection configuration to MN via Xn interface.

- UE needs to distinguish the data logged for MN and the data logged for SN, e.g., send them in separate RRC IE.

- SN needs to request MN to fetch the logged data for SN via Xn interface

- MN needs to distinguish if the received logged data is for MN or for SN.

Considering above complexity and DC scenario is strictly speaking not in the scope of R19, RAN3 is suggested to clarify/confirm that only MN can configure the UE to perform training data collection and reporting in DC scenario.

RAN3 is suggested to clarify/confirm that only MN can configure the UE to perform training data collection and reporting in DC scenario in Rel19.

Based on above analysis, there seems to be no RAN3 impact considering the two agreements captured in the RAN2 LS.

Reply the LS to RAN2 explaining RAN3 understanding and notifying that no relevant RAN3 impact is foreseen.

## 2.2	Dataset and parameter sharing via non-OTA

For the CSI compression use case, two-sided model will be used with an encoder at UE side and decoder at gNB side. For the training of the two sided model, RAN1 considers a scenario [R1-2410922] that

- NW-side shares model parameters and/or dataset to the UE or UE-side via standardized signaling, where the model parameters and/or dataset received at the UE or UE-side go through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., model (re-)training.

For Option 4-1: sharing {target CSI, CSI feedback} dataset:

  - Data size for target CSI depends on the format. There is no agreement on the format or necessary precision of the target CSI. Some examples based on companies’ evaluations are: eType-II format (up to ~1000 bits), eType-II-like format (~ a few 1000 bits), and float32 format (up to ~ 150K bits). The data size may also vary depending on the configuration, and the captured value indicates the order of magnitude of the typical data size per sample as a guideline.

  - There is no agreement on the CSI feedback size. Values in the order of eType II payload size may be assumed (up to ~ 1000 bits) for RAN2 discussion.

  - There is no agreement on the total number of samples N1 in the dataset. The range of the number of samples N1 used in RAN1 evaluations is 40K-1200K. As a representative number, RAN2 may assume that the total number of samples is around N1=600K.

  - Summing up the above, and assuming eType-II-like format of a size 2000 bits, the total dataset size for Option 4-1 may be roughly in the order of 600K * (2000 bits + 1000 bits) / (8bits/Byte) = 225 MB. The size may be smaller for eType-II format. The size may be larger if we consider the dataset consisting of supporting multiple configurations.

For Option 3a-1 without target CSI: sharing encoder parameter:

  - The number of parameters in the encoder models used in RAN1 evaluations have a wide range, ranging from 36K parameters to 13M parameters, with the median value of 5.8M.

  - Based on the above range, the size of encoder parameters may range from 36K*(8/8)=36KB based on the smallest model size in RAN1 evaluations and the use of float8, to 13M*(32/8)=52M based on the largest model size in RAN1 evaluations and the use of float32. Based on the median size, RAN2 may assume the size of 5.8M*(16/8) = 11.6 MB based on the medium model size and the use of float16.

For Option 3a-1 with target CSI: sharing encoder parameters, along with {target CSI} dataset:

  - The size of the encoder parameters is the same as in the Option 3a-1 without target CSI. The size of the dataset is smaller than the Option 4-1, because the dataset only contains the target CSI but not the CSI feedback. The number of samples N2 in the dataset may be the same or smaller than N1. The size becomes N2 * (2000 bits) / (8bits/Byte) + 11.6 MB.

The size of the model parameters/dataset can range from 11.6MB to 225MB which could be challenging for NGAP message transmission.

To support above model related parameters/dataset transfer, two approaches are further identified and analysed by RAN2 as explained in R3-253010.

Alternative 1 (non-OTA approach):

gNB -> NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server)

Alternative 2 (OTA approach):

gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (a server inside MNO or an OTT server)

NOTE: The data transfer (up to RAN1 further details on what to transfer) between gNB and NW dataset/model parameters collection entity (OAM/CN) in Alternative 1/2, if needed, is up to RAN3/SA2/SA5.

For non-OTA approaches, i.e., ‘NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified the following candidate solutions (see below Table 1).  From RAN2 point of view, it is assumed they can be supported within Rel-19 existing architecture framework. RAN3, SA2, and SA5 can further confirm the above RAN2 assumption. Furthermore, it does not preclude RAN3/SA2/SA5 to identify other candidate solutions beyond options listed below.

Table 1. non-OTA candidate solutions

| Option | Impacted WG | Specification impact/Implementation impact |
| --- | --- | --- |
| OAM -> UE-side training entity (a server inside MNO or an OTT server), where OAM is NW-side dataset/model parameter collection entity | SA5, SA3 | Up to SA5 (any intermediate node between OAM and UE-side OTT server is up to SA5; CN involvement if needed is up to SA2/SA5 discussion) |
| CN -> UE-side training entity (a server inside MNO or an OTT server), where CN is NW-side dataset/model parameter collection entity | SA2, SA3 | Up to SA2 (any intermediate node between CN and UE-side OTT server is up to SA2) |
| gNB -> OAM/CN -> UE-side training entity (a server inside MNO or an OTT server), where gNB is NW-side dataset/model parameter collection entity | RAN3, SA2, SA5, SA3 | Up to RAN3, SA2, SA5 (any intermediate node between gNB/OAM, OAM/UE-side OTT server, CN/UE-side OTT server is up to RAN3/SA2/SA5) |

To RAN3, SA2, SA5:

Action: RAN2 respectfully asks RAN3/SA2/SA5 to confirm RAN2 assumption on non-OTA solutions.

In addition, RAN3 is asked to confirm RAN2’s assumption as indicated in the LS. In particular if following can be supported via the existing NW interface (with possible enhancement):

- gNB can transfer the “raw data” (e.g., CSI report) to OAM/CN that is NW-side dataset/model parameter collection entity (e.g., model training entity)

- gNB can be NW-side dataset/model parameter collection entity itself (e.g., model training entity) and transfer the NW-side dataset/model parameter to OAM/CN.

Considering the size of the model parameters/dataset can vary from 11.6MB to 225MB, it is considered impractical for gNB to transfer the model parameters/dataset to CN via NG interface.

Comparatively, it is feasible for the gNB to transfer the model parameters/dataset to OAM via the IP network similar as how the Tracing/MDT report is transferred from gNB to OAM via IP network in either file based or streaming based manner.

Legacy Trace/MDT report is transferred from gNB to OAM via IP network in either file based or streaming based manner.

RAN3 considers it challenging to transfer the model parameters/dataset (with size up to 225MB) to CN via NG interface.

RAN3 considers it feasible to transfer the model parameters/dataset (with size up to 225MB) to OAM via the IP network.

Reply the LS to RAN2 explaining RAN3’s understanding.

# Conclusion

Based on the discussion above, we observe:

Observation 1	the configuration related to the NW sided data collection is configured via immediate MDT framework in RRC connected state, while the design of data logging and reporting follows the principle of logged MDT:

a.	UE logs the L1 measurements according to the NW-sided data collection configuration

b.	UE indicates to NW if UE has data available

c.	NW fetches the logged L1 measurements via the UEInformationRequest/UEInformationResponse procedures

Observation 2	The size of the model parameters/dataset can range from 11.6MB to 225MB which could be challenging for NGAP message transmission.

Observation 3	Legacy Trace/MDT report is transferred from gNB to OAM via IP network in either file based or streaming based manner.

Based on the discussion above, we propose:

Proposal 1	RAN3 clarifies/confirms that the OAM-centric solution for NW-side data collection is for the AI based beam management and AI based CSI prediction use cases, wherein training data is collected by the OAM to train an gNB-sided AIML model.

Proposal 2	RAN3 is suggested to clarify/confirm that only MN can configure the UE to perform training data collection and reporting in DC scenario in Rel19.

Proposal 3	Reply the LS to RAN2 explaining RAN3 understanding and notifying that no relevant RAN3 impact is foreseen.

Proposal 4	RAN3 considers it challenging to transfer the model parameters/dataset (with size up to 225MB) to CN via NG interface.

Proposal 5	RAN3 considers it feasible to transfer the model parameters/dataset (with size up to 225MB) to OAM via the IP network.

Proposal 6	Reply the LS to RAN2 explaining RAN3’s understanding.

# Reply LS to R3-253011

Title:	Rely LS on OAM-centric solution for NW-side data collection

Response to:	R3-253011

Release:	Rel-19

Work Item:	NR_AIML_air-Core

Source:	RAN3

To:	RAN2

Cc:	SA5

Contact person:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

- Overall description

RAN3 would like to thank RAN2 for informing the RAN2 agreements related to the OAM centric solution for NW-side data collection in R3-253011. RAN3 forms the following understanding:

- The referred “OAM centric solution for NW-side data collection” is for AIML based beam management and AIML based CSI prediction use cases, where OAM can request gNB via management based immediate MDT to configure UE(s) to log and report L1 measurements in RRC connected state.

- In NR-DC scenario, only MN can configure UE(s) to log and report L1 measurements in RRC connected state.

With above understanding, there is no RAN3 impact foreseen.

2	Actions

To RAN2:

ACTION: 	RAN3 respectfully asks RAN2 to take above information into account for their work and notify RAN3 in case of any concern.

3	Dates of next TSG RAN WG2 meetings

TSG RAN3 Meeting #129		25 - 29 Aug 2025					Bangalore, India

TSG RAN3 Meeting #129bis		13 - 17 Oct 2025					Prague, CZ

# Reply LS to R3-253010

Title:	Rely LS on signalling feasibility of dataset and parameter sharing

Response to:	R3-253010

Release:	Rel-19

Work Item:	NR_AIML_air-Core

Source:	RAN3

To:	RAN2

Cc:	RAN1, SA2, SA3, SA5

Contact person:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

- Overall description

RAN3 would like to thank RAN2 for informing the RAN2 agreements related to the dataset and parameter sharing for two-sided model in R3-253010.

RAN3 discussed the feasibility of supporting the transfer of model related dataset/parameter from gNB to OAM or CN via existing network interface and conclude that:

- It is feasible for gNB to transfer the model related dataset/parameter, that ranges from KBs to hundreds of MBs, to OAM via the IP network.

- It is challenging for gNB to transfer the model related dataset/parameter, that ranges from KBs to hundreds of MBs, to CN via the NG interface.

2	Actions

To RAN2:

ACTION: 	RAN3 respectfully asks RAN2 to take above information into account for their work and notify RAN3 in case of any concern.

3	Dates of next TSG RAN WG2 meetings

TSG RAN3 Meeting #129		25 - 29 Aug 2025					Bangalore, India

TSG RAN3 Meeting #129bis		13 - 17 Oct 2025					Prague, CZ