3GPP TSG-RAN WG3 Meeting #128	R3-253333

Malta, MT, 19th – 23rd May, 2025

Agenda item:	21.3

Source: 	Lenovo

Title: 	Discussion on Uplink Bit Rate Control

Document for:	Discussion and Approval

- Introduction

In RAN3#127bis, RAN3 discussed uplink bit rate control and agreed that:

Introduce an indication in NGAP, to allow the SMF to inform the gNB which QoS flow(s) are subject to uplink rate control.

Introduce the same indication in XnAP to inform target gNB.

FFS on per QoS flow indication or per DRB indication over F1AP?

FFS on whether CN may inform gNB the recommended UL rate for specific QoS flows.

Continue the discussion on UL rate control.

This paper continues discussing the open issues on XR UL bit rate control.

- Discussion

RAN2 further discussed whether per flow or per DRB indication for UL bit rate control and concluded that:

From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this. FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

In the F1-U, the gNB-DU cannot directly know the QFI of a PDCP PDU. This limitation arises due to the following reasons:

For UL Data Transmission:

-	The UE may include the QFI in the SDAP header of a packet, especially when there is a many-to-one mapping of QoS flows to a single DRB. However, the gNB-DU usually does not read the SDAP header because the SDAP function is located in gNB-CU. The scheduling is per DRB basis in gNB-DU.

For DL (Downlink) Data Transmission:

-	Before Release-18, the gNB-CU did not provide the QFI of a PDCP PDU to the gNB-DU.

-	Starting from Release-18, the gNB-CU may optionally include the QFI in the GTP-U extension header, specifically within the PDU Set Information container. However, this feature is limited to the F1-U PDU Set Information container and is not universally applicable to all scenarios.

Observation 1: The gNB-DU usually does not read the SDAP header because the SDAP function is located in gNB-CU and the scheduling is per DRB basis in gNB-DU.

It has been agreed to implement XR uplink rate control to dynamically adjust the bit rate for the UE during periods of congestion:

RAN2#128 Agreements on XR rate control - RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

In accordance with the agreement, when the gNB detects uplink congestion for a QoS flow, it can signal a reduced bit rate to the UE for the corresponding QoS flow. If multiple QoS flows are mapped to a single DRB, one proposed solution is to use MAC CE to indicate the bit rate per QoS flow to the UE. Upon receiving this indication, the UE forwards it to the upper layer for bit rate adaptation.

To effectively support rate control per QoS flow using MAC CE, the gNB needs to adapt the bit rate at the QoS flow level based on the current bit rate and the radio congestion status. However, in the case of a gNB CU-DU split architecture, challenges arise due to the separation of responsibilities between gNB-CU and gNB-DU.

Specifically, the gNB-DU is responsible for detecting radio congestion status, as radio scheduling is performed within the gNB-DU. On the other hand, the gNB-CU is responsible for measuring the bit rate for each QoS flow, since it is the entity that can identify the QoS flow ID based on the QFI field in the SDAP header. This separation of responsibilities leads to difficulties when multiple QoS flows are mapped to a single DRB. In such cases, the gNB-DU is unable to measure the bit rate per QoS flow, and consequently, it cannot accurately determine which QoS flow and what rate level should be adapted.

Observation 2: The gNB-CU is better suited for measuring the bit rate of each QoS flow, while the gNB-DU is responsible for detecting radio congestion status.

The proposed solution involves the following steps:

- Bit Rate Measurement and Reporting: The gNB-CU measures the bit rate for each QoS flow and provides this information to the gNB-DU.

- Bit Rate Control and Signaling: Based on the received bit rate measurements and its own detection of radio congestion status, the gNB-DU decides on the necessary bit rate adjustments and sends the corresponding bit rate control MAC CE to the UE.

- Dynamic QoS Flow Measurement Triggering: The gNB-CU may trigger or transmit QoS flow measurements to the gNB-DU based on requests from the gNB-DU. This ensures that the gNB-DU has the most up-to-date information for making informed decisions.

- Configuration and Information Flow: The gNB-CU may configure the gNB-DU to specify which QoS flows are eligible for bit rate adaptation. This configuration information may be derived from the Core Network (CN) or the UE, ensuring that the adaptation process aligns with higher-level requirements and user needs.

Proposal 1: The gNB-CU measures and reports the bit rate for a QoS flow to the gNB-DU, which then determines the necessary bit rate adjustments and sends the corresponding bit rate control MAC CE to the UE.

In the reply LS [1], SA2 responds that:

- SA2 confirms that the CN can indicate to the gNB that a QoS flow is subject to uplink rate control. - SA2 would like to inform RAN WGs that SA2 has not decided yet on whether the existing or a new mechanism can be reused.

In RAN3#127bis meeting, we agreed that

Introduce an indication in NGAP, to allow the SMF to inform the gNB which QoS flow(s) are subject to uplink rate control.

And in the BL CR, we introduced a new indication as:

| Indication of Bitrate Adaptation | O |  | ENUMERATED (uplink, …) | Indicates that the QoS Flow allows rate adaptation in the indicated direction. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

We believe that introducing a new indication to signify that a QoS flow is subject to uplink rate control is the most appropriate approach. Reusing the existing IE would conflate two distinct features. For instance, from the RAN perspective, alternative QoS notification and uplink bit rate control are two separate features, and there is no need to mix them together.

Proposal 2: Reply SA2 that RAN3 has agreed to introduce a new indication to indicate QoS flow is subject to uplink rate control.

3. Conclusion

Based on the discussion, the following proposals are made:

Proposal 1: The gNB-CU measures and reports the bit rate for a QoS flow to the gNB-DU, which then determines the necessary bit rate adjustments and sends the corresponding bit rate control MAC CE to the UE.

Proposal 2: Reply SA2 that RAN3 has agreed to introduce a new indication to indicate QoS flow is subject to uplink rate control.

Reference

[1] R3-253014 LS reply on uplink rate control	SA2(vivo)