3GPP TSG-RAN WG3 Meeting #128	R3-253334

Malta, MT, 19 - 23 May, 2025

Agenda item:	31.1

Source: 	Huawei, LG Electronics, Qualcomm, China Telecom, CATT, Xiaomi, CMCC

Title: 	Discussion on the PDCP SN gap report handling during UE’s handover

Document for:	Discussion and Decision

1. Introduction

---------------Excerpt from RAN2 reply LS in R3-250836---------------

RAN2 thanks RAN3 for investigating SN gap report handling during UE mobility and providing the solutions for the identified issue.

For Rel-18, RAN2 does not think there is a big problem to solve and agreed that alternative 3 from RAN3 LS will not be supported by RAN2. It is up to RAN3 whether to address this issue or not, e.g. based on alternative 1 or 2.

For Rel-19, RAN2 will discuss whether the problem is a critical issue in the context of RLC enhancements. If RAN2 concludes the issue needs to be solved in Rel-19, further discussions are needed on the solutions considering but not limiting to the three alternatives as in the RAN3 LS.

In Rel-18, RAN2 has discussed and agreed to support PDCP SN gap report to enable the transmitting PDCP entity to notify the receiving PDCP entity about the packets discarded before transmitting. However, it is not clear how to handle the PDCP SN gap report during UE’s inter-gNB mobility. In previous meetings, two network-based solutions and one UE-based solution were discussed based on the papers R3-244517 and R3-245384. RAN3 sent LS R3-247891 to RAN2 to check the feasibility of UE-based solution. And RAN2 replied in R3-250836[1] saying the UE-based solution will not be supported in R18. For R19, the issue may be further considered in the context of RLC enhancements. The reply from RAN2 is copied below:

However, there is no further discussion so far in RAN2. Considering the remaining TUs for Rel-19 XR are quite limited, and the handling for handover is RAN3 territory, we think RAN3 should discuss the issue without waiting for RAN2. In this contribution, we suggest to solve the issue in Rel-19 TEI and analyse the impacts for the network-based solutions.

2. Discussion

## 2.1 Problem description

Although the issue about how to handle PDCP SN gap during handover was discussed several times, we still provide the following introduction of the problem below, for better understanding.

PDCP SN gap report has been supported by RAN2 to allow the receiving PDCP entity to get aware of the discarded SDUs so that it does not need to wait for them. The details are specified in TS 38.323 [2]. Considering the intention of PDCP SN gap report, we find that there are some issues during the procedure of handover. Some RAN3 enhancements are needed.

Specifically, for uplink transmission, if the UE discards some packets before handover, it will send the PDCP SN gap report to the source gNB. The source gNB can update the receiving window according to the SN gap report. During the handover procedure, the source gNB forwards the buffered packets to the target gNB and sends the SN STATUS TRANSFER message to the target gNB. The SN STATUS TRANSFER message indicates the PDCP SNs corresponding to those packets that haven't been received, so that the target gNB could sent PDCP status report to UE and UE can retransmit these packets after handover. However, among the packets, there may be some packets which have already been discarded by the UE before handover. The target gNB is not aware of the discarding and will wait for the discarded packets, result in large delay.

For example, the following figure shows a possible situation at the source gNB. The UE indicated to the source gNB that the PDCP SDU 1/2/6/7 were discarded. According to the report, the source gNB updated the RX_DELIV and RX_NEXT. Then during handover, the source gNB will forward the SDU 8 to the target gNB and indicate that SDU 4/5/6/7 are not received yet. However, since SDU 6/7 are already discarded actually, the target gNB cannot receive them from the UE anymore, which will introduce unnecessary waiting delay.

Fig.1 The source gNB received the PDCP SN gap report before handover.

Observation 1: The target gNB is not aware of which packet has been discarded by UE before the handover, it will wait for the discarded packets until the PDCP reordering timer expires and cause large delay at PDCP layer.

## 2.2 Potential Solutions

To solve the issue, the following solutions can be considered.

-	Alternative 1: The source gNB forwards the PDCP SN gap report information to the target gNB.

-	Alternative 2: The source gNB indicates the discarded SDUs as successfully received via the UL received status in the SN STATUS TRANSFER sent to the target gNB.

-	Alternative 3: The UE sends the PDCP SN gap report again after connecting to the target gNB.

Alternative 1 and 2 are network-based solution, while the Alternative 3 is a UE-based solution. According to the reply LS R3-250836 from RAN2, Alternative 3 has been excluded by RAN2 for R18. Furthermore, for Rel-19, RAN2 “will discuss whether the problem is a critical issue in the context of RLC enhancements”, but there is no further discussion so far in RAN2 and the issue is not included in the R19 WID. From our perspective, the issue during the handover procedure is different from the issue in context of RLC enhancements mentioned by RAN2. They cannot be solved with a unify solution. The discussion for the handover scenario should not be impacted due to the R19 RLC enhancements consideration. In the following part, we will explain more details on the difference between the RLC enhancement case and the handover scenario.

In Rel-19 XR, RAN2 is going to introduce enhancements to the RLC protocol to avoid unnecessary RLC retransmission. Specifically, when an RLC SDU is outdated, the transmitting RLC entity will not retransmit it any longer. Meanwhile, the receiving RLC entity will maintain a timer to determine which RLC SDUs are outdated, and for the outdated SDUs, the receiving entity will provide positive acknowledgement to the transmitting side even if the SDUs have not been successfully received. With such enhancements, it is possible that the gNB cannot receive the PDCP SN gap report sent by the UE since the report may be regarded as outdated at the receiving RLC entity before it is successfully received.

The above issue does not relate to the UE handover scenario. It cannot be solved by any one of the proposed three alternatives, since neither the gNB nor the UE knows the loss of the PDCP SN gap report. Besides, it is also doubtful whether it is worth addressing such issue since it sounds like a corner case.

However, the problem described in section 2.1 is more common. It is highly possible to occur when the UE performs handover. RAN3 should focus on the issue during handover no matter RAN2 solve the PDCP SN gap loss due to RLC enhancement or not.

Observation 2: The loss of PDCP SN gap report considering Rel-19 RLC enhancements in RAN2 is totally different from the PDCP SN gap forwarding issue for UE handover, RAN3 is responsible for solving handover issue.

Alternative 1 and 2 can be considered by RAN3 because they only impact network side. Similar issue also exists for RLF recovery scenario, and the above solutions are applicable for UE RLF recovery case also.

For Alternative 1, the target gNB can get aware of the discarding explicitly. The behaviour upon receiving the SN gap report from the source gNB is the same to receiving the SN gap report from the UE. We provide the CR for Alternative 1 in the correlated paper R3-253335 [3].

For Alternative 2, the indication is implicit. There may be a risk that the target gNB still hope to receive the discarded SDUs from the source gNB since it is indicated as “received”. To avoid such issue, some description about the target gNB’s behaviour needs to be added. But this may impact the existing data forwarding behaviour based on the received SN STATUS TRANSFER message, because the target gNB cannot distinguish whether the packets marked correctly received by the source gNB but never reached/forwarded to the target gNB was either discarded by the UE or lost during data forwarding. The CR for Alternative 2 is provided in R3-253336 [4].

Proposal 1: RAN3 to solve the issue on how to inform the target gNB about the PDCP SN gap during handover/RLF recovery, and down-select between the following options:

- Alternative 1: The source gNB forwards the SN gap report to the target gNB.

- Alternative 2: The source gNB indicates the discarded SDUs as received in the SN STATUS TRANSFER.

3. Conclusion

In this contribution, we analyzed the issue of PDCP SN gap report. The following proposals and observations were provided:

Observation 1: The target gNB is not aware of which packet has been discarded by UE before the handover, it will wait for the discarded packets until the PDCP reordering timer expires and cause large delay at PDCP layer.

Observation 2: The loss of PDCP SN gap report considering Rel-19 RLC enhancements in RAN2 is totally different from the PDCP SN gap forwarding issue for UE handover, RAN3 is responsible for solving handover issue.

Proposal 1: RAN3 to solve the issue on how to inform the target gNB about the PDCP SN gap during handover/RLF recovery, and down-select between the following options:

- Alternative 1: The source gNB forwards the SN gap report to the target gNB.

- Alternative 2: The source gNB indicates the discarded SDUs as received in the SN STATUS TRANSFER.

4. Reference

[1]. R3-247891, 3GPP TSG RAN WG3, LS on PDCP SN gap report handling during UE mobility.

[2]. R3-250836, 3GPP TSG RAN WG2, Reply on PDCP SN gap report handling during UE mobility.

[3]. R3-253335, Correction for PDCP SN gap report during UE’s handover – Alt 1, Huawei, LG Electronics, Qualcomm, China Telecom, Xiaomi, CMCC.

[3]. R3-253336, Correction for PDCP SN gap report during UE’s handover – Alt 2, Huawei, China Telecom, CATT, Xiaomi.