3GPP TSG-RAN WG3 Meeting #128	R3-253340

# Malta, MT, 19 – 23 May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.483 | CR | 0166 | rev | 1 | Current version: | 18.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for MDBV in alternative QoS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson, Nokia, Nokia Shanghai Bell, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SA2 agreed that the MDBV can be included in the Alternative QoS Profile for Delay-critical traffic, the following sentence is excerpted from TS 23.501 “For a GBR QoS Flow using the Delay-critical resource type, an Alternative QoS Profile may also include the QoS parameter MDBV.” To align with SA2, RAN3 also included the MDBV in the Alternative QoS Parameters, the semantics description part includes “This IE shall be included if the Delay Critical IE is set to "delay critical" and is ignored otherwise.” Such statement indicates that the MDBV will always be included in the alternative QoS parameter if the traffic is delay critial. But this is not align with SA2’s specification because the MDBV is optionally included in the alternative QoS parameter for the delay critical traffic. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the semantics description for the Maximum Data Burst Volume to be “This IE may be included if the Delay Critical IE is set to "delay critical" and is ignored otherwise”. Impact Analysis: Impact assessment towards the previous version of the specification (same release): - This CR has no impact with the previous version of the specification (same release). - This CR has impact on the functional point of view. The impact can be considered isolated because the change only affects the alternative QoS parameter configuration for delay critical traffic. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Will cause some unpredictable error at CU-UP, who expects that the MDBV will always be included in the alternative QoS parameter for delay critial QoS flow, if the CN does not provide MDBV in alternative QoS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.93 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.413 CR1273, TS 38.473 CR1561, TS38.423 CR1481 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Resubmitted to RAN3 #128 meeting. |  |  |  |  |  |  |  |  |

Start of Change

9.3.1.93	Alternative QoS Parameters Set List

This IE contains alternative sets of QoS parameters which the NG-RAN node can indicate to be fulfilled when notification control is enabled and it cannot fulfil the requested list of QoS parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Alternative QoS Parameters Set Item |  | 1..<maxnoofQoSparaSets> |  |  |  |  |
| >Alternative QoS Parameters Set Index | M |  | INTEGER (1..8,..) |  | – |  |
| >Guaranteed Flow Bit Rate Downlink | O |  | Bit Rate 9.3.1.20 |  | – |  |
| >Guaranteed Flow Bit Rate Uplink | O |  | Bit Rate 9.3.1.20 |  | – |  |
| >Packet Delay Budget | O |  | 9.3.1.47 |  | – |  |
| >Packet Error Rate | O |  | 9.3.1.48 |  | – |  |
| >Maximum Data Burst Volume | O |  | 9.3.1.50 | For details see TS 23.501 [20]. This IE may be included if the Delay Critical IE is set to “delay critical” and is ignored otherwise. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSParaSets | Maximum no. of alternative sets of QoS Parameters allowed for the QoS under Notification Control. Value is 8. |

End of Change