3GPP TSG-RAN WG3 #128																		R3-253405

# Malta, MT, 19-23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1558 | rev | 3 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | F1 support of L3 measurements-based LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Vodafone, CATT, ZTE, NTT DoCoMo, Verizon Wireless, Nokia, Samsung, LG Electronics, Lenovo, Google, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to RAN2’s LS in R3-247013 both LTM execution and early synchronisation for UL and/or DL can be triggered using Layer 3 measurements from the UE, F1AP signaling needs to be updated. This involves triggering a new Class-2 procedure from the gNB-CU to the gNB-DU to instruct the gNB-DU to send a MAC CE command to the UE, and/or trigger early DL or UL synchronization. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new Class 2 procedure named CU-DU Mobility Initiation to trigger cell switch and/or early synchronization for the UE. Include assistance information from the gNB-CU to the gNB-DU, i.e., triggering indication and measurements quantities. Impact Analysis: Impact assessment towards the previous version of the specification (same release): The CR has isolated impact because the change affects only the LTM function. The CR is backwards compatible from a protocol point of view. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LTM execution and early synchronization cannot be triggered based on Layer 3 measurements in gNB split architecture. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1, 8.2.x (new), 9.2.2.x (new), 9.3.1.xxx (new), 9.3.1.xxy (new), 9.3.1.xyy (new), 9.4.3, 9.4.4, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.401 CR 0428, TS 38.470 CR 0158 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 3: remove the FFSs Rev 2: change category to F Rev 1: updates towards Option 5 |  |  |  |  |  |  |  |  |

# 8	F1AP procedures

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| F1 Setup | F1 SETUP REQUEST | F1 SETUP RESPONSE | F1 SETUP FAILURE |
| gNB-DU Configuration Update | GNB-DU CONFIGURATION UPDATE | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-DU CONFIGURATION UPDATE FAILURE |
| gNB-CU Configuration Update | GNB-CU CONFIGURATION UPDATE | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-CU CONFIGURATION UPDATE FAILURE |
| UE Context Setup | UE CONTEXT SETUP REQUEST | UE CONTEXT SETUP RESPONSE | UE CONTEXT SETUP FAILURE |
| UE Context Release (gNB-CU initiated) | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| UE Context Modification (gNB-CU initiated) | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Modification Required (gNB-DU initiated) | UE CONTEXT MODIFICATION REQUIRED | UE CONTEXT MODIFICATION CONFIRM | UE CONTEXT MODIFICATION REFUSE |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| gNB-DU Resource Coordination | GNB-DU RESOURCE COORDINATION REQUEST | GNB-DU RESOURCE COORDINATION RESPONSE |  |
| F1 Removal | F1 REMOVAL REQUEST | F1 REMOVAL RESPONSE | F1 REMOVAL FAILURE |
| BAP Mapping Configuration | BAP MAPPING CONFIGURATION | BAP MAPPING CONFIGURATION ACKNOWLEDGE | BAP MAPPING CONFIGURATION FAILURE |
| GNB-DU Resource Configuration | GNB-DU RESOURCE CONFIGURATION | GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE | GNB-DU RESOURCE CONFIGURATION FAILURE |
| IAB TNL Address Allocation | IAB TNL ADDRESS REQUEST | IAB TNL ADDRESS RESPONSE | IAB TNL ADDRESS FAILURE |
| IAB UP Configuration Update | IAB UP CONFIGURATION UPDATE REQUEST | IAB UP CONFIGURATION UPDATE RESPONSE | IAB UP CONFIGURATION UPDATE FAILURE |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Positioning Measurement | POSITIONING MEASUREMENT REQUEST | POSITIONING MEASUREMENT RESPONSE | POSITIONING MEASUREMENT FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| Broadcast Context Setup | BROADCAST CONTEXT SETUP REQUEST | BROADCAST CONTEXT SETUP RESPONSE | BROADCAST CONTEXT SETUP FAILURE |
| Broadcast Context Release | BROADCAST CONTEXT RELEASE COMMAND | BROADCAST CONTEXT RELEASE COMPLETE |  |
| Broadcast Context Modification | BROADCAST CONTEXT MODIFICATION REQUEST | BROADCAST CONTEXT MODIFICATION RESPONSE | BROADCAST CONTEXT MODIFICATION FAILURE |
| Multicast Context Setup | MULTICAST CONTEXT SETUP REQUEST | MULTICAST CONTEXT SETUP RESPONSE | MULTICAST CONTEXT SETUP FAILURE |
| Multicast Context Release | MULTICAST CONTEXT RELEASE COMMAND | MULTICAST CONTEXT RELEASE COMPLETE |  |
| Multicast Context Modification | MULTICAST CONTEXT MODIFICATION REQUEST | MULTICAST CONTEXT MODIFICATION RESPONSE | MULTICAST CONTEXT MODIFICATION FAILURE |
| Multicast Distribution Setup | MULTICAST DISTRIBUTION SETUP REQUEST | MULTICAST DISTRIBUTION SETUP RESPONSE | MULTICAST DISTRIBUTION SETUP FAILURE |
| Multicast Distribution Release | MULTICAST DISTRIBUTION RELEASE COMMAND | MULTICAST DISTRIBUTION RELEASE COMPLETE |  |
| PDC Measurement Initiation | PDC MEASUREMENT INITIATION REQUEST | PDC MEASUREMENT INITIATION RESPONSE | PDC MEASUREMENT INITIATION FAILURE |
| PRS Configuration Exchange | PRS CONFIGURATION REQUEST | PRS CONFIGURATION RESPONSE | PRS CONFIGURATION FAILURE |
| Measurement Preconfiguration | MEASUREMENT PRECONFIGURATION REQUIRED | MEASUREMENT PRECONFIGURATION CONFIRM | MEASUREMENT PRECONFIGURATION REFUSE |
| Timing Synchronisation Status | TIMING SYNCHRONISATION STATUS REQUEST | TIMING SYNCHRONISATION STATUS RESPONSE | TIMING SYNCHRONISATION STATUS FAILURE |
| Multicast Context Notification | MULTICAST CONTEXT NOTIFICATION INDICATION | MULTICAST CONTEXT NOTIFICATION CONFIRM | MULTICAST CONTEXT NOTIFICATION REFUSE |
| Multicast Common Configuration | MULTICAST COMMON CONFIGURATION REQUEST | MULTICAST COMMON CONFIGURATION RESPONSE | MULTICAST COMMON CONFIGURATION REFUSE |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| Notify | NOTIFY |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |
| RRC Delivery Report | RRC DELIVERY REPORT |
| Network Access Rate Reduction | NETWORK ACCESS RATE REDUCTION |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| DU-CU Radio Information Transfer | DU-CU RADIO INFORMATION TRANSFER |
| CU-DU Radio Information Transfer | CU-DU RADIO INFORMATION TRANSFER |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Reference Time Information Reporting Control | REFERENCE TIME INFORMATION REPORTING CONTROL |
| Reference Time Information Report | REFERENCE TIME INFORMATION REPORT |
| Access Success | ACCESS SUCCESS |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Positioning Assistance Information Control | POSITIONING ASSISTANCE INFORMATION CONTROL |
| Positioning Assistance Information Feedback | POSITIONING ASSISTANCE INFORMATION FEEDBACK |
| Positioning Measurement Report | POSITIONING MEASUREMENT REPORT |
| Positioning Measurement Abort | POSITIONING MEASUREMENT ABORT |
| Positioning Measurement Failure Indication | POSITIONING MEASUREMENT FAILURE INDICATION |
| Positioning Measurement Update | POSITIONING MEASUREMENT UPDATE |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Multicast Group Paging | MULTICAST GROUP PAGING |
| Broadcast Context Release Request | BROADCAST CONTEXT RELEASE REQUEST |
| Multicast Context Release Request | MULTICAST CONTEXT RELEASE REQUEST |
| PDC Measurement Report | PDC MEASUREMENT REPORT |
| PDC Measurement Failure Indication | PDC MEASUREMENT FAILURE INDICATION |
| PDC Measurement Termination | PDC MEASUREMENT TERMINATION COMMAND |
| Measurement Activation | MEASUREMENT ACTIVATION |
| QoE Information Transfer | QOE INFORMATION TRANSFER |
| Positioning System Information Delivery | POSITIONING SYSTEM INFORMATION DELIVERY COMMAND |
| DU-CU Cell Switch Notification | DU-CU CELL SWITCH NOTIFICATION |
| CU-DU Cell Switch Notification | CU-DU CELL SWITCH NOTIFICATION |
| DU-CU TA Information Transfer | DU-CU TA INFORMATION TRANSFER |
| CU-DU TA Information Transfer | CU-DU TA INFORMATION TRANSFER |
| QoE Information Transfer Control | QOE INFORMATION TRANSFER CONTROL |
| RACH Indication | RACH INDICATION |
| Timing Synchronisation Status Report | TIMING SYNCHRONISATION STATUS REPORT |
| Mobile IAB F1 Setup Triggering | MIAB F1 SETUP TRIGGERING |
| Mobile IAB F1 Setup Outcome Notification | MIAB F1 SETUP OUTCOME NOTIFICATION |
| Broadcast Transport Resource Request | BROADCAST TRANSPORT RESOURCE REQUEST |
| SRS Information Reservation Notification | SRS INFORMATION RESERVATION NOTIFICATION |
| DU-CU Access And Mobility Indication | DU-CU ACCESS AND MOBILITY INDICATION |
| CU-DU Mobility Initiation | CU-DU MOBILITY INITIATION REQUEST |

<<<<<< NEXT CHANGE >>>>>>

### 8.2.x	CU-DU Mobility Initiation

#### 8.2.x.1	General

The purpose of the CU-DU Mobility Initiation procedure is to trigger cell switch command and/or early synchronization for the UE. The procedure uses UE-associated signalling.

#### 8.2.x.2	Successful Operation

Figure 8.3.x.2-1: CU-DU Mobility Initiation procedure. Successful operation.

The gNB-CU initiates the procedure by sending a CU-DU MOBILITY INITIATION REQUEST message.

Upon reception of the CU-DU MOBILITY INITIATION REQUEST message, the gNB-DU shall trigger LTM.

If the Additional Mobility Assistance Information IE is contained in the CU-DU MOBILITY INITIATION REQUEST message, the gNB-DU may take the information into account for triggering LTM.

#### 8.2.x.3	Unsuccessful Operation

Not applicable.

#### 8.2.x.4	Abnormal Conditions

If the Triggering Indication IE bitmap is set to "0" (all bits are set to "0") in the CU-DU MOBILITY INITIATION REQUEST message, then gNB-DU shall initiate an ERROR INDICATION message with an appropriate cause value.

<<<<<< NEXT CHANGE >>>>>>

#### 9.2.2.x	CU-DU MOBILITY INITIATION REQUEST

This message is sent by the gNB-CU to the gNB-DU to trigger a cell switch command and/or early synchronization for the UE.

Direction: gNB-CU ® gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Mobility Triggering Indication | M |  | BIT STRING (SIZE(8)) | Indicates the triggering of the CU-DU Mobility Initiation procedure. First bit = early UL synchronization, second bit = early DL synchronization, third bit = cell switch, other bits reserved for future use. | YES | reject |
| Mobility Assistance Information Cell Switch | C-ifTrigger IndicationCellSwitch |  |  |  | YES | ignore |
| >Candidate Cell with Beam Information | M |  | 9.3.1.xxx |  | - |  |
| Mobility Assistance Information Early UL sync | C-ifTrigger IndicationEarlyULSync |  |  |  | YES | ignore |
| >Candidate Cell List with Beam Information | M |  | 9.3.1.xxy |  | - |  |
| Mobility Assistance Information Early DL sync | C-ifTrigger IndicationEarlyDLSync |  |  |  | YES | ignore |
| >Candidate Cell List with Beam Information | M |  | 9.3.1.xxy |  | - |  |
| Additional Mobility Assistance Information |  | 0..1 |  |  | YES | ignore |
| >Candidate Cell ID List | M | 1 .. < maxnoofCandidateCells> |  |  | - |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>SSB List |  | 1..<maxnoofSSBs> |  |  |  |  |
| >>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>Selected Measurement Quantities | M |  | 9.3.1.xyy |  | - |  |

| Condition | Explanation |
| --- | --- |
| ifTriggeringIndicationCellSwitch | This IE shall be present if the Triggering Indication IE is set to the value "cell switch". |
| ifTriggeringIndicationEarlyULSync | This IE shall be present if the Triggering Indication IE is set to the value "early UL synchronization ". |
| ifTriggeringIndicationEarlyDLSync | This IE shall be present if the Triggering Indication IE is set to the value "early DL synchronization ". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCandidateCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |
| maxnoofSSBs | Maximum no. of SSBs that can be served by a NG-RAN node cell. Value is 64. |

<<<<< NEXT CHANGE >>>>>>

#### 9.3.1.xxx	Candidate Cell with Beam Information

This IE includes a list of candidate cells and associated SSB indexes information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |
| SSB Index | M |  | INTEGER (0..63) |  |

#### 9.3.1.xxy	Candidate Cell with Beam Information List

This IE includes a list of candidate cells and associated SSB indexes information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Candidate Cell List |  | 1 .. < maxnoofCandidateCells> |  |  |
| >Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |
| >SSB Index List |  | 0..<maxnoofSSBIndices> |  |  |
| >>SSB Index | M |  | INTEGER (0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCandidateCells | Maximum no. of Cells configured as candidates allowed towards one UE, the maximum value is 8. |
| maxnoofSSBIndices | Maximum no. of SSBs that can be served by a NG-RAN node cell. Value is 64. |

#### 9.3.1.xyy	Measurement Quantities

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | IE/Group Name | Presence |
| --- | --- | --- | --- | --- | --- | --- |
| RSRP | O |  | INTEGER (0..127) | Corresponds to information provided in the RSRP-Range IE as defined in TS 38.331 [10]. |  |  |
| RSRQ | O |  | INTEGER (0..127) | Corresponds to information provided in the RSRQ-Range IE as defined in TS 38.331 [10]. |  |  |
| SINR | O |  | INTEGER (0..127) | Corresponds to information provided in the SINR-Range IE as defined in TS 38.331 [10]. |  |  |

<<<<< NEXT CHANGE >>>>>>

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

F1AP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM F1AP-CommonDataTypes

Reset,

ResetAcknowledge,

F1SetupRequest,

F1SetupResponse,

F1SetupFailure,

GNBDUConfigurationUpdate,

GNBDUConfigurationUpdateAcknowledge,

GNBDUConfigurationUpdateFailure,

GNBCUConfigurationUpdate,

GNBCUConfigurationUpdateAcknowledge,

GNBCUConfigurationUpdateFailure,

UEContextSetupRequest,

UEContextSetupResponse,

UEContextSetupFailure,

UEContextReleaseCommand,

UEContextReleaseComplete,

UEContextModificationRequest,

UEContextModificationResponse,

UEContextModificationFailure,

UEContextModificationRequired,

UEContextModificationConfirm,

ErrorIndication,

UEContextReleaseRequest,

DLRRCMessageTransfer,

ULRRCMessageTransfer,

GNBDUResourceCoordinationRequest,

GNBDUResourceCoordinationResponse,

PrivateMessage,

UEInactivityNotification,

InitialULRRCMessageTransfer,

SystemInformationDeliveryCommand,

Paging,

Notify,

WriteReplaceWarningRequest,

WriteReplaceWarningResponse,

PWSCancelRequest,

PWSCancelResponse,

PWSRestartIndication,

PWSFailureIndication,

GNBDUStatusIndication,

RRCDeliveryReport,

UEContextModificationRefuse,

F1RemovalRequest,

F1RemovalResponse,

F1RemovalFailure,

NetworkAccessRateReduction,

TraceStart,

DeactivateTrace,

DUCURadioInformationTransfer,

CUDURadioInformationTransfer,

BAPMappingConfiguration,

BAPMappingConfigurationAcknowledge,

BAPMappingConfigurationFailure,

GNBDUResourceConfiguration,

GNBDUResourceConfigurationAcknowledge,

GNBDUResourceConfigurationFailure,

IABTNLAddressRequest,

IABTNLAddressResponse,

IABTNLAddressFailure,

IABUPConfigurationUpdateRequest,

IABUPConfigurationUpdateResponse,

IABUPConfigurationUpdateFailure,

ResourceStatusRequest,

ResourceStatusResponse,

ResourceStatusFailure,

ResourceStatusUpdate,

AccessAndMobilityIndication,

ReferenceTimeInformationReportingControl,

ReferenceTimeInformationReport,

AccessSuccess,

CellTrafficTrace,

PositioningMeasurementRequest,

PositioningMeasurementResponse,

PositioningMeasurementFailure,

PositioningAssistanceInformationControl,

PositioningAssistanceInformationFeedback,

PositioningMeasurementReport,

PositioningMeasurementAbort,

PositioningMeasurementFailureIndication,

PositioningMeasurementUpdate,

TRPInformationRequest,

TRPInformationResponse,

TRPInformationFailure,

PositioningInformationRequest,

PositioningInformationResponse,

PositioningInformationFailure,

PositioningActivationRequest,

PositioningActivationResponse,

PositioningActivationFailure,

PositioningDeactivation,

PositioningInformationUpdate,

E-CIDMeasurementInitiationRequest,

E-CIDMeasurementInitiationResponse,

E-CIDMeasurementInitiationFailure,

E-CIDMeasurementFailureIndication,

E-CIDMeasurementReport,

E-CIDMeasurementTerminationCommand,

BroadcastContextSetupRequest,

BroadcastContextSetupResponse,

BroadcastContextSetupFailure,

BroadcastContextReleaseCommand,

BroadcastContextReleaseComplete,

BroadcastContextReleaseRequest,

BroadcastContextModificationRequest,

BroadcastContextModificationResponse,

BroadcastContextModificationFailure,

MulticastGroupPaging,

MulticastContextSetupRequest,

MulticastContextSetupResponse,

MulticastContextSetupFailure,

MulticastContextReleaseCommand,

MulticastContextReleaseComplete,

MulticastContextReleaseRequest,

MulticastContextModificationRequest,

MulticastContextModificationResponse,

MulticastContextModificationFailure,

MulticastDistributionSetupRequest,

MulticastDistributionSetupResponse,

MulticastDistributionSetupFailure,

MulticastDistributionReleaseCommand,

MulticastDistributionReleaseComplete,

PDCMeasurementInitiationRequest,

PDCMeasurementInitiationResponse,

PDCMeasurementInitiationFailure,

PDCMeasurementReport,

PDCMeasurementTerminationCommand,

PDCMeasurementFailureIndication,

PRSConfigurationRequest,

PRSConfigurationResponse,

PRSConfigurationFailure,

MeasurementPreconfigurationRequired,

MeasurementPreconfigurationConfirm,

MeasurementPreconfigurationRefuse,

MeasurementActivation,

QoEInformationTransfer,

PosSystemInformationDeliveryCommand,

DUCUCellSwitchNotification,

CUDUCellSwitchNotification,

DUCUTAInformationTransfer,

CUDUTAInformationTransfer,

QoEInformationTransferControl,

RachIndication,

TimingSynchronisationStatusRequest,

TimingSynchronisationStatusResponse,

TimingSynchronisationStatusFailure,

TimingSynchronisationStatusReport,

MIABF1SetupTriggering,

MIABF1SetupOutcomeNotification,

MulticastContextNotificationIndication,

MulticastContextNotificationConfirm,

MulticastContextNotificationRefuse,

MulticastCommonConfigurationRequest,

MulticastCommonConfigurationResponse,

MulticastCommonConfigurationRefuse,

BroadcastTransportResourceRequest,

DUCUAccessAndMobilityIndication,

SRSInformationReservationNotification,

CUDUMobilityInitiationRequest

FROM F1AP-PDU-Contents

id-Reset,

id-F1Setup,

id-gNBDUConfigurationUpdate,

id-gNBCUConfigurationUpdate,

id-UEContextSetup,

id-UEContextRelease,

id-UEContextModification,

id-UEContextModificationRequired,

id-DUCUAccessAndMobilityIndication,

id-ErrorIndication,

id-UEContextReleaseRequest,

id-DLRRCMessageTransfer,

id-ULRRCMessageTransfer,

id-GNBDUResourceCoordination,

id-privateMessage,

id-UEInactivityNotification,

id-InitialULRRCMessageTransfer,

id-SystemInformationDeliveryCommand,

id-Paging,

id-Notify,

id-WriteReplaceWarning,

id-PWSCancel,

id-PWSRestartIndication,

id-PWSFailureIndication,

id-GNBDUStatusIndication,

id-RRCDeliveryReport,

id-F1Removal,

id-NetworkAccessRateReduction,

id-TraceStart,

id-DeactivateTrace,

id-DUCURadioInformationTransfer,

id-CUDURadioInformationTransfer,

id-BAPMappingConfiguration,

id-GNBDUResourceConfiguration,

id-IABTNLAddressAllocation,

id-IABUPConfigurationUpdate,

id-resourceStatusReportingInitiation,

id-resourceStatusReporting,

id-accessAndMobilityIndication,

id-ReferenceTimeInformationReportingControl,

id-ReferenceTimeInformationReport,

id-accessSuccess,

id-cellTrafficTrace,

id-PositioningMeasurementExchange,

id-PositioningAssistanceInformationControl,

id-PositioningAssistanceInformationFeedback,

id-PositioningMeasurementReport,

id-PositioningMeasurementAbort,

id-PositioningMeasurementFailureIndication,

id-PositioningMeasurementUpdate,

id-TRPInformationExchange,

id-PositioningInformationExchange,

id-PositioningActivation,

id-PositioningDeactivation,

id-PositioningInformationUpdate,

id-E-CIDMeasurementInitiation,

id-E-CIDMeasurementFailureIndication,

id-E-CIDMeasurementReport,

id-E-CIDMeasurementTermination,

id-BroadcastContextSetup,

id-BroadcastContextRelease,

id-BroadcastContextReleaseRequest,

id-BroadcastContextModification,

id-MulticastGroupPaging,

id-MulticastContextSetup,

id-MulticastContextRelease,

id-MulticastContextReleaseRequest,

id-MulticastContextModification,

id-MulticastDistributionSetup,

id-MulticastDistributionRelease,

id-PDCMeasurementInitiation,

id-PDCMeasurementTerminationCommand,

id-PDCMeasurementFailureIndication,

id-PDCMeasurementReport,

id-pRSConfigurationExchange,

id-measurementPreconfiguration,

id-measurementActivation,

id-QoEInformationTransfer,

id-PosSystemInformationDeliveryCommand,

id-DUCUCellSwitchNotification,

id-CUDUCellSwitchNotification,

id-DUCUTAInformationTransfer,

id-CUDUTAInformationTransfer,

id-QoEInformationTransferControl,

id-RachIndication,

id-TimingSynchronisationStatus,

id-TimingSynchronisationStatusReport,

id-MIABF1SetupTriggering,

id-MIABF1SetupOutcomeNotification,

id-MulticastContextNotification,

id-MulticastCommonConfiguration,

id-BroadcastTransportResourceRequest,

id-SRSInformationReservationNotification,

id-CUDUMobilityInitiationRequest

FROM F1AP-Constants

ProtocolIE-SingleContainer{},

F1AP-PROTOCOL-IES

FROM F1AP-Containers;

-- **************************************************************

--

-- Interface Elementary Procedure Class

--

-- **************************************************************

F1AP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome							OPTIONAL,

&UnsuccessfulOutcome						OPTIONAL,

&procedureCode				ProcedureCode 	UNIQUE,

&criticality				Criticality 	DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE			&InitiatingMessage

[SUCCESSFUL OUTCOME			&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE				&procedureCode

[CRITICALITY				&criticality]

}

-- **************************************************************

--

-- Interface PDU Definition

--

-- **************************************************************

F1AP-PDU ::= CHOICE {

initiatingMessage	InitiatingMessage,

successfulOutcome	SuccessfulOutcome,

unsuccessfulOutcome	UnsuccessfulOutcome,

choice-extension	ProtocolIE-SingleContainer { { F1AP-PDU-ExtIEs} }

}

F1AP-PDU-ExtIEs F1AP-PROTOCOL-IES ::= { -- this extension is not used

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode	F1AP-ELEMENTARY-PROCEDURE.&procedureCode		({F1AP-ELEMENTARY-PROCEDURES}),

criticality		F1AP-ELEMENTARY-PROCEDURE.&criticality			({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			F1AP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode	F1AP-ELEMENTARY-PROCEDURE.&procedureCode		({F1AP-ELEMENTARY-PROCEDURES}),

criticality		F1AP-ELEMENTARY-PROCEDURE.&criticality			({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			F1AP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode	F1AP-ELEMENTARY-PROCEDURE.&procedureCode		({F1AP-ELEMENTARY-PROCEDURES}),

criticality		F1AP-ELEMENTARY-PROCEDURE.&criticality			({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			F1AP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

-- Interface Elementary Procedure List

--

-- **************************************************************

F1AP-ELEMENTARY-PROCEDURES F1AP-ELEMENTARY-PROCEDURE ::= {

F1AP-ELEMENTARY-PROCEDURES-CLASS-1			|

F1AP-ELEMENTARY-PROCEDURES-CLASS-2,

...

}

F1AP-ELEMENTARY-PROCEDURES-CLASS-1 F1AP-ELEMENTARY-PROCEDURE ::= {

reset							|

f1Setup							|

gNBDUConfigurationUpdate		|

gNBCUConfigurationUpdate		|

uEContextSetup					|

uEContextRelease				|

uEContextModification			|

uEContextModificationRequired	|

writeReplaceWarning				|

pWSCancel						|

gNBDUResourceCoordination		|

f1Removal						|

bAPMappingConfiguration			|

gNBDUResourceConfiguration		|

iABTNLAddressAllocation			|

iABUPConfigurationUpdate		|

resourceStatusReportingInitiation	|

positioningMeasurementExchange	|

tRPInformationExchange			|

positioningInformationExchange	|

positioningActivation			|

e-CIDMeasurementInitiation		|

broadcastContextSetup			|

broadcastContextRelease			|

broadcastContextModification	|

multicastContextSetup			|

multicastContextRelease			|

multicastContextModification	|

multicastDistributionSetup		|

multicastDistributionRelease	|

pDCMeasurementInitiation		|

pRSConfigurationExchange		|

measurementPreconfiguration	|

timingSynchronisationStatus	|

multicastContextNotification	|

multicastCommonConfiguration	,

...

}

F1AP-ELEMENTARY-PROCEDURES-CLASS-2 F1AP-ELEMENTARY-PROCEDURE ::= {

errorIndication							|

uEContextReleaseRequest					|

dLRRCMessageTransfer					|

uLRRCMessageTransfer					|

uEInactivityNotification				|

privateMessage							|

initialULRRCMessageTransfer				|

systemInformationDelivery				|

paging									|

notify									|

pWSRestartIndication					|

pWSFailureIndication					|

gNBDUStatusIndication					|

rRCDeliveryReport						|

networkAccessRateReduction				|

traceStart								|

deactivateTrace							|

dUCURadioInformationTransfer			|

cUDURadioInformationTransfer			|

resourceStatusReporting					|

accessAndMobilityIndication				|

referenceTimeInformationReportingControl|

referenceTimeInformationReport			|

accessSuccess							|

cellTrafficTrace						|

positioningAssistanceInformationControl		|

positioningAssistanceInformationFeedback	|

positioningMeasurementReport				|

positioningMeasurementAbort					|

positioningMeasurementFailureIndication		|

positioningMeasurementUpdate				|

positioningDeactivation						|

e-CIDMeasurementFailureIndication			|

e-CIDMeasurementReport						|

e-CIDMeasurementTermination					|

positioningInformationUpdate				|

multicastGroupPaging						|

broadcastContextReleaseRequest				|

multicastContextReleaseRequest				|

pDCMeasurementReport						|

pDCMeasurementTerminationCommand			|

pDCMeasurementFailureIndication				|

measurementActivation						|

qoEInformationTransfer						|

posSystemInformationDelivery					|

dUCUCellSwitchNotification					|

cUDUCellSwitchNotification					|

dUCUTAInformationTransfer					|

cUDUTAInformationTransfer					|

qoEInformationTransferControl					|

rachIndication								|

timingSynchronisationStatusReport				|

mIABF1SetupTriggering						|

mIABF1SetupOutcomeNotification				|

broadcastTransportResourceRequest			|

dUCUAccessAndMobilityIndication			|

sRSInformationReservationNotification		|

cUDUMobilityInitiation,

...

}

-- **************************************************************

--

-- Interface Elementary Procedures

--

-- **************************************************************

reset F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Reset

SUCCESSFUL OUTCOME		ResetAcknowledge

PROCEDURE CODE			id-Reset

CRITICALITY				reject

}

f1Setup F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		F1SetupRequest

SUCCESSFUL OUTCOME		F1SetupResponse

UNSUCCESSFUL OUTCOME	F1SetupFailure

PROCEDURE CODE			id-F1Setup

CRITICALITY				reject

}

gNBDUConfigurationUpdate F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNBDUConfigurationUpdate

SUCCESSFUL OUTCOME		GNBDUConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	GNBDUConfigurationUpdateFailure

PROCEDURE CODE			id-gNBDUConfigurationUpdate

CRITICALITY				reject

}

gNBCUConfigurationUpdate F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNBCUConfigurationUpdate

SUCCESSFUL OUTCOME		GNBCUConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	GNBCUConfigurationUpdateFailure

PROCEDURE CODE			id-gNBCUConfigurationUpdate

CRITICALITY				reject

}

uEContextSetup F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextSetupRequest

SUCCESSFUL OUTCOME		UEContextSetupResponse

UNSUCCESSFUL OUTCOME	UEContextSetupFailure

PROCEDURE CODE			id-UEContextSetup

CRITICALITY				reject

}

uEContextRelease F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextReleaseCommand

SUCCESSFUL OUTCOME		UEContextReleaseComplete

PROCEDURE CODE			id-UEContextRelease

CRITICALITY				reject

}

uEContextModification F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextModificationRequest

SUCCESSFUL OUTCOME		UEContextModificationResponse

UNSUCCESSFUL OUTCOME	UEContextModificationFailure

PROCEDURE CODE			id-UEContextModification

CRITICALITY				reject

}

uEContextModificationRequired F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextModificationRequired

SUCCESSFUL OUTCOME		UEContextModificationConfirm

UNSUCCESSFUL OUTCOME	UEContextModificationRefuse

PROCEDURE CODE			id-UEContextModificationRequired

CRITICALITY				reject

}

writeReplaceWarning F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		WriteReplaceWarningRequest

SUCCESSFUL OUTCOME		WriteReplaceWarningResponse

PROCEDURE CODE			id-WriteReplaceWarning

CRITICALITY				reject

}

pWSCancel F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PWSCancelRequest

SUCCESSFUL OUTCOME		PWSCancelResponse

PROCEDURE CODE			id-PWSCancel

CRITICALITY				reject

}

errorIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-ErrorIndication

CRITICALITY				ignore

}

uEContextReleaseRequest F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextReleaseRequest

PROCEDURE CODE			id-UEContextReleaseRequest

CRITICALITY				ignore

}

initialULRRCMessageTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		InitialULRRCMessageTransfer

PROCEDURE CODE			id-InitialULRRCMessageTransfer

CRITICALITY				ignore

}

dLRRCMessageTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DLRRCMessageTransfer

PROCEDURE CODE			id-DLRRCMessageTransfer

CRITICALITY				ignore

}

uLRRCMessageTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ULRRCMessageTransfer

PROCEDURE CODE			id-ULRRCMessageTransfer

CRITICALITY				ignore

}

uEInactivityNotification  F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEInactivityNotification

PROCEDURE CODE			id-UEInactivityNotification

CRITICALITY				ignore

}

gNBDUResourceCoordination F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNBDUResourceCoordinationRequest

SUCCESSFUL OUTCOME		GNBDUResourceCoordinationResponse

PROCEDURE CODE			id-GNBDUResourceCoordination

CRITICALITY				reject

}

privateMessage F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-privateMessage

CRITICALITY				ignore

}

systemInformationDelivery F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SystemInformationDeliveryCommand

PROCEDURE CODE			id-SystemInformationDeliveryCommand

CRITICALITY				ignore

}

paging F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Paging

PROCEDURE CODE			id-Paging

CRITICALITY				ignore

}

notify F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Notify

PROCEDURE CODE			id-Notify

CRITICALITY				ignore

}

networkAccessRateReduction F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NetworkAccessRateReduction

PROCEDURE CODE			id-NetworkAccessRateReduction

CRITICALITY				ignore

}

pWSRestartIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PWSRestartIndication

PROCEDURE CODE			id-PWSRestartIndication

CRITICALITY				ignore

}

pWSFailureIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PWSFailureIndication

PROCEDURE CODE			id-PWSFailureIndication

CRITICALITY				ignore

}

gNBDUStatusIndication 	F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNBDUStatusIndication

PROCEDURE CODE			id-GNBDUStatusIndication

CRITICALITY				ignore

}

rRCDeliveryReport F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RRCDeliveryReport

PROCEDURE CODE			id-RRCDeliveryReport

CRITICALITY				ignore

}

f1Removal F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		F1RemovalRequest

SUCCESSFUL OUTCOME		F1RemovalResponse

UNSUCCESSFUL OUTCOME	F1RemovalFailure

PROCEDURE CODE			id-F1Removal

CRITICALITY				reject

}

traceStart F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TraceStart

PROCEDURE CODE			id-TraceStart

CRITICALITY				ignore

}

deactivateTrace F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DeactivateTrace

PROCEDURE CODE			id-DeactivateTrace

CRITICALITY				ignore

}

dUCURadioInformationTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DUCURadioInformationTransfer

PROCEDURE CODE			id-DUCURadioInformationTransfer

CRITICALITY				ignore

}

cUDURadioInformationTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CUDURadioInformationTransfer

PROCEDURE CODE			id-CUDURadioInformationTransfer

CRITICALITY				ignore

}

bAPMappingConfiguration F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BAPMappingConfiguration

SUCCESSFUL OUTCOME		BAPMappingConfigurationAcknowledge

UNSUCCESSFUL OUTCOME	BAPMappingConfigurationFailure

PROCEDURE CODE			id-BAPMappingConfiguration

CRITICALITY				reject

}

gNBDUResourceConfiguration F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNBDUResourceConfiguration

SUCCESSFUL OUTCOME		GNBDUResourceConfigurationAcknowledge

UNSUCCESSFUL OUTCOME	GNBDUResourceConfigurationFailure

PROCEDURE CODE			id-GNBDUResourceConfiguration

CRITICALITY				reject

}

iABTNLAddressAllocation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		IABTNLAddressRequest

SUCCESSFUL OUTCOME		IABTNLAddressResponse

UNSUCCESSFUL OUTCOME	IABTNLAddressFailure

PROCEDURE CODE			id-IABTNLAddressAllocation

CRITICALITY				reject

}

iABUPConfigurationUpdate F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		IABUPConfigurationUpdateRequest

SUCCESSFUL OUTCOME		IABUPConfigurationUpdateResponse

UNSUCCESSFUL OUTCOME	IABUPConfigurationUpdateFailure

PROCEDURE CODE			id-IABUPConfigurationUpdate

CRITICALITY				reject

}

resourceStatusReportingInitiation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResourceStatusRequest

SUCCESSFUL OUTCOME		ResourceStatusResponse

UNSUCCESSFUL OUTCOME	ResourceStatusFailure

PROCEDURE CODE			id-resourceStatusReportingInitiation

CRITICALITY				reject

}

resourceStatusReporting F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResourceStatusUpdate

PROCEDURE CODE			id-resourceStatusReporting

CRITICALITY				ignore

}

accessAndMobilityIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AccessAndMobilityIndication

PROCEDURE CODE			id-accessAndMobilityIndication

CRITICALITY				ignore

}

referenceTimeInformationReportingControl F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ReferenceTimeInformationReportingControl

PROCEDURE CODE			id-ReferenceTimeInformationReportingControl

CRITICALITY				ignore

}

referenceTimeInformationReport F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ReferenceTimeInformationReport

PROCEDURE CODE			id-ReferenceTimeInformationReport

CRITICALITY				ignore

}

accessSuccess F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AccessSuccess

PROCEDURE CODE			id-accessSuccess

CRITICALITY				ignore

}

cellTrafficTrace F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CellTrafficTrace

PROCEDURE CODE			id-cellTrafficTrace

CRITICALITY				ignore

}

positioningAssistanceInformationControl F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningAssistanceInformationControl

PROCEDURE CODE			id-PositioningAssistanceInformationControl

CRITICALITY				ignore

}

positioningAssistanceInformationFeedback F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningAssistanceInformationFeedback

PROCEDURE CODE			id-PositioningAssistanceInformationFeedback

CRITICALITY				ignore

}

positioningMeasurementExchange F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningMeasurementRequest

SUCCESSFUL OUTCOME		PositioningMeasurementResponse

UNSUCCESSFUL OUTCOME	PositioningMeasurementFailure

PROCEDURE CODE			id-PositioningMeasurementExchange

CRITICALITY				reject

}

positioningMeasurementReport F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningMeasurementReport

PROCEDURE CODE			id-PositioningMeasurementReport

CRITICALITY				ignore

}

positioningMeasurementAbort F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningMeasurementAbort

PROCEDURE CODE			id-PositioningMeasurementAbort

CRITICALITY				ignore

}

positioningMeasurementFailureIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningMeasurementFailureIndication

PROCEDURE CODE			id-PositioningMeasurementFailureIndication

CRITICALITY				ignore

}

positioningMeasurementUpdate F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningMeasurementUpdate

PROCEDURE CODE			id-PositioningMeasurementUpdate

CRITICALITY				ignore

}

tRPInformationExchange F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TRPInformationRequest

SUCCESSFUL OUTCOME		TRPInformationResponse

UNSUCCESSFUL OUTCOME	TRPInformationFailure

PROCEDURE CODE			id-TRPInformationExchange

CRITICALITY				reject

}

positioningInformationExchange F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningInformationRequest

SUCCESSFUL OUTCOME		PositioningInformationResponse

UNSUCCESSFUL OUTCOME	PositioningInformationFailure

PROCEDURE CODE			id-PositioningInformationExchange

CRITICALITY				reject

}

positioningActivation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningActivationRequest

SUCCESSFUL OUTCOME		PositioningActivationResponse

UNSUCCESSFUL OUTCOME	PositioningActivationFailure

PROCEDURE CODE			id-PositioningActivation

CRITICALITY				reject

}

positioningDeactivation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningDeactivation

PROCEDURE CODE			id-PositioningDeactivation

CRITICALITY				ignore

}

e-CIDMeasurementInitiation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementInitiationRequest

SUCCESSFUL OUTCOME		E-CIDMeasurementInitiationResponse

UNSUCCESSFUL OUTCOME	E-CIDMeasurementInitiationFailure

PROCEDURE CODE			id-E-CIDMeasurementInitiation

CRITICALITY				reject

}

e-CIDMeasurementFailureIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementFailureIndication

PROCEDURE CODE			id-E-CIDMeasurementFailureIndication

CRITICALITY				ignore

}

e-CIDMeasurementReport F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementReport

PROCEDURE CODE			id-E-CIDMeasurementReport

CRITICALITY				ignore

}

e-CIDMeasurementTermination F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementTerminationCommand

PROCEDURE CODE			id-E-CIDMeasurementTermination

CRITICALITY				ignore

}

positioningInformationUpdate F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningInformationUpdate

PROCEDURE CODE			id-PositioningInformationUpdate

CRITICALITY				ignore

}

broadcastContextSetup F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastContextSetupRequest

SUCCESSFUL OUTCOME		BroadcastContextSetupResponse

UNSUCCESSFUL OUTCOME	BroadcastContextSetupFailure

PROCEDURE CODE			id-BroadcastContextSetup

CRITICALITY				reject

}

broadcastContextRelease F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastContextReleaseCommand

SUCCESSFUL OUTCOME		BroadcastContextReleaseComplete

PROCEDURE CODE			id-BroadcastContextRelease

CRITICALITY				reject

}

broadcastContextReleaseRequest F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastContextReleaseRequest

PROCEDURE CODE			id-BroadcastContextReleaseRequest

CRITICALITY				reject

}

broadcastContextModification F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastContextModificationRequest

SUCCESSFUL OUTCOME		BroadcastContextModificationResponse

UNSUCCESSFUL OUTCOME	BroadcastContextModificationFailure

PROCEDURE CODE			id-BroadcastContextModification

CRITICALITY				reject

}

multicastGroupPaging F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastGroupPaging

PROCEDURE CODE			id-MulticastGroupPaging

CRITICALITY				ignore

}

multicastContextSetup F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastContextSetupRequest

SUCCESSFUL OUTCOME		MulticastContextSetupResponse

UNSUCCESSFUL OUTCOME	MulticastContextSetupFailure

PROCEDURE CODE			id-MulticastContextSetup

CRITICALITY				reject

}

multicastContextRelease F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastContextReleaseCommand

SUCCESSFUL OUTCOME		MulticastContextReleaseComplete

PROCEDURE CODE			id-MulticastContextRelease

CRITICALITY				reject

}

multicastContextReleaseRequest F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastContextReleaseRequest

PROCEDURE CODE			id-MulticastContextReleaseRequest

CRITICALITY				reject

}

multicastContextModification F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastContextModificationRequest

SUCCESSFUL OUTCOME		MulticastContextModificationResponse

UNSUCCESSFUL OUTCOME	MulticastContextModificationFailure

PROCEDURE CODE			id-MulticastContextModification

CRITICALITY				reject

}

multicastDistributionSetup F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastDistributionSetupRequest

SUCCESSFUL OUTCOME		MulticastDistributionSetupResponse

UNSUCCESSFUL OUTCOME	MulticastDistributionSetupFailure

PROCEDURE CODE			id-MulticastDistributionSetup

CRITICALITY				reject

}

multicastDistributionRelease F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastDistributionReleaseCommand

SUCCESSFUL OUTCOME		MulticastDistributionReleaseComplete

PROCEDURE CODE			id-MulticastDistributionRelease

CRITICALITY				reject

}

pDCMeasurementInitiation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDCMeasurementInitiationRequest

SUCCESSFUL OUTCOME		PDCMeasurementInitiationResponse

UNSUCCESSFUL OUTCOME	PDCMeasurementInitiationFailure

PROCEDURE CODE			id-PDCMeasurementInitiation

CRITICALITY				reject

}

pDCMeasurementReport F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDCMeasurementReport

PROCEDURE CODE			id-PDCMeasurementReport

CRITICALITY				ignore

}

pDCMeasurementTerminationCommand F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDCMeasurementTerminationCommand

PROCEDURE CODE			id-PDCMeasurementTerminationCommand

CRITICALITY				ignore

}

pDCMeasurementFailureIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDCMeasurementFailureIndication

PROCEDURE CODE			id-PDCMeasurementFailureIndication

CRITICALITY				ignore

}

pRSConfigurationExchange F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PRSConfigurationRequest

SUCCESSFUL OUTCOME		PRSConfigurationResponse

UNSUCCESSFUL OUTCOME	PRSConfigurationFailure

PROCEDURE CODE			id-pRSConfigurationExchange

CRITICALITY				reject

}

measurementPreconfiguration F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementPreconfigurationRequired

SUCCESSFUL OUTCOME		MeasurementPreconfigurationConfirm

UNSUCCESSFUL OUTCOME	MeasurementPreconfigurationRefuse

PROCEDURE CODE			id-measurementPreconfiguration

CRITICALITY				reject

}

measurementActivation		F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementActivation

PROCEDURE CODE			id-measurementActivation

CRITICALITY				ignore

}

qoEInformationTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		QoEInformationTransfer

PROCEDURE CODE			id-QoEInformationTransfer

CRITICALITY				ignore

}

posSystemInformationDelivery F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PosSystemInformationDeliveryCommand

PROCEDURE CODE			id-PosSystemInformationDeliveryCommand

CRITICALITY				ignore

}

dUCUCellSwitchNotification 	F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DUCUCellSwitchNotification

PROCEDURE CODE			id-DUCUCellSwitchNotification

CRITICALITY				ignore

}

cUDUCellSwitchNotification 	F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CUDUCellSwitchNotification

PROCEDURE CODE			id-CUDUCellSwitchNotification

CRITICALITY				ignore

}

dUCUTAInformationTransfer 	F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DUCUTAInformationTransfer

PROCEDURE CODE			id-DUCUTAInformationTransfer

CRITICALITY				ignore

}

cUDUTAInformationTransfer 	F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CUDUTAInformationTransfer

PROCEDURE CODE			id-CUDUTAInformationTransfer

CRITICALITY				ignore

}

qoEInformationTransferControl F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		QoEInformationTransferControl

PROCEDURE CODE			id-QoEInformationTransferControl

CRITICALITY				ignore

}

rachIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RachIndication

PROCEDURE CODE			id-RachIndication

CRITICALITY				ignore

}

timingSynchronisationStatus F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TimingSynchronisationStatusRequest

SUCCESSFUL OUTCOME		TimingSynchronisationStatusResponse

UNSUCCESSFUL OUTCOME	TimingSynchronisationStatusFailure

PROCEDURE CODE			id-TimingSynchronisationStatus

CRITICALITY				reject

}

timingSynchronisationStatusReport F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TimingSynchronisationStatusReport

PROCEDURE CODE			id-TimingSynchronisationStatusReport

CRITICALITY				ignore

}

mIABF1SetupTriggering F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MIABF1SetupTriggering

PROCEDURE CODE			id-MIABF1SetupTriggering

CRITICALITY				ignore

}

mIABF1SetupOutcomeNotification F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MIABF1SetupOutcomeNotification

PROCEDURE CODE			id-MIABF1SetupOutcomeNotification

CRITICALITY				ignore

}

multicastContextNotification F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastContextNotificationIndication

SUCCESSFUL OUTCOME		MulticastContextNotificationConfirm

UNSUCCESSFUL OUTCOME	MulticastContextNotificationRefuse

PROCEDURE CODE			id-MulticastContextNotification

CRITICALITY				reject

}

multicastCommonConfiguration F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastCommonConfigurationRequest

SUCCESSFUL OUTCOME		MulticastCommonConfigurationResponse

UNSUCCESSFUL OUTCOME	MulticastCommonConfigurationRefuse

PROCEDURE CODE			id-MulticastCommonConfiguration

CRITICALITY				reject

}

broadcastTransportResourceRequest F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastTransportResourceRequest

PROCEDURE CODE			id-BroadcastTransportResourceRequest

CRITICALITY				reject

}

dUCUAccessAndMobilityIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DUCUAccessAndMobilityIndication

PROCEDURE CODE			id-DUCUAccessAndMobilityIndication

CRITICALITY				ignore

}

sRSInformationReservationNotification F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SRSInformationReservationNotification

PROCEDURE CODE			id-SRSInformationReservationNotification

CRITICALITY				reject

}

cUDUMobilityInitiation		F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CUDUMobilityInitiationRequest

PROCEDURE CODE			id-CUDUMobilityInitiationRequest

CRITICALITY				ignore

}

END

-- ASN1STOP

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AssociatedSessionID,

BroadcastMRBs-FailedToBeModified-Item,

BroadcastMRBs-FailedToBeSetup-Item,

BroadcastMRBs-FailedToBeSetupMod-Item,

BroadcastMRBs-Modified-Item,

BroadcastMRBs-Setup-Item,

BroadcastMRBs-SetupMod-Item,

BroadcastMRBs-ToBeModified-Item,

BroadcastMRBs-ToBeReleased-Item,

BroadcastMRBs-ToBeSetup-Item,

BroadcastMRBs-ToBeSetupMod-Item,

Candidate-SpCell-Item,

Cause,

Cells-Allowed-to-be-Deactivated-List-Item,

Cells-Failed-to-be-Activated-List-Item,

Cells-Status-Item,

Cells-to-be-Activated-List-Item,

Cells-to-be-Deactivated-List-Item,

CellULConfigured,

CriticalityDiagnostics,

C-RNTI,

CUtoDURRCInformation,

DRB-Activity-Item,

DRBs-FailedToBeModified-Item,

DRBs-FailedToBeSetup-Item,

DRBs-FailedToBeSetupMod-Item,

DRB-Notify-Item,

DRBs-ModifiedConf-Item,

DRBs-Modified-Item,

DRBs-Required-ToBeModified-Item,

DRBs-Required-ToBeReleased-Item,

DRBs-Setup-Item,

DRBs-SetupMod-Item,

DRBs-ToBeModified-Item,

DRBs-ToBeReleased-Item,

DRBs-ToBeSetup-Item,

DRBs-ToBeSetupMod-Item,

DRXCycle,

DRXConfigurationIndicator,

DUtoCURRCInformation,

ExecuteDuplication,

FullConfiguration,

GNB-CU-MBS-F1AP-ID,

GNB-CU-UE-F1AP-ID,

GNB-DU-MBS-F1AP-ID,

GNB-DU-UE-F1AP-ID,

GNB-DU-ID,

GNB-DU-Served-Cells-Item,

GNB-CU-Name,

GNB-DU-Name,

InactivityMonitoringRequest,

InactivityMonitoringResponse,

LowerLayerPresenceStatusChange,

MBS-CUtoDURRCInformation,

MBSMulticastF1UContextDescriptor,

MBS-Session-ID,

MBS-ServiceArea,

MulticastF1UContextReferenceCU,

MulticastF1UContext-ToBeSetup-Item,

MulticastF1UContext-Setup-Item,

MulticastF1UContext-FailedToBeSetup-Item,

MulticastMBSSessionList,

MulticastMRBs-ToBeSetup-Item,

MulticastMRBs-Setup-Item,

MulticastMRBs-FailedToBeSetup-Item,

MulticastMRBs-ToBeSetupMod-Item,

MulticastMRBs-ToBeModified-Item,

MulticastMRBs-ToBeReleased-Item,

MulticastMRBs-SetupMod-Item,

MulticastMRBs-FailedToBeSetupMod-Item,

MulticastMRBs-Modified-Item,

MulticastMRBs-FailedToBeModified-Item,

BroadcastAreaScope,

NetworkControlledRepeaterAuthorized,

NRCGI,

UEContextNotRetrievable,

Potential-SpCell-Item,

RANSharingAssistanceInformation,

RAT-FrequencyPriorityInformation,

RequestedSRSTransmissionCharacteristics,

ResourceCoordinationTransferContainer,

RRCContainer,

RRCContainer-RRCSetupComplete,

RRCReconfigurationCompleteIndicator,

SCell-ToBeRemoved-Item,

SCell-ToBeSetup-Item,

SCell-ToBeSetupMod-Item,

SCell-FailedtoSetup-Item,

SCell-FailedtoSetupMod-Item,

SDT-Volume-Threshold,

ServCellIndex,

Served-Cells-To-Add-Item,

Served-Cells-To-Delete-Item,

Served-Cells-To-Modify-Item,

ServingCellMO,

SNSSAI,

SRBID,

SRBs-FailedToBeSetup-Item,

SRBs-FailedToBeSetupMod-Item,

SRBs-Required-ToBeReleased-Item,

SRBs-ToBeReleased-Item,

SRBs-ToBeSetup-Item,

SRBs-ToBeSetupMod-Item,

SRBs-Modified-Item,

SRBs-Setup-Item,

SRBs-SetupMod-Item,

SupportedUETypeList,

TimeToWait,

TransactionID,

TransmissionActionIndicator,

UE-associatedLogicalF1-ConnectionItem,

UEIdentity-List-For-Paging-Item,

DUtoCURRCContainer,

PagingCell-Item,

SItype-List,

UEIdentityIndexValue,

GNB-CU-TNL-Association-Setup-Item,

GNB-CU-TNL-Association-Failed-To-Setup-Item,

GNB-CU-TNL-Association-To-Add-Item,

GNB-CU-TNL-Association-To-Remove-Item,

GNB-CU-TNL-Association-To-Update-Item,

MaskedIMEISV,

PagingDRX,

PagingPriority,

PagingIdentity,

Cells-to-be-Barred-Item,

PWSSystemInformation,

Broadcast-To-Be-Cancelled-Item,

Cells-Broadcast-Cancelled-Item,

NR-CGI-List-For-Restart-Item,

PWS-Failed-NR-CGI-Item,

RepetitionPeriod,

NumberofBroadcastRequest,

Cells-To-Be-Broadcast-Item,

Cells-Broadcast-Completed-Item,

Cancel-all-Warning-Messages-Indicator,

EUTRA-NR-CellResourceCoordinationReq-Container,

EUTRA-NR-CellResourceCoordinationReqAck-Container,

RequestType,

PLMN-Identity,

RLCFailureIndication,

UplinkTxDirectCurrentListInformation,

SULAccessIndication,

Protected-EUTRA-Resources-Item,

GNB-DUConfigurationQuery,

BitRate,

RRC-Version,

GNBDUOverloadInformation,

RRCDeliveryStatusRequest,

NeedforGap,

RRCDeliveryStatus,

ResourceCoordinationTransferInformation,

Dedicated-SIDelivery-NeededUE-Item,

Associated-SCell-Item,

IgnoreResourceCoordinationContainer,

PagingOrigin,

UAC-Assistance-Info,

RANUEID,

GNB-DU-TNL-Association-To-Remove-Item,

NotificationInformation,

TraceActivation,

TraceID,

Neighbour-Cell-Information-Item,

AdditionalRRMPriorityIndex,

DUCURadioInformationType,

CUDURadioInformationType,

Transport-Layer-Address-Info,

BHChannels-ToBeSetup-Item,

BHChannels-Setup-Item,

BHChannels-FailedToBeSetup-Item,

BHChannels-ToBeModified-Item,

BHChannels-ToBeReleased-Item,

BHChannels-ToBeSetupMod-Item,

BHChannels-FailedToBeModified-Item,

BHChannels-FailedToBeSetupMod-Item,

BHChannels-Modified-Item,

BHChannels-SetupMod-Item,

BHChannels-Required-ToBeReleased-Item,

BAPAddress,

BH-Routing-Information-Added-List-Item,

BH-Routing-Information-Removed-List-Item,

Child-Nodes-List,

Activated-Cells-to-be-Updated-List,

UL-BH-Non-UP-Traffic-Mapping,

IABIPv6RequestType,

IAB-TNL-Addresses-To-Remove-Item,

IABTNLAddress,

IAB-Allocated-TNL-Address-Item,

IABv4AddressesRequested,

TrafficMappingInfo,

UL-UP-TNL-Information-to-Update-List-Item,

UL-UP-TNL-Address-to-Update-List-Item,

DL-UP-TNL-Address-to-Update-List-Item,

NRV2XServicesAuthorized,

LTEV2XServicesAuthorized,

NRUESidelinkAggregateMaximumBitrate,

LTEUESidelinkAggregateMaximumBitrate,

SLDRBs-SetupMod-Item,

SLDRBs-ModifiedConf-Item,

SLDRBs-FailedToBeModified-Item,

SLDRBs-FailedToBeSetup-Item,

SLDRBs-FailedToBeSetupMod-Item,

SLDRBs-Modified-Item,

SLDRBs-Required-ToBeModified-Item,

SLDRBs-Required-ToBeReleased-Item,

SLDRBs-Setup-Item,

SLDRBs-ToBeModified-Item,

SLDRBs-ToBeReleased-Item,

SLDRBs-ToBeSetup-Item,

SLDRBs-ToBeSetupMod-Item,

GNBCUMeasurementID,

GNBDUMeasurementID,

RegistrationRequest,

ReportCharacteristics,

CellToReportList,

HardwareLoadIndicator,

CellMeasurementResultList,

ReportingPeriodicity,

TNLCapacityIndicator,

RAReportList,

RLFReportInformationList,

ReportingRequestType,

TimeReferenceInformation,

ConditionalInterDUMobilityInformation,

ConditionalIntraDUMobilityInformation,

TargetCellList,

MDTPLMNList,

PrivacyIndicator,

TransportLayerAddress,

URI-address,

NID,

PosAssistance-Information,

PosBroadcast,

PositioningBroadcastCells,

RoutingID,

PosAssistanceInformationFailureList,

PosMeasurementQuantities,

PosMeasurementResultList,

PosReportCharacteristics,

TRPInformationTypeItem,

TRPInformationItem,

LMF-MeasurementID,

RAN-MeasurementID,

SDT-Termination-Request,

SRSResourceSetID,

SpatialRelationInfo,

SRSResourceTrigger,

SRSConfiguration,

TRPList,

E-CID-MeasurementQuantities,

MeasurementPeriodicity,

E-CID-MeasurementResult,

Cell-Portion-ID,

LMF-UE-MeasurementID,

RAN-UE-MeasurementID,

RelativeTime1900,

SystemFrameNumber,

SlotNumber,

AbortTransmission,

TRP-MeasurementRequestList,

MeasurementBeamInfoRequest,

E-CID-ReportCharacteristics,

Extended-GNB-CU-Name,

Extended-GNB-DU-Name,

F1CTransferPath,

SCGIndicator,

SpatialRelationPerSRSResource,

MeasurementPeriodicityExtended,

SuccessfulHOReportInformationList,

Coverage-Modification-Notification,

CCO-Assistance-Information,

CellsForSON-List,

IABCongestionIndication,

IABConditionalRRCMessageDeliveryIndication,

F1CTransferPathNRDC,

BufferSizeThresh,

IAB-TNL-Addresses-Exception,

BAP-Header-Rewriting-Added-List-Item,

Re-routingEnableIndicator,

Neighbour-Node-Cells-List,

Serving-Cells-List,

RBSetConfiguration,

PDCMeasurementPeriodicity,

PDCMeasurementQuantities,

PDCMeasurementResult,

PDCReportType,

RAN-UE-PDC-MeasID,

SCGActivationRequest,

SCGActivationStatus,

TRP-MeasurementUpdateList,

PRSTRPList,

PRSTransmissionTRPList,

ResponseTime,

TRP-PRS-Info-List,

PRS-Measurement-Info-List,

PRSConfigRequestType,

MeasurementCharacteristicsRequestIndicator,

MeasurementTimeOccasion,

UEReportingInformation,

PosContextRevIndication,

NRRedCapUEIndication,

NRPagingeDRXInformation,

NRPagingeDRXInformationforRRCINACTIVE,

QoEInformation,

CG-SDTQueryIndication,

CG-SDTKeptIndicator,

CG-SDTSessionInfo,

SDTInformation,

FiveG-ProSeAuthorized,

UuRLCChannelToBeSetupList,

UuRLCChannelToBeModifiedList,

UuRLCChannelToBeReleasedList,

UuRLCChannelSetupList,

UuRLCChannelFailedToBeSetupList,

UuRLCChannelModifiedList,

UuRLCChannelFailedToBeModifiedList,

UuRLCChannelRequiredToBeModifiedList,

UuRLCChannelRequiredToBeReleasedList,

PC5RLCChannelToBeSetupList,

PC5RLCChannelToBeModifiedList,

PC5RLCChannelToBeReleasedList,

PC5RLCChannelSetupList,

PC5RLCChannelFailedToBeSetupList,

PC5RLCChannelFailedToBeModifiedList,

PC5RLCChannelRequiredToBeModifiedList,

PC5RLCChannelRequiredToBeReleasedList,

PC5RLCChannelModifiedList,

RemoteUELocalID,

PathSwitchConfiguration,

SidelinkRelayConfiguration,

PagingCause,

PEIPSAssistanceInfo,

UEPagingCapability,

GNBDUUESliceMaximumBitRateList,

MDTPollutedMeasurementIndicator,

UE-MulticastMRBs-ConfirmedToBeModified-Item,

UE-MulticastMRBs-RequiredToBeModified-Item,

UE-MulticastMRBs-RequiredToBeReleased-Item,

UE-MulticastMRBs-Setup-Item,

UE-MulticastMRBs-Setupnew-Item,

UE-MulticastMRBs-ToBeReleased-Item,

UE-MulticastMRBs-ToBeSetup-Item,

UE-MulticastMRBs-ToBeSetup-atModify-Item,

PosMeasurementAmount,

BAP-Header-Rewriting-Removed-List-Item,

SLDRXCycleList,

MDTPLMNModificationList,

ActivationRequestType,

PosMeasGapPreConfigList,

PosMeasurementPeriodicityNR-AoA,

SRSPosRRCInactiveConfig,

SDTBearerConfigurationQueryIndication,

SDTBearerConfigurationInfo,

ServingCellMO-List-Item,

ServingCellMO-encoded-in-CGC-List,

PosSItypeList,

DAPS-HO-Status,

UuRLCChannelID,

UplinkTxDirectCurrentTwoCarrierListInfo,

SRSPosRRCInactiveQueryIndication,

MC-PagingCell-Item,

UlTxDirectCurrentMoreCarrierInformation,

CPACMCGInformation,

ExtendedUEIdentityIndexValue,

HashedUEIdentityIndexValue,

DedicatedSIDeliveryIndication,

Configured-BWP-List,

MT-SDT-Information,

LTMInformation-Setup,

LTMConfigurationIDMappingList,

LTMInformation-Modify,

LTMCells-ToBeReleased-List,

LTMCFRAResourceConfig-List,

LTMConfiguration,

EarlySyncInformation-Request,

EarlySyncInformation,

EarlySyncCandidateCellInformation-List,

EarlySyncServingCellInformation,

LTMCellSwitchInformation,

DUtoCUTAInformation-List,

CUtoDUTAInformation-List,

DeactivationIndication,

RAReportIndicationList,

SuccessfulPSCellChangeReportInformationList,

PathAdditionInformation,

RANTSSRequestType,

RANTimingSynchronisationStatusInfo,

GlobalGNB-ID,

Activated-Cells-Mapping-List-Item,

RRC-Terminating-IAB-Donor-Related-Info,

NCGI-to-be-Updated-List-Item,

Mobile-IAB-MTUserLocationInformation,

TAI,

IndicationMCInactiveReception,

MulticastCU2DURRCInfo,

MulticastDU2CURRCInfo,

MBSMulticastSessionReceptionState,

MulticastCU2DUCommonRRCInfo,

NRA2XServicesAuthorized,

LTEA2XServicesAuthorized,

NReRedCapUEIndication,

NRPaginglongeDRXInformationforRRCINACTIVE,

Cells-With-SSBs-Activated-List,

Recommended-SSBs-for-Paging-List,

S-CPAC-Configuration,

DLLBTFailureInformationRequest,

DLLBTFailureInformationList,

SLPositioning-Ranging-Service-Info,

TimeWindowInformation-SRS-List,

TimeWindowInformation-Measurement-List,

SRSPosRRCInactiveValidityAreaConfig,

SRSReservationType,

RequestedSRSPreconfigurationCharacteristics-List,

SRSPreconfiguration-List,

Broadcast-MRBs-Transport-Request-Item,

TAInformation-List,

NonIntegerDRXCycle,

AggregatedPosSRSResourceSetList,

F1U-PathFailure,

LTMResetInformation,

MobilityTriggeringIndication,

MobilityAssistanceInfo-CellSwitch,

MobilityAssistanceInfo-EarlyULSync,

MobilityAssistanceInfo-EarlyDLSync,

MobilityAssistanceInfo-Additional

FROM F1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

F1AP-PRIVATE-IES,

F1AP-PROTOCOL-EXTENSION,

F1AP-PROTOCOL-IES,

F1AP-PROTOCOL-IES-PAIR

FROM F1AP-Containers

id-AssociatedSessionID,

id-BroadcastMRBs-FailedToBeModified-List,

id-BroadcastMRBs-FailedToBeModified-Item,

id-BroadcastMRBs-FailedToBeSetup-List,

id-BroadcastMRBs-FailedToBeSetup-Item,

id-BroadcastMRBs-FailedToBeSetupMod-List,

id-BroadcastMRBs-FailedToBeSetupMod-Item,

id-BroadcastMRBs-Modified-List,

id-BroadcastMRBs-Modified-Item,

id-BroadcastMRBs-Setup-List,

id-BroadcastMRBs-Setup-Item,

id-BroadcastMRBs-SetupMod-List,

id-BroadcastMRBs-SetupMod-Item,

id-BroadcastMRBs-ToBeModified-List,

id-BroadcastMRBs-ToBeModified-Item,

id-BroadcastMRBs-ToBeReleased-List,

id-BroadcastMRBs-ToBeReleased-Item,

id-BroadcastMRBs-ToBeSetup-List,

id-BroadcastMRBs-ToBeSetup-Item,

id-BroadcastMRBs-ToBeSetupMod-List,

id-BroadcastMRBs-ToBeSetupMod-Item,

id-Candidate-SpCell-Item,

id-Candidate-SpCell-List,

id-Cause,

id-Cancel-all-Warning-Messages-Indicator,

id-Cells-Failed-to-be-Activated-List,

id-Cells-Failed-to-be-Activated-List-Item,

id-Cells-Status-Item,

id-Cells-Status-List,

id-Cells-to-be-Activated-List,

id-Cells-to-be-Activated-List-Item,

id-Cells-to-be-Deactivated-List,

id-Cells-to-be-Deactivated-List-Item,

id-Cells-Allowed-to-be-Deactivated-List,

id-Cells-Allowed-to-be-Deactivated-List-Item,

id-Cells-With-SSBs-Activated-List,

id-Recommended-SSBs-for-Paging-List,

id-ConfirmedUEID,

id-CriticalityDiagnostics,

id-C-RNTI,

id-CUtoDURRCInformation,

id-DRB-Activity-Item,

id-DRB-Activity-List,

id-DRBs-FailedToBeModified-Item,

id-DRBs-FailedToBeModified-List,

id-DRBs-FailedToBeSetup-Item,

id-DRBs-FailedToBeSetup-List,

id-DRBs-FailedToBeSetupMod-Item,

id-DRBs-FailedToBeSetupMod-List,

id-DRBs-ModifiedConf-Item,

id-DRBs-ModifiedConf-List,

id-DRBs-Modified-Item,

id-DRBs-Modified-List,

id-DRB-Notify-Item,

id-DRB-Notify-List,

id-DRBs-Required-ToBeModified-Item,

id-DRBs-Required-ToBeModified-List,

id-DRBs-Required-ToBeReleased-Item,

id-DRBs-Required-ToBeReleased-List,

id-DRBs-Setup-Item,

id-DRBs-Setup-List,

id-DRBs-SetupMod-Item,

id-DRBs-SetupMod-List,

id-DRBs-ToBeModified-Item,

id-DRBs-ToBeModified-List,

id-DRBs-ToBeReleased-Item,

id-DRBs-ToBeReleased-List,

id-DRBs-ToBeSetup-Item,

id-DRBs-ToBeSetup-List,

id-DRBs-ToBeSetupMod-Item,

id-DRBs-ToBeSetupMod-List,

id-DRXCycle,

id-DUtoCURRCInformation,

id-ExecuteDuplication,

id-FullConfiguration,

id-gNB-CU-MBS-F1AP-ID,

id-gNB-CU-UE-F1AP-ID,

id-gNB-DU-MBS-F1AP-ID,

id-gNB-DU-UE-F1AP-ID,

id-gNB-DU-ID,

id-GNB-DU-Served-Cells-Item,

id-gNB-DU-Served-Cells-List,

id-gNB-CU-Name,

id-gNB-DU-Name,

id-Extended-GNB-CU-Name,

id-Extended-GNB-DU-Name,

id-InactivityMonitoringRequest,

id-InactivityMonitoringResponse,

id-MBS-CUtoDURRCInformation,

id-MBS-Session-ID,

id-MBS-ServiceArea,

id-MBSMulticastF1UContextDescriptor,

id-MC-PagingCell-Item,

id-MC-PagingCell-List,

id-MulticastF1UContextReferenceCU,

id-MulticastMBSSessionSetupList,

id-MulticastMBSSessionRemoveList,

id-MulticastMRBs-FailedToBeModified-List,

id-MulticastMRBs-FailedToBeModified-Item,

id-MulticastMRBs-FailedToBeSetup-List,

id-MulticastMRBs-FailedToBeSetup-Item,

id-MulticastMRBs-FailedToBeSetupMod-List,

id-MulticastMRBs-FailedToBeSetupMod-Item,

id-MulticastMRBs-Modified-List,

id-MulticastMRBs-Modified-Item,

id-MulticastMRBs-Setup-List,

id-MulticastMRBs-Setup-Item,

id-MulticastMRBs-SetupMod-List,

id-MulticastMRBs-SetupMod-Item,

id-MulticastMRBs-ToBeModified-List,

id-MulticastMRBs-ToBeModified-Item,

id-MulticastMRBs-ToBeReleased-List,

id-MulticastMRBs-ToBeReleased-Item,

id-MulticastMRBs-ToBeSetup-List,

id-MulticastMRBs-ToBeSetup-Item,

id-MulticastMRBs-ToBeSetupMod-List,

id-MulticastMRBs-ToBeSetupMod-Item,

id-MulticastF1UContext-ToBeSetup-List,

id-MulticastF1UContext-ToBeSetup-Item,

id-MulticastF1UContext-Setup-List,

id-MulticastF1UContext-Setup-Item,

id-MulticastF1UContext-FailedToBeSetup-List,

id-MulticastF1UContext-FailedToBeSetup-Item,

id-BroadcastAreaScope,

id-new-gNB-CU-UE-F1AP-ID,

id-new-gNB-DU-UE-F1AP-ID,

id-oldgNB-DU-UE-F1AP-ID,

id-PLMNAssistanceInfoForNetShar,

id-Potential-SpCell-Item,

id-Potential-SpCell-List,

id-RAT-FrequencyPriorityInformation,

id-RedirectedRRCmessage,

id-ResetType,

id-RequestedSRSTransmissionCharacteristics,

id-ResourceCoordinationTransferContainer,

id-RRCContainer,

id-RRCContainer-RRCSetupComplete,

id-RRCReconfigurationCompleteIndicator,

id-SCell-FailedtoSetup-List,

id-SCell-FailedtoSetup-Item,

id-SCell-FailedtoSetupMod-List,

id-SCell-FailedtoSetupMod-Item,

id-SCell-ToBeRemoved-Item,

id-SCell-ToBeRemoved-List,

id-SCell-ToBeSetup-Item,

id-SCell-ToBeSetup-List,

id-SCell-ToBeSetupMod-Item,

id-SCell-ToBeSetupMod-List,

id-SDT-Termination-Request,

id-SDT-Volume-Threshold,

id-SelectedPLMNID,

id-Served-Cells-To-Add-Item,

id-Served-Cells-To-Add-List,

id-Served-Cells-To-Delete-Item,

id-Served-Cells-To-Delete-List,

id-Served-Cells-To-Modify-Item,

id-Served-Cells-To-Modify-List,

id-ServCellIndex,

id-ServingCellMO,

id-SNSSAI,

id-SpCell-ID,

id-SpCellULConfigured,

id-SRBID,

id-SRBs-FailedToBeSetup-Item,

id-SRBs-FailedToBeSetup-List,

id-SRBs-FailedToBeSetupMod-Item,

id-SRBs-FailedToBeSetupMod-List,

id-SRBs-Required-ToBeReleased-Item,

id-SRBs-Required-ToBeReleased-List,

id-SRBs-ToBeReleased-Item,

id-SRBs-ToBeReleased-List,

id-SRBs-ToBeSetup-Item,

id-SRBs-ToBeSetup-List,

id-SRBs-ToBeSetupMod-Item,

id-SRBs-ToBeSetupMod-List,

id-SRBs-Modified-Item,

id-SRBs-Modified-List,

id-SRBs-Setup-Item,

id-SRBs-Setup-List,

id-SRBs-SetupMod-Item,

id-SRBs-SetupMod-List,

id-SupportedUETypeList,

id-TimeToWait,

id-TransactionID,

id-TransmissionActionIndicator,

id-UEContextNotRetrievable,

id-UE-associatedLogicalF1-ConnectionItem,

id-UE-associatedLogicalF1-ConnectionListResAck,

id-UEIdentity-List-For-Paging-List,

id-UEIdentity-List-For-Paging-Item,

id-UE-MulticastMRBs-ConfirmedToBeModified-List,

id-UE-MulticastMRBs-ConfirmedToBeModified-Item,

id-UE-MulticastMRBs-RequiredToBeModified-List,

id-UE-MulticastMRBs-RequiredToBeModified-Item,

id-UE-MulticastMRBs-RequiredToBeReleased-List,

id-UE-MulticastMRBs-RequiredToBeReleased-Item,

id-UE-MulticastMRBs-Setup-List,

id-UE-MulticastMRBs-Setup-Item,

id-UE-MulticastMRBs-Setupnew-List,

id-UE-MulticastMRBs-Setupnew-Item,

id-UE-MulticastMRBs-ToBeReleased-List,

id-UE-MulticastMRBs-ToBeReleased-Item,

id-UE-MulticastMRBs-ToBeSetup-atModify-List,

id-UE-MulticastMRBs-ToBeSetup-atModify-Item,

id-UE-MulticastMRBs-ToBeSetup-List,

id-UE-MulticastMRBs-ToBeSetup-Item,

id-DUtoCURRCContainer,

id-NRCGI,

id-PagingCell-Item,

id-PagingCell-List,

id-PagingDRX,

id-PagingPriority,

id-SItype-List,

id-UEIdentityIndexValue,

id-GNB-CU-TNL-Association-Setup-List,

id-GNB-CU-TNL-Association-Setup-Item,

id-GNB-CU-TNL-Association-Failed-To-Setup-List,

id-GNB-CU-TNL-Association-Failed-To-Setup-Item,

id-GNB-CU-TNL-Association-To-Add-Item,

id-GNB-CU-TNL-Association-To-Add-List,

id-GNB-CU-TNL-Association-To-Remove-Item,

id-GNB-CU-TNL-Association-To-Remove-List,

id-GNB-CU-TNL-Association-To-Update-Item,

id-GNB-CU-TNL-Association-To-Update-List,

id-MaskedIMEISV,

id-PagingIdentity,

id-Cells-to-be-Barred-List,

id-Cells-to-be-Barred-Item,

id-PWSSystemInformation,

id-RepetitionPeriod,

id-NumberofBroadcastRequest,

id-Cells-To-Be-Broadcast-List,

id-Cells-To-Be-Broadcast-Item,

id-Cells-Broadcast-Completed-List,

id-Cells-Broadcast-Completed-Item,

id-Broadcast-To-Be-Cancelled-List,

id-Broadcast-To-Be-Cancelled-Item,

id-Cells-Broadcast-Cancelled-List,

id-Cells-Broadcast-Cancelled-Item,

id-NR-CGI-List-For-Restart-List,

id-NR-CGI-List-For-Restart-Item,

id-PWS-Failed-NR-CGI-List,

id-PWS-Failed-NR-CGI-Item,

id-EUTRA-NR-CellResourceCoordinationReq-Container,

id-EUTRA-NR-CellResourceCoordinationReqAck-Container,

id-Protected-EUTRA-Resources-List,

id-RequestType,

id-ServingPLMN,

id-DRXConfigurationIndicator,

id-RLCFailureIndication,

id-UplinkTxDirectCurrentListInformation,

id-SULAccessIndication,

id-Protected-EUTRA-Resources-Item,

id-GNB-DUConfigurationQuery,

id-GNB-DU-UE-AMBR-UL,

id-GNB-CU-RRC-Version,

id-GNB-DU-RRC-Version,

id-GNBDUOverloadInformation,

id-NeedforGap,

id-RRCDeliveryStatusRequest,

id-RRCDeliveryStatus,

id-Dedicated-SIDelivery-NeededUE-List,

id-Dedicated-SIDelivery-NeededUE-Item,

id-ResourceCoordinationTransferInformation,

id-Associated-SCell-List,

id-Associated-SCell-Item,

id-IgnoreResourceCoordinationContainer,

id-UAC-Assistance-Info,

id-RANUEID,

id-PagingOrigin,

id-GNB-DU-TNL-Association-To-Remove-Item,

id-GNB-DU-TNL-Association-To-Remove-List,

id-NotificationInformation,

id-TraceActivation,

id-TraceID,

id-Neighbour-Cell-Information-List,

id-Neighbour-Cell-Information-Item,

id-AdditionalRRMPriorityIndex,

id-DUCURadioInformationType,

id-CUDURadioInformationType,

id-LowerLayerPresenceStatusChange,

id-Transport-Layer-Address-Info,

id-BHChannels-ToBeSetup-List,

id-BHChannels-ToBeSetup-Item,

id-BHChannels-Setup-List,

id-BHChannels-Setup-Item,

id-BHChannels-ToBeModified-Item,

id-BHChannels-ToBeModified-List,

id-BHChannels-ToBeReleased-Item,

id-BHChannels-ToBeReleased-List,

id-BHChannels-ToBeSetupMod-Item,

id-BHChannels-ToBeSetupMod-List,

id-BHChannels-FailedToBeSetup-Item,

id-BHChannels-FailedToBeSetup-List,

id-BHChannels-FailedToBeModified-Item,

id-BHChannels-FailedToBeModified-List,

id-BHChannels-FailedToBeSetupMod-Item,

id-BHChannels-FailedToBeSetupMod-List,

id-BHChannels-Modified-Item,

id-BHChannels-Modified-List,

id-BHChannels-SetupMod-Item,

id-BHChannels-SetupMod-List,

id-BHChannels-Required-ToBeReleased-Item,

id-BHChannels-Required-ToBeReleased-List,

id-BAPAddress,

id-ConfiguredBAPAddress,

id-BH-Routing-Information-Added-List,

id-BH-Routing-Information-Added-List-Item,

id-BH-Routing-Information-Removed-List,

id-BH-Routing-Information-Removed-List-Item,

id-UL-BH-Non-UP-Traffic-Mapping,

id-Child-Nodes-List,

id-Activated-Cells-to-be-Updated-List,

id-IABIPv6RequestType,

id-IAB-TNL-Addresses-To-Remove-List,

id-IAB-TNL-Addresses-To-Remove-Item,

id-IAB-Allocated-TNL-Address-List,

id-IAB-Allocated-TNL-Address-Item,

id-IABv4AddressesRequested,

id-TrafficMappingInformation,

id-UL-UP-TNL-Information-to-Update-List,

id-UL-UP-TNL-Information-to-Update-List-Item,

id-UL-UP-TNL-Address-to-Update-List,

id-UL-UP-TNL-Address-to-Update-List-Item,

id-DL-UP-TNL-Address-to-Update-List,

id-DL-UP-TNL-Address-to-Update-List-Item,

id-NRV2XServicesAuthorized,

id-LTEV2XServicesAuthorized,

id-NRUESidelinkAggregateMaximumBitrate,

id-LTEUESidelinkAggregateMaximumBitrate,

id-PC5LinkAMBR,

id-SLDRBs-FailedToBeModified-Item,

id-SLDRBs-FailedToBeModified-List,

id-SLDRBs-FailedToBeSetup-Item,

id-SLDRBs-FailedToBeSetup-List,

id-SLDRBs-Modified-Item,

id-SLDRBs-Modified-List,

id-SLDRBs-Required-ToBeModified-Item,

id-SLDRBs-Required-ToBeModified-List,

id-SLDRBs-Required-ToBeReleased-Item,

id-SLDRBs-Required-ToBeReleased-List,

id-SLDRBs-Setup-Item,

id-SLDRBs-Setup-List,

id-SLDRBs-ToBeModified-Item,

id-SLDRBs-ToBeModified-List,

id-SLDRBs-ToBeReleased-Item,

id-SLDRBs-ToBeReleased-List,

id-SLDRBs-ToBeSetup-Item,

id-SLDRBs-ToBeSetup-List,

id-SLDRBs-ToBeSetupMod-Item,

id-SLDRBs-ToBeSetupMod-List,

id-SLDRBs-SetupMod-List,

id-SLDRBs-FailedToBeSetupMod-List,

id-SLDRBs-SetupMod-Item,

id-SLDRBs-FailedToBeSetupMod-Item,

id-SLDRBs-ModifiedConf-List,

id-SLDRBs-ModifiedConf-Item,

id-gNBCUMeasurementID,

id-gNBDUMeasurementID,

id-RegistrationRequest,

id-ReportCharacteristics,

id-CellToReportList,

id-CellMeasurementResultList,

id-HardwareLoadIndicator,

id-ReportingPeriodicity,

id-TNLCapacityIndicator,

id-RAReportList,

id-RLFReportInformationList,

id-ReportingRequestType,

id-TimeReferenceInformation,

id-ConditionalInterDUMobilityInformation,

id-ConditionalIntraDUMobilityInformation,

id-targetCellsToCancel,

id-requestedTargetCellGlobalID,

id-TraceCollectionEntityIPAddress,

id-ManagementBasedMDTPLMNList,

id-PrivacyIndicator,

id-TraceCollectionEntityURI,

id-ServingNID,

id-PosAssistance-Information,

id-PosBroadcast,

id-PositioningBroadcastCells,

id-RoutingID,

id-PosAssistanceInformationFailureList,

id-PosMeasurementQuantities,

id-PosMeasurementResultList,

id-PosMeasurementPeriodicity,

id-PosReportCharacteristics,

id-TRPInformationTypeListTRPReq,

id-TRPInformationTypeItem,

id-TRPInformationListTRPResp,

id-TRPInformationItem,

id-LMF-MeasurementID,

id-RAN-MeasurementID,

id-SRSType,

id-ActivationTime,

id-AbortTransmission,

id-SRSConfiguration,

id-TRPList,

id-E-CID-MeasurementQuantities,

id-E-CID-MeasurementPeriodicity,

id-E-CID-MeasurementResult,

id-Cell-Portion-ID,

id-LMF-UE-MeasurementID,

id-RAN-UE-MeasurementID,

id-SFNInitialisationTime,

id-SystemFrameNumber,

id-SlotNumber,

id-TRP-MeasurementRequestList,

id-MeasurementBeamInfoRequest,

id-E-CID-ReportCharacteristics,

id-F1CTransferPath,

id-SCGIndicator,

id-SRSSpatialRelationPerSRSResource,

id-PosMeasurementPeriodicityExtended,

id-SuccessfulHOReportInformationList,

id-Coverage-Modification-Notification,

id-CCO-Assistance-Information,

id-CellsForSON-List,

id-IABCongestionIndication,

id-IABConditionalRRCMessageDeliveryIndication,

id-F1CTransferPathNRDC,

id-BufferSizeThresh,

id-IAB-TNL-Addresses-Exception,

id-BAP-Header-Rewriting-Added-List,

id-BAP-Header-Rewriting-Added-List-Item,

id-Re-routingEnableIndicator,

id-Neighbour-Node-Cells-List,

id-Serving-Cells-List,

id-MDTPollutedMeasurementIndicator,

id-PDCMeasurementPeriodicity,

id-PDCMeasurementQuantities,

id-PDCMeasurementResult,

id-PDCReportType,

id-RAN-UE-PDC-MeasID,

id-SCGActivationRequest,

id-SCGActivationStatus,

id-TRP-MeasurementUpdateList,

id-PRSTRPList,

id-PRSTransmissionTRPList,

id-ResponseTime,

id-TRP-PRS-Info-List,

id-PRS-Measurement-Info-List,

id-PRSConfigRequestType,

id-MeasurementCharacteristicsRequestIndicator,

id-MeasurementTimeOccasion,

id-UEReportingInformation,

id-PosContextRevIndication,

id-NRRedCapUEIndication,

id-RANUEPagingDRX,

id-CNUEPagingDRX,

id-NRPagingeDRXInformation,

id-NRPagingeDRXInformationforRRCINACTIVE,

id-QoEInformation,

id-CG-SDTQueryIndication,

id-CG-SDTKeptIndicator,

id-CG-SDTSessionInfoOld,

id-SDTInformation,

id-FiveG-ProSeAuthorized,

id-FiveG-ProSePC5LinkAMBR,

id-FiveG-ProSeUEPC5AggregateMaximumBitrate,

id-UuRLCChannelToBeSetupList,

id-UuRLCChannelToBeModifiedList,

id-UuRLCChannelToBeReleasedList,

id-UuRLCChannelSetupList,

id-UuRLCChannelFailedToBeSetupList,

id-UuRLCChannelModifiedList,

id-UuRLCChannelFailedToBeModifiedList,

id-UuRLCChannelRequiredToBeModifiedList,

id-UuRLCChannelRequiredToBeReleasedList,

id-PC5RLCChannelToBeSetupList,

id-PC5RLCChannelToBeModifiedList,

id-PC5RLCChannelToBeReleasedList,

id-PC5RLCChannelSetupList,

id-PC5RLCChannelFailedToBeSetupList,

id-PC5RLCChannelModifiedList,

id-PC5RLCChannelFailedToBeModifiedList,

id-PC5RLCChannelRequiredToBeModifiedList,

id-PC5RLCChannelRequiredToBeReleasedList,

id-SidelinkRelayConfiguration,

id-UpdatedRemoteUELocalID,

id-PathSwitchConfiguration,

id-PagingCause,

id-PEIPSAssistanceInfo,

id-UEPagingCapability,

id-GNBDUUESliceMaximumBitRateList,

id-PosMeasurementAmount,

id-BAP-Header-Rewriting-Removed-List,

id-BAP-Header-Rewriting-Removed-List-Item,

id-SLDRXCycleList,

id-ManagementBasedMDTPLMNModificationList,

id-ActivationRequestType,

id-PosMeasGapPreConfigList,

id-PosMeasurementPeriodicityNR-AoA,

id-SRSPosRRCInactiveConfig,

id-SDTBearerConfigurationQueryIndication,

id-SDTBearerConfigurationInfo,

id-ServingCellMO-List,

id-ServingCellMO-List-Item,

id-ServingCellMO-encoded-in-CGC-List,

id-PosSItypeList,

id-DAPS-HO-Status,

id-SRBMappingInfo,

id-UplinkTxDirectCurrentTwoCarrierListInfo,

id-SRSPosRRCInactiveQueryIndication,

id-UlTxDirectCurrentMoreCarrierInformation,

id-CPACMCGInformation,

id-ExtendedUEIdentityIndexValue,

id-HashedUEIdentityIndexValue,

id-DedicatedSIDeliveryIndication,

id-Configured-BWP-List,

id-NetworkControlledRepeaterAuthorized,

id-MT-SDT-Information,

id-LTMInformation-Setup,

id-LTMConfigurationIDMappingList,

id-LTMInformation-Modify,

id-LTMCells-ToBeReleased-List,

id-LTMConfiguration,

id-LTMCFRAResourceConfig-List,

id-EarlySyncInformation-Request,

id-EarlySyncInformation,

id-EarlySyncCandidateCellInformation-List,

id-EarlySyncServingCellInformation,

id-LTMCellSwitchInformation,

id-DUtoCUTAInformation-List,

id-CUtoDUTAInformation-List,

id-DeactivationIndication,

id-RAReportIndicationList,

id-SuccessfulPSCellChangeReportInformationList,

id-PathAdditionInformation,

id-RANTSSRequestType,

id-RANTimingSynchronisationStatusInfo,

id-Target-gNB-ID,

id-Target-gNB-IP-address,

id-Target-SeGW-IP-address,

id-Activated-Cells-Mapping-List,

id-Activated-Cells-Mapping-List-Item,

id-F1SetupOutcome,

id-RRC-Terminating-IAB-Donor-Related-Info,

id-RRC-Terminating-IAB-Donor-gNB-ID,

id-NCGI-to-be-Updated-List,

id-NCGI-to-be-Updated-List-Item,

id-Mobile-IAB-MTUserLocationInformation,

id-IndicationMCInactiveReception,

id-MulticastCU2DURRCInfo,

id-MulticastDU2CURRCInfo,

id-MBSMulticastSessionReceptionState,

id-MulticastCU2DUCommonRRCInfo,

id-NRA2XServicesAuthorized,

id-LTEA2XServicesAuthorized,

id-NRUESidelinkAggregateMaximumBitrateForA2X,

id-LTEUESidelinkAggregateMaximumBitrateForA2X,

id-NReRedCapUEIndication,

id-NRPaginglongeDRXInformationforRRCINACTIVE,

id-Target-F1-Terminating-Donor-gNB-ID,

id-Broadcast-MRBs-Transport-Request-List,

id-Broadcast-MRBs-Transport-Request-Item,

id-S-CPAC-Configuration,

id-DLLBTFailureInformationRequest,

id-DLLBTFailureInformationList,

id-SLPositioning-Ranging-Service-Info,

id-TimeWindowInformation-SRS-List,

id-TimeWindowInformation-Measurement-List,

id-SRSPosRRCInactiveValidityAreaConfig,

id-SRSReservationType,

id-RequestedSRSPreconfigurationCharacteristics-List,

id-SRSPreconfiguration-List,

id-SRSInformation,

id-TAInformation-List,

id-NonIntegerDRXCycle,

id-AggregatedPosSRSResourceSetList,

id-RANSharingAssistanceInformation,

id-F1U-PathFailure,

id-LTMResetInformation,

id-PreconfiguredSRSInformation,

id-MobilityTriggeringIndication,

id-MobilityAssistanceInfo-CellSwitch,

id-MobilityAssistanceInfo-EarlyULSync,

id-MobilityAssistanceInfo-EarlyDLSync,

id-MobilityAssistanceInfo-Additional,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofIndividualF1ConnectionsToReset,

maxnoofPotentialSpCells,

maxnoofSCells,

maxnoofSRBs,

maxnoofPagingCells,

maxnoofTNLAssociations,

maxCellineNB,

maxnoofUEIDs,

maxnoofBHRLCChannels,

maxnoofRoutingEntries,

maxnoofTLAsIAB,

maxnoofULUPTNLInformationforIAB,

maxnoofUPTNLAddresses,

maxnoofSLDRBs,

maxnoofTRPInfoTypes,

maxnoofTRPs,

maxnoofMRBs,

maxnoofUEIDforPaging,

maxnoofMRBsforUE,

maxnoofServingCellMOs

FROM F1AP-Constants;

-- **************************************************************

--

#### -- RESET ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Reset

--

-- **************************************************************

Reset ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ResetIEs} },

...

}

<<<<<< Skip unchanged >>>>>>

-- **************************************************************

--

#### -- Timing Synchronisation Status Reporting Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- TIMING SYNCHRONISATION STATUS REPORT

--

-- **************************************************************

TimingSynchronisationStatusReport::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ TimingSynchronisationStatusReport-IEs}},

...

}

TimingSynchronisationStatusReport-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID									CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-RANTimingSynchronisationStatusInfo				CRITICALITY ignore	TYPE RANTimingSynchronisationStatusInfo							PRESENCE mandatory },

...

}

-- **************************************************************

--

#### --  DU-CU Access And Mobility Indication ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- DU-CU Access And Mobility Indication

--

-- **************************************************************

DUCUAccessAndMobilityIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { DUCUAccessAndMobilityIndicationIEs} },

...

}

DUCUAccessAndMobilityIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory }|

{ ID id-DLLBTFailureInformationList				CRITICALITY ignore	TYPE DLLBTFailureInformationList	PRESENCE optional},

...

}

-- **************************************************************

--

#### -- CU-DU MOBILITY INITIATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- CU-DU Mobility Initiation Request

--

-- **************************************************************

CUDUMobilityInitiationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ CUDUMobilityInitiationRequestIEs }},

...

}

CUDUMobilityInitiationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-MobilityTriggeringIndication			CRITICALITY reject	TYPE MobilityTriggeringIndication				PRESENCE mandatory}|

{ ID id-MobilityAssistanceInfo-CellSwitch		CRITICALITY ignore	TYPE MobilityAssistanceInfo-CellSwitch		PRESENCE conditional}|

{ ID id-MobilityAssistanceInfo-EarlyULSync		CRITICALITY ignore	TYPE MobilityAssistanceInfo-EarlyULSync		PRESENCE conditional}|

{ ID id-MobilityAssistanceInfo-EarlyDLSync		CRITICALITY ignore	TYPE MobilityAssistanceInfo-EarlyDLSync		PRESENCE conditional}|

{ ID id-MobilityAssistanceInfo-Additional		CRITICALITY ignore	TYPE MobilityAssistanceInfo-Additional			PRESENCE optional	},

...

}

END

-- ASN1STOP

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-gNB-CUSystemInformation,

id-HandoverPreparationInformation,

id-TAISliceSupportList,

id-RANAC,

id-BearerTypeChange,

id-Coverage-Modification-Cause,

id-Cell-Direction,

id-Cell-Type,

id-CellGroupConfig,

id-AvailablePLMNList,

id-PDUSessionID,

id-ULPDUSessionAggregateMaximumBitRate,

id-DC-Based-Duplication-Configured,

id-DC-Based-Duplication-Activation,

id-Duplication-Activation,

id-DLPDCPSNLength,

id-ULPDCPSNLength,

id-RLC-Status,

id-MeasurementTimingConfiguration,

id-DRB-Information,

id-QoSFlowMappingIndication,

id-ServingCellMO,

id-RLCMode,

id-ExtendedServedPLMNs-List,

id-ExtendedAvailablePLMN-List,

id-DRX-LongCycleStartOffset,

id-SelectedBandCombinationIndex,

id-SelectedFeatureSetEntryIndex,

id-Ph-InfoSCG,

id-latest-RRC-Version-Enhanced,

id-RequestedBandCombinationIndex,

id-RequestedFeatureSetEntryIndex,

id-DRX-Config,

id-UEAssistanceInformation,

id-PDCCH-BlindDetectionSCG,

id-Requested-PDCCH-BlindDetectionSCG,

id-BPLMN-ID-Info-List,

id-NotificationInformation,

id-TNLAssociationTransportLayerAddressgNBDU,

id-portNumber,

id-AdditionalSIBMessageList,

id-IgnorePRACHConfiguration,

id-CG-Config,

id-Ph-InfoMCG,

id-AggressorgNBSetID,

id-VictimgNBSetID,

id-MeasGapSharingConfig,

id-systemInformationAreaID,

id-areaScope,

id-IntendedTDD-DL-ULConfig,

id-QosMonitoringRequest,

id-BHInfo,

id-IAB-Info-IAB-DU,

id-IAB-Info-IAB-donor-CU,

id-IAB-Barred,

id-SIB12-message,

id-SIB13-message,

id-SIB14-message,

id-UEAssistanceInformationEUTRA,

id-SL-PHY-MAC-RLC-Config,

id-SL-ConfigDedicatedEUTRA-Info,

id-AlternativeQoSParaSetList,

id-CurrentQoSParaSetIndex,

id-CarrierList,

id-ULCarrierList,

id-FrequencyShift7p5khz,

id-SSB-PositionsInBurst,

id-NRPRACHConfig,

id-TDD-UL-DLConfigCommonNR,

id-CNPacketDelayBudgetDownlink,

id-CNPacketDelayBudgetUplink,

id-ExtendedPacketDelayBudget,

id-TSCTrafficCharacteristics,

id-AdditionalPDCPDuplicationTNL-List,

id-RLCDuplicationInformation,

id-AdditionalDuplicationIndication,

id-mdtConfiguration,

id-TraceCollectionEntityURI,

id-NID,

id-NPNSupportInfo,

id-NPNBroadcastInformation,

id-AvailableSNPN-ID-List,

id-SIB10-message,

id-RequestedP-MaxFR2,

id-DLCarrierList,

id-ExtendedTAISliceSupportList,

id-E-CID-MeasurementQuantities-Item,

id-ConfiguredTACIndication,

id-NRCGI,

id-SFN-Offset,

id-TransmissionStopIndicator,

id-SrsFrequency,

id-EstimatedArrivalProbability,

id-Supported-MBS-FSA-ID-List,

id-TRPType,

id-SRSSpatialRelationPerSRSResource,

id-MBS-Broadcast-NeighbourCellList,

id-PDCPTerminatingNodeDLTNLAddrInfo,

id-ENBDLTNLAddress,

id-PRS-Resource-ID,

id-LocationMeasurementInformation,

id-SliceRadioResourceStatus,

id-CompositeAvailableCapacity-SUL,

id-NR-U,

id-NR-U-Channel-List,

id-MIMOPRBusageInformation,

id-IngressNonF1terminatingTopologyIndicator,

id-NonF1terminatingTopologyIndicator,

id-EgressNonF1terminatingTopologyIndicator,

id-rBSetConfiguration,

id-frequency-Domain-HSNA-Configuration-List,

id-child-IAB-Nodes-NA-Resource-List,

id-Parent-IAB-Nodes-NA-Resource-Configuration-List,

id-uL-FreqInfo,

id-uL-Transmission-Bandwidth,

id-dL-FreqInfo,

id-dL-Transmission-Bandwidth,

id-uL-NR-Carrier-List,

id-dL-NR-Carrier-List,

id-nRFreqInfo,

id-transmission-Bandwidth,

id-nR-Carrier-List,

id-permutation,

id-M5ReportAmount,

id-M6ReportAmount,

id-M7ReportAmount,

id-SurvivalTime,

id-PDCMeasurementQuantities-Item,

id-OnDemandPRS,

id-AoA-SearchWindow,

id-ZoAInformation,

id-ARPLocationInfo,

id-ARP-ID,

id-MultipleULAoA,

id-UL-SRS-RSRPP,

id-SRSResourcetype,

id-ExtendedAdditionalPathList,

id-LoS-NLoSInformation,

id-NumberOfTRPRxTEG,

id-NumberOfTRPRxTxTEG,

id-TRPTxTEGAssociation,

id-TRPTEGInformation,

id-TRPRx-TEGInformation,

id-TRPBeamAntennaInformation,

id-Redcap-Bcast-Information,

id-NR-TADV,

id-SDT-MAC-PHY-CG-Config,

id-CG-SDTindicatorSetup,

id-CG-SDTindicatorMod,

id-SDTRLCBearerConfiguration,

id-SRBMappingInfo,

id-DRBMappingInfo,

id-LastUsedCellIndication,

id-Recommended-SSBs-List,

id-SSBs-withinTheCell-tobe-Activated-List,

id-SIB17-message,

id-MUSIM-GapConfig,

id-SIB20-message,

id-pathPower,

id-DU-RX-MT-RX-Extend,

id-DU-TX-MT-TX-Extend,

id-DU-RX-MT-TX-Extend,

id-DU-TX-MT-RX-Extend,

id-TAINSAGSupportList,

id-SL-RLC-ChannelToAddModList,

id-SIB15-message,

id-InterFrequencyConfig-NoGap,

id-MBSInterestIndication,

id-L571Info,

id-L1151Info,

id-SCS-480,

id-SCS-960,

id-SRSPortIndex,

id-PEISubgroupingSupportIndication,

id-NeedForGapsInfoNR,

id-NeedForGapNCSGInfoNR,

id-NeedForGapNCSGInfoEUTRA,

id-Source-MRB-ID,

id-RedCapIndication,

id-UL-GapFR2-Config,

id-ConfigRestrictInfoDAPS,

id-MulticastF1UContextReferenceCU,

id-TwoPHRModeMCG,

id-TwoPHRModeSCG,

id-ncd-SSB-RedCapInitialBWP-SDT,

id-nrofSymbolsExtended,

id-repetitionFactorExtended,

id-startRBHopping,

id-startRBIndex,

id-transmissionCombn8,

id-ServCellInfoList,

id-Preconfigured-measurement-GAP-Request,

id-BWP-Id,

id-ExtendedResourceSymbolOffset,

id-MusimCapabilityRestrictionIndication,

id-duplicationIndication,

id-dRB-List,

id-ChannelOccupancyTimePercentageUL,

id-RadioResourceStatusNR-U,

id-FiveG-ProSeLayer2Multipath,

id-FiveG-ProSeLayer2UEtoUERelay,

id-FiveG-ProSeLayer2UEtoUERemote,

id-TSCTrafficCharacteristicsFeedback,

id-RANfeedbacktype,

id-Mobile-TRP-LocationInformation,

id-Mobile-IAB-MT-UE-ID,

id-MobileAccessPointLocation,

id-SIB24-message,

id-PDUSetQoSParameters,

id-N6JitterInformation,

id-ECNMarkingorCongestionInformationReportingRequest,

id-ECNMarkingorCongestionInformationReportingStatus,

id-ERedcap-Bcast-Information,

id-NeedForInterruptionInfoNR,

id-SCPAC-Request,

id-MobileIAB-Barred,

id-F1UTunnelNotEstablished,

id-S-CPACLowerLayerReferenceConfigRequest,

id-MusimCandidateBandList,

id-PSIbasedSDUdiscardUL,

id-SIB22-message,

id-U2URLCChannelQoS,

id-SL-PHY-MAC-RLC-ConfigExt,

id-UL-RSCP,

id-BW-Aggregation-Request-Indication,

id-ReportingGranularitykminus1,

id-ReportingGranularitykminus1additionalpath,

id-ReportingGranularitykminus2,

id-ReportingGranularitykminus2additionalpath,

id-ReportingGranularitykminus3,

id-ReportingGranularitykminus3additionalpath,

id-ReportingGranularitykminus4,

id-ReportingGranularitykminus4additionalpath,

id-ReportingGranularitykminus5,

id-ReportingGranularitykminus5additionalpath,

id-ReportingGranularitykminus6,

id-ReportingGranularitykminus6additionalpath,

id-TimingReportingGranularityFactorExtended,

id-PosValidityAreaCellList,

id-SymbolIndex,

id-AggregatedPosSRSResourceIDList,

id-PhaseQuality,

id-PRSBWAggregationRequestInfoList,

id-AggregatedPRSResourceSetList,

id-MeasuredFrequencyHops,

id-TxHoppingConfiguration,

id-AggregatedPosSRSResourceSetList,

id-ValidityAreaSpecificSRSInformation,

id-PeerUE-ID,

id-MeasBasedOnAggregatedResources,

id-SIB23-message,

id-PointA,

id-SCS-SpecificCarrier,

id-NR-PCI,

id-E-CID-MeasuredResultsAssociatedInfoList,

id-XR-Bcast-Information,

id-MaxDataBurstVolume,

id-BarringExemptionforEmerCallInfo,

id-SIB17bis-message,

id-ReportingIntervalIMs,

id-Transmission-Bandwidth-asymmetric,

id-TagIDPointer,

id-LocalOrigin,

id-SRSPosPeriodicConfigHyperSFNIndex,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

maxnoofBPLMNsNR,

maxnoofDLUPTNLInformation,

maxnoofNrCellBands,

maxnoofULUPTNLInformation,

maxnoofQoSFlows,

maxnoofSliceItems,

maxnoofSIBTypes,

maxnoofSITypes,

maxCellineNB,

maxnoofExtendedBPLMNs,

maxnoofAdditionalSIBs,

maxnoofUACPLMNs,

maxnoofUACperPLMN,

maxCellingNBDU,

maxnoofTLAs,

maxnoofGTPTLAs,

maxnoofslots,

maxnoofNonUPTrafficMappings,

maxnoofServingCells,

maxnoofServedCellsIAB,

maxnoofChildIABNodes,

maxnoofIABSTCInfo,

maxnoofDUFSlots,

maxnoofHSNASlots,

maxnoofEgressLinks,

maxnoofMappingEntries,

maxnoofDSInfo,

maxnoofQoSParaSets,

maxnoofPC5QoSFlows,

maxnoofSSBAreas,

maxnoofNRSCSs,

maxnoofPhysicalResourceBlocks,

maxnoofPhysicalResourceBlocks-1,

maxnoofPRACHconfigs,

maxnoofRAReports,

maxnoofRLFReports,

maxnoofAdditionalPDCPDuplicationTNL,

maxnoofRLCDuplicationState,

maxnoofCHOcells,

maxnoofMDTPLMNs,

maxnoofCAGsupported,

maxnoofNIDsupported,

maxnoofExtSliceItems,

maxnoofPosMeas,

maxnoofTRPInfoTypes,

maxnoofSRSTriggerStates,

maxnoofSpatialRelations,

maxnoBcastCell,

maxnoofTRPs,

maxnooflcs-gcs-translation,

maxnoofPath,

maxnoofMeasE-CID,

maxnoofSSBs,

maxnoSRS-ResourceSets,

maxnoSRS-ResourcePerSet,

maxnoSRS-Carriers,

maxnoSCSs,

maxnoSRS-Resources,

maxnoSRS-PosResources,

maxnoSRS-PosResourceSets,

maxnoSRS-PosResourcePerSet,

maxnoofPRS-ResourceSets,

maxnoofPRS-ResourcesPerSet,

maxNoOfMeasTRPs,

maxnoofPRSresourceSets,

maxnoofPRSresources,

maxnoofSuccessfulHOReports,

maxnoofNR-UChannelIDs,

maxServedCellforSON,

maxNeighbourCellforSON,

maxAffectedCells,

maxnoofMBSQoSFlows,

maxnoofMBSFSAs,

maxnoofMBSAreaSessionIDs,

maxnoofMBSServiceAreaInformation,

maxnoofTAIforMBS,

maxnoofCellsforMBS,

maxnoofIABCongInd,

maxnoofBHRLCChannels,

maxnoofTLAsIAB,

maxnoofRBsetsPerCell,

maxnoofRBsetsPerCell-1,

maxnoofNeighbourNodeCellsIAB,

maxnoofMeasPDC,

maxnoARPs,

maxnoofULAoAs,

maxNoPathExtended,

maxnoTRPTEGs,

maxFreqLayers,

maxNumResourcesPerAngle,

maxnoAzimuthAngles,

maxnoElevationAngles,

maxnoofPRSTRPs,

maxnoofQoEInformation,

maxnoofUuRLCChannels,

maxnoofPC5RLCChannels,

maxnoofSMBRValues,

maxnoofMBSSessionsofUE,

maxnoofSLdestinations,

maxnoofNSAGs,

maxnoofSDTBearers,

maxnoofPosSITypes,

maxnoofMRBs,

maxNrofBWPs,

maxnoofUETypes,

maxnoofLTMCells,

maxnoofLTMgNB-DUs,

maxnoofTAList,

maxnoofDRBs,

maxnoofUEsInQMCTransferControlMessage,

maxnoofUEsforRAReportIndications,

maxnoofSuccessfulPSCellChangeReports,

maxnoofPeriodicities,

maxnoofThresholdMBS-1,

maxMBSSessionsinSessionInfoList,

maxnoofLBTFailureInformation,

maxnoofRSPPQoSFlows,

maxnoVACell,

maxnoAggregatedSRS-Resources,

maxnoAggregatedPosSRSResourceSets,

maxnoAggregatedPosPRSResourceSets,

maxnoofTimeWindowSRS,

maxnoofTimeWindowMea,

maxnoPreconfiguredSRS,

maxnoHopsMinusOne,

maxnoAggCombinations,

maxnoAggregatedPosSRSCombinations,

maxnoofCandidateCells,

maxnoofSSBIndices

<<<<<< Skip unchanged >>>>>>

#### -- C

CAGID ::= BIT STRING (SIZE(32))

Cancel-all-Warning-Messages-Indicator ::= ENUMERATED {true, ...}

Candidate-SpCell-Item ::= SEQUENCE {

candidate-SpCell-ID			NRCGI	,

iE-Extensions	ProtocolExtensionContainer { { Candidate-SpCell-ItemExtIEs } }	OPTIONAL,

...

}

Candidate-SpCell-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

CandidateCellwithBeamInfo	::= SEQUENCE {

nRCGI				NRCGI,

sSBIndex			SSBIndex,

iE-Extensions		ProtocolExtensionContainer { { CandidateCellwithBeamInfo-ExtIEs } }	OPTIONAL

}

CandidateCellwithBeamInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CandidateCellwithBeamInfoList ::= SEQUENCE (SIZE(1..maxnoofCandidateCells)) OF CandidateCellwithBeamInfo-Item

CandidateCellwithBeamInfo-Item ::= SEQUENCE {

nRCGI				NRCGI,

sSBIndexList		SSBIndexList,

iE-Extensions		ProtocolExtensionContainer { { CandidateCellwithBeamInfo-Item-ExtIEs } }	OPTIONAL

}

CandidateCellwithBeamInfo-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CandidateCellwithMeasurementsList ::= SEQUENCE (SIZE(1..maxnoofCandidateCells)) OF CandidateCellwithMeasurements-Item

CandidateCellwithMeasurements-Item	::= SEQUENCE {

nRCGI				NRCGI,

sSBIndexwithMeasurementsList		SSBIndexwithMeasurementsList,

iE-Extensions		ProtocolExtensionContainer { { CandidateCellwithMeasurements-Item-ExtIEs } }	OPTIONAL

}

CandidateCellwithMeasurements-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CapacityValue::= SEQUENCE {

capacityValue				INTEGER (0..100),

sSBAreaCapacityValueList	SSBAreaCapacityValueList		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { CapacityValue-ExtIEs} } OPTIONAL

}

CapacityValue-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cause ::= CHOICE {

radioNetwork		CauseRadioNetwork,

transport			CauseTransport,

protocol			CauseProtocol,

misc				CauseMisc,

choice-extension	ProtocolIE-SingleContainer { { Cause-ExtIEs} }

}

Cause-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

CauseMisc ::= ENUMERATED {

control-processing-overload,

not-enough-user-plane-processing-resources,

hardware-failure,

om-intervention,

unspecified,

...

}

CauseProtocol ::= ENUMERATED {

transfer-syntax-error,

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

abstract-syntax-error-falsely-constructed-message,

unspecified,

...

}

CauseRadioNetwork ::= ENUMERATED {

unspecified,

rl-failure-rlc,

unknown-or-already-allocated-gnb-cu-ue-f1ap-id,

unknown-or-already-allocated-gnb-du-ue-f1ap-id,

unknown-or-inconsistent-pair-of-ue-f1ap-id,

interaction-with-other-procedure,

not-supported-qci-Value,

action-desirable-for-radio-reasons,

no-radio-resources-available,

procedure-cancelled,

normal-release,

...,

cell-not-available,

rl-failure-others,

ue-rejection,

resources-not-available-for-the-slice,

amf-initiated-abnormal-release,

release-due-to-pre-emption,

plmn-not-served-by-the-gNB-CU,

multiple-drb-id-instances,

unknown-drb-id,

multiple-bh-rlc-ch-id-instances,

unknown-bh-rlc-ch-id,

cho-cpc-resources-tobechanged,

nPN-not-supported,

nPN-access-denied,

gNB-CU-Cell-Capacity-Exceeded,

report-characteristics-empty,

existing-measurement-ID,

measurement-temporarily-not-available,

measurement-not-supported-for-the-object,

unknown-bh-address,

unknown-bap-routing-id,

insufficient-ue-capabilities,

scg-activation-deactivation-failure,

scg-deactivation-failure-due-to-data-transmission,

requested-item-not-supported-on-time,

unknown-or-already-allocated-gNB-CU-MBS-F1AP-ID,

unknown-or-already-allocated-gNB-DU-MBS-F1AP-ID,

unknown-or-inconsistent-pair-of-MBS-F1AP-ID,

unknown-or-inconsistent-MRB-ID,

tat-sdt-expiry,

lTM-command-triggered,

sSB-not-available

}

CauseTransport ::= ENUMERATED {

unspecified,

transport-resource-unavailable,

...,

unknown-TNL-address-for-IAB,

unknown-UP-TNL-information-for-IAB

}

CellGroupConfig ::= OCTET STRING

CellCapacityClassValue ::= INTEGER (1..100,...)

Cell-Direction ::= ENUMERATED {dl-only, ul-only}

CellMeasurementResultList ::= SEQUENCE (SIZE(1.. maxCellingNBDU)) OF CellMeasurementResultItem

CellMeasurementResultItem ::= SEQUENCE {

cellID							NRCGI,

radioResourceStatus				RadioResourceStatus 			OPTIONAL,

compositeAvailableCapacityGroup	CompositeAvailableCapacityGroup	OPTIONAL,

sliceAvailableCapacity			SliceAvailableCapacity 			OPTIONAL,

numberofActiveUEs 				NumberofActiveUEs				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CellMeasurementResultItem-ExtIEs} } OPTIONAL

}

CellMeasurementResultItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-NR-U-Channel-List	CRITICALITY ignore	EXTENSION NR-U-Channel-List PRESENCE optional },

...

}

Cell-Portion-ID ::= INTEGER (0..4095,...)

CellsForSON-List ::= SEQUENCE (SIZE(1.. maxServedCellforSON)) OF CellsForSON-Item

CellsForSON-Item ::= SEQUENCE {

nRCGI											NRCGI,

neighbourNR-CellsForSON-List					NeighbourNR-CellsForSON-List									OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { { CellsForSON-Item-ExtIEs} }	OPTIONAL,

...

}

CellsForSON-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-Failed-to-be-Activated-List-Item ::= SEQUENCE {

nRCGI				NRCGI,

cause				Cause,

iE-Extensions		ProtocolExtensionContainer { { Cells-Failed-to-be-Activated-List-ItemExtIEs } }	OPTIONAL,

...

}

Cells-Failed-to-be-Activated-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-Status-Item ::= SEQUENCE {

nRCGI			NRCGI,

service-status		Service-Status,

iE-Extensions				ProtocolExtensionContainer { { Cells-Status-ItemExtIEs } }	OPTIONAL,

...

}

Cells-Status-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-To-Be-Broadcast-Item ::= SEQUENCE {

nRCGI				NRCGI,

iE-Extensions		ProtocolExtensionContainer { { Cells-To-Be-Broadcast-ItemExtIEs } }	OPTIONAL,

...

}

Cells-To-Be-Broadcast-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-Broadcast-Completed-Item ::= SEQUENCE {

nRCGI				NRCGI,

iE-Extensions		ProtocolExtensionContainer { { Cells-Broadcast-Completed-ItemExtIEs } }	OPTIONAL,

...

}

Cells-Broadcast-Completed-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Broadcast-To-Be-Cancelled-Item ::= SEQUENCE {

nRCGI				NRCGI,

iE-Extensions		ProtocolExtensionContainer { { Broadcast-To-Be-Cancelled-ItemExtIEs } }	OPTIONAL,

...

}

Broadcast-To-Be-Cancelled-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-Broadcast-Cancelled-Item ::= SEQUENCE {

nRCGI				NRCGI,

numberOfBroadcasts	NumberOfBroadcasts,

iE-Extensions		ProtocolExtensionContainer { { Cells-Broadcast-Cancelled-ItemExtIEs } }	OPTIONAL,

...

}

Cells-Broadcast-Cancelled-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-to-be-Activated-List-Item ::= SEQUENCE {

nRCGI		NRCGI,

nRPCI		NRPCI		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { Cells-to-be-Activated-List-ItemExtIEs} }	OPTIONAL,

...

}

Cells-to-be-Activated-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-gNB-CUSystemInformation			CRITICALITY reject	EXTENSION GNB-CUSystemInformation				PRESENCE optional }|

{ ID id-AvailablePLMNList				CRITICALITY ignore	EXTENSION AvailablePLMNList						PRESENCE optional }|

{ ID id-ExtendedAvailablePLMN-List		CRITICALITY ignore	EXTENSION ExtendedAvailablePLMN-List			PRESENCE optional }|

{ ID id-IAB-Info-IAB-donor-CU			CRITICALITY ignore	EXTENSION IAB-Info-IAB-donor-CU					PRESENCE optional}|

{ ID id-AvailableSNPN-ID-List			CRITICALITY ignore	EXTENSION AvailableSNPN-ID-List					PRESENCE optional }|

{ ID id-MBS-Broadcast-NeighbourCellList	CRITICALITY ignore	EXTENSION MBS-Broadcast-NeighbourCellList	PRESENCE optional }|

{ ID id-SSBs-withinTheCell-tobe-Activated-List			CRITICALITY reject	EXTENSION SSBs-toBeActivated-List				PRESENCE optional },

...

}

Cells-With-SSBs-Activated-List ::= SEQUENCE (SIZE(1.. maxCellingNBDU)) OF Cells-With-SSBs-Activated-List-Item

Cells-With-SSBs-Activated-List-Item::= SEQUENCE {

nRCGI			 			NRCGI,

sSBs-activated-List 		SSBs-activated-List,

iE-Extensions				ProtocolExtensionContainer { { Cells-With-SSBs-Activated-List-Item-ExtIEs} } OPTIONAL

}

Cells-With-SSBs-Activated-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-Allowed-to-be-Deactivated-List-Item ::= SEQUENCE {

nRCGI		NRCGI,

iE-Extensions				ProtocolExtensionContainer { { Cells-Allowed-to-be-Deactivated-List-ItemExtIEs} }	OPTIONAL,

...

}

Cells-Allowed-to-be-Deactivated-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-to-be-Deactivated-List-Item ::= SEQUENCE {

nRCGI			NRCGI	,

iE-Extensions				ProtocolExtensionContainer { { Cells-to-be-Deactivated-List-ItemExtIEs } }	OPTIONAL,

...

}

Cells-to-be-Deactivated-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-to-be-Barred-Item::= SEQUENCE {

nRCGI			NRCGI	,

cellBarred		CellBarred,

iE-Extensions				ProtocolExtensionContainer { { Cells-to-be-Barred-Item-ExtIEs } }	OPTIONAL

}

Cells-to-be-Barred-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-IAB-Barred	CRITICALITY ignore	EXTENSION IAB-Barred		PRESENCE optional }|

{ ID id-MobileIAB-Barred	CRITICALITY ignore	EXTENSION MobileIAB-Barred		PRESENCE optional },

...

}

CellBarred	::=	ENUMERATED {barred, not-barred, ...}

CellSize ::= ENUMERATED {verysmall, small, medium, large, ...}

CellToReportList ::= SEQUENCE (SIZE(1.. maxCellingNBDU)) OF CellToReportItem

CellToReportItem ::= SEQUENCE {

cellID		NRCGI,

sSBToReportList		SSBToReportList		 OPTIONAL,

sliceToReportList	SliceToReportList	 OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { CellToReportItem-ExtIEs} } OPTIONAL

}

CellToReportItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

CellType ::= SEQUENCE {

cellSize		CellSize,

iE-Extensions		ProtocolExtensionContainer { {CellType-ExtIEs} }	OPTIONAL,

...

}

CellType-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CellULConfigured ::=  ENUMERATED {none, ul, sul, ul-and-sul, ...}

CG-SDTQueryIndication ::=  ENUMERATED {true, ...}

CG-SDTKeptIndicator ::= ENUMERATED {true, ...}

CG-SDTindicatorSetup ::= ENUMERATED {true, ...}

CG-SDTindicatorMod ::= ENUMERATED {true, false, ...}

CG-SDTSessionInfo ::= SEQUENCE {

gNB-CU-UE-F1AP-ID					GNB-CU-UE-F1AP-ID,

gNB-DU-UE-F1AP-ID					GNB-DU-UE-F1AP-ID,

iE-Extensions						ProtocolExtensionContainer {{CG-SDTSessionInfo-ExtIEs}}		OPTIONAL,

...

}

CG-SDTSessionInfo-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

ChannelOccupancyTimePercentage ::= INTEGER (0..100,...)

Child-IAB-Nodes-NA-Resource-List ::= SEQUENCE (SIZE(1..maxnoofChildIABNodes)) OF Child-IAB-Nodes-NA-Resource-List-Item

Child-IAB-Nodes-NA-Resource-List-Item::= SEQUENCE {

gNB-CU-UE-F1AP-ID	GNB-CU-UE-F1AP-ID,

gNB-DU-UE-F1AP-ID	GNB-DU-UE-F1AP-ID,

nA-Resource-Configuration-List		NA-Resource-Configuration-List 		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { Child-IAB-Nodes-NA-Resource-List-Item-ExtIEs} } OPTIONAL

}

Child-IAB-Nodes-NA-Resource-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Child-Node-Cells-List ::= SEQUENCE (SIZE(1..maxnoofChildIABNodes)) OF Child-Node-Cells-List-Item

Child-Node-Cells-List-Item ::=	SEQUENCE{

nRCGI 								NRCGI,

iAB-DU-Cell-Resource-Configuration-Mode-Info 	IAB-DU-Cell-Resource-Configuration-Mode-Info	OPTIONAL,

iAB-STC-Info						IAB-STC-Info			OPTIONAL,

rACH-Config-Common					RACH-Config-Common		OPTIONAL,

rACH-Config-Common-IAB				RACH-Config-Common-IAB	OPTIONAL,

cSI-RS-Configuration				OCTET STRING			OPTIONAL,

sR-Configuration					OCTET STRING			OPTIONAL,

pDCCH-ConfigSIB1					OCTET STRING			OPTIONAL,

sCS-Common							OCTET STRING	OPTIONAL,

multiplexingInfo					MultiplexingInfo	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer {{Child-Node-Cells-List-Item-ExtIEs}}		OPTIONAL

}

Child-Node-Cells-List-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Child-Nodes-List ::= SEQUENCE (SIZE(1..maxnoofChildIABNodes)) OF Child-Nodes-List-Item

Child-Nodes-List-Item ::= SEQUENCE{

gNB-CU-UE-F1AP-ID	GNB-CU-UE-F1AP-ID,

gNB-DU-UE-F1AP-ID	GNB-DU-UE-F1AP-ID,

child-Node-Cells-List 	Child-Node-Cells-List	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer {{Child-Nodes-List-Item-ExtIEs}}		OPTIONAL

}

Child-Nodes-List-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

CHOtrigger-InterDU ::= ENUMERATED {

cho-initiation,

cho-replace,

...

}

CHOtrigger-IntraDU ::= ENUMERATED {

cho-initiation,

cho-replace,

cho-cancel,

...

}

CNSubgroupID ::= INTEGER (0..7, ...)

CNUEPagingIdentity ::= CHOICE {

fiveG-S-TMSI			BIT STRING (SIZE(48)),

choice-extension			ProtocolIE-SingleContainer { { CNUEPagingIdentity-ExtIEs } }

}

CNUEPagingIdentity-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

CompositeAvailableCapacityGroup ::= SEQUENCE {

compositeAvailableCapacityDownlink	CompositeAvailableCapacity,

compositeAvailableCapacityUplink 	CompositeAvailableCapacity,

iE-Extensions	ProtocolExtensionContainer { { CompositeAvailableCapacityGroup-ExtIEs} } OPTIONAL

}

CompositeAvailableCapacityGroup-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-CompositeAvailableCapacity-SUL		CRITICALITY ignore	EXTENSION CompositeAvailableCapacity		PRESENCE optional	},

...

}

CompositeAvailableCapacity ::= SEQUENCE {

cellCapacityClassValue 	CellCapacityClassValue		OPTIONAL,

capacityValue			CapacityValue,

iE-Extensions	ProtocolExtensionContainer { { CompositeAvailableCapacity-ExtIEs} } OPTIONAL

}

CompositeAvailableCapacity-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

CHO-Probability ::= INTEGER (1..100)

ConditionalInterDUMobilityInformation ::= SEQUENCE {

cho-trigger						CHOtrigger-InterDU,

targetgNB-DUUEF1APID			GNB-DU-UE-F1AP-ID							OPTIONAL

-- The above IE shall be present if the cho-trigger IE is present and set to "cho-replace" --,

iE-Extensions					ProtocolExtensionContainer { { ConditionalInterDUMobilityInformation-ExtIEs} }	OPTIONAL,

...

}

ConditionalInterDUMobilityInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

{ ID id-EstimatedArrivalProbability		CRITICALITY ignore		EXTENSION CHO-Probability				PRESENCE optional	}|

{ ID id-SCPAC-Request					CRITICALITY reject		EXTENSION SCPAC-Request			PRESENCE optional	}|

{ ID id-S-CPACLowerLayerReferenceConfigRequest	CRITICALITY reject		EXTENSION S-CPACLowerLayerReferenceConfigRequest		PRESENCE optional	},

...

}

ConditionalIntraDUMobilityInformation ::= SEQUENCE {

cho-trigger						CHOtrigger-IntraDU,

targetCellsTocancel				TargetCellList								OPTIONAL,

-- The above IE shall be present if the cho-trigger IE is present and set to "cho-cancel"

iE-Extensions					ProtocolExtensionContainer { { ConditionalIntraDUMobilityInformation-ExtIEs} }	OPTIONAL,

...

}

ConditionalIntraDUMobilityInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

{ ID id-EstimatedArrivalProbability		CRITICALITY ignore		EXTENSION CHO-Probability				PRESENCE optional	}|

{ ID id-SCPAC-Request					CRITICALITY reject		EXTENSION SCPAC-Request			PRESENCE optional	}|

{ ID id-S-CPACLowerLayerReferenceConfigRequest	CRITICALITY reject		EXTENSION S-CPACLowerLayerReferenceConfigRequest		PRESENCE optional	},

...

}

ConfigRestrictInfoDAPS ::= OCTET STRING

ConfiguredTACIndication ::= ENUMERATED {

true,

...

}

Configured-BWP-List ::= SEQUENCE (SIZE(1.. maxNrofBWPs)) OF Configured-BWP-Item

Configured-BWP-Item ::= SEQUENCE {

bWP-Id									BWP-Id,

bWP-Location-and-bandwidth				INTEGER (0..37949),

iE-Extensions					ProtocolExtensionContainer { { Configured-BWP-Item-ExtIEs } }	OPTIONAL,

...

}

Configured-BWP-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

CoordinateID ::= INTEGER (0..511, ...)

Coverage-Modification-Notification ::= SEQUENCE {

coverage-Modification-List				Coverage-Modification-List,

iE-Extensions								ProtocolExtensionContainer { { Coverage-Modification-Notification-ExtIEs} }	OPTIONAL,

...

}

Coverage-Modification-Notification-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

...

}

Coverage-Modification-List ::= SEQUENCE (SIZE (1..maxCellingNBDU)) OF Coverage-Modification-Item

Coverage-Modification-Item ::= SEQUENCE {

nRCGI								NRCGI,

cellCoverageState					CellCoverageState,

sSBCoverageModificationList			SSBCoverageModification-List OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { Coverage-Modification-Item-ExtIEs} } 				OPTIONAL,

...

}

Coverage-Modification-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-Coverage-Modification-Cause	CRITICALITY ignore	EXTENSION CCO-issue-detection		PRESENCE optional },

...

}

CellCoverageState ::= INTEGER (0..63, ...)

CCO-Assistance-Information ::= SEQUENCE {

cCO-issue-detection				CCO-issue-detection		OPTIONAL,

affectedCellsAndBeams-List		AffectedCellsAndBeams-List 		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CCO-Assistance-Information-ExtIEs} }	OPTIONAL,

...

}

CCO-Assistance-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

...

}

CCO-issue-detection	::=	ENUMERATED {

coverage,

cell-edge-capacity,

...,

network-energy-saving}

CP-TransportLayerAddress ::= CHOICE {

endpoint-IP-address				TransportLayerAddress,

endpoint-IP-address-and-port	Endpoint-IP-address-and-port,

choice-extension				ProtocolIE-SingleContainer { { CP-TransportLayerAddress-ExtIEs } }

}

CP-TransportLayerAddress-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

CPACMCGInformation ::= SEQUENCE {

cpac-trigger					CPAC-trigger,

pscellid						NRCGI,

iE-Extensions					ProtocolExtensionContainer { { CPACMCGInformation-ExtIEs} }	OPTIONAL,

...

}

CPACMCGInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CPAC-trigger ::= ENUMERATED {

cpac-preparation,

cpac-executed,

...

}

CPTrafficType ::= INTEGER (1..3,...)

CriticalityDiagnostics ::= SEQUENCE {

procedureCode					ProcedureCode														OPTIONAL,

triggeringMessage				TriggeringMessage													OPTIONAL,

procedureCriticality			Criticality															OPTIONAL,

transactionID					TransactionID														OPTIONAL,

iEsCriticalityDiagnostics		CriticalityDiagnostics-IE-List										OPTIONAL,

iE-Extensions					ProtocolExtensionContainer {{CriticalityDiagnostics-ExtIEs}}		OPTIONAL,

...

}

CriticalityDiagnostics-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CriticalityDiagnostics-IE-List ::= SEQUENCE (SIZE (1.. maxnoofErrors)) OF CriticalityDiagnostics-IE-Item

CriticalityDiagnostics-IE-Item ::= SEQUENCE {

iECriticality			Criticality,

iE-ID					ProtocolIE-ID,

typeOfError 			TypeOfError,

iE-Extensions			ProtocolExtensionContainer {{CriticalityDiagnostics-IE-Item-ExtIEs}}	OPTIONAL,

...

}

CriticalityDiagnostics-IE-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

C-RNTI ::= INTEGER (0..65535, ...)

CUDURadioInformationType ::= CHOICE {

rIM								CUDURIMInformation,

choice-extension				ProtocolIE-SingleContainer { { CUDURadioInformationType-ExtIEs} }

}

CUDURadioInformationType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

CUDURIMInformation ::= SEQUENCE {

victimgNBSetID			GNBSetID,

rIMRSDetectionStatus	RIMRSDetectionStatus,

iE-Extensions			ProtocolExtensionContainer { { CUDURIMInformation-ExtIEs} }	OPTIONAL

}

CUDURIMInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CUtoDURRCInformation ::= SEQUENCE {

cG-ConfigInfo						CG-ConfigInfo						OPTIONAL,

uE-CapabilityRAT-ContainerList		UE-CapabilityRAT-ContainerList		OPTIONAL,

measConfig							MeasConfig							OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { CUtoDURRCInformation-ExtIEs} } OPTIONAL,

...

}

CUtoDURRCInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-HandoverPreparationInformation	CRITICALITY ignore	EXTENSION HandoverPreparationInformation		PRESENCE optional }|

{ ID id-CellGroupConfig					CRITICALITY ignore	EXTENSION CellGroupConfig							PRESENCE optional }|

{ ID id-MeasurementTimingConfiguration	CRITICALITY ignore	EXTENSION MeasurementTimingConfiguration		PRESENCE optional }|

{ ID id-UEAssistanceInformation			CRITICALITY ignore	EXTENSION UEAssistanceInformation					PRESENCE optional }|

{ ID id-CG-Config						CRITICALITY ignore	EXTENSION CG-Config									PRESENCE optional }|

{ ID id-UEAssistanceInformationEUTRA	CRITICALITY ignore	EXTENSION UEAssistanceInformationEUTRA			PRESENCE optional }|

{ ID id-LocationMeasurementInformation	CRITICALITY ignore	EXTENSION LocationMeasurementInformation		PRESENCE optional }|

{ ID id-MUSIM-GapConfig					CRITICALITY reject	EXTENSION MUSIM-GapConfig							PRESENCE optional }|

{ ID id-SDT-MAC-PHY-CG-Config			CRITICALITY ignore	EXTENSION SDT-MAC-PHY-CG-Config						PRESENCE optional }|

{ ID id-MBSInterestIndication			CRITICALITY ignore	EXTENSION MBSInterestIndication						PRESENCE optional }|

{ ID id-NeedForGapsInfoNR				CRITICALITY ignore	EXTENSION NeedForGapsInfoNR							PRESENCE optional }|

{ ID id-NeedForGapNCSGInfoNR			CRITICALITY ignore	EXTENSION NeedForGapNCSGInfoNR						PRESENCE optional }|

{ ID id-NeedForGapNCSGInfoEUTRA			CRITICALITY ignore	EXTENSION NeedForGapNCSGInfoEUTRA					PRESENCE optional }|

{ ID id-ConfigRestrictInfoDAPS			CRITICALITY ignore	EXTENSION ConfigRestrictInfoDAPS					PRESENCE optional }|

{ ID id-Preconfigured-measurement-GAP-Request			CRITICALITY ignore	EXTENSION Preconfigured-measurement-GAP-Request					PRESENCE optional }|

{ ID id-NeedForInterruptionInfoNR		CRITICALITY ignore	EXTENSION NeedForInterruptionInfoNR				PRESENCE optional }|

{ ID id-MusimCapabilityRestrictionIndication		CRITICALITY ignore	EXTENSION MusimCapabilityRestrictionIndication				PRESENCE optional }|

{ ID id-MusimCandidateBandList			CRITICALITY ignore	EXTENSION MusimCandidateBandList					PRESENCE optional },

...

}

CUtoDUTAInformation-List ::= SEQUENCE (SIZE(1.. maxnoofTAList)) OF CUtoDUTAInformation-Item

CUtoDUTAInformation-Item	::= SEQUENCE {

nRCGI					NRCGI,

tAValue					TAValue,

preambleIndex			PreambleIndex,

rA-RNTI					RA-RNTI,

tagIDPointer			TagIDPointer		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { CUtoDUTAInformation-Item-ExtIEs} }	OPTIONAL,

...

}

CUtoDUTAInformation-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CSIResourceConfiguration ::= SEQUENCE  {

cSIResourceConfigToAddModList 			OCTET STRING		OPTIONAL,

cSIResourceConfigToReleaseList			OCTET STRING		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { CSIResourceConfiguration-ExtIEs} }	OPTIONAL

}

CSIResourceConfiguration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- D

<<<<<< Skip unchanged >>>>>>

#### -- M

MappingInformationIndex	::= BIT STRING (SIZE (26))

MappingInformationtoRemove	::= SEQUENCE (SIZE(1..maxnoofMappingEntries)) OF MappingInformationIndex

MaskedIMEISV ::= 	BIT STRING (SIZE (64))

MaxDataBurstVolume  ::= INTEGER (0..4095, ..., 4096.. 2000000)

MaxPacketLossRate ::= INTEGER (0..1000)

MBS-Broadcast-NeighbourCellList ::= OCTET STRING

MBS-Flows-Mapped-To-MRB-List	::=	SEQUENCE (SIZE(1.. maxnoofMBSQoSFlows)) OF MBS-Flows-Mapped-To-MRB-Item

MBS-Flows-Mapped-To-MRB-Item 	::= SEQUENCE {

mBS-QoSFlowIdentifier						QoSFlowIdentifier,

mbs-QoSFlowLevelQoSParameters				QoSFlowLevelQoSParameters,

iE-Extensions								ProtocolExtensionContainer { { MBS-Flows-Mapped-To-MRB-Item-ExtIEs} } OPTIONAL

}

MBS-Flows-Mapped-To-MRB-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSF1UInformation ::= SEQUENCE {

mbs-f1u-info				UPTransportLayerInformation,

iE-Extensions					ProtocolExtensionContainer	{ { MBSF1UInformation-ExtIEs } }	OPTIONAL,

...

}

MBSF1UInformation-ExtIEs		F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-F1UTunnelNotEstablished	CRITICALITY		ignore		EXTENSION		F1UTunnelNotEstablished		PRESENCE		optional	},

...

}

MBSInterestIndication ::= OCTET STRING

MBS-Session-ID ::= SEQUENCE {

tMGI						TMGI,

nID							NID					OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MBS-Session-ID-ExtIEs} } OPTIONAL,

...

}

MBS-Session-ID-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-Area-Session-ID  ::= INTEGER (0..65535, ...)

MBS-CUtoDURRCInformation		::= SEQUENCE {

mBS-Broadcast-Cell-List		MBS-Broadcast-Cell-List,

mBS-Broadcast-MRB-List		MBS-Broadcast-MRB-List,

iE-Extensions				ProtocolExtensionContainer { { MBS-CUtoDURRCInformation-ExtIEs } } OPTIONAL,

...

}

MBS-CUtoDURRCInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-Broadcast-Cell-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF  MBS-Broadcast-Cell-Item

MBS-Broadcast-Cell-Item ::= SEQUENCE {

nRCGI						NRCGI,

mtch-neighbourCell			OCTET STRING		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MBS-Broadcast-Cell-Item-ExtIEs} } OPTIONAL,

...

}

MBS-Broadcast-Cell-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-Broadcast-MRB-List	::= SEQUENCE (SIZE(1.. maxnoofMRBs))	OF  MBS-Broadcast-MRB-Item

MBS-Broadcast-MRB-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

mRB-PDCP-Config-Broadcast	OCTET STRING,

iE-Extensions				ProtocolExtensionContainer { { MBS-Broadcast-MRB-Item-ExtIEs} } OPTIONAL,

...

}

MBS-Broadcast-MRB-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSMulticastF1UContextDescriptor ::= SEQUENCE {

multicastF1UContextReferenceF1		MulticastF1UContextReferenceF1,

mc-F1UCtxtusage 	ENUMERATED {ptm, ptp, ptp-retransmission, ptp-forwarding, ...},

mbsAreaSession						MBS-Area-Session-ID				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer	{{MBSMulticastF1UContextDescriptor-ExtIEs}} OPTIONAL,

...

}

MBSMulticastF1UContextDescriptor-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MT-SDT-Information ::= SEQUENCE {

mt-SDT-Indicator	MT-SDT-Indicator,

iE-Extensions		ProtocolExtensionContainer { { MT-SDT-Information-ExtIEs } } OPTIONAL,

...

}

MT-SDT-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MT-SDT-Indicator ::= ENUMERATED {true, ...}

MBSMulticastSessionReceptionState ::= ENUMERATED {start-monitoring-G-RNTI, stop-monitoring-G-RNTI, ...}

MulticastCU2DURRCInfo		::= SEQUENCE {

mBS-Multicast-CU2DU-Cell-List		MBS-Multicast-CU2DU-Cell-List 	OPTIONAL,

mBS-Multicast-MRB-List		MBS-Multicast-MRB-List 					OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MulticastCU2DURRCInfo-ExtIEs } } OPTIONAL,

...

}

MulticastCU2DURRCInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-Multicast-CU2DU-Cell-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF  MBS-Multicast-CU2DU-Cell-Item

MBS-Multicast-CU2DU-Cell-Item ::= SEQUENCE {

nRCGI									NRCGI,

mbsMulticastRRC-INACTIVEReceptionMode	MBSMulticastRRCINACTIVEReceptionMode	OPTIONAL,

mbsMulticastConfigurationRequest		ENUMERATED {query, ...}					OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MBS-Multicast-CU2DU-Cell-Item-ExtIEs} } OPTIONAL,

...

}

MBS-Multicast-CU2DU-Cell-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSMulticastRRCINACTIVEReceptionMode ::= ENUMERATED {activated, deactivated, ...}

MBS-Multicast-MRB-List	::= SEQUENCE (SIZE(1.. maxnoofMRBs))	OF  MBS-Multicast-MRB-Item

MBS-Multicast-MRB-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

mRB-PDCP-Config-Broadcast	OCTET STRING,

iE-Extensions				ProtocolExtensionContainer { { MBS-Multicast-MRB-Item-ExtIEs} } OPTIONAL,

...

}

MBS-Multicast-MRB-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastCU2DUCommonRRCInfo ::= SEQUENCE {

multicastCommonCU2DUCellList		MulticastCommonCU2DUCellList		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {MulticastCU2DUCommonRRCInfo-ExtIEs} } OPTIONAL,

...

}

MulticastCU2DUCommonRRCInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastCommonCU2DUCellList ::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF MulticastCommonCU2DUCell-Item

MulticastCommonCU2DUCell-Item ::= SEQUENCE {

nRCGI									NRCGI,

multicastCommonCu2DUCellInformation		MulticastCommonCu2DUCellInformation,

iE-Extensions				ProtocolExtensionContainer { {MulticastCommonCU2DUCell-Item-ExtIEs} } OPTIONAL,

...

}

MulticastCommonCU2DUCell-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastCommonCu2DUCellInformation ::= SEQUENCE {

mBSMulticastNeighbourCellListItem	MBSMulticastNeighbourCellListItem		OPTIONAL,

thresholdMBS-ListItem				ThresholdMBS-ListItem					OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {MulticastCommonCu2DUCellInformation-ExtIEs} } OPTIONAL,

...

}

MulticastCommonCu2DUCellInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSMulticastNeighbourCellListItem ::= CHOICE {

mbsMulticastNeighbourCellListInformationprovided		UpdateMBSMulticastNeighbourCellListInformation,

nombsMulticastNeighbourCellListInformationprovided		NULL,

choice-extension		ProtocolIE-SingleContainer { {MBSMulticastNeighbourCellListItem-ExtIEs} }

}

MBSMulticastNeighbourCellListItem-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

ThresholdMBS-ListItem ::= CHOICE {

thresholdMBS-ListInformationprovided		UpdateThresholdMBS-ListInformation,

nothresholdMBSListInformationprovided		NULL,

choice-extension		ProtocolIE-SingleContainer { {ThresholdMBS-ListItem-ExtIEs} }

}

ThresholdMBS-ListItem-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

UpdateMBSMulticastNeighbourCellListInformation ::= SEQUENCE {

mbs-NeighbourCellList			OCTET STRING						OPTIONAL,

mbs-MulticastSessionList		MTCH-NeighbourCellSessionList		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {UpdateMBSMulticastNeighbourCellListInformation-ExtIEs} } OPTIONAL,

...

}

UpdateMBSMulticastNeighbourCellListInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MTCH-NeighbourCellSessionList ::= SEQUENCE (SIZE(1..maxMBSSessionsinSessionInfoList))	OF MTCH-NeighbourCellSession-Item

MTCH-NeighbourCellSession-Item ::= SEQUENCE {

mbsSessionID						MBS-Session-ID,

mtch-NeighbourCellInformation			MTCH-NeighbourCellInformation,

iE-Extensions				ProtocolExtensionContainer { {MTCH-NeighbourCellSession-Item-ExtIEs} } OPTIONAL,

...

}

MTCH-NeighbourCellSession-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MTCH-NeighbourCellInformation ::= CHOICE {

mtch-NeighbourCellprovided					OCTET STRING,

mtch-NeighbourCellnotprovided				NULL,

choice-extension		ProtocolIE-SingleContainer { {MTCH-NeighbourCellInformation-ExtIEs} }

}

MTCH-NeighbourCellInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

UpdateThresholdMBS-ListInformation ::= SEQUENCE {

thresholdMBSList				OCTET STRING				OPTIONAL,

thresholdIndexSessionList		ThresholdIndexSessionList	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {UpdateThresholdMBS-ListInformation-ExtIEs} } OPTIONAL,

...

}

UpdateThresholdMBS-ListInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ThresholdIndexSessionList ::= SEQUENCE (SIZE(1..maxMBSSessionsinSessionInfoList))	OF ThresholdIndexSession-Item

ThresholdIndexSession-Item ::= SEQUENCE {

mbsSessionID						MBS-Session-ID,

thresholdIndexInformation			ThresholdIndexInformation,

iE-Extensions				ProtocolExtensionContainer { {ThresholdIndexSession-Item-ExtIEs} } OPTIONAL,

...

}

ThresholdIndexSession-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ThresholdIndexInformation ::= CHOICE {

thresholdIndexprovided				ThresholdIndex,

thresholdIndexnotprovided			NULL,

choice-extension		ProtocolIE-SingleContainer { {ThresholdIndexInformation-ExtIEs} }

}

ThresholdIndexInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

ThresholdIndex ::= INTEGER (0..maxnoofThresholdMBS-1)

MulticastDU2CURRCInfo		::= SEQUENCE {

mBS-Multicast-DU2CU-Cell-List		MBS-Multicast-DU2CU-Cell-List			OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MulticastDU2CURRCInfo-ExtIEs } } OPTIONAL,

...

}

MulticastDU2CURRCInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-Multicast-DU2CU-Cell-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF  MBS-Multicast-DU2CU-Cell-Item

MBS-Multicast-DU2CU-Cell-Item ::= SEQUENCE {

nRCGI									NRCGI,

mbsMulticastConfigurationResponseInfo	MBSMulticastConfigurationResponseInfo		OPTIONAL,

mbsMulticastConfigurationNotification	MBSMulticastConfigurationNotification								OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MBS-Multicast-DU2CU-Cell-Item-ExtIEs} } OPTIONAL,

...

}

MBS-Multicast-DU2CU-Cell-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSMulticastConfigurationResponseInfo ::= CHOICE {

mbsMulticastConfiguration-available			MBSMulticastConfiguration-available,

mbsMulticastConfiguration-notavailable		MBSMulticastConfiguration-notavailable,

choice-extension		ProtocolIE-SingleContainer { {MBSMulticastConfigurationResponseInfo-ExtIEs} }

}

MBSMulticastConfigurationResponseInfo-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

MBSMulticastConfiguration-available ::= SEQUENCE {

mBSMulticastConfiguration				OCTET STRING,

iE-Extensions				ProtocolExtensionContainer { { MBSMulticastConfiguration-available-ExtIEs} } OPTIONAL,

...

}

MBSMulticastConfiguration-available-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSMulticastConfiguration-notavailable ::= SEQUENCE {

mBSMulticastConfiguration-notavailable			ENUMERATED {not-available, ...},

iE-Extensions				ProtocolExtensionContainer { { MBSMulticastConfiguration-notavailable-ExtIEs} } OPTIONAL,

...

}

MBSMulticastConfiguration-notavailable-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSMulticastConfigurationNotification ::= SEQUENCE {

mbsMulticastConfigurationNotificationInfo			MBSMulticastConfigurationNotificationInfo		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBSMulticastConfigurationNotification-ExtIEs} }	OPTIONAL,

...

}

MBSMulticastConfigurationNotification-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSMulticastConfigurationNotificationInfo ::= CHOICE {

mbsMulticastConfigurationChanged			OCTET STRING,

mbsMulticastConfigurationRemoved			NULL,

choice-extension		ProtocolIE-SingleContainer { {MBSMulticastConfigurationNotificationInfo-ExtIEs} }

}

MBSMulticastConfigurationNotificationInfo-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

MulticastF1UContext-ToBeSetup-Item ::= SEQUENCE {

mRB-ID                  MRB-ID,

mbs-f1u-info-at-DU      UPTransportLayerInformation,

mbsProgressInformation		MRB-ProgressInformation						OPTIONAL,

-- The above IE shall be present if the MC F1-U Context usage IE in the MBS Multicast F1-U Context Descriptor IE is set to "ptp forwarding".

iE-Extensions           ProtocolExtensionContainer { {MulticastF1UContext-ToBeSetup-Item-ExtIEs} }	OPTIONAL,

...

}

MulticastF1UContext-ToBeSetup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastF1UContext-Setup-Item ::= SEQUENCE {

mRB-ID                  MRB-ID,

mbs-f1u-info-at-CU      UPTransportLayerInformation,

iE-Extensions           ProtocolExtensionContainer { {MulticastF1UContext-Setup-Item-ExtIEs} }	OPTIONAL,

...

}

MulticastF1UContext-Setup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastF1UContext-FailedToBeSetup-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

cause						Cause							OPTIONAL,

iE-Extensions           ProtocolExtensionContainer { {MulticastF1UContext-FailedToBeSetup-Item-ExtIEs} }	OPTIONAL,

...

}

MulticastF1UContext-FailedToBeSetup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSPTPRetransmissionTunnelRequired ::= ENUMERATED {true,	...}

MBS-ServiceArea ::= CHOICE {

locationindependent		MBS-ServiceAreaInformation,

locationdependent		MBS-ServiceAreaInformationList,

choice-Extensions		ProtocolIE-SingleContainer { {MBSServiceArea-ExtIEs} }

}

MBSServiceArea-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

MBS-ServiceAreaInformation ::= SEQUENCE {

mBS-ServiceAreaCellList		MBS-ServiceAreaCellList																OPTIONAL,

mBS-ServiceAreaTAIList		MBS-ServiceAreaTAIList																OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBS-ServiceAreaInformation-ExtIEs} }	OPTIONAL,

...

}

MBS-ServiceAreaInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-ServiceAreaCellList ::= SEQUENCE (SIZE(1.. maxnoofCellsforMBS)) OF NRCGI

MBS-ServiceAreaTAIList ::= SEQUENCE (SIZE(1.. maxnoofTAIforMBS)) OF MBS-ServiceAreaTAIList-Item

MBS-ServiceAreaTAIList-Item ::= SEQUENCE {

plmn-ID							PLMN-Identity,

fiveGS-TAC						FiveGS-TAC,

iE-Extensions					ProtocolExtensionContainer { {MBS-ServiceAreaTAIList-Item-ExtIEs} }	OPTIONAL,

...

}

MBS-ServiceAreaTAIList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-ServiceAreaInformationList ::= SEQUENCE (SIZE(1..maxnoofMBSServiceAreaInformation)) OF MBS-ServiceAreaInformationItem

MBS-ServiceAreaInformationItem ::= SEQUENCE {

mBS-AreaSessionID					MBS-Area-Session-ID,

mBS-ServiceAreaInformation 			MBS-ServiceAreaInformation,

iE-Extensions						ProtocolExtensionContainer { { MBS-ServiceAreaInformationItem-ExtIEs} }	OPTIONAL,

...

}

MBS-ServiceAreaInformationItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MC-PagingCell-Item ::= SEQUENCE {

nRCGI			NRCGI,

iE-Extensions	ProtocolExtensionContainer { { MC-PagingCell-ItemExtIEs } }	OPTIONAL

}

MC-PagingCell-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

MIB-message ::= OCTET STRING

MeasConfig ::= OCTET STRING

MeasGapConfig ::= OCTET STRING

MeasGapSharingConfig ::= OCTET STRING

PosMeasurementAmount ::= ENUMERATED {ma0, ma1, ma2, ma4, ma8, ma16, ma32, ma64}

MeasurementBeamInfoRequest ::= ENUMERATED {true, ...}

MeasurementBeamInfo	 ::= SEQUENCE {

pRS-Resource-ID				PRS-Resource-ID		OPTIONAL,

pRS-Resource-Set-ID			PRS-Resource-Set-ID	OPTIONAL,

sSB-Index					SSB-Index			OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MeasurementBeamInfo-ExtIEs} } OPTIONAL

}

MeasurementBeamInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MeasurementTimingConfiguration ::= OCTET STRING

MessageIdentifier ::= BIT STRING (SIZE (16))

MeasurementTimeOccasion ::= ENUMERATED {o1, o4, ...}

MeasurementCharacteristicsRequestIndicator ::= BIT STRING (SIZE (16))

MRB-ProgressInformation ::= CHOICE {

pdcp-SN12			INTEGER (0..4095),

pdcp-SN18			INTEGER (0..262143),

choice-extension		ProtocolIE-SingleContainer { { MRB-ProgressInformation-ExtIEs} }

}

MRB-ProgressInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

MulticastF1UContextReferenceF1 ::= OCTET STRING (SIZE(4))

MulticastF1UContextReferenceCU ::= OCTET STRING (SIZE(4))

MultipleULAoA ::= SEQUENCE {

multipleULAoA					MultipleULAoA-List,

iE-Extensions					ProtocolExtensionContainer { { MultipleULAoA-ExtIEs} } OPTIONAL,

...

}

MultipleULAoA-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MultipleULAoA-List ::= SEQUENCE (SIZE(1.. maxnoofULAoAs)) OF MultipleULAoA-Item

MultipleULAoA-Item ::= CHOICE {

uL-AoA		UL-AoA,

ul-ZoA		ZoAInformation,

choice-extension ProtocolIE-SingleContainer { { MultipleULAoA-Item-ExtIEs } }

}

MultipleULAoA-Item-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

MDTPollutedMeasurementIndicator	::= ENUMERATED {iDC,no-IDC, ...}

MRB-ID ::= INTEGER (1..512, ...)

MulticastMBSSessionList ::= SEQUENCE (SIZE(1..maxnoofMBSSessionsofUE)) OF MulticastMBSSessionList-Item

MulticastMBSSessionList-Item ::= SEQUENCE {

mbsSessionId				MBS-Session-ID,

iE-Extensions				ProtocolExtensionContainer { { MulticastMBSSessionList-Item-ExtIEs } } OPTIONAL,

...

}

MulticastMBSSessionList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-FailedToBeModified-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

cause						Cause							OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MulticastMRBs-FailedtoBeModified-Item-ExtIEs} } OPTIONAL,

...

}

MulticastMRBs-FailedtoBeModified-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-FailedToBeSetup-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

cause						Cause							OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MulticastMRBs-FailedToBeSetup-Item-ExtIEs} } OPTIONAL,

...

}

MulticastMRBs-FailedToBeSetup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-FailedToBeSetupMod-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

cause						Cause							OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MulticastMRBs-FailedToBeSetupMod-Item-ExtIEs} } OPTIONAL,

...

}

MulticastMRBs-FailedToBeSetupMod-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-Modified-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

iE-Extensions				ProtocolExtensionContainer { { MulticastMRBs-Modified-Item-ExtIEs} } OPTIONAL,

...

}

MulticastMRBs-Modified-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-Setup-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

iE-Extensions				ProtocolExtensionContainer { { MulticastMRBs-Setup-Item-ExtIEs} } OPTIONAL,

...

}

MulticastMRBs-Setup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-SetupMod-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

iE-Extensions				ProtocolExtensionContainer { { MulticastMRBs-SetupMod-Item-ExtIEs} } OPTIONAL,

...

}

MulticastMRBs-SetupMod-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-ToBeModified-Item ::= SEQUENCE {

mRB-ID							MRB-ID,

mRB-QoSInformation				QoSFlowLevelQoSParameters		OPTIONAL,

mBS-Flows-Mapped-To-MRB-List	MBS-Flows-Mapped-To-MRB-List	OPTIONAL,

mBS-DL-PDCP-SN-Length			PDCPSNLength 					OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { MulticastMRBs-ToBeModified-Item-ExtIEs} } OPTIONAL,

...

}

MulticastMRBs-ToBeModified-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-ToBeReleased-Item	::= SEQUENCE {

mRB-ID						MRB-ID,

iE-Extensions				ProtocolExtensionContainer { { MulticastMRBs-ToBeReleased-ItemExtIEs } }	OPTIONAL,

...

}

MulticastMRBs-ToBeReleased-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-ToBeSetup-Item ::= SEQUENCE {

mRB-ID							MRB-ID,

mRB-QoSInformation				QoSFlowLevelQoSParameters,

mBS-Flows-Mapped-To-MRB-List	MBS-Flows-Mapped-To-MRB-List,

mBS-DL-PDCP-SN-Length			PDCPSNLength,

iE-Extensions					ProtocolExtensionContainer { { MulticastMRBs-ToBeSetup-Item-ExtIEs} } OPTIONAL,

...

}

MulticastMRBs-ToBeSetup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-ToBeSetupMod-Item ::= SEQUENCE {

mRB-ID							MRB-ID,

mRB-QoSInformation				QoSFlowLevelQoSParameters,

mBS-Flows-Mapped-To-MRB-List	MBS-Flows-Mapped-To-MRB-List,

mBS-DL-PDCP-SN-Length			PDCPSNLength,

iE-Extensions					ProtocolExtensionContainer { { MulticastMRBs-ToBeSetupMod-Item-ExtIEs} } OPTIONAL,

...

}

MulticastMRBs-ToBeSetupMod-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MultiplexingInfo 	::=	SEQUENCE{

iAB-MT-Cell-List 	IAB-MT-Cell-List,

iE-Extensions		ProtocolExtensionContainer { {MultiplexingInfo-ExtIEs} } OPTIONAL

}

MultiplexingInfo-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

MusimCapabilityRestrictionIndication ::= ENUMERATED {true, ...}

MusimCandidateBandList ::= OCTET STRING

M2Configuration ::= ENUMERATED {true, ...}

M5Configuration ::= SEQUENCE {

m5period			M5period,

m5-links-to-log		M5-Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M5Configuration-ExtIEs} } OPTIONAL,

...

}

M5Configuration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID	id-M5ReportAmount	CRITICALITY ignore	EXTENSION M5ReportAmount PRESENCE optional },

...

}

M5period ::= ENUMERATED { ms1024, ms2048, ms5120, ms10240, min1, ... }

M5ReportAmount	::= ENUMERATED { r1, r2, r4, r8, r16, r32, r64, infinity, ... }

M5-Links-to-log	::= ENUMERATED {uplink, downlink, both-uplink-and-downlink, ...}

M6Configuration ::= SEQUENCE {

m6report-Interval	M6report-Interval,

m6-links-to-log		M6-Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M6Configuration-ExtIEs} } OPTIONAL,

...

}

M6Configuration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID	id-M6ReportAmount	CRITICALITY ignore	EXTENSION M6ReportAmount PRESENCE optional },

...

}

M6report-Interval ::= ENUMERATED { ms120, ms240, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1, min6, min12, min30, ..., ms480}

M6ReportAmount	::= ENUMERATED { r1, r2, r4, r8, r16, r32, r64, infinity, ... }

M6-Links-to-log	::= ENUMERATED {uplink, downlink, both-uplink-and-downlink, ...}

M7Configuration ::= SEQUENCE {

m7period			M7period,

m7-links-to-log		M7-Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M7Configuration-ExtIEs} } OPTIONAL,

...

}

M7Configuration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID	id-M7ReportAmount	CRITICALITY ignore	EXTENSION M7ReportAmount PRESENCE optional},

...

}

M7period	::= INTEGER(1..60, ...)

M7ReportAmount	::= ENUMERATED { r1, r2, r4, r8, r16, r32, r64, infinity, ... }

M7-Links-to-log	::= ENUMERATED {downlink, ...}

MDT-Activation ::= ENUMERATED {

immediate-MDT-only,

immediate-MDT-and-Trace,

...

}

MDTConfiguration ::= SEQUENCE {

mdt-Activation				MDT-Activation,

measurementsToActivate		MeasurementsToActivate,

m2Configuration				M2Configuration		OPTIONAL,

--  The above IE shall be present if the Measurements to Activate IE has the second bit set to "1".

m5Configuration				M5Configuration		OPTIONAL,

-- The above IE shall be present if the Measurements to Activate IE has the fifth bit set to "1".

m6Configuration				M6Configuration		OPTIONAL,

--  The above IE shall be present if the Measurements to Activate IE has the seventh bit set to "1".

m7Configuration				M7Configuration		OPTIONAL,

--  The above IE shall be present if the Measurements to Activate IE has the eighth bit set to "1".

iE-Extensions				ProtocolExtensionContainer { { MDTConfiguration-ExtIEs} } OPTIONAL,

...

}

MDTConfiguration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MDTPLMNList ::= SEQUENCE (SIZE(1..maxnoofMDTPLMNs)) OF PLMN-Identity

MDTPLMNModificationList ::= SEQUENCE (SIZE(0..maxnoofMDTPLMNs)) OF PLMN-Identity

MeasuredFrequencyHops ::= ENUMERATED {singleHop, multiHop, ...}

MeasuredResultsValue ::= CHOICE {

uL-AngleOfArrival	UL-AoA,

uL-SRS-RSRP			UL-SRS-RSRP,

uL-RTOA				UL-RTOA-Measurement,

gNB-RxTxTimeDiff	GNB-RxTxTimeDiff,

choice-extension	ProtocolIE-SingleContainer { { MeasuredResultsValue-ExtIEs } }

}

MeasuredResultsValue-ExtIEs F1AP-PROTOCOL-IES ::= {

{ ID id-ZoAInformation	CRITICALITY reject TYPE ZoAInformation	PRESENCE mandatory}|

{ ID id-MultipleULAoA	CRITICALITY reject TYPE MultipleULAoA	PRESENCE mandatory}|

{ ID id-UL-SRS-RSRPP	CRITICALITY reject TYPE UL-SRS-RSRPP	PRESENCE mandatory}|

{ ID id-UL-RSCP			CRITICALITY reject TYPE UL-RSCP			PRESENCE mandatory},

...

}

MeasurementsToActivate ::= BIT STRING (SIZE (8))

Mobile-TRP-LocationInformation ::= SEQUENCE {

location-Information			OCTET STRING,

velocity-Information			OCTET STRING	OPTIONAL,

location-time-stamp				TimeStamp		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Mobile-TRP-LocationInformation-ExtIEs} } OPTIONAL,

...

}

Mobile-TRP-LocationInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Mobile-IAB-MT-UE-ID ::= OCTET STRING

MUSIM-GapConfig ::= OCTET STRING

MobileIAB-Barred	::=	ENUMERATED {barred, not-barred, ...}

MeasBasedOnAggregatedResources ::= ENUMERATED { true, ... }

MobilityTriggeringIndication ::=  BIT STRING (SIZE(8))

MobilityAssistanceInfo-CellSwitch ::= SEQUENCE {

candidateCellwithBeamInfo		CandidateCellwithBeamInfo,

iE-Extensions					ProtocolExtensionContainer { { MobilityAssistanceInfo-CellSwitch-ExtIEs } }	OPTIONAL,

...

}

MobilityAssistanceInfo-CellSwitch-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

MobilityAssistanceInfo-EarlyULSync ::= SEQUENCE {

candidateCellwithBeamInfoList		CandidateCellwithBeamInfoList,

iE-Extensions						ProtocolExtensionContainer { { MobilityAssistanceInfo-EarlyULSync-ExtIEs } }	OPTIONAL,

...

}

MobilityAssistanceInfo-EarlyULSync-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

MobilityAssistanceInfo-EarlyDLSync ::= SEQUENCE {

candidateCellwithBeamInfoList		CandidateCellwithBeamInfoList,

iE-Extensions						ProtocolExtensionContainer { { MobilityAssistanceInfo-EarlyDLSync-ExtIEs } }	OPTIONAL,

...

}

MobilityAssistanceInfo-EarlyDLSync-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

MobilityAssistanceInfo-Additional ::= SEQUENCE {

candidateCellwithMeasurementsList	CandidateCellwithMeasurementsList,

iE-Extensions						ProtocolExtensionContainer { { MobilityAssistanceInfo-Additional-ExtIEs } }	OPTIONAL,

...

}

MobilityAssistanceInfo-Additional-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- N

<<<<<< Skip unchanged >>>>>>

#### -- S

SCell-FailedtoSetup-Item	::= SEQUENCE {

sCell-ID			NRCGI	,

cause		Cause			OPTIONAL ,

iE-Extensions	ProtocolExtensionContainer { { SCell-FailedtoSetup-ItemExtIEs } }	OPTIONAL,

...

}

SCell-FailedtoSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SCell-FailedtoSetupMod-Item	::= SEQUENCE {

sCell-ID			NRCGI	,

cause		Cause			OPTIONAL ,

iE-Extensions	ProtocolExtensionContainer { { SCell-FailedtoSetupMod-ItemExtIEs } }	OPTIONAL,

...

}

SCell-FailedtoSetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SCell-ToBeRemoved-Item	::= SEQUENCE {

sCell-ID			NRCGI	,

iE-Extensions	ProtocolExtensionContainer { { SCell-ToBeRemoved-ItemExtIEs } }	OPTIONAL,

...

}

SCell-ToBeRemoved-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SCell-ToBeSetup-Item ::= SEQUENCE {

sCell-ID			NRCGI	,

sCellIndex			SCellIndex,

sCellULConfigured		CellULConfigured 	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SCell-ToBeSetup-ItemExtIEs } }	OPTIONAL,

...

}

SCell-ToBeSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ServingCellMO		CRITICALITY ignore	EXTENSION ServingCellMO		PRESENCE optional	},

...

}

SCell-ToBeSetupMod-Item	::= SEQUENCE {

sCell-ID			NRCGI	,

sCellIndex			SCellIndex,

sCellULConfigured		CellULConfigured 	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SCell-ToBeSetupMod-ItemExtIEs } }	OPTIONAL,

...

}

SCell-ToBeSetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ServingCellMO		CRITICALITY ignore	EXTENSION ServingCellMO		PRESENCE optional	},

...

}

SCellIndex ::=INTEGER (1..31, ...)

SCGActivationRequest ::= ENUMERATED {activate-scg, deactivate-scg, ...}

SCGActivationStatus ::= ENUMERATED {scg-activated, scg-deactivated, ...}

SCGIndicator	::=	ENUMERATED{released, ...}

SCPAC-Request ::= ENUMERATED {initiation, ...}

S-CPAC-Configuration	::= SEQUENCE {

referenceConfigurationInformation 	ReferenceConfigurationInformation			OPTIONAL,

completeCandidateConfigurationIndicator		CompleteCandidateConfigurationIndicator 	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { S-CPAC-Configuration-ExtIEs } }	OPTIONAL,

...

}

S-CPAC-Configuration-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

S-CPACLowerLayerReferenceConfigRequest	::=	ENUMERATED{true, ...}

SCS-480	::= INTEGER(0..319)

SCS-960	::= INTEGER(0..639)

SCS-SpecificCarrier ::=					SEQUENCE {

offsetToCarrier						INTEGER (0..2199,...),

subcarrierSpacing					ENUMERATED {kHz15, kHz30, kHz60, kHz120,..., kHz480, kHz960},

carrierBandwidth					INTEGER (1..275,...),

iE-Extensions						ProtocolExtensionContainer { { SCS-SpecificCarrier-ExtIEs } } OPTIONAL

}

SCS-SpecificCarrier-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SDTBearerConfigurationQueryIndication ::=	ENUMERATED {true, ...}

SDTBearerConfigurationInfo ::= SEQUENCE {

sDTBearerConfig-List			SDTBearerConfig-List,

iE-Extensions					ProtocolExtensionContainer { { SDTBearerConfigurationInfo-ExtIEs } } OPTIONAL

}

SDTBearerConfigurationInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SDTBearerConfig-List ::= SEQUENCE (SIZE(1..maxnoofSDTBearers)) OF SDTBearerConfig-List-Item

SDTBearerConfig-List-Item ::= SEQUENCE{

sDTBearerType 							SDTBearerType,

sDTRLCBearerConfiguration				SDTRLCBearerConfiguration,

iE-Extensions			ProtocolExtensionContainer {{ SDTBearerConfig-List-Item-ExtIEs}}		OPTIONAL

}

SDTBearerConfig-List-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SDTBearerType ::= CHOICE {

sRB					SRBID,

dRB					DRBID,

choice-extension				ProtocolIE-SingleContainer {{ SDTBearerType-ExtIEs }}

}

SDTBearerType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

SDT-MAC-PHY-CG-Config ::= OCTET STRING

SDTInformation ::= SEQUENCE {

sdtIndicator					ENUMERATED {true,...},

sdtAssistantInformation			ENUMERATED {singlepacket, multiplepackets,...}	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SDTInformation-ExtIEs } } OPTIONAL

}

SDTInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SDTRLCBearerConfiguration ::= OCTET STRING

SDT-Termination-Request	::= ENUMERATED {radio-link-problem, normal, ...,sdt-volume-threshold-crossed}

SDT-Volume-Threshold ::= INTEGER(1.. 192000,...)

Search-window-information ::= SEQUENCE {

expectedPropagationDelay		INTEGER (-3841..3841,...),

delayUncertainty				INTEGER (1..246,...),

iE-Extensions					ProtocolExtensionContainer { { Search-window-information-ExtIEs } } OPTIONAL

}

Search-window-information-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SerialNumber ::= BIT STRING (SIZE (16))

SIBType-PWS ::=INTEGER (6..8, ...)

SelectedBandCombinationIndex ::= OCTET STRING

SelectedFeatureSetEntryIndex ::= OCTET STRING

CG-ConfigInfo ::= OCTET STRING

ServCellInfoList ::= OCTET STRING

ServCellIndex ::= INTEGER (0..31, ...)

ServingCellMO ::= INTEGER (1..64, ...)

ServingCellMO-List-Item ::= SEQUENCE {

servingCellMO					ServingCellMO,

sSB-Frequency					INTEGER (0..3279165),

iE-Extensions					ProtocolExtensionContainer { { ServingCellMO-List-Item-ExtIEs } } OPTIONAL

}

ServingCellMO-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ServingCellMO-encoded-in-CGC-List ::= SEQUENCE (SIZE(1.. maxNrofBWPs)) OF ServingCellMO-encoded-in-CGC-Item

ServingCellMO-encoded-in-CGC-Item ::= SEQUENCE {

servingCellMO					ServingCellMO,

iE-Extensions					ProtocolExtensionContainer { { ServingCellMO-encoded-in-CGC-Item-ExtIEs } }	OPTIONAL,

...

}

ServingCellMO-encoded-in-CGC-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-BWP-Id	CRITICALITY ignore	EXTENSION BWP-Id	PRESENCE optional },

...

}

Served-Cell-Information ::= SEQUENCE {

nRCGI							NRCGI,

nRPCI							NRPCI,

fiveGS-TAC						FiveGS-TAC			OPTIONAL,

configured-EPS-TAC				Configured-EPS-TAC 		OPTIONAL,

servedPLMNs						ServedPLMNs-List,

nR-Mode-Info					NR-Mode-Info,

measurementTimingConfiguration	OCTET STRING,

iE-Extensions		ProtocolExtensionContainer { {Served-Cell-Information-ExtIEs} } OPTIONAL,

...

}

Served-Cell-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-RANAC							CRITICALITY ignore	EXTENSION RANAC							PRESENCE optional }|

{	ID id-ExtendedServedPLMNs-List		CRITICALITY ignore	EXTENSION ExtendedServedPLMNs-List	PRESENCE optional }|

{	ID id-Cell-Direction				CRITICALITY ignore	EXTENSION Cell-Direction				PRESENCE optional }|

{	ID id-BPLMN-ID-Info-List			CRITICALITY ignore	EXTENSION BPLMN-ID-Info-List			PRESENCE optional }|

{	ID id-Cell-Type						CRITICALITY ignore	EXTENSION CellType						PRESENCE optional}|

{	ID id-ConfiguredTACIndication		CRITICALITY ignore	EXTENSION ConfiguredTACIndication		PRESENCE optional }|

{	ID id-AggressorgNBSetID				CRITICALITY ignore	EXTENSION AggressorgNBSetID				PRESENCE optional}|

{	ID id-VictimgNBSetID				CRITICALITY ignore	EXTENSION VictimgNBSetID				PRESENCE optional}|

{	ID id-IAB-Info-IAB-DU				CRITICALITY ignore	EXTENSION IAB-Info-IAB-DU				PRESENCE optional}|

{	ID id-SSB-PositionsInBurst			CRITICALITY ignore	EXTENSION SSB-PositionsInBurst			PRESENCE optional }|

{	ID id-NRPRACHConfig					CRITICALITY ignore	EXTENSION NRPRACHConfig					PRESENCE optional }|

{	ID id-SFN-Offset					CRITICALITY ignore	EXTENSION SFN-Offset					PRESENCE optional }|

{	ID id-NPNBroadcastInformation		CRITICALITY reject 	EXTENSION NPNBroadcastInformation		PRESENCE optional }|

{	ID id-Supported-MBS-FSA-ID-List				CRITICALITY ignore	EXTENSION Supported-MBS-FSA-ID-List						PRESENCE optional }|

{	ID id-Redcap-Bcast-Information		CRITICALITY ignore	EXTENSION Redcap-Bcast-Information	PRESENCE optional }|

{	ID id-ERedcap-Bcast-Information		CRITICALITY ignore	EXTENSION ERedcap-Bcast-Information	PRESENCE optional }|

{	ID id-XR-Bcast-Information			CRITICALITY ignore	EXTENSION XR-Bcast-Information			PRESENCE optional }|

{	ID id-BarringExemptionforEmerCallInfo				CRITICALITY ignore	EXTENSION BarringExemptionforEmerCallInfo				PRESENCE optional },

...

}

Serving-Cells-List ::= SEQUENCE (SIZE(1..maxnoofServingCells)) OF Serving-Cells-List-Item

Serving-Cells-List-Item ::= SEQUENCE{

nRCGI 								NRCGI,

iAB-MT-Cell-NA-Resource-Configuration-Mode-Info       IAB-MT-Cell-NA-Resource-Configuration-Mode-Info		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer {{Serving-Cells-List-Item-ExtIEs}}		OPTIONAL

}

Serving-Cells-List-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Supported-MBS-FSA-ID-List::= SEQUENCE (SIZE(1.. maxnoofMBSFSAs)) OF MBS-FrequencySelectionArea-Identity

MBS-FrequencySelectionArea-Identity::= OCTET STRING (SIZE(3))

SFN-Offset ::= SEQUENCE {

sFN-Time-Offset					BIT STRING (SIZE(24)),

iE-Extensions		ProtocolExtensionContainer { {SFN-Offset-ExtIEs} } OPTIONAL,

...

}

SFN-Offset-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Served-Cells-To-Add-Item ::= SEQUENCE {

served-Cell-Information		Served-Cell-Information,

gNB-DU-System-Information	GNB-DU-System-Information	 OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { Served-Cells-To-Add-ItemExtIEs} }	OPTIONAL,

...

}

Served-Cells-To-Add-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Served-Cells-To-Delete-Item ::= SEQUENCE {

oldNRCGI					NRCGI	,

iE-Extensions				ProtocolExtensionContainer { { Served-Cells-To-Delete-ItemExtIEs } }	OPTIONAL,

...

}

Served-Cells-To-Delete-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Served-Cells-To-Modify-Item ::= SEQUENCE {

oldNRCGI					NRCGI							,

served-Cell-Information		Served-Cell-Information		,

gNB-DU-System-Information	GNB-DU-System-Information 	OPTIONAL	,

iE-Extensions				ProtocolExtensionContainer { { Served-Cells-To-Modify-ItemExtIEs } }	OPTIONAL,

...

}

Served-Cells-To-Modify-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Served-EUTRA-Cells-Information::= SEQUENCE {

eUTRA-Mode-Info						EUTRA-Mode-Info,

protectedEUTRAResourceIndication	ProtectedEUTRAResourceIndication,

iE-Extensions						ProtocolExtensionContainer { {Served-EUTRA-Cell-Information-ExtIEs} } OPTIONAL,

...

}

Served-EUTRA-Cell-Information-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Service-State ::= ENUMERATED {

in-service,

out-of-service,

...

}

Service-Status ::= SEQUENCE {

service-state				Service-State,

switchingOffOngoing			ENUMERATED {true, ...}	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { Service-Status-ExtIEs } }	OPTIONAL,

...

}

Service-Status-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SelectedMeasurementQuantities ::= SEQUENCE {

rSRP			INTEGER (0..127) 	OPTIONAL,

rSRQ			INTEGER (0..127) 	OPTIONAL,

sINR			INTEGER (0..127) 	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SelectedMeasurementQuantities-ExtIEs } }	OPTIONAL,

...

}

SelectedMeasurementQuantities-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SSBIndex  ::= INTEGER(0..63)

SSBIndexList	::=  SEQUENCE (SIZE(1..maxnoofSSBIndices)) OF SSBIndexList-Item

SSBIndexList-Item	::= SEQUENCE {

sSBIndex			SSBIndex,

iE-Extensions		ProtocolExtensionContainer { { SSBIndex-Item-ExtIEs } }	OPTIONAL}

SSBIndex-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SSBIndexwithMeasurementsList	::=  SEQUENCE (SIZE(1..maxnoofSSBIndices)) OF SSBIndexwithMeasurements-Item

SSBIndexwithMeasurements-Item	::= SEQUENCE {

sSBIndex			SSBIndex,

selectedMeasurementQuantities	SelectedMeasurementQuantities		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { SSBIndexwithMeasurements-Item-ExtIEs } }	OPTIONAL}

SSBIndexwithMeasurements-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<< Skip unchanged >>>>>>

#### -- T

TAI ::= SEQUENCE {

pLMN-Identity				         PLMN-Identity,

fiveGS-TAC					         FiveGS-TAC,

iE-Extensions					ProtocolExtensionContainer { {TAI-ExtIEs} } OPTIONAL,

...

}

<<<<<< Skip unchanged >>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM F1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-Reset									ProcedureCode ::= 0

id-F1Setup									ProcedureCode ::= 1

id-ErrorIndication							ProcedureCode ::= 2

id-gNBDUConfigurationUpdate					ProcedureCode ::= 3

id-gNBCUConfigurationUpdate					ProcedureCode ::= 4

id-UEContextSetup							ProcedureCode ::= 5

id-UEContextRelease							ProcedureCode ::= 6

id-UEContextModification					ProcedureCode ::= 7

id-UEContextModificationRequired			ProcedureCode ::= 8

id-procedure-code-9-not-to-be-used			ProcedureCode ::= 9

id-UEContextReleaseRequest					ProcedureCode ::= 10

id-InitialULRRCMessageTransfer				ProcedureCode ::= 11

id-DLRRCMessageTransfer						ProcedureCode ::= 12

id-ULRRCMessageTransfer						ProcedureCode ::= 13

id-privateMessage							ProcedureCode ::= 14

id-UEInactivityNotification					ProcedureCode ::= 15

id-GNBDUResourceCoordination				ProcedureCode ::= 16

id-SystemInformationDeliveryCommand			ProcedureCode ::= 17

id-Paging									ProcedureCode ::= 18

id-Notify									ProcedureCode ::= 19

id-WriteReplaceWarning						ProcedureCode ::= 20

id-PWSCancel								ProcedureCode ::= 21

id-PWSRestartIndication						ProcedureCode ::= 22

id-PWSFailureIndication						ProcedureCode ::= 23

id-GNBDUStatusIndication 					ProcedureCode ::= 24

id-RRCDeliveryReport		 				ProcedureCode ::= 25

id-F1Removal								ProcedureCode ::= 26

id-NetworkAccessRateReduction				ProcedureCode ::= 27

id-TraceStart								ProcedureCode ::= 28

id-DeactivateTrace							ProcedureCode ::= 29

id-DUCURadioInformationTransfer				ProcedureCode ::= 30

id-CUDURadioInformationTransfer				ProcedureCode ::= 31

id-BAPMappingConfiguration					ProcedureCode ::= 32

id-GNBDUResourceConfiguration				ProcedureCode ::= 33

id-IABTNLAddressAllocation					ProcedureCode ::= 34

id-IABUPConfigurationUpdate					ProcedureCode ::= 35

id-resourceStatusReportingInitiation		ProcedureCode ::= 36

id-resourceStatusReporting					ProcedureCode ::= 37

id-accessAndMobilityIndication				ProcedureCode ::= 38

id-accessSuccess							ProcedureCode ::= 39

id-cellTrafficTrace 						ProcedureCode ::= 40

id-PositioningMeasurementExchange			ProcedureCode ::= 41

id-PositioningAssistanceInformationControl	ProcedureCode ::= 42

id-PositioningAssistanceInformationFeedback	ProcedureCode ::= 43

id-PositioningMeasurementReport				ProcedureCode ::= 44

id-PositioningMeasurementAbort				ProcedureCode ::= 45

id-PositioningMeasurementFailureIndication	ProcedureCode ::= 46

id-PositioningMeasurementUpdate				ProcedureCode ::= 47

id-TRPInformationExchange					ProcedureCode ::= 48

id-PositioningInformationExchange			ProcedureCode ::= 49

id-PositioningActivation					ProcedureCode ::= 50

id-PositioningDeactivation					ProcedureCode ::= 51

id-E-CIDMeasurementInitiation				ProcedureCode ::= 52

id-E-CIDMeasurementFailureIndication		ProcedureCode ::= 53

id-E-CIDMeasurementReport					ProcedureCode ::= 54

id-E-CIDMeasurementTermination				ProcedureCode ::= 55

id-PositioningInformationUpdate				ProcedureCode ::= 56

id-ReferenceTimeInformationReport			ProcedureCode ::= 57

id-ReferenceTimeInformationReportingControl	ProcedureCode ::= 58

id-BroadcastContextSetup					ProcedureCode ::= 59

id-BroadcastContextRelease					ProcedureCode ::= 60

id-BroadcastContextReleaseRequest			ProcedureCode ::= 61

id-BroadcastContextModification				ProcedureCode ::= 62

id-MulticastGroupPaging						ProcedureCode ::= 63

id-MulticastContextSetup					ProcedureCode ::= 64

id-MulticastContextRelease					ProcedureCode ::= 65

id-MulticastContextReleaseRequest			ProcedureCode ::= 66

id-MulticastContextModification				ProcedureCode ::= 67

id-MulticastDistributionSetup				ProcedureCode ::= 68

id-MulticastDistributionRelease				ProcedureCode ::= 69

id-PDCMeasurementInitiation					ProcedureCode ::= 70

id-PDCMeasurementReport						ProcedureCode ::= 71

id-procedure-code-72-not-to-be-used			ProcedureCode ::= 72

id-procedure-code-73-not-to-be-used			ProcedureCode ::= 73

id-procedure-code-74-not-to-be-used			ProcedureCode ::= 74

id-pRSConfigurationExchange					ProcedureCode ::= 75

id-measurementPreconfiguration				ProcedureCode ::= 76

id-measurementActivation					ProcedureCode ::= 77

id-QoEInformationTransfer					ProcedureCode ::= 78

id-PDCMeasurementTerminationCommand			ProcedureCode ::= 79

id-PDCMeasurementFailureIndication 			ProcedureCode ::= 80

id-PosSystemInformationDeliveryCommand		ProcedureCode ::= 81

id-DUCUCellSwitchNotification				ProcedureCode ::= 82

id-CUDUCellSwitchNotification				ProcedureCode ::= 83

id-DUCUTAInformationTransfer				ProcedureCode ::= 84

id-CUDUTAInformationTransfer				ProcedureCode ::= 85

id-QoEInformationTransferControl			ProcedureCode ::= 86

id-RachIndication							ProcedureCode ::= 87

id-TimingSynchronisationStatus				ProcedureCode ::= 88

id-TimingSynchronisationStatusReport		ProcedureCode ::= 89

id-MIABF1SetupTriggering					ProcedureCode ::= 90

id-MIABF1SetupOutcomeNotification			ProcedureCode ::= 91

id-MulticastContextNotification 			ProcedureCode ::= 92

id-MulticastCommonConfiguration				ProcedureCode ::= 93

id-BroadcastTransportResourceRequest		ProcedureCode ::= 94

id-DUCUAccessAndMobilityIndication			ProcedureCode ::= 95

id-SRSInformationReservationNotification	ProcedureCode ::= 96

id-CUDUMobilityInitiationRequest			ProcedureCode ::= xx

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs							INTEGER ::= 65535

maxProtocolExtensions					INTEGER ::= 65535

maxProtocolIEs							INTEGER ::= 65535

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxNRARFCN								INTEGER ::= 3279165

maxnoofErrors							INTEGER ::= 256

maxnoofIndividualF1ConnectionsToReset	INTEGER ::= 65536

maxCellingNBDU							INTEGER ::= 512

maxnoofSCells							INTEGER ::= 32

maxnoofSRBs								INTEGER ::= 8

maxnoofDRBs								INTEGER ::= 64

maxnoofULUPTNLInformation				INTEGER ::= 2

maxnoofDLUPTNLInformation				INTEGER ::= 2

maxnoofBPLMNs							INTEGER ::= 6

maxnoofCandidateSpCells					INTEGER ::= 64

maxnoofPotentialSpCells					INTEGER ::= 64

maxnoofNrCellBands						INTEGER ::= 32

maxnoofSIBTypes							INTEGER ::= 32

maxnoofSITypes							INTEGER ::= 32

maxnoofPagingCells						INTEGER ::= 512

maxnoofTNLAssociations					INTEGER ::= 32

maxnoofQoSFlows							INTEGER ::= 64

maxnoofSliceItems						INTEGER ::= 1024

maxCellineNB							INTEGER ::= 256

maxnoofExtendedBPLMNs					INTEGER ::= 6

maxnoofUEIDs							INTEGER ::= 65536

maxnoofBPLMNsNR							INTEGER ::= 12

maxnoofUACPLMNs							INTEGER ::= 12

maxnoofUACperPLMN						INTEGER ::= 64

maxnoofAdditionalSIBs					INTEGER ::= 63

maxnoofslots							INTEGER ::= 5120

maxnoofTLAs								INTEGER ::=	16

maxnoofGTPTLAs							INTEGER ::=	16

maxnoofBHRLCChannels					INTEGER ::= 65536

maxnoofRoutingEntries					INTEGER ::= 1024

maxnoofIABSTCInfo						INTEGER ::= 45

maxnoofSymbols							INTEGER ::= 14

maxnoofServingCells						INTEGER ::= 32

maxnoofDUFSlots							INTEGER ::= 320

maxnoofHSNASlots						INTEGER ::= 5120

maxnoofServedCellsIAB					INTEGER ::= 512

maxnoofSSBarea							INTEGER ::=64

maxnoofChildIABNodes					INTEGER ::= 1024

maxnoofNonUPTrafficMappings				INTEGER ::= 32

maxnoofTLAsIAB							INTEGER ::= 1024

maxnoofMappingEntries					INTEGER ::= 67108864

maxnoofDSInfo							INTEGER ::= 64

maxnoofEgressLinks						INTEGER ::= 2

maxnoofULUPTNLInformationforIAB			INTEGER ::= 32678

maxnoofUPTNLAddresses					INTEGER ::= 8

maxnoofSLDRBs							INTEGER ::= 512

maxnoofQoSParaSets						INTEGER ::= 8

maxnoofPC5QoSFlows						INTEGER ::= 2048

maxnoofSSBAreas							INTEGER ::=	64

maxnoofPhysicalResourceBlocks			INTEGER ::= 275

maxnoofPhysicalResourceBlocks-1			INTEGER ::= 274

maxnoofPRACHconfigs						INTEGER ::= 16

maxnoofRAReports						INTEGER ::= 64

maxnoofRLFReports						INTEGER ::= 64

maxnoofAdditionalPDCPDuplicationTNL		INTEGER ::=	2

maxnoofRLCDuplicationState				INTEGER ::=	3

maxnoofCHOcells							INTEGER ::= 8

maxnoofMDTPLMNs							INTEGER ::=	16

maxnoofCAGsupported						INTEGER ::= 12

maxnoofNIDsupported						INTEGER ::= 12

maxnoofNRSCSs							INTEGER ::= 5

maxnoofExtSliceItems					INTEGER ::= 65535

maxnoofPosMeas							INTEGER ::=	16384

maxnoofTRPInfoTypes						INTEGER ::=	64

maxnoofTRPs								INTEGER ::=	65535

maxnoofSRSTriggerStates					INTEGER ::= 3

maxnoofSpatialRelations					INTEGER ::= 64

maxnoBcastCell							INTEGER ::= 16384

maxnoofAngleInfo						INTEGER ::= 65535

maxnooflcs-gcs-translation				INTEGER ::= 3

maxnoofPath								INTEGER ::= 2

maxnoofMeasE-CID						INTEGER ::= 64

maxnoofSSBs								INTEGER ::= 255

maxnoSRS-ResourceSets					INTEGER ::= 16

maxnoSRS-ResourcePerSet					INTEGER ::= 16

maxnoSRS-Carriers						INTEGER ::= 32

maxnoSCSs								INTEGER ::= 5

maxnoSRS-Resources						INTEGER ::= 64

maxnoSRS-PosResources					INTEGER ::= 64

maxnoSRS-PosResourceSets				INTEGER ::= 16

maxnoSRS-PosResourcePerSet				INTEGER ::= 16

maxnoofPRS-ResourceSets					INTEGER ::= 2

maxnoofPRS-ResourcesPerSet				INTEGER ::= 64

maxNoOfMeasTRPs							INTEGER ::= 64

maxnoofPRSresourceSets					INTEGER ::= 8

maxnoofPRSresources						INTEGER ::= 64

maxnoofSuccessfulHOReports				INTEGER ::= 64

maxnoofNR-UChannelIDs					INTEGER ::= 16

maxServedCellforSON						INTEGER ::= 256

maxNeighbourCellforSON					INTEGER ::= 32

maxAffectedCells						INTEGER ::= 32

maxnoofMRBs								INTEGER ::= 32

maxnoofMBSQoSFlows						INTEGER ::= 64

maxnoofMBSFSAs 							INTEGER ::= 256

maxnoofUEIDforPaging 					INTEGER ::= 4096

maxnoofCellsforMBS						INTEGER ::= 512

maxnoofTAIforMBS						INTEGER ::= 512

maxnoofMBSAreaSessionIDs				INTEGER ::= 256

maxnoofMBSServiceAreaInformation		INTEGER ::= 256

maxnoofIABCongInd						INTEGER ::= 1024

maxnoofNeighbourNodeCellsIAB			INTEGER ::= 1024

maxnoofRBsetsPerCell					INTEGER ::= 8

maxnoofRBsetsPerCell-1					INTEGER ::= 7

maxnoofMeasPDC							INTEGER ::= 16

maxnoARPs								INTEGER ::=	16

maxnoofULAoAs							INTEGER ::= 8

maxNoPathExtended						INTEGER ::= 8

maxnoTRPTEGs							INTEGER ::= 8

maxFreqLayers							INTEGER ::= 4

maxNumResourcesPerAngle					INTEGER ::= 24

maxnoAzimuthAngles						INTEGER ::= 3600

maxnoElevationAngles					INTEGER ::= 1801

maxnoofPRSTRPs							INTEGER ::= 256

maxnoofQoEInformation					INTEGER ::= 16

maxnoofUuRLCChannels					INTEGER ::= 32

maxnoofPC5RLCChannels					INTEGER ::= 512

maxnoofSMBRValues						INTEGER ::= 8

maxnoofMRBsforUE						INTEGER ::= 64

maxnoofMBSSessionsofUE					INTEGER ::= 256

maxnoofSLdestinations					INTEGER ::= 32

maxnoofNSAGs							INTEGER ::= 256

maxnoofSDTBearers						INTEGER ::= 72

maxnoofServingCellMOs					INTEGER ::= 16

maxNrofBWPs								INTEGER ::= 8

maxnoofPosSITypes						INTEGER ::= 32

maxnoofUETypes							INTEGER ::= 8

maxnoofLTMCells							INTEGER	::= 8

maxnoofTAList							INTEGER	::= 8

maxnoofLTMgNB-DUs						INTEGER	::= 8

maxnoofUEsInQMCTransferControlMessage	INTEGER ::= 512

maxnoofUEsforRAReportIndications		INTEGER ::= 64

maxnoofSuccessfulPSCellChangeReports	INTEGER ::= 64

maxnoofPeriodicities					INTEGER ::= 8

maxnoofThresholdMBS-1					INTEGER ::= 7

maxMBSSessionsinSessionInfoList			INTEGER ::= 1024

maxnoofLBTFailureInformation			INTEGER ::= 64

maxnoofRSPPQoSFlows						INTEGER ::= 2048

maxnoVACell								INTEGER ::= 32

maxnoAggregatedSRS-Resources			INTEGER ::= 3

maxnoAggregatedPosSRSResourceSets		INTEGER ::= 48

maxnoAggregatedPosPRSResourceSets		INTEGER ::= 3

maxnoofTimeWindowSRS					INTEGER ::= 16

maxnoofTimeWindowMea					INTEGER ::= 16

maxnoPreconfiguredSRS					INTEGER ::= 16

maxnoHopsMinusOne						INTEGER ::= 5

maxnoAggCombinations					INTEGER ::= 2

maxnoofCandidateCells					INTEGER ::= 8

maxnoofSSBIndices						INTEGER ::= 64

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause											ProtocolIE-ID ::= 0

id-Cells-Failed-to-be-Activated-List				ProtocolIE-ID ::= 1

id-Cells-Failed-to-be-Activated-List-Item			ProtocolIE-ID ::= 2

id-Cells-to-be-Activated-List						ProtocolIE-ID ::= 3

id-Cells-to-be-Activated-List-Item					ProtocolIE-ID ::= 4

id-Cells-to-be-Deactivated-List						ProtocolIE-ID ::= 5

id-Cells-to-be-Deactivated-List-Item				ProtocolIE-ID ::= 6

id-CriticalityDiagnostics							ProtocolIE-ID ::= 7

id-CUtoDURRCInformation								ProtocolIE-ID ::= 9

id-DRBs-FailedToBeModified-Item						ProtocolIE-ID ::= 12

id-DRBs-FailedToBeModified-List						ProtocolIE-ID ::= 13

id-DRBs-FailedToBeSetup-Item						ProtocolIE-ID ::= 14

id-DRBs-FailedToBeSetup-List						ProtocolIE-ID ::= 15

id-DRBs-FailedToBeSetupMod-Item						ProtocolIE-ID ::= 16

id-DRBs-FailedToBeSetupMod-List						ProtocolIE-ID ::= 17

id-DRBs-ModifiedConf-Item							ProtocolIE-ID ::= 18

id-DRBs-ModifiedConf-List							ProtocolIE-ID ::= 19

id-DRBs-Modified-Item								ProtocolIE-ID ::= 20

id-DRBs-Modified-List								ProtocolIE-ID ::= 21

id-DRBs-Required-ToBeModified-Item					ProtocolIE-ID ::= 22

id-DRBs-Required-ToBeModified-List					ProtocolIE-ID ::= 23

id-DRBs-Required-ToBeReleased-Item					ProtocolIE-ID ::= 24

id-DRBs-Required-ToBeReleased-List					ProtocolIE-ID ::= 25

id-DRBs-Setup-Item									ProtocolIE-ID ::= 26

id-DRBs-Setup-List									ProtocolIE-ID ::= 27

id-DRBs-SetupMod-Item								ProtocolIE-ID ::= 28

id-DRBs-SetupMod-List								ProtocolIE-ID ::= 29

id-DRBs-ToBeModified-Item							ProtocolIE-ID ::= 30

id-DRBs-ToBeModified-List							ProtocolIE-ID ::= 31

id-DRBs-ToBeReleased-Item							ProtocolIE-ID ::= 32

id-DRBs-ToBeReleased-List							ProtocolIE-ID ::= 33

id-DRBs-ToBeSetup-Item								ProtocolIE-ID ::= 34

id-DRBs-ToBeSetup-List								ProtocolIE-ID ::= 35

id-DRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 36

id-DRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 37

id-DRXCycle											ProtocolIE-ID ::= 38

id-DUtoCURRCInformation								ProtocolIE-ID ::= 39

id-gNB-CU-UE-F1AP-ID								ProtocolIE-ID ::= 40

id-gNB-DU-UE-F1AP-ID								ProtocolIE-ID ::= 41

id-gNB-DU-ID										ProtocolIE-ID ::= 42

id-GNB-DU-Served-Cells-Item							ProtocolIE-ID ::= 43

id-gNB-DU-Served-Cells-List							ProtocolIE-ID ::= 44

id-gNB-DU-Name										ProtocolIE-ID ::= 45

id-ProtocolIE-ID-46-not-to-be-used					ProtocolIE-ID ::= 46

id-oldgNB-DU-UE-F1AP-ID								ProtocolIE-ID ::= 47

id-ResetType										ProtocolIE-ID ::= 48

id-ResourceCoordinationTransferContainer			ProtocolIE-ID ::= 49

id-RRCContainer										ProtocolIE-ID ::= 50

id-SCell-ToBeRemoved-Item							ProtocolIE-ID ::= 51

id-SCell-ToBeRemoved-List							ProtocolIE-ID ::= 52

id-SCell-ToBeSetup-Item								ProtocolIE-ID ::= 53

id-SCell-ToBeSetup-List								ProtocolIE-ID ::= 54

id-SCell-ToBeSetupMod-Item							ProtocolIE-ID ::= 55

id-SCell-ToBeSetupMod-List							ProtocolIE-ID ::= 56

id-Served-Cells-To-Add-Item							ProtocolIE-ID ::= 57

id-Served-Cells-To-Add-List							ProtocolIE-ID ::= 58

id-Served-Cells-To-Delete-Item						ProtocolIE-ID ::= 59

id-Served-Cells-To-Delete-List						ProtocolIE-ID ::= 60

id-Served-Cells-To-Modify-Item						ProtocolIE-ID ::= 61

id-Served-Cells-To-Modify-List						ProtocolIE-ID ::= 62

id-SpCell-ID										ProtocolIE-ID ::= 63

id-SRBID											ProtocolIE-ID ::= 64

id-SRBs-FailedToBeSetup-Item						ProtocolIE-ID ::= 65

id-SRBs-FailedToBeSetup-List						ProtocolIE-ID ::= 66

id-SRBs-FailedToBeSetupMod-Item						ProtocolIE-ID ::= 67

id-SRBs-FailedToBeSetupMod-List						ProtocolIE-ID ::= 68

id-SRBs-Required-ToBeReleased-Item					ProtocolIE-ID ::= 69

id-SRBs-Required-ToBeReleased-List					ProtocolIE-ID ::= 70

id-SRBs-ToBeReleased-Item							ProtocolIE-ID ::= 71

id-SRBs-ToBeReleased-List							ProtocolIE-ID ::= 72

id-SRBs-ToBeSetup-Item								ProtocolIE-ID ::= 73

id-SRBs-ToBeSetup-List								ProtocolIE-ID ::= 74

id-SRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 75

id-SRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 76

id-TimeToWait										ProtocolIE-ID ::= 77

id-TransactionID									ProtocolIE-ID ::= 78

id-TransmissionActionIndicator						ProtocolIE-ID ::= 79

id-UE-associatedLogicalF1-ConnectionItem 			ProtocolIE-ID ::= 80

id-UE-associatedLogicalF1-ConnectionListResAck		ProtocolIE-ID ::= 81

id-gNB-CU-Name										ProtocolIE-ID ::= 82

id-SCell-FailedtoSetup-List							ProtocolIE-ID ::= 83

id-SCell-FailedtoSetup-Item							ProtocolIE-ID ::= 84

id-SCell-FailedtoSetupMod-List						ProtocolIE-ID ::= 85

id-SCell-FailedtoSetupMod-Item						ProtocolIE-ID ::= 86

id-RRCReconfigurationCompleteIndicator 				ProtocolIE-ID ::= 87

id-Cells-Status-Item								ProtocolIE-ID ::= 88

id-Cells-Status-List								ProtocolIE-ID ::= 89

id-Candidate-SpCell-List							ProtocolIE-ID ::= 90

id-Candidate-SpCell-Item							ProtocolIE-ID ::= 91

id-Potential-SpCell-List							ProtocolIE-ID ::= 92

id-Potential-SpCell-Item							ProtocolIE-ID ::= 93

id-FullConfiguration								ProtocolIE-ID ::= 94

id-C-RNTI											ProtocolIE-ID ::= 95

id-SpCellULConfigured								ProtocolIE-ID ::= 96

id-InactivityMonitoringRequest						ProtocolIE-ID ::= 97

id-InactivityMonitoringResponse						ProtocolIE-ID ::= 98

id-DRB-Activity-Item								ProtocolIE-ID ::= 99

id-DRB-Activity-List								ProtocolIE-ID ::= 100

id-EUTRA-NR-CellResourceCoordinationReq-Container	ProtocolIE-ID ::= 101

id-EUTRA-NR-CellResourceCoordinationReqAck-Container	ProtocolIE-ID ::= 102

id-Protected-EUTRA-Resources-List					ProtocolIE-ID ::= 105

id-RequestType 										ProtocolIE-ID ::= 106

id-ServCellIndex									ProtocolIE-ID ::= 107

id-RAT-FrequencyPriorityInformation					ProtocolIE-ID ::= 108

id-ExecuteDuplication								ProtocolIE-ID ::= 109

id-NRCGI											ProtocolIE-ID ::= 111

id-PagingCell-Item									ProtocolIE-ID ::= 112

id-PagingCell-List									ProtocolIE-ID ::= 113

id-PagingDRX										ProtocolIE-ID ::= 114

id-PagingPriority 									ProtocolIE-ID ::= 115

id-SItype-List										ProtocolIE-ID ::= 116

id-UEIdentityIndexValue								ProtocolIE-ID ::= 117

id-gNB-CUSystemInformation							ProtocolIE-ID ::= 118

id-HandoverPreparationInformation					ProtocolIE-ID ::= 119

id-GNB-CU-TNL-Association-To-Add-Item				ProtocolIE-ID ::= 120

id-GNB-CU-TNL-Association-To-Add-List				ProtocolIE-ID ::= 121

id-GNB-CU-TNL-Association-To-Remove-Item			ProtocolIE-ID ::= 122

id-GNB-CU-TNL-Association-To-Remove-List			ProtocolIE-ID ::= 123

id-GNB-CU-TNL-Association-To-Update-Item			ProtocolIE-ID ::= 124

id-GNB-CU-TNL-Association-To-Update-List			ProtocolIE-ID ::= 125

id-MaskedIMEISV										ProtocolIE-ID ::= 126

id-PagingIdentity									ProtocolIE-ID ::= 127

id-DUtoCURRCContainer								ProtocolIE-ID ::= 128

id-Cells-to-be-Barred-List							ProtocolIE-ID ::= 129

id-Cells-to-be-Barred-Item							ProtocolIE-ID ::= 130

id-TAISliceSupportList								ProtocolIE-ID ::= 131

id-GNB-CU-TNL-Association-Setup-List				ProtocolIE-ID ::= 132

id-GNB-CU-TNL-Association-Setup-Item				ProtocolIE-ID ::= 133

id-GNB-CU-TNL-Association-Failed-To-Setup-List		ProtocolIE-ID ::= 134

id-GNB-CU-TNL-Association-Failed-To-Setup-Item		ProtocolIE-ID ::= 135

id-DRB-Notify-Item									ProtocolIE-ID ::= 136

id-DRB-Notify-List									ProtocolIE-ID ::= 137

id-ProtocolIE-ID-138-not-to-be-used				ProtocolIE-ID ::= 138

id-RANAC											ProtocolIE-ID ::= 139

id-PWSSystemInformation								ProtocolIE-ID ::= 140

id-RepetitionPeriod									ProtocolIE-ID ::= 141

id-NumberofBroadcastRequest							ProtocolIE-ID ::= 142

id-Cells-To-Be-Broadcast-List						ProtocolIE-ID ::= 144

id-Cells-To-Be-Broadcast-Item						ProtocolIE-ID ::= 145

id-Cells-Broadcast-Completed-List 					ProtocolIE-ID ::= 146

id-Cells-Broadcast-Completed-Item 					ProtocolIE-ID ::= 147

id-Broadcast-To-Be-Cancelled-List 					ProtocolIE-ID ::= 148

id-Broadcast-To-Be-Cancelled-Item 					ProtocolIE-ID ::= 149

id-Cells-Broadcast-Cancelled-List 					ProtocolIE-ID ::= 150

id-Cells-Broadcast-Cancelled-Item 					ProtocolIE-ID ::= 151

id-NR-CGI-List-For-Restart-List 					ProtocolIE-ID ::= 152

id-NR-CGI-List-For-Restart-Item 					ProtocolIE-ID ::= 153

id-PWS-Failed-NR-CGI-List 							ProtocolIE-ID ::= 154

id-PWS-Failed-NR-CGI-Item 							ProtocolIE-ID ::= 155

id-ConfirmedUEID									ProtocolIE-ID ::= 156

id-Cancel-all-Warning-Messages-Indicator			ProtocolIE-ID ::= 157

id-GNB-DU-UE-AMBR-UL								ProtocolIE-ID ::= 158

id-DRXConfigurationIndicator						ProtocolIE-ID ::= 159

id-RLC-Status										ProtocolIE-ID ::= 160

id-DLPDCPSNLength									ProtocolIE-ID ::= 161

id-GNB-DUConfigurationQuery							ProtocolIE-ID ::= 162

id-MeasurementTimingConfiguration					ProtocolIE-ID ::= 163

id-DRB-Information									ProtocolIE-ID ::= 164

id-ServingPLMN										ProtocolIE-ID ::= 165

id-Protected-EUTRA-Resources-Item					ProtocolIE-ID ::= 168

id-GNB-CU-RRC-Version								ProtocolIE-ID ::= 170

id-GNB-DU-RRC-Version								ProtocolIE-ID ::= 171

id-GNBDUOverloadInformation							ProtocolIE-ID ::= 172

id-CellGroupConfig									ProtocolIE-ID ::= 173

id-RLCFailureIndication								ProtocolIE-ID ::= 174

id-UplinkTxDirectCurrentListInformation				ProtocolIE-ID ::= 175

id-DC-Based-Duplication-Configured					ProtocolIE-ID ::= 176

id-DC-Based-Duplication-Activation					ProtocolIE-ID ::= 177

id-SULAccessIndication								ProtocolIE-ID ::= 178

id-AvailablePLMNList								ProtocolIE-ID ::= 179

id-PDUSessionID										ProtocolIE-ID ::= 180

id-ULPDUSessionAggregateMaximumBitRate				ProtocolIE-ID ::= 181

id-ServingCellMO									ProtocolIE-ID ::= 182

id-QoSFlowMappingIndication							ProtocolIE-ID ::= 183

id-RRCDeliveryStatusRequest							ProtocolIE-ID ::= 184

id-RRCDeliveryStatus								ProtocolIE-ID ::= 185

id-BearerTypeChange									ProtocolIE-ID ::= 186

id-RLCMode											ProtocolIE-ID ::= 187

id-Duplication-Activation							ProtocolIE-ID ::= 188

id-Dedicated-SIDelivery-NeededUE-List				ProtocolIE-ID ::= 189

id-Dedicated-SIDelivery-NeededUE-Item				ProtocolIE-ID ::= 190

id-DRX-LongCycleStartOffset							ProtocolIE-ID ::= 191

id-ULPDCPSNLength									ProtocolIE-ID ::= 192

id-SelectedBandCombinationIndex						ProtocolIE-ID ::= 193

id-SelectedFeatureSetEntryIndex						ProtocolIE-ID ::= 194

id-ResourceCoordinationTransferInformation			ProtocolIE-ID ::= 195

id-ExtendedServedPLMNs-List							ProtocolIE-ID ::= 196

id-ExtendedAvailablePLMN-List						ProtocolIE-ID ::= 197

id-Associated-SCell-List							ProtocolIE-ID ::= 198

id-latest-RRC-Version-Enhanced						ProtocolIE-ID ::= 199

id-Associated-SCell-Item							ProtocolIE-ID ::= 200

id-Cell-Direction									ProtocolIE-ID ::= 201

id-SRBs-Setup-List									ProtocolIE-ID ::= 202

id-SRBs-Setup-Item									ProtocolIE-ID ::= 203

id-SRBs-SetupMod-List								ProtocolIE-ID ::= 204

id-SRBs-SetupMod-Item								ProtocolIE-ID ::= 205

id-SRBs-Modified-List								ProtocolIE-ID ::= 206

id-SRBs-Modified-Item								ProtocolIE-ID ::= 207

id-Ph-InfoSCG										ProtocolIE-ID ::= 208

id-RequestedBandCombinationIndex					ProtocolIE-ID ::= 209

id-RequestedFeatureSetEntryIndex					ProtocolIE-ID ::= 210

id-RequestedP-MaxFR2								ProtocolIE-ID ::= 211

id-DRX-Config										ProtocolIE-ID ::= 212

id-IgnoreResourceCoordinationContainer				ProtocolIE-ID ::= 213

id-UEAssistanceInformation							ProtocolIE-ID ::= 214

id-NeedforGap										ProtocolIE-ID ::= 215

id-PagingOrigin										ProtocolIE-ID ::= 216

id-new-gNB-CU-UE-F1AP-ID							ProtocolIE-ID ::= 217

id-RedirectedRRCmessage								ProtocolIE-ID ::= 218

id-new-gNB-DU-UE-F1AP-ID							ProtocolIE-ID ::= 219

id-NotificationInformation							ProtocolIE-ID ::= 220

id-PLMNAssistanceInfoForNetShar						ProtocolIE-ID ::= 221

id-UEContextNotRetrievable							ProtocolIE-ID ::= 222

id-BPLMN-ID-Info-List								ProtocolIE-ID ::= 223

id-SelectedPLMNID									ProtocolIE-ID ::= 224

id-UAC-Assistance-Info								ProtocolIE-ID ::= 225

id-RANUEID											ProtocolIE-ID ::= 226

id-GNB-DU-TNL-Association-To-Remove-Item			ProtocolIE-ID ::= 227

id-GNB-DU-TNL-Association-To-Remove-List			ProtocolIE-ID ::= 228

id-TNLAssociationTransportLayerAddressgNBDU			ProtocolIE-ID ::= 229

id-portNumber										ProtocolIE-ID ::= 230

id-AdditionalSIBMessageList							ProtocolIE-ID ::= 231

id-Cell-Type										ProtocolIE-ID ::= 232

id-IgnorePRACHConfiguration							ProtocolIE-ID ::= 233

id-CG-Config										ProtocolIE-ID ::= 234

id-PDCCH-BlindDetectionSCG							ProtocolIE-ID ::= 235

id-Requested-PDCCH-BlindDetectionSCG				ProtocolIE-ID ::= 236

id-Ph-InfoMCG										ProtocolIE-ID ::= 237

id-MeasGapSharingConfig								ProtocolIE-ID ::= 238

id-systemInformationAreaID							ProtocolIE-ID ::= 239

id-areaScope										ProtocolIE-ID ::= 240

id-RRCContainer-RRCSetupComplete					ProtocolIE-ID ::= 241

id-TraceActivation									ProtocolIE-ID ::= 242

id-TraceID											ProtocolIE-ID ::= 243

id-Neighbour-Cell-Information-List					ProtocolIE-ID ::= 244

id-ProtocolIE-ID-246-not-to-be-used					ProtocolIE-ID ::= 246

id-ProtocolIE-ID-247-not-to-be-used					ProtocolIE-ID ::= 247

id-AdditionalRRMPriorityIndex						ProtocolIE-ID ::= 248

id-DUCURadioInformationType							ProtocolIE-ID ::= 249

id-CUDURadioInformationType 						ProtocolIE-ID ::= 250

id-AggressorgNBSetID								ProtocolIE-ID ::= 251

id-VictimgNBSetID									ProtocolIE-ID ::= 252

id-LowerLayerPresenceStatusChange					ProtocolIE-ID ::= 253

id-Transport-Layer-Address-Info						ProtocolIE-ID ::= 254

id-Neighbour-Cell-Information-Item					ProtocolIE-ID ::= 255

id-IntendedTDD-DL-ULConfig							ProtocolIE-ID ::= 256

id-QosMonitoringRequest								ProtocolIE-ID ::= 257

id-BHChannels-ToBeSetup-List						ProtocolIE-ID ::= 258

id-BHChannels-ToBeSetup-Item						ProtocolIE-ID ::= 259

id-BHChannels-Setup-List							ProtocolIE-ID ::= 260

id-BHChannels-Setup-Item							ProtocolIE-ID ::= 261

id-BHChannels-ToBeModified-Item						ProtocolIE-ID ::= 262

id-BHChannels-ToBeModified-List						ProtocolIE-ID ::= 263

id-BHChannels-ToBeReleased-Item						ProtocolIE-ID ::= 264

id-BHChannels-ToBeReleased-List						ProtocolIE-ID ::= 265

id-BHChannels-ToBeSetupMod-Item						ProtocolIE-ID ::= 266

id-BHChannels-ToBeSetupMod-List						ProtocolIE-ID ::= 267

id-BHChannels-FailedToBeModified-Item				ProtocolIE-ID ::= 268

id-BHChannels-FailedToBeModified-List				ProtocolIE-ID ::= 269

id-BHChannels-FailedToBeSetupMod-Item				ProtocolIE-ID ::= 270

id-BHChannels-FailedToBeSetupMod-List				ProtocolIE-ID ::= 271

id-BHChannels-Modified-Item							ProtocolIE-ID ::= 272

id-BHChannels-Modified-List							ProtocolIE-ID ::= 273

id-BHChannels-SetupMod-Item							ProtocolIE-ID ::= 274

id-BHChannels-SetupMod-List							ProtocolIE-ID ::= 275

id-BHChannels-Required-ToBeReleased-Item			ProtocolIE-ID ::= 276

id-BHChannels-Required-ToBeReleased-List			ProtocolIE-ID ::= 277

id-BHChannels-FailedToBeSetup-Item					ProtocolIE-ID ::= 278

id-BHChannels-FailedToBeSetup-List					ProtocolIE-ID ::= 279

id-BHInfo											ProtocolIE-ID ::= 280

id-BAPAddress										ProtocolIE-ID ::= 281

id-ConfiguredBAPAddress								ProtocolIE-ID ::= 282

id-BH-Routing-Information-Added-List				ProtocolIE-ID ::= 283

id-BH-Routing-Information-Added-List-Item			ProtocolIE-ID ::= 284

id-BH-Routing-Information-Removed-List				ProtocolIE-ID ::= 285

id-BH-Routing-Information-Removed-List-Item			ProtocolIE-ID ::= 286

id-UL-BH-Non-UP-Traffic-Mapping						ProtocolIE-ID ::= 287

id-Activated-Cells-to-be-Updated-List				ProtocolIE-ID ::= 288

id-Child-Nodes-List									ProtocolIE-ID ::= 289

id-IAB-Info-IAB-DU									ProtocolIE-ID ::= 290

id-IAB-Info-IAB-donor-CU							ProtocolIE-ID ::= 291

id-IAB-TNL-Addresses-To-Remove-List					ProtocolIE-ID ::= 292

id-IAB-TNL-Addresses-To-Remove-Item					ProtocolIE-ID ::= 293

id-IAB-Allocated-TNL-Address-List					ProtocolIE-ID ::= 294

id-IAB-Allocated-TNL-Address-Item					ProtocolIE-ID ::= 295

id-IABIPv6RequestType								ProtocolIE-ID ::= 296

id-IABv4AddressesRequested							ProtocolIE-ID ::= 297

id-IAB-Barred										ProtocolIE-ID ::= 298

id-TrafficMappingInformation						ProtocolIE-ID ::= 299

id-UL-UP-TNL-Information-to-Update-List				ProtocolIE-ID ::= 300

id-UL-UP-TNL-Information-to-Update-List-Item		ProtocolIE-ID ::= 301

id-UL-UP-TNL-Address-to-Update-List					ProtocolIE-ID ::= 302

id-UL-UP-TNL-Address-to-Update-List-Item			ProtocolIE-ID ::= 303

id-DL-UP-TNL-Address-to-Update-List					ProtocolIE-ID ::= 304

id-DL-UP-TNL-Address-to-Update-List-Item			ProtocolIE-ID ::= 305

id-NRV2XServicesAuthorized							ProtocolIE-ID ::= 306

id-LTEV2XServicesAuthorized							ProtocolIE-ID ::= 307

id-NRUESidelinkAggregateMaximumBitrate				ProtocolIE-ID ::= 308

id-LTEUESidelinkAggregateMaximumBitrate				ProtocolIE-ID ::= 309

id-SIB12-message									ProtocolIE-ID ::= 310

id-SIB13-message									ProtocolIE-ID ::= 311

id-SIB14-message									ProtocolIE-ID ::= 312

id-SLDRBs-FailedToBeModified-Item					ProtocolIE-ID ::= 313

id-SLDRBs-FailedToBeModified-List					ProtocolIE-ID ::= 314

id-SLDRBs-FailedToBeSetup-Item						ProtocolIE-ID ::= 315

id-SLDRBs-FailedToBeSetup-List						ProtocolIE-ID ::= 316

id-SLDRBs-Modified-Item								ProtocolIE-ID ::= 317

id-SLDRBs-Modified-List								ProtocolIE-ID ::= 318

id-SLDRBs-Required-ToBeModified-Item				ProtocolIE-ID ::= 319

id-SLDRBs-Required-ToBeModified-List				ProtocolIE-ID ::= 320

id-SLDRBs-Required-ToBeReleased-Item				ProtocolIE-ID ::= 321

id-SLDRBs-Required-ToBeReleased-List				ProtocolIE-ID ::= 322

id-SLDRBs-Setup-Item								ProtocolIE-ID ::= 323

id-SLDRBs-Setup-List								ProtocolIE-ID ::= 324

id-SLDRBs-ToBeModified-Item							ProtocolIE-ID ::= 325

id-SLDRBs-ToBeModified-List							ProtocolIE-ID ::= 326

id-SLDRBs-ToBeReleased-Item							ProtocolIE-ID ::= 327

id-SLDRBs-ToBeReleased-List							ProtocolIE-ID ::= 328

id-SLDRBs-ToBeSetup-Item							ProtocolIE-ID ::= 329

id-SLDRBs-ToBeSetup-List							ProtocolIE-ID ::= 330

id-SLDRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 331

id-SLDRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 332

id-SLDRBs-SetupMod-List								ProtocolIE-ID ::= 333

id-SLDRBs-FailedToBeSetupMod-List					ProtocolIE-ID ::= 334

id-SLDRBs-SetupMod-Item								ProtocolIE-ID ::= 335

id-SLDRBs-FailedToBeSetupMod-Item					ProtocolIE-ID ::= 336

id-SLDRBs-ModifiedConf-List							ProtocolIE-ID ::= 337

id-SLDRBs-ModifiedConf-Item							ProtocolIE-ID ::= 338

id-UEAssistanceInformationEUTRA						ProtocolIE-ID ::= 339

id-PC5LinkAMBR										ProtocolIE-ID ::= 340

id-SL-PHY-MAC-RLC-Config							ProtocolIE-ID ::= 341

id-SL-ConfigDedicatedEUTRA-Info						ProtocolIE-ID ::= 342

id-AlternativeQoSParaSetList						ProtocolIE-ID ::= 343

id-CurrentQoSParaSetIndex							ProtocolIE-ID ::= 344

id-gNBCUMeasurementID								ProtocolIE-ID ::= 345

id-gNBDUMeasurementID								ProtocolIE-ID ::= 346

id-RegistrationRequest								ProtocolIE-ID ::= 347

id-ReportCharacteristics							ProtocolIE-ID ::= 348

id-CellToReportList									ProtocolIE-ID ::= 349

id-CellMeasurementResultList						ProtocolIE-ID ::= 350

id-HardwareLoadIndicator							ProtocolIE-ID ::= 351

id-ReportingPeriodicity								ProtocolIE-ID ::= 352

id-TNLCapacityIndicator								ProtocolIE-ID ::= 353

id-CarrierList										ProtocolIE-ID ::= 354

id-ULCarrierList									ProtocolIE-ID ::= 355

id-FrequencyShift7p5khz								ProtocolIE-ID ::= 356

id-SSB-PositionsInBurst								ProtocolIE-ID ::= 357

id-NRPRACHConfig									ProtocolIE-ID ::= 358

id-RAReportList										ProtocolIE-ID ::= 359

id-RLFReportInformationList							ProtocolIE-ID ::= 360

id-TDD-UL-DLConfigCommonNR							ProtocolIE-ID ::= 361

id-CNPacketDelayBudgetDownlink						ProtocolIE-ID ::= 362

id-ExtendedPacketDelayBudget						ProtocolIE-ID ::= 363

id-TSCTrafficCharacteristics						ProtocolIE-ID ::= 364

id-ReportingRequestType								ProtocolIE-ID ::= 365

id-TimeReferenceInformation							ProtocolIE-ID ::= 366

id-CNPacketDelayBudgetUplink						ProtocolIE-ID ::= 369

id-AdditionalPDCPDuplicationTNL-List				ProtocolIE-ID ::= 370

id-RLCDuplicationInformation						ProtocolIE-ID ::= 371

id-AdditionalDuplicationIndication					ProtocolIE-ID ::= 372

id-ConditionalInterDUMobilityInformation			ProtocolIE-ID ::= 373

id-ConditionalIntraDUMobilityInformation			ProtocolIE-ID ::= 374

id-targetCellsToCancel								ProtocolIE-ID ::= 375

id-requestedTargetCellGlobalID						ProtocolIE-ID ::= 376

id-ManagementBasedMDTPLMNList						ProtocolIE-ID ::= 377

id-TraceCollectionEntityIPAddress 					ProtocolIE-ID ::= 378

id-PrivacyIndicator									ProtocolIE-ID ::= 379

id-TraceCollectionEntityURI							ProtocolIE-ID ::= 380

id-mdtConfiguration									ProtocolIE-ID ::= 381

id-ServingNID										ProtocolIE-ID ::= 382

id-NPNBroadcastInformation							ProtocolIE-ID ::= 383

id-NPNSupportInfo									ProtocolIE-ID ::= 384

id-NID												ProtocolIE-ID ::= 385

id-AvailableSNPN-ID-List							ProtocolIE-ID ::= 386

id-SIB10-message									ProtocolIE-ID ::= 387

id-DLCarrierList									ProtocolIE-ID ::= 389

id-ExtendedTAISliceSupportList					ProtocolIE-ID ::= 390

id-RequestedSRSTransmissionCharacteristics			ProtocolIE-ID ::= 391

id-PosAssistance-Information						ProtocolIE-ID ::= 392

id-PosBroadcast										ProtocolIE-ID ::= 393

id-RoutingID										ProtocolIE-ID ::= 394

id-PosAssistanceInformationFailureList				ProtocolIE-ID ::= 395

id-PosMeasurementQuantities							ProtocolIE-ID ::= 396

id-PosMeasurementResultList							ProtocolIE-ID ::= 397

id-TRPInformationTypeListTRPReq						ProtocolIE-ID ::= 398

id-TRPInformationTypeItem							ProtocolIE-ID ::= 399

id-TRPInformationListTRPResp						ProtocolIE-ID ::= 400

id-TRPInformationItem								ProtocolIE-ID ::= 401

id-LMF-MeasurementID								ProtocolIE-ID ::= 402

id-SRSType											ProtocolIE-ID ::= 403

id-ActivationTime									ProtocolIE-ID ::= 404

id-AbortTransmission								ProtocolIE-ID ::= 405

id-PositioningBroadcastCells						ProtocolIE-ID ::= 406

id-SRSConfiguration									ProtocolIE-ID ::= 407

id-PosReportCharacteristics							ProtocolIE-ID ::= 408

id-PosMeasurementPeriodicity						ProtocolIE-ID ::= 409

id-TRPList											ProtocolIE-ID ::= 410

id-RAN-MeasurementID								ProtocolIE-ID ::= 411

id-LMF-UE-MeasurementID								ProtocolIE-ID ::= 412

id-RAN-UE-MeasurementID								ProtocolIE-ID ::= 413

id-E-CID-MeasurementQuantities						ProtocolIE-ID ::= 414

id-E-CID-MeasurementQuantities-Item					ProtocolIE-ID ::= 415

id-E-CID-MeasurementPeriodicity						ProtocolIE-ID ::= 416

id-E-CID-MeasurementResult							ProtocolIE-ID ::= 417

id-Cell-Portion-ID									ProtocolIE-ID ::= 418

id-SFNInitialisationTime							ProtocolIE-ID ::= 419

id-SystemFrameNumber								ProtocolIE-ID ::= 420

id-SlotNumber										ProtocolIE-ID ::= 421

id-TRP-MeasurementRequestList						ProtocolIE-ID ::= 422

id-MeasurementBeamInfoRequest						ProtocolIE-ID ::= 423

id-E-CID-ReportCharacteristics						ProtocolIE-ID ::= 424

id-ConfiguredTACIndication							ProtocolIE-ID ::= 425

id-Extended-GNB-CU-Name								ProtocolIE-ID ::= 426

id-Extended-GNB-DU-Name								ProtocolIE-ID ::= 427

id-F1CTransferPath									ProtocolIE-ID ::= 428

id-SFN-Offset										ProtocolIE-ID ::= 429

id-TransmissionStopIndicator						ProtocolIE-ID ::= 430

id-SrsFrequency										ProtocolIE-ID ::= 431

id-SCGIndicator										ProtocolIE-ID ::= 432

id-EstimatedArrivalProbability						ProtocolIE-ID ::= 433

id-TRPType											ProtocolIE-ID ::= 434

id-SRSSpatialRelationPerSRSResource					ProtocolIE-ID ::= 435

id-PDCPTerminatingNodeDLTNLAddrInfo					ProtocolIE-ID ::= 436

id-ENBDLTNLAddress									ProtocolIE-ID ::= 437

id-PosMeasurementPeriodicityExtended				ProtocolIE-ID ::= 438

id-PRS-Resource-ID									ProtocolIE-ID ::= 439

id-LocationMeasurementInformation					ProtocolIE-ID ::= 440

id-SliceRadioResourceStatus							ProtocolIE-ID ::= 441

id-CompositeAvailableCapacity-SUL					ProtocolIE-ID ::= 442

id-SuccessfulHOReportInformationList				ProtocolIE-ID ::= 443

id-NR-U-Channel-List								ProtocolIE-ID ::= 444

id-NR-U												ProtocolIE-ID ::= 445

id-Coverage-Modification-Notification				ProtocolIE-ID ::= 446

id-CCO-Assistance-Information						ProtocolIE-ID ::= 447

id-ProtocolIE-ID-448-not-to-be-used				ProtocolIE-ID ::= 448

id-CellsForSON-List									ProtocolIE-ID ::= 449

id-MIMOPRBusageInformation							ProtocolIE-ID ::= 450

id-gNB-CU-MBS-F1AP-ID								ProtocolIE-ID ::= 451

id-gNB-DU-MBS-F1AP-ID								ProtocolIE-ID ::= 452

id-ProtocolIE-ID-453-not-to-be-used				ProtocolIE-ID ::= 453

id-MBS-CUtoDURRCInformation							ProtocolIE-ID ::= 454

id-MBS-Session-ID									ProtocolIE-ID ::= 455

id-SNSSAI											ProtocolIE-ID ::= 456

id-MBS-Broadcast-NeighbourCellList					ProtocolIE-ID ::= 457

id-BroadcastMRBs-FailedToBeModified-List			ProtocolIE-ID ::= 458

id-BroadcastMRBs-FailedToBeModified-Item			ProtocolIE-ID ::= 459

id-BroadcastMRBs-FailedToBeSetup-List				ProtocolIE-ID ::= 460

id-BroadcastMRBs-FailedToBeSetup-Item				ProtocolIE-ID ::= 461

id-BroadcastMRBs-FailedToBeSetupMod-List			ProtocolIE-ID ::= 462

id-BroadcastMRBs-FailedToBeSetupMod-Item			ProtocolIE-ID ::= 463

id-BroadcastMRBs-Modified-List						ProtocolIE-ID ::= 464

id-BroadcastMRBs-Modified-Item						ProtocolIE-ID ::= 465

id-BroadcastMRBs-Setup-List							ProtocolIE-ID ::= 466

id-BroadcastMRBs-Setup-Item							ProtocolIE-ID ::= 467

id-BroadcastMRBs-SetupMod-List						ProtocolIE-ID ::= 468

id-BroadcastMRBs-SetupMod-Item						ProtocolIE-ID ::= 469

id-BroadcastMRBs-ToBeModified-List					ProtocolIE-ID ::= 470

id-BroadcastMRBs-ToBeModified-Item					ProtocolIE-ID ::= 471

id-BroadcastMRBs-ToBeReleased-List					ProtocolIE-ID ::= 472

id-BroadcastMRBs-ToBeReleased-Item					ProtocolIE-ID ::= 473

id-BroadcastMRBs-ToBeSetup-List						ProtocolIE-ID ::= 474

id-BroadcastMRBs-ToBeSetup-Item						ProtocolIE-ID ::= 475

id-BroadcastMRBs-ToBeSetupMod-List					ProtocolIE-ID ::= 476

id-BroadcastMRBs-ToBeSetupMod-Item					ProtocolIE-ID ::= 477

id-Supported-MBS-FSA-ID-List						ProtocolIE-ID ::= 478

id-UEIdentity-List-For-Paging-List 					ProtocolIE-ID ::= 479

id-UEIdentity-List-For-Paging-Item 					ProtocolIE-ID ::= 480

id-MBS-ServiceArea									ProtocolIE-ID ::= 481

id-MulticastMRBs-FailedToBeModified-List			ProtocolIE-ID ::= 482

id-MulticastMRBs-FailedToBeModified-Item			ProtocolIE-ID ::= 483

id-MulticastMRBs-FailedToBeSetup-List				ProtocolIE-ID ::= 484

id-MulticastMRBs-FailedToBeSetup-Item				ProtocolIE-ID ::= 485

id-MulticastMRBs-FailedToBeSetupMod-List			ProtocolIE-ID ::= 486

id-MulticastMRBs-FailedToBeSetupMod-Item			ProtocolIE-ID ::= 487

id-MulticastMRBs-Modified-List						ProtocolIE-ID ::= 488

id-MulticastMRBs-Modified-Item						ProtocolIE-ID ::= 489

id-MulticastMRBs-Setup-List							ProtocolIE-ID ::= 490

id-MulticastMRBs-Setup-Item							ProtocolIE-ID ::= 491

id-MulticastMRBs-SetupMod-List						ProtocolIE-ID ::= 492

id-MulticastMRBs-SetupMod-Item						ProtocolIE-ID ::= 493

id-MulticastMRBs-ToBeModified-List					ProtocolIE-ID ::= 494

id-MulticastMRBs-ToBeModified-Item					ProtocolIE-ID ::= 495

id-MulticastMRBs-ToBeReleased-List					ProtocolIE-ID ::= 496

id-MulticastMRBs-ToBeReleased-Item					ProtocolIE-ID ::= 497

id-MulticastMRBs-ToBeSetup-List						ProtocolIE-ID ::= 498

id-MulticastMRBs-ToBeSetup-Item						ProtocolIE-ID ::= 499

id-MulticastMRBs-ToBeSetupMod-List					ProtocolIE-ID ::= 500

id-MulticastMRBs-ToBeSetupMod-Item					ProtocolIE-ID ::= 501

id-MBSMulticastF1UContextDescriptor					ProtocolIE-ID ::= 502

id-MulticastF1UContext-ToBeSetup-List				ProtocolIE-ID ::= 503

id-MulticastF1UContext-ToBeSetup-Item				ProtocolIE-ID ::= 504

id-MulticastF1UContext-Setup-List					ProtocolIE-ID ::= 505

id-MulticastF1UContext-Setup-Item					ProtocolIE-ID ::= 506

id-MulticastF1UContext-FailedToBeSetup-List			ProtocolIE-ID ::= 507

id-MulticastF1UContext-FailedToBeSetup-Item			ProtocolIE-ID ::= 508

id-IABCongestionIndication							ProtocolIE-ID ::= 509

id-IABConditionalRRCMessageDeliveryIndication		ProtocolIE-ID ::= 510

id-F1CTransferPathNRDC								ProtocolIE-ID ::= 511

id-BufferSizeThresh 								ProtocolIE-ID ::= 512

id-IAB-TNL-Addresses-Exception						ProtocolIE-ID ::= 513

id-BAP-Header-Rewriting-Added-List					ProtocolIE-ID ::= 514

id-BAP-Header-Rewriting-Added-List-Item				ProtocolIE-ID ::= 515

id-Re-routingEnableIndicator						ProtocolIE-ID ::= 516

id-NonF1terminatingTopologyIndicator				ProtocolIE-ID ::= 517

id-EgressNonF1terminatingTopologyIndicator			ProtocolIE-ID ::= 518

id-IngressNonF1terminatingTopologyIndicator			ProtocolIE-ID ::= 519

id-rBSetConfiguration 								ProtocolIE-ID ::= 520

id-frequency-Domain-HSNA-Configuration-List			ProtocolIE-ID ::= 521

id-child-IAB-Nodes-NA-Resource-List					ProtocolIE-ID ::= 522

id-Parent-IAB-Nodes-NA-Resource-Configuration-List	ProtocolIE-ID ::= 523

id-uL-FreqInfo										ProtocolIE-ID ::= 524

id-uL-Transmission-Bandwidth						ProtocolIE-ID ::= 525

id-dL-FreqInfo										ProtocolIE-ID ::= 526

id-dL-Transmission-Bandwidth						ProtocolIE-ID ::= 527

id-uL-NR-Carrier-List								ProtocolIE-ID ::= 528

id-dL-NR-Carrier-List								ProtocolIE-ID ::= 529

id-nRFreqInfo										ProtocolIE-ID ::= 530

id-transmission-Bandwidth							ProtocolIE-ID ::= 531

id-nR-Carrier-List									ProtocolIE-ID ::= 532

id-Neighbour-Node-Cells-List						ProtocolIE-ID ::= 533

id-Serving-Cells-List								ProtocolIE-ID ::= 534

id-permutation										ProtocolIE-ID ::= 535

id-MDTPollutedMeasurementIndicator					ProtocolIE-ID ::= 536

id-M5ReportAmount 									ProtocolIE-ID ::= 537

id-M6ReportAmount 									ProtocolIE-ID ::= 538

id-M7ReportAmount 									ProtocolIE-ID ::= 539

id-SurvivalTime										ProtocolIE-ID ::= 540

id-PDCMeasurementPeriodicity						ProtocolIE-ID ::= 541

id-PDCMeasurementQuantities							ProtocolIE-ID ::= 542

id-PDCMeasurementQuantities-Item					ProtocolIE-ID ::= 543

id-PDCMeasurementResult								ProtocolIE-ID ::= 544

id-PDCReportType									ProtocolIE-ID ::= 545

id-RAN-UE-PDC-MeasID								ProtocolIE-ID ::= 546

id-SCGActivationRequest								ProtocolIE-ID ::= 547

id-SCGActivationStatus								ProtocolIE-ID ::= 548

id-PRSTRPList										ProtocolIE-ID ::= 549

id-PRSTransmissionTRPList							ProtocolIE-ID ::= 550

id-OnDemandPRS										ProtocolIE-ID ::= 551

id-AoA-SearchWindow									ProtocolIE-ID ::= 552

id-TRP-MeasurementUpdateList						ProtocolIE-ID ::= 553

id-ZoAInformation									ProtocolIE-ID ::= 554

id-ResponseTime										ProtocolIE-ID ::= 555

id-ARPLocationInfo									ProtocolIE-ID ::= 556

id-ARP-ID											ProtocolIE-ID ::= 557

id-MultipleULAoA									ProtocolIE-ID ::= 558

id-UL-SRS-RSRPP										ProtocolIE-ID ::= 559

id-SRSResourcetype									ProtocolIE-ID ::= 560

id-ExtendedAdditionalPathList						ProtocolIE-ID ::= 561

id-LoS-NLoSInformation								ProtocolIE-ID ::= 562

id-NumberOfTRPRxTEG									ProtocolIE-ID ::= 564

id-NumberOfTRPRxTxTEG								ProtocolIE-ID ::= 565

id-TRPTxTEGAssociation								ProtocolIE-ID ::= 566

id-TRPTEGInformation								ProtocolIE-ID ::= 567

id-TRPRx-TEGInformation								ProtocolIE-ID ::= 568

id-TRP-PRS-Info-List								ProtocolIE-ID ::= 569

id-PRS-Measurement-Info-List						ProtocolIE-ID ::= 570

id-PRSConfigRequestType								ProtocolIE-ID ::= 571

id-MeasurementTimeOccasion							ProtocolIE-ID ::= 573

id-MeasurementCharacteristicsRequestIndicator		ProtocolIE-ID ::= 574

id-UEReportingInformation							ProtocolIE-ID ::= 575

id-PosContextRevIndication							ProtocolIE-ID ::= 576

id-TRPBeamAntennaInformation						ProtocolIE-ID ::= 577

id-NRRedCapUEIndication 							ProtocolIE-ID ::= 578

id-Redcap-Bcast-Information							ProtocolIE-ID ::= 579

id-RANUEPagingDRX									ProtocolIE-ID ::= 580

id-CNUEPagingDRX									ProtocolIE-ID ::= 581

id-NRPagingeDRXInformation							ProtocolIE-ID ::= 582

id-NRPagingeDRXInformationforRRCINACTIVE			ProtocolIE-ID ::= 583

id-NR-TADV											ProtocolIE-ID ::= 584

id-QoEInformation									ProtocolIE-ID ::= 585

id-CG-SDTQueryIndication							ProtocolIE-ID ::= 586

id-SDT-MAC-PHY-CG-Config							ProtocolIE-ID ::= 587

id-CG-SDTKeptIndicator								ProtocolIE-ID ::= 588

id-CG-SDTindicatorSetup								ProtocolIE-ID ::= 589

id-CG-SDTindicatorMod								ProtocolIE-ID ::= 590

id-CG-SDTSessionInfoOld								ProtocolIE-ID ::= 591

id-SDTInformation									ProtocolIE-ID ::= 592

id-SDTRLCBearerConfiguration						ProtocolIE-ID ::= 593

id-FiveG-ProSeAuthorized							ProtocolIE-ID ::= 594

id-FiveG-ProSeUEPC5AggregateMaximumBitrate			ProtocolIE-ID ::= 595

id-FiveG-ProSePC5LinkAMBR	 						ProtocolIE-ID ::= 596

id-SRBMappingInfo									ProtocolIE-ID ::= 597

id-DRBMappingInfo									ProtocolIE-ID ::= 598

id-UuRLCChannelToBeSetupList						ProtocolIE-ID ::= 599

id-UuRLCChannelToBeModifiedList						ProtocolIE-ID ::= 600

id-UuRLCChannelToBeReleasedList						ProtocolIE-ID ::= 601

id-UuRLCChannelSetupList							ProtocolIE-ID ::= 602

id-UuRLCChannelFailedToBeSetupList					ProtocolIE-ID ::= 603

id-UuRLCChannelModifiedList							ProtocolIE-ID ::= 604

id-UuRLCChannelFailedToBeModifiedList				ProtocolIE-ID ::= 605

id-UuRLCChannelRequiredToBeModifiedList				ProtocolIE-ID ::= 606

id-UuRLCChannelRequiredToBeReleasedList				ProtocolIE-ID ::= 607

id-PC5RLCChannelToBeSetupList						ProtocolIE-ID ::= 608

id-PC5RLCChannelToBeModifiedList					ProtocolIE-ID ::= 609

id-PC5RLCChannelToBeReleasedList					ProtocolIE-ID ::= 610

id-PC5RLCChannelSetupList							ProtocolIE-ID ::= 611

id-PC5RLCChannelFailedToBeSetupList					ProtocolIE-ID ::= 612

id-PC5RLCChannelFailedToBeModifiedList				ProtocolIE-ID ::= 613

id-PC5RLCChannelRequiredToBeModifiedList			ProtocolIE-ID ::= 614

id-PC5RLCChannelRequiredToBeReleasedList			ProtocolIE-ID ::= 615

id-PC5RLCChannelModifiedList						ProtocolIE-ID ::= 616

id-SidelinkRelayConfiguration						ProtocolIE-ID ::= 617

id-UpdatedRemoteUELocalID							ProtocolIE-ID ::= 618

id-PathSwitchConfiguration							ProtocolIE-ID ::= 619

id-PagingCause										ProtocolIE-ID ::= 620

id-MUSIM-GapConfig									ProtocolIE-ID ::= 621

id-PEIPSAssistanceInfo								ProtocolIE-ID ::= 622

id-UEPagingCapability								ProtocolIE-ID ::= 623

id-LastUsedCellIndication							ProtocolIE-ID ::= 624

id-SIB17-message									ProtocolIE-ID ::= 625

id-GNBDUUESliceMaximumBitRateList					ProtocolIE-ID ::= 626

id-SIB20-message									ProtocolIE-ID ::= 627

id-UE-MulticastMRBs-ToBeReleased-List				ProtocolIE-ID ::= 628

id-UE-MulticastMRBs-ToBeReleased-Item				ProtocolIE-ID ::= 629

id-UE-MulticastMRBs-ToBeSetup-List					ProtocolIE-ID ::= 630

id-UE-MulticastMRBs-ToBeSetup-Item					ProtocolIE-ID ::= 631

id-MulticastMBSSessionSetupList						ProtocolIE-ID ::= 632

id-MulticastMBSSessionRemoveList					ProtocolIE-ID ::= 633

id-PosMeasurementAmount								ProtocolIE-ID ::= 634

id-SDT-Termination-Request							ProtocolIE-ID ::= 635

id-pathPower										ProtocolIE-ID ::= 636

id-DU-RX-MT-RX-Extend								ProtocolIE-ID ::= 637

id-DU-TX-MT-TX-Extend								ProtocolIE-ID ::= 638

id-DU-RX-MT-TX-Extend								ProtocolIE-ID ::= 639

id-DU-TX-MT-RX-Extend								ProtocolIE-ID ::= 640

id-BAP-Header-Rewriting-Removed-List				ProtocolIE-ID ::= 641

id-BAP-Header-Rewriting-Removed-List-Item			ProtocolIE-ID ::= 642

id-SLDRXCycleList									ProtocolIE-ID ::= 643

id-TAINSAGSupportList								ProtocolIE-ID ::= 644

id-SL-RLC-ChannelToAddModList						ProtocolIE-ID ::= 645

id-BroadcastAreaScope								ProtocolIE-ID ::= 646

id-ManagementBasedMDTPLMNModificationList 			ProtocolIE-ID ::= 647

id-SIB15-message									ProtocolIE-ID ::= 648

id-ActivationRequestType							ProtocolIE-ID ::= 649

id-PosMeasGapPreConfigList							ProtocolIE-ID ::= 650

id-InterFrequencyConfig-NoGap						ProtocolIE-ID ::= 651

id-MBSInterestIndication							ProtocolIE-ID ::= 652

id-UE-MulticastMRBs-ConfirmedToBeModified-List		ProtocolIE-ID ::= 653

id-UE-MulticastMRBs-ConfirmedToBeModified-Item		ProtocolIE-ID ::= 654

id-UE-MulticastMRBs-RequiredToBeModified-List		ProtocolIE-ID ::= 655

id-UE-MulticastMRBs-RequiredToBeModified-Item		ProtocolIE-ID ::= 656

id-UE-MulticastMRBs-RequiredToBeReleased-List		ProtocolIE-ID ::= 657

id-UE-MulticastMRBs-RequiredToBeReleased-Item		ProtocolIE-ID ::= 658

id-L571Info											ProtocolIE-ID ::= 659

id-L1151Info										ProtocolIE-ID ::= 660

id-SCS-480											ProtocolIE-ID ::= 661

id-SCS-960											ProtocolIE-ID ::= 662

id-SRSPortIndex										ProtocolIE-ID ::= 663

id-PEISubgroupingSupportIndication					ProtocolIE-ID ::= 664

id-NeedForGapsInfoNR								ProtocolIE-ID ::= 665

id-NeedForGapNCSGInfoNR								ProtocolIE-ID ::= 666

id-NeedForGapNCSGInfoEUTRA							ProtocolIE-ID ::= 667

id-ProtocolIE-ID-668-not-to-be-used				ProtocolIE-ID ::= 668

id-ProtocolIE-ID-669-not-to-be-used				ProtocolIE-ID ::= 669

id-ProtocolIE-ID-670-not-to-be-used				ProtocolIE-ID ::= 670

id-Source-MRB-ID									ProtocolIE-ID ::= 671

id-PosMeasurementPeriodicityNR-AoA					ProtocolIE-ID ::= 672

id-RedCapIndication									ProtocolIE-ID ::= 673

id-SRSPosRRCInactiveConfig							ProtocolIE-ID ::= 674

id-SDTBearerConfigurationQueryIndication			ProtocolIE-ID ::= 675

id-SDTBearerConfigurationInfo						ProtocolIE-ID ::= 676

id-UL-GapFR2-Config									ProtocolIE-ID ::= 677

id-ConfigRestrictInfoDAPS							ProtocolIE-ID ::= 678

id-UE-MulticastMRBs-Setup-List						ProtocolIE-ID ::= 679

id-UE-MulticastMRBs-Setup-Item						ProtocolIE-ID ::= 680

id-MulticastF1UContextReferenceCU					ProtocolIE-ID ::= 681

id-PosSItypeList									ProtocolIE-ID ::= 682

id-DAPS-HO-Status									ProtocolIE-ID ::= 683

id-UplinkTxDirectCurrentTwoCarrierListInfo			ProtocolIE-ID ::= 684

id-UE-MulticastMRBs-ToBeSetup-atModify-List			ProtocolIE-ID ::= 685

id-UE-MulticastMRBs-ToBeSetup-atModify-Item			ProtocolIE-ID ::= 686

id-MC-PagingCell-List								ProtocolIE-ID ::= 687

id-MC-PagingCell-Item								ProtocolIE-ID ::= 688

id-SRSPosRRCInactiveQueryIndication					ProtocolIE-ID ::= 689

id-UlTxDirectCurrentMoreCarrierInformation	        ProtocolIE-ID ::= 690

id-CPACMCGInformation								ProtocolIE-ID ::= 691

id-TwoPHRModeMCG									ProtocolIE-ID ::= 692

id-TwoPHRModeSCG									ProtocolIE-ID ::= 693

id-ExtendedUEIdentityIndexValue						ProtocolIE-ID ::= 694

id-ServingCellMO-List								ProtocolIE-ID ::= 695

id-ServingCellMO-List-Item							ProtocolIE-ID ::= 696

id-ServingCellMO-encoded-in-CGC-List				ProtocolIE-ID ::= 697

id-HashedUEIdentityIndexValue						ProtocolIE-ID ::= 698

id-UE-MulticastMRBs-Setupnew-List					ProtocolIE-ID ::= 699

id-UE-MulticastMRBs-Setupnew-Item					ProtocolIE-ID ::= 700

id-ncd-SSB-RedCapInitialBWP-SDT						ProtocolIE-ID ::= 701

id-nrofSymbolsExtended								ProtocolIE-ID ::= 702

id-repetitionFactorExtended							ProtocolIE-ID ::= 703

id-startRBHopping									ProtocolIE-ID ::= 704

id-startRBIndex										ProtocolIE-ID ::= 705

id-transmissionCombn8								ProtocolIE-ID ::= 706

id-ServCellInfoList									ProtocolIE-ID ::= 707

id-DedicatedSIDeliveryIndication					ProtocolIE-ID ::= 708

id-Configured-BWP-List								ProtocolIE-ID ::= 709

id-Preconfigured-measurement-GAP-Request			ProtocolIE-ID ::= 710

id-BWP-Id											ProtocolIE-ID ::= 711

id-NetworkControlledRepeaterAuthorized				ProtocolIE-ID ::= 712

id-MT-SDT-Information								ProtocolIE-ID ::= 713

id-ExtendedResourceSymbolOffset						ProtocolIE-ID ::= 714

id-NeedForInterruptionInfoNR						ProtocolIE-ID ::= 715

id-SDT-Volume-Threshold								ProtocolIE-ID ::= 716

id-SupportedUETypeList								ProtocolIE-ID ::= 717

id-MusimCapabilityRestrictionIndication				ProtocolIE-ID ::= 718

id-duplicationIndication							ProtocolIE-ID ::= 719

id-LTMInformation-Setup								ProtocolIE-ID ::= 720

id-LTMConfigurationIDMappingList 					ProtocolIE-ID ::= 721

id-LTMInformation-Modify							ProtocolIE-ID ::= 722

id-LTMCells-ToBeReleased-List						ProtocolIE-ID ::= 723

id-ProtocolIE-ID-724-not-to-be-used						ProtocolIE-ID ::= 724

id-LTMConfiguration									ProtocolIE-ID ::= 725

id-EarlySyncInformation-Request						ProtocolIE-ID ::= 726

id-EarlySyncInformation								ProtocolIE-ID ::= 727

id-EarlySyncCandidateCellInformation-List			ProtocolIE-ID ::= 728

id-LTMCellSwitchInformation							ProtocolIE-ID ::= 729

id-DUtoCUTAInformation-List							ProtocolIE-ID ::= 730

id-ProtocolIE-ID-731-not-to-be-used					ProtocolIE-ID ::= 731

id-dRB-List											ProtocolIE-ID ::= 732

id-DeactivationIndication							ProtocolIE-ID ::= 733

id-RAReportIndicationList							ProtocolIE-ID ::= 734

id-ChannelOccupancyTimePercentageUL					ProtocolIE-ID ::= 735

id-SuccessfulPSCellChangeReportInformationList		ProtocolIE-ID ::= 736

id-RadioResourceStatusNR-U							ProtocolIE-ID ::= 737

id-FiveG-ProSeLayer2Multipath						ProtocolIE-ID ::= 738

id-FiveG-ProSeLayer2UEtoUERelay						ProtocolIE-ID ::= 739

id-FiveG-ProSeLayer2UEtoUERemote					ProtocolIE-ID ::= 740

id-PathAdditionInformation							ProtocolIE-ID ::= 741

id-Recommended-SSBs-List							ProtocolIE-ID ::= 742

id-Recommended-SSBs-for-Paging-List					ProtocolIE-ID ::= 743

id-SSBs-withinTheCell-tobe-Activated-List			ProtocolIE-ID ::= 744

id-Cells-With-SSBs-Activated-List					ProtocolIE-ID ::= 745

id-Cells-Allowed-to-be-Deactivated-List				ProtocolIE-ID ::= 746

id-Cells-Allowed-to-be-Deactivated-List-Item		ProtocolIE-ID ::= 747

id-Coverage-Modification-Cause						ProtocolIE-ID ::= 748

id-RANTSSRequestType								ProtocolIE-ID ::= 749

id-RANTimingSynchronisationStatusInfo				ProtocolIE-ID ::= 750

id-TSCTrafficCharacteristicsFeedback				ProtocolIE-ID ::= 751

id-RANfeedbacktype									ProtocolIE-ID ::= 752

id-Mobile-TRP-LocationInformation					ProtocolIE-ID ::= 753

id-Mobile-IAB-MT-UE-ID								ProtocolIE-ID ::= 754

id-Target-gNB-ID									ProtocolIE-ID ::= 755

id-Target-gNB-IP-address							ProtocolIE-ID ::= 756

id-Target-SeGW-IP-address							ProtocolIE-ID ::= 757

id-Activated-Cells-Mapping-List						ProtocolIE-ID ::= 758

id-Activated-Cells-Mapping-List-Item				ProtocolIE-ID ::= 759

id-F1SetupOutcome									ProtocolIE-ID ::= 760

id-RRC-Terminating-IAB-Donor-Related-Info			ProtocolIE-ID ::= 761

id-RRC-Terminating-IAB-Donor-gNB-ID					ProtocolIE-ID ::= 762

id-NCGI-to-be-Updated-List							ProtocolIE-ID ::= 763

id-NCGI-to-be-Updated-List-Item						ProtocolIE-ID ::= 764

id-Mobile-IAB-MTUserLocationInformation				ProtocolIE-ID ::= 765

id-MobileAccessPointLocation						ProtocolIE-ID ::= 766

id-AssociatedSessionID								ProtocolIE-ID ::= 767

id-IndicationMCInactiveReception					ProtocolIE-ID ::= 768

id-MulticastCU2DURRCInfo							ProtocolIE-ID ::= 769

id-MBSMulticastSessionReceptionState				ProtocolIE-ID ::= 770

id-F1UTunnelNotEstablished							ProtocolIE-ID ::= 771

id-MulticastDU2CURRCInfo							ProtocolIE-ID ::= 772

id-SIB24-message										ProtocolIE-ID ::= 773

id-MulticastCU2DUCommonRRCInfo						ProtocolIE-ID ::= 774

id-PDUSetQoSParameters								ProtocolIE-ID ::= 775

id-N6JitterInformation								ProtocolIE-ID ::= 776

id-ECNMarkingorCongestionInformationReportingRequest	ProtocolIE-ID ::= 777

id-ECNMarkingorCongestionInformationReportingStatus	ProtocolIE-ID ::= 778

id-NRA2XServicesAuthorized							ProtocolIE-ID ::= 779

id-LTEA2XServicesAuthorized							ProtocolIE-ID ::= 780

id-NRUESidelinkAggregateMaximumBitrateForA2X		ProtocolIE-ID ::= 781

id-LTEUESidelinkAggregateMaximumBitrateForA2X		ProtocolIE-ID ::= 782

id-NReRedCapUEIndication 							ProtocolIE-ID ::= 783

id-ERedcap-Bcast-Information 						ProtocolIE-ID ::= 784

id-NRPaginglongeDRXInformationforRRCINACTIVE		ProtocolIE-ID ::= 785

id-SCPAC-Request									ProtocolIE-ID ::= 786

id-Target-F1-Terminating-Donor-gNB-ID				ProtocolIE-ID ::= 787

id-MobileIAB-Barred									ProtocolIE-ID ::= 788

id-Broadcast-MRBs-Transport-Request-List 			ProtocolIE-ID ::= 789

id-Broadcast-MRBs-Transport-Request-Item			ProtocolIE-ID ::= 790

id-S-CPACLowerLayerReferenceConfigRequest			ProtocolIE-ID ::= 791

id-S-CPAC-Configuration								ProtocolIE-ID ::= 792

id-MusimCandidateBandList							ProtocolIE-ID ::= 793

id-DLLBTFailureInformationRequest					ProtocolIE-ID ::= 794

id-DLLBTFailureInformationList						ProtocolIE-ID ::= 795

id-PSIbasedSDUdiscardUL								ProtocolIE-ID ::= 796

id-SIB22-message 									ProtocolIE-ID ::= 797

id-CUtoDUTAInformation-List							ProtocolIE-ID ::= 798

id-U2URLCChannelQoS								ProtocolIE-ID ::= 799

id-SL-PHY-MAC-RLC-ConfigExt							ProtocolIE-ID ::= 800

id-SLPositioning-Ranging-Service-Info				ProtocolIE-ID ::= 801

id-TimeWindowInformation-SRS-List					ProtocolIE-ID ::= 802

id-TimeWindowInformation-Measurement-List			ProtocolIE-ID ::= 803

id-UL-RSCP											ProtocolIE-ID ::= 804

id-BW-Aggregation-Request-Indication				ProtocolIE-ID ::= 805

id-ReportingGranularitykminus1						ProtocolIE-ID ::= 806

id-ReportingGranularitykminus2						ProtocolIE-ID ::= 807

id-ReportingGranularitykminus1additionalpath		ProtocolIE-ID ::= 808

id-ReportingGranularitykminus2additionalpath		ProtocolIE-ID ::= 809

id-TimingReportingGranularityFactorExtended			ProtocolIE-ID ::= 810

id-SRSPosRRCInactiveValidityAreaConfig				ProtocolIE-ID ::= 811

id-PosValidityAreaCellList							ProtocolIE-ID ::= 812

id-SRSReservationType							ProtocolIE-ID ::= 813

id-SymbolIndex										ProtocolIE-ID ::= 814

id-PRSBandwidthAggregationRequestIndication			ProtocolIE-ID ::= 815

id-AggregatedPosSRSResourceIDList					ProtocolIE-ID ::= 816

id-AggregatedPRSResourceSetList						ProtocolIE-ID ::= 817

id-PhaseQuality										ProtocolIE-ID ::= 818

id-MeasuredFrequencyHops							ProtocolIE-ID ::= 819

id-TxHoppingConfiguration							ProtocolIE-ID ::= 820

id-ReportingGranularitykminus3						ProtocolIE-ID ::= 821

id-ReportingGranularitykminus4						ProtocolIE-ID ::= 822

id-ReportingGranularitykminus5						ProtocolIE-ID ::= 823

id-ReportingGranularitykminus6						ProtocolIE-ID ::= 824

id-ReportingGranularitykminus3additionalpath		ProtocolIE-ID ::= 825

id-ReportingGranularitykminus4additionalpath		ProtocolIE-ID ::= 826

id-ReportingGranularitykminus5additionalpath		ProtocolIE-ID ::= 827

id-ReportingGranularitykminus6additionalpath		ProtocolIE-ID ::= 828

id-AggregatedPosSRSResourceSetList					ProtocolIE-ID ::= 829

id-RequestedSRSPreconfigurationCharacteristics-List	ProtocolIE-ID ::= 830

id-SRSPreconfiguration-List							ProtocolIE-ID ::= 831

id-SRSInformation									ProtocolIE-ID ::= 832

id-ValidityAreaSpecificSRSInformation				ProtocolIE-ID ::= 833

id-E-CID-MeasuredResultsAssociatedInfoList			ProtocolIE-ID ::= 834

id-XR-Bcast-Information								ProtocolIE-ID ::= 835

id-MaxDataBurstVolume 								ProtocolIE-ID ::= 836

id-TAInformation-List								ProtocolIE-ID ::= 837

id-NonIntegerDRXCycle								ProtocolIE-ID ::= 838

id-PointA											ProtocolIE-ID ::= 839

id-SCS-SpecificCarrier								ProtocolIE-ID ::= 840

id-NR-PCI											ProtocolIE-ID ::= 841

id-PeerUE-ID											ProtocolIE-ID ::= 842

id-EarlySyncServingCellInformation					ProtocolIE-ID ::= 843

id-RANSharingAssistanceInformation					ProtocolIE-ID ::= 844

id-LTMCFRAResourceConfig-List						ProtocolIE-ID ::= 845

id-F1U-PathFailure									ProtocolIE-ID ::= 846

id-MeasBasedOnAggregatedResources					ProtocolIE-ID ::= 847

id-SIB23-message									ProtocolIE-ID ::= 848

id-BarringExemptionforEmerCallInfo					ProtocolIE-ID ::= 849

id-SIB17bis-message									ProtocolIE-ID ::= 850

id-ReportingIntervalIMs								ProtocolIE-ID ::= 851

id-Transmission-Bandwidth-asymmetric				ProtocolIE-ID ::= 852

id-TagIDPointer										ProtocolIE-ID ::= 853

id-LocalOrigin										ProtocolIE-ID ::= 854

id-LTMResetInformation								ProtocolIE-ID ::= 855

id-SRSPosPeriodicConfigHyperSFNIndex				ProtocolIE-ID ::= 856

id-PreconfiguredSRSInformation						ProtocolIE-ID ::= 857

id-MobilityTriggeringIndication						ProtocolIE-ID ::= xx1

id-MobilityAssistanceInfo-CellSwitch				ProtocolIE-ID ::= xx2

id-MobilityAssistanceInfo-EarlyULSync				ProtocolIE-ID ::= xx3

id-MobilityAssistanceInfo-EarlyDLSync				ProtocolIE-ID ::= xx4

id-MobilityAssistanceInfo-Additional				ProtocolIE-ID ::= xx5

END

-- ASN1STOP