3GPP TSG-RAN WG3 Meeting #128	R3-253406

St.Julian, Malta, May 19 - 23, 2025

Title:	Discussion on L3 measurement based LTM

Agenda Item:	9.2

Source:	CEWiT

Document for:	Discussion/Agreement

- Introduction

In RAN#107, L3 measurement support for LTM was discussed and the following agreement was made providing the following guidance to RAN3.

RAN kindly requests RAN3 to agree by RAN#108, a single standard solution as Rel-18 correction, to support the L3 measurement based LTM over the F1 interface (see WI Code above).

RAN3#127bis Agreements:

CU triggers the new procedure with the L3 measurement information and trigger indicator towards DU for early synchronization and cell switch.

DU takes it into account if supported.

FFS on the presence and details of the L3 measurement information and trigger indicator.

SSB Index list is per candidate Cell.

FFS: It has to be clarified if CU-DU Mobility Initiation message includes CSI-RS Index List.

To be continued...

R3-252412 : Taking this CR as the baseline, the FFSes and the measurements structure to be continued…

In this document, we will be looking into whether the CU-DU mobility initiation message to include CSI-RS index list or not. We will also explain the presence and details of L3 measurement information and trigger indicator.

- Discussion

The R18 LTM was designed with SSB based L1 measurements from the UE. Considering the unfiltered nature of the measurements, this was intended to make the handover decision more reliable, avoiding ping-pong after the handover.

Also considering the amount of time we have for the enhancement of bringing in the L3 measurement support for the R18 LTM in the agenda, it is desired not to opt for CSI-RS measurements, just because the L3 measurements support it.

This will lead to more impact in the specification work and may not necessarily improve the performance of the handover. So we intend not to support CSI-RS based L3 measurements for R18 LTM.

Proposal 1: RAN3 to agree that SSB measurements are sufficient. Need not have CSI-RS measurements as a part of R18 LTM correction for L3 measurement-based LTM

On the contrary, if we continue to bring the support for L3 measurements in R19 LTM, considering the framework already supports CSI-RS based L1 measurements, bringing in CSI-RS based L3 measurements is acceptable. Hence for R19 LTM, we propose the option to have CSI-RS index list in the F1AP message.

Proposal 2 : RAN3 to consider for R19 LTM, CSI-RS index to be included in the Mobility Assistance Information Cell Switch IE along with Candidate ID and SSB index

For the L3 measurement-based LTM, the CU must send a triggering indication every time it initiates the message. DU, however, still has the discretion to use this information for decision making.

Proposal 3 : RAN3 to agree that triggering indication would have mandatory presence in the mobility initiation request.

Similarly, the L3 measurement information would also contain the essential information such as RSRP, RSRQ, as mandatory parameters. SINR could be optional based on availability.

Proposal 4 : RSRP, RSRQ will be mandatory IEs in measurement information. SINR could be optional.

- Conclusion

Below are the proposals we make:

Proposal 1: RAN3 to agree that SSB measurements are sufficient. Need not have CSI-RS measurements as a part of R18 LTM correction for L3 measurement-based LTM

Proposal 2 : RAN3 to consider for R19 LTM, CSI-RS index to be included in the Mobility Assistance Information Cell Switch IE along with Candidate ID and SSB index

Proposal 3 : RAN3 to agree that triggering indication would have mandatory presence in the mobility initiation request.

Proposal 4 : RSRP, RSRQ will be mandatory IEs in measurement information. SINR could be optional.

- References

- Baseline CR R3-252412

- Meeting report of RAN3#127bis