3GPP TSG-RAN WG3 Meeting #128	R3-253407

Malta, MT, 19 – 23 May, 2025

Title: 	Further discussion on common signal adaptation

Source: 	Huawei, NEC, CATT, CMCC

Agenda item:	17.4

Document Type:	 Discussion

# 1. Introduction

In this contribution, we provide our further considerations on the common signal/channel adaptation, including SSB, PRACH and paging aspects.

# 2. Discussion

## 2.1 Paging adaptation

The previous RAN3-127bis meeting made the following agreement, based on the RAN2 liaison in [1].

- Introduce a new indication for NES paging adaptation in UE Paging Capability over F1.

- Meanwhile the RAN2-129bis meeting made the following agreements.

Agreements @ RAN2#129bis meeting 1. For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17. 2. For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17. 3. Paging clustering/bundling/adaptation is not supported/applied in RRC_CONNECTED.

- For PEI, it was agreed that a separate PEI configuration is needed for R19 NES UEs. R19 UEs supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 when configured. Regarding the PEI capability, it is not clear whether a unified new energy saving capability or a dedicated new PEI capability is needed. Given that the discussion is ongoing in RAN2, RAN3 can be pending till further progress on the detailed separate PEI configuration/capability is made.

RAN3 is pending on the separate PEI configuration/capability.

## 2.2 SSB adaptation

For SSB adaptation, RAN1 and RAN2 made the following agreements:

Agreement@ RAN1#119 meeting Conclusion There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19 Conclusion There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode) Conclusion There is no RAN1 consensus to support time domain adaptation for CD-SSB in Rel-19 for SCell Agreement@ RAN1#120 meeting For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell - UE is configured with SSB burst periodicity using legacy signalling for the SCell - UE is configured with X additional SSB burst periodicity for the SCell - FFS: Value of X - SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity - FFS: Whether there is specification impact - Note: This does not impact the discussion on OD-SSB - For switching the periodicity, down-select between - (MAC-CE) - MAC-CE details for SSB burst periodicity adaptation is up to RAN2. - (DCI) - Alt 1-3: DCI based signalling is used Agreement@ RAN1#120 meeting For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell - Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI] - FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1) Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation Agreement@ RAN2#129 meeting 1. RAN2 confirms SSB adaptation in time domain is not supported for RRC idle/inactive UEs and Rel-19 NES-capable UE’s PCell. 2. RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation. 3. For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC. Agreement@ RAN2#129bis meeting 1. Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1. 2. For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration. 3. Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

From the above agreements made in other WGs, it can be observed that for the SSB adaptation:

- it is only applicable for RRC connected UEs, but not for RRC idle/inactive UEs;

- Only the SSB burst periodicity adaptation for the NCD-SSB is supported.

- The SSB adaption is applicable only for SCells.

- The DCI format 2_9 is utilized to indicate the SSB adaptation.

From RAN3 perspective, since the NCD-SSB is always broadcasted with the periodicity adaptation, this cell can be considered as a normal cell. For the indicated SSB adaptation contents, it is the DU that is responsible to determine the SSB adaptation and send DCI format 2_9, hence there is no F1AP impact.

In addition, for the L3 measurement, RAN2 agreed that multiple additional SMTC configurations are configured to the UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration. In the existing F1AP specification,

- the cell level Measurement Timing Configuration IE referring to MeasurementTimingConfiguration is included in the Served Cell Information IE, then propagate to neighbour cell

- the CU includes the UE level MeasurementTimingConfiguration IE of the CU to DU RRC Information IE as a container to the DU to generate the measurement gaps.

Given that RAN2 agreed that the legacy SMTC is used for neighbour cell measurement, and there is no impact on the measurement gap which can decided based on the legacy SMTC parameters, RAN3 can assume that there is no RAN3 impact.  This can be revisited upon further progress.

For SSB adaptation over SCell, RAN3 assumes there is no RAN3 impact, which can be revisited upon further progress in other WGs.

## 2.3 PRACH adaptation

For PRACH adaptation, significant agreements were made in other WGs and provided as follows.  At previous RAN3-127bis meeting, the PRACH adaptation capability indication was agreed and captured in the F1AP BLCR.

Agreement@ RAN1#116bis meeting Support adaptation mechanisms of PRACH in time-domain for following: - UE in idle/inactive mode - UE in connected mode Agreement@ RAN1#120 meeting For DCI-based adaptation for additional PRACH resources, DCI 1_0 with P-RNTI indicates the availability information for additional PRACH resource from a reference point and for a validity time duration - FFS: Validity time duration for availability is configured by higher layer signaling or predefined - FFS: Location of the reference point defined in the specification - FFS: Value/granularity of the validity time duration. - FFS: Whether DCI can be used to explicitly deactivate the additional PRACH resources Agreement@ RAN1#120bis meeting For adaptation of PRACH in time-domain, at least for 4-step RACH, at least for DCI 1_0 with P-RNTI, - Support configuration of the additional PRACH resources within [the same] RACH-ConfigCommon in SIB1 used to configure the legacy PRACH resources - the legacy PRACH resources used for ‘additional RO validation before the SSB-RO mapping’ are configured in the RACH-ConfigCommon For DCI-based adaptation for additional PRACH resources, the following 1-bit field is used for adaptation indication in DCI format 1_0 with P-RNTI - Use one bit from the Bits 5-8 within the Short Message field (from upper layers) - Send LS to RAN2 to confirm the use of this bit. Above applies for cell that transmits the DCI for connected UEs and IDLE/INACTIVE mode UEs For DCI-based adaptation for additional PRACH resources, for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI - the validity duration is configured via higher layer signalling. Agreements @ RAN2#129 meeting 1. Legacy UEs and UEs non-capable of time domain PRACH adaptation are expected to use legacy PRACH resources per legacy configuration and procedures. No barring is needed. 2a. RAN2 starts CBRA for RACH adaptation. 2b. RAN2 starts 4-step RACH adaptation. 3. From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request. 4. From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources. 5a. From R2 perspective, RACH adaptation is not modelled as RA feature(s). 5b. From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain. 6. Will follow legacy mechanism regarding how to select RACH resource.

From the above agreements, it can be observed that:

- DCI 1_0 with P-RNTI (one bit in the short message field) is used to indicate the availability information for additional PRACH resource from a reference point and for a validity time duration.

- The validity duration is configured via higher layer signalling.

In the current F1AP and XnAP specification, the NR PRACH Configuration is included in Served Cell information IE, containing the NR PRACH Configuration List for the UL carrier or the SUL carrier.  And in the SON aspects, in the F1AP GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message and the GNB-CU CONFIGURATION UPDATE message, the Cells for SON IE is present to indicate the PRACH configuration of the served cells or neighbour cells. For the PRACH adaptation, RAN3 needs to discuss at least the following issues.

- If the additional PRACH configuration should be contained in the NR PRACH Configuration in the Served Cell information IE, and indicated as “additional PRACH resources”?

- Each time the DCI 1_0 with P-RNTI was sent to indicate the availability of the additional PRACH resources, or the validity duration is expired, whether the gNB-CU should be notified and neighbour cells be notified?

- Whether the Cells for SON IE should be extended to include the additional PRACH resource?

Though a set of PRACH parameters are provided in the RAN1 LS in [2], the detailed PRACH configuration/signalling are still under discussion. Hence it is suggested for RAN3 to initially discuss the above open issues.

For PRACH adaptation, RAN3 to initially discuss the following open issues:

- If the additional PRACH configurations should be contained in the NR PRACH Configuration in the Served Cell information IE, and indicated as “additional PRACH resources”?

- Each time the DCI 1_0 with P-RNTI was sent to indicate the availability of the additional PRACH resources or the validity duration is expired, whether the gNB-CU should be notified and neighbour cells be notified?

- Whether the Cells for SON IE should be extended to include the additional PRACH resource?

# 4. Conclusion

Based on the discussion in this paper, we make the following proposals.

Paging adaptation

- RAN3 is pending on the separate PEI configuration/capability.

SSB adaptation

For SSB adaptation over SCell, RAN3 assumes there is no RAN3 impact, which can be revisited upon further progress in other WGs.

- PRACH adaptation

For PRACH adaptation, RAN3 to initially discuss the following open issues:

- If the additional PRACH configurations should be contained in the NR PRACH Configuration in the Served Cell information IE, and indicated as “additional PRACH resources”?

- Each time the DCI 1_0 with P-RNTI was sent to indicate the availability of the additional PRACH resources or the validity duration is expired, whether the gNB-CU should be notified and neighbour cells be notified?

- Whether the Cells for SON IE should be extended to include the additional PRACH resource?

# 5. Reference

- R2-2501483 LS on paging enhancement in R19 NES, RAN2

- R1-2503154	LS on Rel-19 higher layers parameters list Post RAN1#120bis, RAN1