3GPP TSG-RAN WG3 Meeting #128	R3-253437

# St Julian, Malta, 19th – 23rd May, 2025

Title: 	On collection of throughput measurements for UE performance

Source: 	Ericsson, Jio Platforms, InterDigital

Agenda item:	11.4

Document Type:	Discussion and decision

# 1. Introduction

During RAN3#127bis, the following agreements and ways forward were captured:

- WA: In case of both split architecture and non-split architecture, UE throughput UL/DL at PDCP level can be considered for the AI/ML for NG-RAN function in Rel-19.

- Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over E1 interface to transfer UL/DL UE throughput and UL/DL packet delay from CU-UP to CU-CP upon request.

This paper discusses ways forward based on the above, primarily discussing the collection and signalling of Throughput as part of UE performance reporting in split architecture.

# 2. Discussion

During Release 18, RAN3 formalised the Data Collection Reporting Initiation and the Data Collection Reporting procedures for non-split architectures. One of the intended use-cases for the procedures was UE performance collection, where information/feedback could be collected by the source node from a neighbour node to which UEs were handed over to. The defined UE performance metrics are Average UE Throughput UL, Average UE Throughput DL, Average Packet Delay, and Average Packet Loss DL.

In the context of throughput measurements, it is important to note that the throughput metrics defined during Release 18 had the following definition (TS 38.423, clause 9.2.3.4)- ‘This IE indicates the number of bits delivered by NG-RAN in UL or to NG-RAN in DL or by the UE in sidelink within a period of time, divided by the duration of the period.’.

Observation 1: The Release 18 metrics for Average UE Throughput were not defined at any particular level, e.g., PDCP, RLC.

In line with the LS sent to SA5 during R3#127bis (R3-252436), any performance measure computed at a higher layer is generally considered closer to the application-level or service-level perceived performance. Measuring throughput at a lower layer has the drawback that it may not consider higher-layer interactions in the measure. In the context of UE performance, the source gNB is interested in obtaining data (to be used either as training, or feedback data) that clearly describes the service level performance experienced by a UE after a handover event. Based on the reasoning described above, obtaining measurements at a lower layer does not always unequivocally express the service- or application-level performance experienced by the UE.

Observation 2: When possible, it is better to choose a higher-layer measurement as it follows the service- or application-level performance better than a lower-layer measurement.

Proposal 1: RAN3 to convert the following WA into an agreement: “In case of both split architecture and non-split architecture, UE throughput UL/DL at PDCP level can be considered for the AI/ML for NG-RAN function in Rel-19.”

3GPP TS 28.558, and other specifications defining UE measurements, define no PDCP level UE throughput measurements. The LS sent to SA5 (R3-252436) requested for clarifications in terms of using DL UE Buffered Throughput at the PDCP level and the feasibility of measuring a corresponding UL UE throughput measurement at PDCP level for use as UE performance metrics for throughput. The decision in RAN3 to use the above metrics is pending based on the outcomes in SA5.

In the above context, we would however like to note that 3GPP TS 37.483 (E1 AP) defines the following IEs and corresponding functionality as part of the BEARER CONTEXT SETUP REQUEST and BEARER CONTEXT MODIFICATION request messages:

If the PDU Session Resource DL Aggregate Maximum Bit Rate IE is contained in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall store and use the information for the down link traffic policing for the Non-GBR QoS flows for the concerned UE as specified in TS 23.501. […] If the PDU Session Resource DL Aggregate Maximum Bit Rate IE is contained in the PDU Session Resource To Setup List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall replace the information in the UE context and use it when enforcing downlink traffic policing for the non GBR QoS flows for the concerned UE, as specified in TS 23.501. […] If the UE Slice Maximum Bit Rate List IE is included in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, store and use the information for the downlink traffic policing for each concerned slice as specified in TS 23.501. […] If the UE Slice Maximum Bit Rate List IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, store and replace the previously provided UE Slice Maximum Bit Rate List by the received UE Slice Maximum Bit Rate List in the UE context, and use the received UE Slice Maximum Bit Rate List for the downlink traffic policing for each concerned slice as specified in TS 23.501.

It is evident from the above that a gNB-CU-UP must measure the UE throughput at the PDCP level and for the DRBs corresponding to the slice for which the Maximum Bitrate was specified. However, it needs to be noted that no specific PDCP measurements have been defined to make such functionality work.

Observation 3: There is already a requirement on the gNB-CU-UP to measure PDCP throughput without explicit definition of such measurements in the specifications.

Based on the above findings, for the problem at hand, i.e., throughput measurements for UE performance, it is not necessary that PDCP-level throughput measurements shall be specified for this purpose, as in practice the gNB-CU-UP can collect these measurements using existing functionality.

Proposal 2: RAN3 to agree that the gNB-CU-UP can compute the UL and DL PDCP throughput for a UE. This functionality is considered already available.

For the purpose of defining UE throughput for UE performance, throughput at PDCP level may be computed per DRB for UL or DL. The Average per DRB UE throughput UL and Average per DRB UE throughput DL may then be signalled (per DRB) by the gNB-CU-UP to the gNB-CU-CP.

Proposal 3: Depending on the outcome from SA5, an alternative measurement for the average PDCP throughput per DRB in UL and DL may be considered as the measured throughput for UE Performance at the gNB-CU-UP.

Signalling aspects

During RAN3#127bis, the following agreement was reached:

- Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over E1 interface to transfer UL/DL UE throughput and UL/DL packet delay from CU-UP to CU-CP upon request.

The introduction of the Data Collectionn Reporting Initiation and Data Collection Reporting procedures addresses the problem of collection of UE performance metrics and signalling from the gNB-CU-UP to the gNB-CU-CP. However, the details of how a request to collect these measurements for specific UEs/DRBs is signalled from the gNB-CU-CP to the gNB-CU-UP has not yet been agreed.

As a solution to the above, we propose the following. On the E1 interface, the Bearer Context Setup procedure is used to allow the gNB-CU-CP to establish a bearer context in the gNB-CU-UP. This procedure is initiated immediately after a successful handover. A gNB-CU-CP which has received a request from a neighbor gNB to collect UE performance for a UE, may then indicate to the gNB-CU-UP to collect the required measurements, e.g., throughput, delay, by including a request for UE performance monitoring along with the IEs in the Bearer Context Setup Request message. The indication may also contain the DataCollectionId corresponding to the Data Collection Reporting procedure over which the measured UE performance metrics shall be signalled.

Proposal 4: RAN3 to agree to extend E1 AP Bearer Context Setup procedure and Bearer Context Modification procedure for the gNB-CU-CP to indicate to the gNB-CU-UP to collect throughput measurements for UE performance.

A TP reflecting the above proposals is available in R3-253438.

# 3. Conclusion

# This paper makes the following observations and proposals:

Observation 1: The Release 18 metrics for Average UE Throughput were not defined at any particular level, e.g., PDCP, RLC.

Observation 2: When possible, it is better to choose a higher-layer measurement as it follows the service- or application-level performance better than a lower-layer measurement.

Proposal 1: RAN3 to convert the following WA into an agreement: “In case of both split architecture and non-split architecture, UE throughput UL/DL at PDCP level can be considered for the AI/ML for NG-RAN function in Rel-19.”

Observation 3: There is already a requirement on the gNB-CU-UP to measure PDCP throughput without explicit definition of such measurements in the specifications.

Proposal 2: RAN3 to agree that the gNB-CU-UP can compute the UL and DL PDCP throughput for a UE. This functionality is considered already available.

Proposal 3: Depending on the outcome from SA5, an alternative measurement for the average PDCP throughput per DRB in UL and DL may be considered as the measured throughput for UE Performance at the gNB-CU-UP.

Proposal 4: RAN3 to agree to extend E1 AP Bearer Context Setup procedure and Bearer Context Modification procedure for the gNB-CU-CP to indicate to the gNB-CU-UP to collect throughput measurements for UE performance.