3GPP TSG-RAN WG3 #128                                                 R3-253443

Malta, MT, 19th – 23rd May, 2025

Source:	CATT

Title:	Discussion on Evolution of Duplex Operation

Agenda Item:	19.2

Document for:	Discussion and decision

- Introduction

This document discusses RACH configuration exchange and DC case of Evolution of Duplex Operation

- Discussion

  - RACH configuration exchange

RACH-ConfigCommonSBFD-r19 = SEQUENCE {

sbfd-RACH-SingleConfig-r19               ENUMERATED {enabled}                                             OPTIONAL,  -- Need R

sbfd-RACH-DualConfig-r19                 SBFD-RACH-DualConfig-r19                                         OPTIONAL,  -- Need R

sbfd-RSRP-ThresholdRO-Type-r19                RSRP-Range                                                 OPTIONAL,  -- Need R

sbfd-RSRP-ThresholdRO-TypeUsage-r19           ENUMERATED {above,below}                                   OPTIONAL  -- Need R

sbfd-RACH-SingleConfig: Indicates whether RACH configuration Option 1 for SBFD random access operation is enabled or not, see clause x in TS 38.211 [16] and clause y in TS 38.213 [13].

sbfd-RACH-DualConfig: Used to configure dual RACH configurations and configure random access parameters in SBFD symbols by setting up one additional RACH configuration, see RACH configuration Option 2 for SBFD random access operation in clause x in TS 38.211 [16] and clause y in TS 38.213 [13].

According to RAN 1/2 agreement, two options for RACH configuration are supported. Option 1 is reuse/enable leagcy tdd-UL-DL-ConfigurationCommon IE, option 2 requires an additional RACH configuration. For option 2, RAN3 need to exchange SBFD RACH configuration between NG-RAN (Xn) and CU/DU (F1).

Proposal 1: For option 2, DU provides additional RACH configuration to CU in Served Cell Information IE, CU further sends to neighbour CU in Served Cell Information NR. Neighbour CU provides additional RACH configuration to DU via Neighbour Cell Information List.

  - Simultaneous configuration of SBFD and DC

LS [1] received from RAN2 as below

RAN2 has discussed whether SBFD and dual connectivity (DC) can be configured simultaneously for an SBFD-aware UE. However, no consensus has been reached. RAN2 would like to seek inputs from RAN1 by asking the following question:

Question: Whether SBFD and DC can be configured simultaneously, and whether there is any issue with such configuration for an SBFD-aware UE?

Note that CA has been supported in RAN1 but it may be different from DC. There are some thing need to be considered in other group e.g., maximum power value may different for MCG and SCG, and RAN2 may update their RRC specification and may impact UE.

As RAN3 in CC, we can make some initial analysis about RAN3 specification impact and take RAN1/RAN2 conclusion into account.

From RAN3 point view, SBFD UE is required to reduce interference between MN and SN. They should exchange SBFD time and location information and SSB/NZP CSI-RS/strongest DL beam/CLI mitigation request.

- Introduce SBFD time and location information in S-NODE ADDITION/MODIFICATION REQUEST/RESPONSE message.

- Victim gNB sends strongest DL beam and CLI Mitigation Indication to aggressor gNB by reusing new introduced message i.e., CLI MEASUREMENT UPDATE in Xn.

Proposal 2: The following RAN3 impact is identified to support SBFD DC, whether to support SBFD DC should wait for other group:

- Introduce SBFD time and location information in S-NODE ADDITION/MODIFICATION REQUEST/RESPONSE message.

- Victim gNB sends strongest DL beam and CLI Mitigation Indication to aggressor gNB by reusing new introduced message i.e., CLI MEASUREMENT UPDATE in Xn

  - Additional information exchange

In case aggressor gNB or victim gNB does not support SBFD, the neighbour SBFD configuration can be deleted by “not receive” SBFD configuration in modification message. Different companies may have different implementation but this is not a SBFD specific topic e.g., XR/NPN Broadcast Information. We do not need to add on/off indication for SBFD configuration.

Proposal 3a: No need to add on/off for SBFD configuration.

Also, for CLI mitigation request, RAN1 explicitly say that there is no need to add “stop” message.

- Notes: Following are assumed for information exchange among gNBs

  - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3

  - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact

Proposal 3b: No need to add start/stop for CLI mitigation request.

Base on the RAN1 LS, the strongest DL beam is enough for aggressor gNB to do CLI mitigation. There is no need to introduce DL beam list and corresponding measured value from victim to aggressor.

Proposal 3c: No need to add additional measured DL beam information from victim to aggressor.

- Conclusion

Proposal 1: For option 2, DU provides additional RACH configuration to CU in Served Cell Information IE, CU further sends to neighbour CU in Served Cell Information NR. Neighbour CU provides additional RACH configuration to DU via Neighbour Cell Information List.

Proposal 2: The following RAN3 impact is identified to support SBFD DC, whether to support SBFD DC should wait for other group:

- Introduce SBFD time and location information in S-NODE ADDITION/MODIFICATION REQUEST/RESPONSE message.

- Victim gNB sends strongest DL beam and CLI Mitigation Indication to aggressor gNB by resuing new introduced message i.e., CLI MEASUREMENT UPDATE in Xn

Proposal 3a: No need to add on/off for SBFD configuration.

Proposal 3b: No need to add start/stop for CLI mitigation request.

Proposal 3c: No need to add additional measured DL beam information from victim to aggressor.

- Reference

[1] R3-253028, LS on simultaneous configuration of SBFD and DC, RAN2.