3GPP TSG-RAN WG3 #128                                                                                            R3-253445

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	21.3

Source:	CATT

Title: 	Discussion on NR XR enhancement for others

Document for:	Discussion and Approval

- Introduction

This paper discusses NR XR enhancement for others including following issue.

For Support of exposure of available data rate:

FFS on non-homogeneous deployment.

For Specify uplink congestion signalling:

FFS on per QoS flow indication or per DRB indication over F1AP?

FFS on whether CN may inform gNB the recommended UL rate for specific QoS flows.

For avoid RLC unnecessary retransmission

FFS on whether the gNB-CU sends an indication to the gNB-DU to stop the transmission and retransmission of a RLC SDU or the segment of a RLC SDU.

- Discussion

  - UL bit rate control

We think DU is able to detect per QoS flow UL bit rate by proporation split per DRB UL bit rate into per QoS flow according to QoS to DRB mapping. For example, DU knows per DRB bit rate is 40 mbps and there are three QoS flows mapped to one DRB with 2:1:1 QoS requirement. DU will indicate UL bit rate of QoS 1-3 with 20 mbps, 10 mbps and10 mbps.

Proposal 1: GNB-DU is able to identify per QoS flow bit rate according to per DRB bit rate and QoS to DRB mapping. And further send it to UE via MAC CE

Another solution is that CU detects per QoS flow UL bit rate and sends to DU via F1. The following question need to be discussed.

Q1: whether CU can measure per QoS flow UL bit rate?

CU cannot identify Uu channel condition in time. The DDDS has latency which cannot reflect the real time buffer situation in DU. Therefore, CU may not be able to provide suitable per QoS flow UL bit rate.

Observation 1: CU may be not able to provide real time UL bit rate because CU does not know Uu channel condition in time. But the MAC CE method is aim to achieve real time and fast UL bit rate control.

Q2: when does CU send per QoS flow UL bit rate to DU?

CU can provide the recommended UL bit rate to DU when it thinks the UL bit need to be adapted. In other word, it bases on CU implementation.

Observation 2: CU can decide when to send per QoS flow recommended UL bit rate to DU base on CU’s implementation.

For case 2 i.e., use RRC message together with per DRB MAC CE, CU and DU may have different trigger condition of UL bit rate control. If CU thinks the UL bit rate of QoS 1 need to be adapated, it will send QoS 1 to UE via RRC message, while DU may not think such DRB tend to congestion, in this case, DU will not send per DRB bit rate control to UE via MAC CE. The misalignment may exist in CU and DU. UE cannot do UL bit rate adapation. From other point view, case 2 just move the behaviour of proporation split per DRB bit rate into per Qos flow bit rate from DU to UE. We do not see the benefit to let UE do the split compared with DU.

Observation 3: For case 2, CU and DU may be not aligned because they have different trigger condition of UL bit rate control, and it requires UE to do proportion split per DRB bit rate into per QoS flow which can be done by DU.

  - Support of exposure of available data rate

In last meeting, we support threshold based reporting by introduing threshold and direction in NG/Xn/F1/E1. NG-RAN will report the reference to the respective threshold of “available data rate” to core network in case per QoS flow available data rate” crosses one of the thresholds.

Based on the QoS monitoring request, if the bitrate that NG-RAN is able to provide changes and matches or exceeds indicated Reporting threshold(s), the NG-RAN reports the UL and/or DL available bitrate information of the highest fulfilled threshold(s) for the UL and/or DL direction to the PSA UPF via the GTP-U header of UL packets (or dummy UL packets). The NG-RAN shall only send a report when the available bitrate information is different from the previously reported value(s) for at least one direction. If thresholds exist for both directions, the NG-RAN shall always indicate the available bitrate information for both directions when sending a report.

If CN configures available data rate to NG-RAN but NG-RAN does not active such capability, it will not report the reference of threshold via user plane but CN cannot distinguish whether the available date rate has not cross threshold or NG-RAN does not support this feature.

Based on configuration, the AMF may know whether the QoS monitoring feature (i.e. each of Packet delay monitoring and/or Congestion monitoring and/or Available Bitrate monitoring) is supported by all NG RAN nodes in an area.

According to SA2 specification, AMF may know whether Available Bit rate monitoring and Congestion monitoring are supported by all NG-RAN. OAM can do such configuration but it not preclude NG-RAN based indicator. It should be noted that the “capability” is different from “active”. The design of Available Bitrate monitoring should align with Congestion monitoring by including “active” and “not active”.

Proposal 2: RAN3 consider to introduce available data rate monitor status including “active” and “not active” for non-homogeneous case in 38.413.

  - RLC unnecessary retransmission

When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment.

Based on RAN2 agreement, PDCP will send a discard indication to Tx RLC to stop any transmission and retransmission when timer expired in PDCP. For F1, PDCP is located at CU while RLC is located at DU. F1 need to be enhanced to support discard indication.

Proposal 3: CU sends discard indication to DU to support RLC unnecessary (re)transmission

3 Conclusions

In this contribution the following observations and proposal were derived:

Proposal 1: GNB-DU is able to identify per QoS flow bit rate according to per DRB bit rate and QoS to DRB mapping. And further send it to UE via MAC CE

Observation 1: CU may be not able to provide real time UL bit rate because CU does not know Uu channel condition in time. But the MAC CE method is aim to achieve real time and fast UL bit rate control.

Observation 2: CU can decide when to send per QoS flow recommended UL bit rate to DU base on CU’s implementation

Observation 3: For case 2, CU and DU may be not aligned because they have different trigger condition of UL bit rate control, and it requires UE to do proportion split per DRB bit rate into per QoS flow which can be done by DU.

Proposal 2: RAN3 consider to introduce available data rate monitor status including “active” and “not active” for non-homogeneous case in 38.413.

Proposal 3: CU sends discard indication to DU to support RLC unnecessary (re)transmission.

4 References