3GPP TSG-RAN WG3 Meeting #128	R3-253457

Malta, MT, 19-23, May, 2025

Agenda Item:	18.2

Source:	Ericsson

Title:	Discussion on remaining aspects on LP-WUS

Document for:	Discussion

# 1	Introduction

This paper discusses further progress to consider in RAN3 to support low power radio WUS considering below open points:

- the LP-WUS node capability. - Including Extended UE Identity Index、 - Emergency call back.

# 2	Discussion on RAN3 impacts

## 2.2	LP-WUS node capability.

On the issue of UE radio capability for paging information, RAN2 sent a LS reply [1] to SA2 and RAN3 with following response:

For UE Radio Paging Capabilities introduced up to Rel-18, RAN2 agreed not to introduce any standard solution, i.e., rely on proper network implementation, as follows. - We will not change legacy UE capabilities and rely on network implementation To address the issue for UE Radio Paging Capabilities introduced in Rel-19 onwards, RAN2 made the following agreement: - Use a “container” using OCTET STRING with content generated by UE when new UE Radio Paging Capabilities are introduced. This is applicable only to new Rel-19 and future capabilities added to paging capabilities. This will only work for gNB that support this Rel-19 “container” In particular for Rel-19 LP-WUS feature, if it is deployed, all the gNBs belonging to a certain area need to support the Rel-19 “container”.

This new Rel-19 Container will be included in the UERadioPagingInformation message as defined in TS 38.331 (green-marked in the table below).

9.3.1.68             UE Radio Capability for Paging

This IE contains paging specific UE Radio Capability information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Radio Capability for Paging of NR | O |  | OCTET STRING | Includes the UERadioPagingInformation message as defined in TS 38.331 [18]. |
| UE Radio Capability for Paging of E-UTRA | O |  | OCTET STRING | Includes the UERadioPagingInformation message as defined in TS 36.331 [21]. |
| UE Radio Capability for Paging of NB-IoT | O |  | OCTET STRING | Includes the UERadioPagingInformation-NB message as defined in TS 36.331 [21]. |

No new NGAP IE is needed from RAN3 side.

RAN3 can confirm RAN2 relying on network implementation to solve the issues of mixed deployments with different releases. A note to TS 38.300 can be added.

Proposal 1: RAN3 to confirm RAN2 relying on network implementation to solve the issues of mixed deployments with different releases. A note to TS 38.300 can be added.

## 2.3	Including Extended UE Identity Index

RAN2 have finally concluded on the UE ID based subgrouping issue, and agreed on the number of X below:

UE-ID calculation (RAN2#129bis): - Use 5G-S-TMSI to determine the UE_ID in the formula of UE_ID based subgrouping for LP-WUS, i.e., UE_ID=5G-S-TMSI mod X. - X is based on 32 subgrouping number. Details can be discussed in the running CR.

This means that 5 extra bits are needed to support the 32 subgroup IDs. Current Extended UE identity index value IE is limited to 16 bits. Hence a further extended IE is needed.

A question however, is whether the legacy Extended UE Identity Index Value IE can still be used for LP-WUS subgroup ID when eDRX and PEI are not configured. Below are the different combinations:

In case eDRX and PEI are not configured (or not supported), then AMF can still signal the legacy IE that contains enough bits for LP-WUS subgroup ID to work. Otherwise sending always the new Further extended UE_ID IE will occasion extra unused bits signalled over NGAP. (extra 5 bits will not be used if X = 32768).

RAN3 should therefore discuss if the legacy Extended UE Identity Index Value IE can still be re-used for LP-WUS in case eDRX and PEI are not configured.

Proposal 2: RAN3 to discuss if the legacy Extended UE Identity Index Value IE can still be re-used for LP-WUS in case eDRX and PEI are not configured.

## 2.4	Emergency callback

In last meeting the restriction of using LP-WUS during an emergency PDU session was removed. In previous releases, the additional latency introduced by PEI was deemed tolerable.

For LP-WUS however, the average paging delay is increased due to the time offset between LP-WUS and PO. When a paging message is ready to be transmitted during the time offset, then the paging message is delayed with one DRX cycle. The time offset for LP-WUS is considerably larger (400-800 ms) compared to the time offset for PEI.

In our view considering that LP-WUS is allowed during an emergency PDU session, paging can be delayed by up to one second. It should be discussed whether this also tolerable, of if should be prohibited. During paging, the gNB can be aware that LP-WUS is prohibited explicitly due to an emergency PDU session by the CN and disable the use of UE-ID based subgrouping.

Proposal 3: RAN3 to discuss whether the CN can indicate to RAN during Paging and NGAP context setup request that UE-ID based LP-WUS is not allowed (e.g. during an emergency PDU session).

# 3	Conclusion

Proposal 1: RAN3 to confirm RAN2 relying on network implementation to solve the issues of mixed deployments with different releases. A note to TS 38.300 can be added.

Proposal 2: RAN3 to discuss if the legacy Extended UE Identity Index Value IE can still be re-used for LP-WUS in case eDRX and PEI are not configured.

Proposal 3: RAN3 to discuss whether the CN can indicate to RAN during Paging and NGAP context setup request that UE-ID based LP-WUS is not allowed (e.g. during an emergency PDU session).

# 4	References

- R3-253012, Reply LS on paging capability loss issue, RAN2