# 3GPP TSG-RAN WG3 Meeting #128					R3-253464

# Malta, MT, 19-23, May, 2025

Agenda Item:	21.3

Source:	Ericsson

Title:	Discussion on XR UL bit rate control

Document for:	Discussion

# 1	Introduction

In this paper we continue the discussion on the topic of XR UL bit rate control and provide our views on the RAN2 solutions.

# 2	Discussion

Based on RAN2 discussion, two options are available:

- Option 1: QoS flows are indicated by MAC CE;

- Or Option 2: QoS Flows are indicated by RRC while MAC CE is per DRB.

Based on discussions from last RAN3 meeting, both options require F1 enhancements, as there is no dedicated F1AP IE for the purpose of recommended bitrate control indication, either on a per QoS Flow or per DRB basis.

Observation 1: Both options require F1 enhancements.

The end-to-end signalling solution of Option 1 is presented below:

Fig. 1 Option 1: QoS Flow via MAC CE indication with representation of new impacts in red.

The main impact of this option is on CU functionality, where the CU needs to derive the QoS flow based bit rate based on the presence of the SDAP header, which is configured by the network and is not always present. The CU then signals to the DU a per QoS Flow bitrate information. This could be set either periodically (configured at step 7) or in response to a request from the DU (not shown in figure 1). If the request is coming from the DU then additional impacts are required over F1 for bitrate control indication request, e.g., new request IE in the UE CONTEXT MODIFICATION REQUIRED message (DU-initiated) message.

Observation 2: Option1 (QoS Flow via MAC CE indication) adds new impact on CU design to derive the QoS flow-based bit rate monitoring and relies on the presence of the SDAP header, which is configured by the network and is not always present/ supported by all implementations.

Once the DU knows from the CU the per QoS Flow bitrate, the DU can trigger a MAC CE towards UE. The advantage of this solution from a RAN2’s perspective was that MAC CE is more dynamic and flexible than RRC, however the F1 delay was not taken into account. In above figure, the F1 delay can be at its worst 30ms one way, 60ms two ways. If we besides consider the solution where the DU has to request for per QoS flow bitrate control indication from the CU, then the additional delay will be doubled, making the information finally available at the DU stale and inefficient.

Observation 3: RAN2 did not consider the F1 delay for the QoS Flow indication via MAC CE solution, which could make the information finally available at the DU stale and inefficient. Thereby rendering the solution less flexible than expected.

The end-to-end signalling solution of Option 2 is presented below:

Fig. 2 Option 2 QoS Flow via RRC indication and DRB via MAC CE with representation of new impacts in red.

In this option, the CU initially informs the DU of the list of DRB IDs that can be rate adaptive (step4), based on the assistance information from CN and the result of the QoS flow to DRB mapping. The CU also informs the UE via RRC of the QoS Flows that are rate adaptive. This indication can be considered achieved in a static way at the setup of the DRBs, to avoid many RRC reconfigurations. Afterwards, the DU performs congestion monitoring and when it considers that one of the indicated bitrate adaptive DRB ID(s) can be rate controlled, it informs the UE via MAC CE. As can be seen from the above signalling, this relies on both a static CU signalling via RRC and a dynamic activation via MAC CE. But it should be noted that the number of steps is lesser than option 1. Also the delay caused by the F1 delay is halved compared to option 1.

Further, as can be seen from Figs. 1 and 2, both messages require an RRC reconfiguration step (message 5 in both figures). So the argument that option 1 is faster due to no RRC configuration needed is moot, as anyways the UE needs to be configured at the initial DRB to QoS flow mapping.

Observation 4: Option 2 relies on both a static transmission via RRC of the QoS flows and dynamic activation via MAC CE of the DRB ID that can be rate controlled. This option has lesser delay over F1 interface and is as flexible as option 1.

Observation 5: Both options require an RRC reconfiguration step. So the argument that option 1 removes the RRC message is moot.

We propose to inform RAN2 via LS that option 1 will not work as intended in terms of flexibility as additional delay over the F1 interface will make the bitrate control information at the DU hosting the MAC function stale and inefficient for their intended purpose.

Proposal 1: Inform RAN2 via LS that:

- Regarding Option 1 (QoS Flow Bitrate control indication via MAC CE): the solution relies on the presence of the SDAP header, which is configured by the network and is not always present nor supported by all gNB-CU implementations. It has also additional delay over F1 interface to provide DU with the bitrate control information, which may risk making the information finally available at the DU stale and inefficient for the desired purpose of MAC activation.

- Regarding option 2: in RAN3’s understanding, this solution may have similar flexibility as option 1, as an RRCReconfiguration message is needed in both options.

# 3	Conclusion

In this paper, we have made the following observation and conclusions:

Observation 1: Both options require F1 enhancements.

Observation 2: Option1 (QoS Flow via MAC CE indication) adds new impact on CU design to derive the QoS flow-based bit rate monitoring and relies on the presence of the SDAP header, which is configured by the network and is not always present/ supported by all implementations.

Observation 3: RAN2 did not consider the F1 delay for the QoS Flow indication via MAC CE solution, which could make the information finally available at the DU stale and inefficient. Thereby rendering the solution less flexible than expected.

Observation 4: Option 2 relies on both a static transmission via RRC of the QoS flows and dynamic activation via MAC CE of the DRB ID that can be rate controlled. This option has lesser delay over F1 interface and is as flexible as option 1.

Observation 5: Both options require an RRC reconfiguration step. So the argument that option 1 removes the RRC message is moot.

Proposal 1: Inform RAN2 via LS that:

- Regarding Option 1 (QoS Flow Bitrate control indication via MAC CE): the solution relies on the presence of the SDAP header, which is configured by the network and is not always present nor supported by all gNB-CU implementations. It has also additional delay over F1 interface to provide DU with the bitrate control information, which may risk making the information finally available at the DU stale and inefficient for the desired purpose of MAC activation.

- Regarding option 2: in RAN3’s understanding, this solution may have similar flexibility as option 1, as an RRCReconfiguration message is needed in both options.

# 4	References

[1] R3-251513/R2-2501347 LS on uplink rate control, Nokia

# 5	LS to RAN2

3GPP TSG-RAN WG3 Meeting #128					R3-25xxxx

Malta, MT, 19-23, May, 2025

# Title:	[Draft] RAN3 feedback on support of XR UL bit rate control

# Response to:	Reply LS on XR UL bit rate control (R3-251513/R2-2501347)

# Release:	Release 19

# Work Item:	NR_XR_Ph3-Core

Source:	[Ericsson] to be RAN3

To:	RAN2

Cc:	SA2

Contact Person:

#### Name:	Yazid Lyazidi

#### Tel. Number:

#### E-mail Address:	Yazid.lyazidi@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

RAN3 has discussed the support of XR UL bit rate control would like to provide the following feedback to RAN2:

- Regarding Option 1 (QoS Flow Bitrate control indication via MAC CE): the solution relies on the presence of the SDAP header, which is configured by the network and is not always present nor supported by all gNB-CU implementations. It has also additional delay over F1 interface to provide DU with the bitrate control information, which may risk making the information finally available at the DU stale and inefficient for the desired purpose of MAC activation.

- Regarding option 2: in RAN3’s understanding, this solution may have similar flexibility as option 1, as an RRCReconfiguration message is needed in both options.

2. Actions:

To RAN2:

ACTION: 	RAN3 kindly asks RAN2 to take the above feedback into consideration.

3. Date of Next RAN3 Meetings:

TSG-RAN3 Meeting #129		August 25-28, 2025			Bangalore, India