3GPP TSG RAN WG3 Meeting #128	R3-253479

Malta, MT, 19-23 May 2025

Title:	Reply LS to LS on Continuous MDT

Response to:	R3-253024/ S5-252114

Release:	Rel-19

Work Item:	NR_AIML_NGRAN_enh-Core

Source:	RAN3

To:	SA5

Cc:

Contact Person:

#### Name:                Angelo Centonza

E-mail Address:   angelo.centonza@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	R3-25xxxx (TP to 37.320) - Continuous MDT support; R3-25xxxx - (TP for AIML BLCR to TS38.423) Continuous MDT support

# Overall Description

RAN3 would like to thank SA5 for its LS on the Reply LS to LS on Continuous MDT. In its Reply LS, SA5 provided responses to the set of questions posed by RAN3 on guidance on possible solutions for Continuous Management Based MDT. In particular, the following comments/answers have been provided:

For Measurement Continuity Problem, regarding RAN3 agreements:

RAN3: “- The OAM provides to the RAN means to identify that the Management Based MDT is for Continuous MDT, e.g. reusing existing Management Based MDT configuration information.”

Answer from SA5: In SA5 point of view, it is feasible for OAM to provide the configuration mechanisms to RAN to identify Management based MDT for continuous MDT. SA5 expects that RAN3 could provide the configuration details as OAM requirements.

Based on the received guidance RAN3 understands that existing configuration mechanisms are agreed by SA5 to be used as Continuous MDT identification. Based on that RAN3 agrees that the legacy TR can be used by the RAN to identify that a management based MDT process is for continuous MDT.

RAN3: “- RAN3 will derive solutions for the Measurement Continuity Problem across RRC_Connected mobility. SA5 is requested to focus on the Measurement Continuity Problem across different RRC states.”

Answer from SA5: SA5 would like RAN3 to provide some clarifications on what problem is referred by “the Measurement Continuity Problem across different RRC states”.

RAN3 would like to clarify that the Measurement Continuity Problem across different RRC states refers to how to correlate management based MDT measurements collected while in RRC_Connected, RRC_Inactive and RRC_Idle. This is independent from the nature of the collected measurements.

RAN3: “- A solution should minimise impacts on the current Management Based MDT framework.”

Answer from SA5: SA5 agrees that the solution should minimise impacts on the current Management Based MDT framework.

Based on the received statement RAN3 understands and agrees that the management based MDT framework should ideally not be changed. Solutions that minimise or avoid such changes shall be prioritised.

For Trace Correlation Problem,

RAN3: “To solve the Trace Correlation Problem, namely to enable correlation of MDT measurements generated by the same UE, it is required to use a temporary identifier for the Continuous MDT, not revealing the UE identity. Such identifier should be made available at the TCE to enable correlation of the management-based MDT reports collected during the Continuous MDT, in a way that minimises the impacts on the Management Based MDT solution.”

Answer: SA5 agrees that at lease TR and TRSR (which does not revealing UE identity) could be used for the correlation of the management-based MDT reports at TCE. SA5 would like RAN3 to elaborate the details of how to avoid overlapping

RAN3 would like to confirm that a solution to the Trace Correlation problem reusing TR+TRSR would minimise management based MDT impacts and it would ensure anonymisation of UEs. Hence this solution is optimal.

In reply to the question from SA5 to elaborate the details of how to avoid overlapping, RAN3 has identified to possible solutions:

Solution A: Based on TR/TRSR:

- A TraceJob is sent to all participating RAN nodes at the time of Continuous management-based MDT initiation. The TraceJob includes a specific Trace Reference identifier (i.e. TR) to indicate this is a Continuous management-based MDT procedure. The same TR is used in all TraceJob signalled to the participating RAN nodes. A dedicated TRSR range is assigned to each participating RAN node, which is used by RAN node during Trace Recording Session Reference (TRSR) assignment.

- When a UE is selected for the continuous management-based MDT, and a Trace Recording session is initiated, the gNB allocates a TRSR from the received range. The same TRSR will be used for the whole duration of the Continuous MDT trace of a UE for correlation purpose. Namely, the TRSR is kept for the UE even if the UE moves to a different participating node.

- If the selected UE is handed over to a participating RAN node, the target RAN node recognize the UE as a Continuous management-based MDT UE based on its TR. Therefore, the UE will be reselected in the target RAN node for the Continuous management-based MDT.

  - If the UE has been configured with Immediate Continuous management-based MDT and the UE moves to a different RAN node, the source RAN node may signal to the target RAN node, e.g. as part of the Handover Request message, the TR and TRSR assigned to the UE. If the target RAN node is a participating RAN node, it understands that the UE was selected for Continuous management-based MDT (from the received TR). Hence the target RAN would select the UE again for further Continuous management-based MDT measurements. The target RAN node assigns to the UE the TRSR received from the source RAN node.

  - If the UE has been configured with Logged Continuous management-based MDT, and the UE moves to RRC_Idle mode or RRC_Inactive state, the UE will collect measurements corresponding to the measurement configuration the UE has received for the logged continuous management-based MDT process. Such measurements will be stored by the UE together with the assigned TR, and TRSR.     
Once the UE moves to RRC_Connected mode, the UE will report to the serving RAN node the logs of measurements collected while in RRC_Idle or RRC_Inactive state. If the serving RAN node is a participating RAN node, it will therefore identify that the UE was selected for Continuous management-based MDT by looking at the reported TR. The serving RAN would therefore further select the UE for Continuous management-based MDT, i.e., the UE will be selected for further continuous management-based MDT measurements. The serving RAN node will assign to the UE the same TRSR contained in the reported MDT logs.

- Trace Correlation is achieved by the assigned TR/TRSR which can identify that the trace records belong to the same selected UE. The same TRSR will be used for the whole duration of the Continuous management-based MDT trace of a UE.

Solution B: Based on List of TRs/TRSR

- Similar as solution A) with the following differences

  - each TraceJob signalled to a participating RAN node will have a dedicated TR, together with a TR list. The TR list contains all dedicated TRs being used in the same continuous MDT Job in each participated gNB. This TR list is used by the RAN node to deduce that a UE establishing connection or handing over from a neighbour node needs to be configured with continuous MDT. 	
Each selected UE has a TRSR assigned. The TR/TRSR combination is unchanged for a selected UE during the lifetime of this continues MDT.

  - At handover, the target gNB recognizes the UE is a continues MDT UE from its TR. The target gNB receives from the source gNB the TR+TRSR for the UE and it reassigns to the UE the same TR+TRSR.

  - When the UE moves from RRC_Idle/Inactive to RRC_Connected and it reports Logged MDT logs, the new serving gNB detects the TR+TRSR for the UE and re-assigns the same TR+TRSR for the immediate continuous MDT process for the same UE.

RAN3 has discussed the received guidance on the Measurement Continuity Problem and decided to agree on the attached “(TP to 37.320) – Continuous MDT support”; with the intention to

- provide continuous MDT configuration details as OAM requirements as requested by SA5 in the first item on this list;

- provide clarifications on the measurement Continuity problem as requested on the second item;

- and agree on a solution that minimized impacts on the current Management Based MDT framework as requested by the third item.

Furthermore, RAN3 agreed on the attached “(TP for AIML BLCR to TS38.423) Continuous MDT support” to enable support of the Continuous MDT identification as part of the Handover Signalling.

# 2. Actions

To SA5

ACTION: RAN3 kindly asks SA5 to take the above inputs into account, to update their specifications accordingly to achieve support of Continuous MDT and to confirm to RAN3 that support of continuous MDT in SA5 specifications is in place.

# 3. Date of Next TSG-RAN3 Meetings

TSG-RAN WG3 Meeting RAN3#129			25 - 29 August 2025		India (TBC), IN

TSG-RAN WG3 Meeting RAN3#129-bis			13 - 17 October 2025		Prague, CZ