3GPP TSG-RAN WG3 Meeting #128	R3-253497

St. Julian’s, Malta, 19th – 23rd May, 2025

Title: 	Discussion on split architecture support for Rel-18 “AI/ML for NG-RAN” use cases

Source: 	Huawei, CMCC, China Unicom

Agenda item:	11.4

Document Type:	Discussion and decision

# 1. Introduction

In the last meeting RAN3 kept on discussing the objectives of the Rel-19 WI “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to introduce support of the leftovers in Rel-18 AI/ML for NG-RAN, one of which is related to split architecture support for Rel-18 use cases.

This contribution analyzes the technical enablers for specifying support of split architecture for Rel-18 use cases, and provides proposals for discussion based on the outcome from the last RAN3 meeting.

# 2. Discussion

In the last meeting RAN3 discussed the split architecture support for Rel-18 RAN AI/ML use cases and agreed the following [4]:

Packet delay measured for UE Performance feedback is sent from DU to CU via ASSISTANCE INFORMATION DATA frame over F1-U in the case of CU-DU split architecture. Packet delay measured for UE performance feedback sent from CU-UP to CU-CP is per DRB level. WA: In case of both split architecture and non-split architecture, UE throughput UL/DL at PDCP level can be considered for the AI/ML for NG-RAN function in Rel-19. Send the LS to SA5 to seek the clarification on UE throughput UL/DL at PDCP level and UL/DL Packet Loss Rate at PDCP layer. Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over E1 interface to transfer UL/DL UE throughput and UL/DL packet delay from CU-UP to CU-CP upon request. Agree the TP to 37.480 and TP to 37.483 in this meeting for the new procedure over E1 interface. Change the agreement: Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU. To the following WA: WA: Using the Legacy procedure, i.e., resource status initiation procedure & resource status report procedure to request and report measured EC over F1 interface. Defer the discussion on packet loss rate in the split architecture in the next meeting. For Packet delay, whether/how to enhance UE Context Setup/Modification procedure to introduce the configuration of data collection, e.g., start, stop, UL, DL, and periodicity?

In the next Section 2.1 we discuss further about the reporting of the packet delay measurements from gNB-DU to gNB-CU-UP over F1-U and, after aggregation with the packet delay measurements performed by the gNB-CU-UP, the reporting of the overall packet delay measurement from the gNB-CU-UP to the gNB-CU-CP over E1 taking the above agreements into account. In Section 2.2, instead, we provide our views on the WA related to the reporting of the gNB-DU’s EC to the gNB-CU(-CP) over F1.

## 2.1 On reporting of measured UE performance over E1

The scenario to consider for UE performance metrics collection and reporting is illustrated in Figure 1: the measurement collection is triggered e.g. after an AI/ML-based mobility decision (i.e., handover) taken by the source gNB towards a split target gNB is successfully executed.

Figure 1 - Scenario for reporting UE performance from split target gNB to source gNB.

In the last meeting, and based on the agreements from RAN3#127 meeting [5] concerning the packet delay DL/UL measurements – for which a “gNB-CU-UP-centric” approach should be considered – RAN3 decided to send the LS to SA5 in [6] to check the feasibility to specify UE throughput DL/UL and packet loss rate DL/UL measured at PDCP (hence in the gNB-CU-UP). In this way all UE performance metrics could be transferred over the E1 interface, by means of new Data Collection Reporting (Initiation) procedures agreed in the last meeting, without impacting the F1 interface. There is however a FFS on discussing at this meeting the packet loss rate in split architecture as follows [4]:

Defer the discussion on packet loss rate in the split architecture in the next meeting.

However, we think we should wait for the SA5 feedback on whether PDCP-level packet loss rate (and UE throughput) measurements can be specified and hence referred in RAN3 specifications for the UE performance.

Proposal 1: RAN3 to agree to wait for the SA5 feedback on whether PDCP-level UE packet loss rate and UE throughput measurements can be specified and hence referred in RAN3 specifications for the UE performance.

For the packet delay DL/UL, instead, the following FFS from the last meeting should be addressed [4]:

For Packet delay, whether/how to enhance UE Context Setup/Modification procedure to introduce the configuration of data collection, e.g., start, stop, UL, DL, and periodicity?

The above FFS also refers to the following agreements from RAN3#127 [5]:

Packet delay measured for UE Performance feedback is sent from CU-UP (the delay in DU side is aggregated in the CU-UP) to the CU-CP in the case of CP-UP split architecture. Packet delay measured for UE Performance feedback is sent from DU to CU via DDDS in the case of CU-DU split architecture.

where the latter agreement was corrected in the last meeting as follows [4]:

Packet delay measured for UE Performance feedback is sent from DU to CU via ASSISTANCE INFORMATION DATA frame over F1-U in the case of CU-DU split architecture.

Basically the packet delay components measured at gNB-DU and gNB-CU-UP are expected to be aggregated at the gNB-CU-UP level and then the aggregated result is sent to the gNB-CU-CP over the E1 interface. Concerning the packet delay measurement at gNB-DU level, and based on TS 38.425 [8], the UL Delay DL Result and DL Delay DU Result IEs are measured per-DRB per-UE and are provided from the gNB-DU to the gNB-CU-UP over F1-U within the ASSISTANCE INFORMATION DATA (PDU Type 2) frame.

Once the UL Delay DU Result and DL Delay DU Result measurements are made available at the gNB-CU-UP level, the latter will aggregate them with the corresponding UL and DL delay components measured at the gNB-CU-UP level – that is, average delay over F1-U, average delay DL in CU-CP and average PDCP re-ordering delay (all defined per-DRB per-UE) – in order to derive the overall DL/UL packet delay measurements to be sent over E1 to the gNB-CU-CP as per the agreement from the last RAN3 meeting.

What still needs to be discussed is:

- how to configure the collection and reporting over F1-U of the packet delay DL/UL (per-DRB per-UE) measured by gNB-DU,

- how to configure the collection at the gNB-CU-UP of the packet delay DL/UL (per-DRB per-UE) measured by gNB-CU-UP, and

- how to configure the reporting over E1 of the overall packet delay DL/UL (per-DRB per-UE) once the gNB-CU-UP received the UL Delay DU Result and DL Delay DU Result measurements from the gNB-DU and it aggregated such measurements with the DL/UL delay measurements performed by the gNB-CU-UP itself.

### 2.1.1 On how to configure the collection and reporting over F1-U of the packet delay DL/UL (per-DRB per-UE) measured by gNB-DU

By still considering the scenario as in Figure 1 of a target split gNB being involved in XnAP Data Collection Reporting (Initiation) procedures from a source gNB which configures such target split gNB to collect and request UE performance for a specific UE, RAN3 already agreed that the reporting is performed over F1-U by re-using the existing QoS Monitoring framework. Therefore, we only need to discuss how to configure such reporting (gNB-CU-CP  gNB-DU), and which parameters should be provided to the gNB-DU as part of the configuration. The target gNB-CU-CP receives over Xn the UE performance collection time duration and reporting periodicity (i.e., UE Performance Collection Configuration IE and Reporting Periodicity for Data Collection IE, respectively, within the DATA COLLECTION REQUEST message) from the source gNB over Xn, hence the gNB-CU-CP has full control of the measurements collection process towards the gNB-DU. As a consequence, we think there is no need to provide the UE Performance Collection Configuration IE to the gNB-DU, but we could rely on simple indications to ‘start’ and ‘stop’ the measurement collection as in the existing QoS Monitoring framework. However, in such existing framework, the periodicity for reporting of delay measurements from the gNB-DU to the gNB-CU-UP is not part of the configuration provided to the gNB-DU, and we think that this should be added based on the information received by the target gNB-CU-CP over Xn as provided by the source gNB.

Observation 1: The target gNB-CU-CP has full control of the measurement collection process towards the gNB-DU, based on the collection time duration and reporting periodicity as provided over Xn from the source gNB.

Observation 2: The gNB-DU should only be provided with ‘start’ and ‘stop’ indications (to trigger the start and stop of the delay measurements in the gNB-DU, based on the collection time duration being known at the gNB-CU-CP) and reporting periodicity.

From the existing QoS Monitoring framework, the reporting of either ‘UL’, ‘DL’ or ‘both DL and UL’ delay measurement can be configured. However, as part of the request for UE performance reporting over Xn, a single bit (6th bit) in the Report Characteristics for Data Collection IE of the DATA COLLECTION REQUEST message is used to request for reporting of both DL and UL average packet delay. We should follow the same approach, and hence consider only the request for reporting both DL and UL average packet delay. When received, this request also represents the indication to be considered by the gNB-DU to start the measurement collection.

Observation 3: The ‘start’ indication to trigger the collection of delay measurements at gNB-DU can be a ‘both DL and UL’ measurement indication.

The above observations can be implemented by introducing a new Packet Delay for UE Performance IE which takes the existing QoS Monitoring Request IE as defined in TS 38.473 [7] as baseline, where only the codepoints ‘both’ (renamed as “DL and UL”) and ‘stop’ are kept and the reporting periodicity (aligned with the XnAP’s Reporting Periodicity for Data Collection IE) is added.

Proposal 2: RAN3 to agree to introduce a new Packet Delay for UE Performance which includes the codepoints ‘DL and UL’ and ‘stop’ to start and stop the collection at gNB-DU of both DL and UL delay measurements, as well as the reporting periodicity.

In the last meeting we agreed that the reporting of measurements from gNB-CU-UP to gNB-CU-CP is on a per-DRB level [4] and, in our view, the same should be applied to the reporting of measurements from gNB-DU to gNB-CU-UP over F1-U, as already specified in TS 38.425 clauses 5.5.3.48 and 5.5.3.49 [8].

Proposal 3: RAN3 to agree that the reporting of DL and UL delay measurements from gNB-DU to gNB-CU-UP over F1-U is per-DRB per-UE, as already specified in TS 38.425.

Following the FFS from last meeting, it needs to be discussed which F1 procedure(s) should be used to provide the delay measurements configuration. We think it is reasonable to reuse the existing F1AP UE Context Setup and UE Context Modification (gNB-CU initiated) procedures, which are enhanced to include the new IE as proposed in Proposal 1. The presence of the new IE triggers the gNB-DU to report DL and UL packet delay measurements to the gNB-CU-UP with the specified reporting periodicity over the F1-U Transfer of Assistance Information procedure (as agreed in RAN3#127 [5]).

Proposal 4: RAN3 to agree that the existing F1AP UE Context Setup and UE Context Modification (gNB-CU initiated) procedures are enhanced to include the new Packet Delay for UE Performance IE for the configuration of DL/UL delay measurements at gNB-DU within the UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION REQUEST messages.

A TP for the F1AP BLCR reflecting proposals above in provided in [9].

We also propose a TP for the AI/ML BLCR to TS 38.425 in [18] to support the reporting of DU’s DL/UL delay measurements for the purpose of aggregation and reporting of the overall DL/UL delay.

### 2.1.2 On how to configure the collection at the gNB-CU-UP of the packet delay DL/UL (per-DRB per-UE)

Similar considerations as previously provided for the configuration to collect (and also report) DL/UL packet delay measurements at the gNB-DU apply. Hence, a new IE similar to the one as proposed in Proposal 1 should also be introduced in E1, with the only difference with respect to the new F1AP IE is that the reporting periodicity does need to be included because the new E1AP IE is needed just to trigger the collection of measurements at the gNB-CU-UP, since the reporting to the gNB-CU-CP is only applicable to the aggregated DL/UL packet delay measurements – refer to the next 2.1.3 Section.

Proposal 5: RAN3 to agree to introduce a new Packet Delay for UE Performance IE which includes the codepoints ‘DL and UL’ and ‘stop’ to start and stop the collection at gNB-CU-UP of both DL and UL delay measurements, respectively.

Concerning the E1 procedure(s) to be used to provide the DL/UL delay measurements collection configuration, we think it is reasonable to reuse the existing E1AP Bearer Context Setup and Bearer Context Modification (gNB-CU-CP initiated) procedures, which are enhanced to include the new IE as proposed in Proposal 4. The presence of the new IE triggers the gNB-CU-UP to collect DL and UL packet delay measurements, which are then aggregated with the gNB-DU delay measurements and eventually reported to the gNB-CU-CP over E1.

Proposal 6: RAN3 to agree that the existing E1AP Bearer Context Setup and Bearer Context Modification (gNB-CU-CP initiated) procedures are enhanced to include the new Packet Delay for UE Performance IE for the collection configuration of DL/UL delay measurements at gNB-DU within the BEARER CONTEXT SETUP REQUEST and BEARER CONTEXT MODIFICATION REQUEST messages.

### 2.1.3 On how to configure the reporting over E1 of the overall packet delay DL/UL (per-DRB per-UE) to the gNB-CU-CP

For this purpose, and still taking into account the scenario as in Figure 1 of a target split gNB being involved in XnAP Data Collection Reporting (Initiation) procedures from a source gNB, we should follow an approach similar to the one used since Rel-18:

- rely on an initiation procedure (target gNB-CU-CP  target gNB-CU-UP) to provide the configuration to be used by the target gNB-CU-UP to configure the reporting of the overall (aggregated) DL/UL packet delay measurements, including the reporting periodicity;

- identify procedure(s) which triggers the reporting of measurements, which is(are) properly linked with the initiation procedure as in step a. via a Data Collection ID IE;

- identify a procedure for the actual reporting of the overall DL/UL packet delay measurements.

For aspects a. and c., RAN3 already agreed in the last meeting to introduce new Data Collection Reporting Initiation and Data Collection Reporting procedures over E1 [4]:

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over E1 interface to transfer UL/DL UE throughput and UL/DL packet delay from CU-UP to CU-CP upon request.

The latest E1AP BLCR in [10] includes a bit in the Report Characteristics for Data Collection IE (3rd bit) dedicated to the request of the overall DL/UL packet delay (i.e., the aggregated value) and the Reporting Periodicity for Data Collection IE as part of the DATA COLLECTION REQUEST message, while it is yet FFS the reporting of information within the DATA COLLECTION UPDATE message. Our understanding is that the reporting periodicity of the overall DL/UL packet delay measurements should we set according to the value as configured by the source gNB as part of the XnAP DATA COLLECTION REQUEST message.

Proposal 7: RAN3 to agree that the reporting periodicity of the overall DL/UL packet delay measurements over E1 is set by the gNB-CU-CP according to the value as received from the source gNB as part of the XnAP DATA COLLECTION REQUEST message.

For the content of the E1AP DATA COLLECTION UPDATE message, since RAN3 agreed that the reporting of the overall DL/UL packet delay measurements is per-DRB per-UE [4], we think that a new IE similar to the XnAP UE Associated Info Result List IE should be introduced to report the overall DL/UL packet delay measurements on a per-DRB per-UE granularity.

Proposal 8: RAN3 to agree that a new UE Associated Info Result List IE is introduced in the E1AP DATA COLLECTION UPDATE message to report the overall DL/UL packet delay measurements on a per-DRB per-UE granularity.

Finally, for aspect b., we think that the existing E1AP Bearer Context Setup and Bearer Context Modification (gNB-CU-CP initiated) procedures could be enhanced to include a new IE similar to the XnAP Data Collection ID IE in order to trigger the reporting of the overall DL/UL packet delay measurements from gNB-CU-UP to gNB-CU-CP which is eventually performed via the E1AP DATA COLLECTION REPORTING message based on the collection configuration provided by the E1AP DATA COLLECTION REQUEST message. This approach follows the Rel-18 behaviour where the HANDOVER REQUEST includes the Data Collection ID IE to trigger the collection and reporting of UE performance over Xn via the Data Collection Reporting procedure according to the configuration provided as part of the Data Collection Reporting Initiation procedure.

Proposal 9: RAN3 to agree that the existing E1AP Bearer Context Setup and Bearer Context Modification (gNB-CU-CP initiated) procedures are enhanced to include a new Data Collection ID IE associated to the Data Collection Reporting Initiation over E1 used to configure the collection and reporting of the aggregated DL/UL delay measurements from gNB-CU-UP to gNB-CU-CP.

### 2.1.4 Other aspects

Same as for the XnAP Data Collection Reporting Initiation procedure, we think that partial reporting should be supported also as part of the new Data Collection Reporting Initiation procedure over E1AP. For reporting the list of measurements failed to be reported by the gNB-CU-UP, and considering that UE performance metrics are defined at node level, the DATA COLLECTION RESPONSE message in E1AP needs include the Node Measurement Initiation Result List IE which contains a bitmap indicating the measurements failed to be reported and an appropriate cause value.

Proposal 10: RAN3 to agree to include the Node Measurement Initiation Result List IE which lists the measurements failed to be reported by the gNB-CU-UP with an appropriate cause value.

A TP for the E1AP BLCR reflecting proposals above in provided in [11].

Another aspect that should be discussed concerns the granularity of overall DL/UL packet delay measurements once made available at the gNB-CU-CP, which need to be reported over the Xn interface to the source gNB. For Rel-18, the granularity of UE performance (including packet delay) has been specified to be per-UE, but the granularity of the overall DL/UL packet delay measurements at the gNB-CU-CP was agreed to be per-DRB per-UE. Hence, to derive the corresponding per-UE metrics, the RAN3 common understanding from RAN3#124 can be considered, that is, per-UE DL/UL Average Packet Delay is derived as the average value of per-DRB per-UE overall DL/UL packet delay measurements [12]:

#### UE level performance is derived as the average value of per DRB level measurements of the UE. **** Skip non-relevant text ****

As a consequence, it will be then up to the target gNB-CU-CP to derive the corresponding per-UE DL/UL Average Packet Delay measurements following the RAN3 common understanding from RAN3#124 and report over Xn to the source gNB.

Proposal 11: RAN3 to agree that it will be up to the gNB-CU-CP to derive the corresponding per-UE DL/UL Average Packet Delay measurements originating from per-DRB per-UE measurements sent over E1 by the gNB-CU-UP.

## 2.2 On gNB-DU Energy Cost (EC) reporting over F1

In RAN3#125bis meeting, as part of the Rel-18 leftover related to split architecture support for Rel-18 use cases, RAN3 agreed to introduce the Data Collection Reporting Initiation and Data Collection Reporting procedures over F1, as done in Rel-18 for the Xn interface, to enable the reporting of the measured Energy Cost (EC) from the DU to the CU [13]:

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU.

In the RAN3#126 meeting, further agreements for enabling the reporting of the gNB-DU’s EC to the gNB-CU, and for the intended usage of the reported gNB-DU’s EC have been reached [14]:

In split architecture, the Energy Cost of gNB is the sum of the Energy Cost of its gNB-DUs. Mapping rule is only configured at gNB-DU and shall be unified for all the gNB-DUs within a defined area.

Moreover, due to the support needed from the OAM side for configuring the unified “energy consumption – to – energy cost” mapping rule to the gNB-DU, RAN3 agreed to send a LS to SA5 in [15].

Based on the discussion in previous sections related to UE performance reporting for the split architecture case and, pending SA5 feedback on the feasibility to introduce new PDCP-level measurements, it could be plausible to assume that RAN3 may eventually end up in exchanging the UE performance metrics over the E1 interface only, i.e., without any impact on the F1 interface. Based on this assumption, RAN3 captured a WA in the last meeting, which basically suggests revisiting the agreements from previous meetings on the reporting of the gNB-DU’s EC over F1, in order to exchange this information by enhancing an existing procedure such as the Resource Status Reporting rather than introducing a brand new procedure in Rel-19 for purpose of transferring just a single metric (i.e., the gNB-DU’s EC) [4]:

Change the agreement: Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU. To the following WA: WA: Using the Legacy procedure, i.e., resource status initiation procedure & resource status report procedure to request and report measured EC over F1 interface.

We think that, based on the above assumptions, it would be better to turn the WA into an agreement.

Proposal 12: RAN3 to agree to turn the WA from RAN3#127bis into an agreement, hence the exchange of the gNB-DU’s EC is performed by enhancing the existing Resource Status Reporting (Initiation) procedures over F1.

The TP for the F1AP BLCR in [9] also reflects the above Proposal 12.

In Rel-18, and as part the Stage 2 description of the AI/ML for NG-RAN function, clause 16.20.4 was included in TS 38.300 to list the OAM requirements needed to enable the configuration of the “energy consumption – to – energy cost” mapping rule (which is defined by the Operator and it is provided by the OAM to the gNB), as well as the recommended time window which should be considered by the gNB itself when averaging the energy consumption measurements [17]:

### 16.20.4 OAM Requirements OAM configures the following: - The minimum and maximum energy consumption values corresponding to the minimum and maximum EC index values respectively, based on an implementation-specific mapping rule, which is unified within a defined area; and - The recommended time interval within which an NG-RAN node selects an implementation-specific time window for averaging of the measurements of the NG-RAN node's consumed energy.

As previously stated, in RAN3#126 it was agreed that the “energy consumption – to – energy cost” mapping rule should also be configured in the gNB-DU by the OAM, and as a consequence RAN3 also sent the LS to SA5 in [15] to ask SA5 to introduce changes in their specifications to enable such OAM configuration. SA5 replied to RAN3 in [16] to inform RAN3 of the changes agreed in SA5’s relevant specifications to accomplish RAN3 requirements.

In the LS, however, the recommended time interval which should be configured in the gNB-DU along with the mapping rule was not mentioned. To make the support of the gNB-DU’s EC reporting consistent with the reporting of the gNB’s EC in case of non-split architecture, we think that RAN3 should agree to include the recommended time interval in the list of OAM requirements to be then captured in TS 38.401 properly.

Proposal 13: RAN3 to agree to include the recommended time interval for averaging of the gNB-DU’s energy consumption measurements in the list of OAM requirements, along with the “energy consumption – to – energy cost” mapping rule previously agreed.

If Proposal 13 is agreed, then RAN3 should make SA5 aware of the above agreement, so that SA5 could finalize the work needed to introduce the OAM support to enable the reporting of the gNB-DU’s EC.

Proposal 14: RAN3 to agree to send a LS to SA5 (reply LS to R3-251529) to make SA5 aware of the need to introduce the recommended time interval for averaging of the gNB-DU’s energy consumption measurements in their relevant specifications.

A draft Reply LS to SA5 is provided in [19].

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals:

On reporting measured UE performance over E1

Proposal 1: RAN3 to agree to wait for the SA5 feedback on whether PDCP-level UE packet loss rate and UE throughput measurements can be specified and hence referred in RAN3 specifications for the UE performance.

On how to configure the collection and reporting over F1-U of the packet delay DL/UL (per-DRB per-UE) measured by gNB-DU

Observation 1: The target gNB-CU-CP has full control of the measurement collection process towards the gNB-DU, based on the collection time duration and reporting periodicity as provided over Xn from the source gNB.

Observation 2: The gNB-DU should only be provided with ‘start’ and ‘stop’ indications (to trigger the start and stop of the delay measurements in the gNB-DU, based on the collection time duration being known at the gNB-CU-CP) and reporting periodicity.

Observation 3: The ‘start’ indication to trigger the collection of delay measurements at gNB-DU can be a ‘both DL and UL’ measurement indication.

Proposal 2: RAN3 to agree to introduce a new Packet Delay for UE Performance which includes the codepoints ‘DL and UL’ and ‘stop’ to start and stop the collection at gNB-DU of both DL and UL delay measurements, as well as the reporting periodicity.

Proposal 3: RAN3 to agree that the reporting of DL and UL delay measurements from gNB-DU to gNB-CU-UP over F1-U is per-DRB per-UE, as already specified in TS 38.425.

Proposal 4: RAN3 to agree that the existing F1AP UE Context Setup and UE Context Modification (gNB-CU initiated) procedures are enhanced to include the new Packet Delay for UE Performance IE for the configuration of DL/UL delay measurements at gNB-DU within the UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION REQUEST messages.

A TP for the F1AP BLCR reflecting proposals above in provided in [9].

A TP for the AI/ML BLCR to TS 38.425 in [18].

On how to configure the collection at the gNB-CU-UP of the packet delay DL/UL (per-DRB per-UE)

Proposal 5: RAN3 to agree to introduce a new Packet Delay for UE Performance IE which includes the codepoints ‘DL and UL’ and ‘stop’ to start and stop the collection at gNB-CU-UP of both DL and UL delay measurements, respectively.

Proposal 6: RAN3 to agree that the existing E1AP Bearer Context Setup and Bearer Context Modification (gNB-CU-CP initiated) procedures are enhanced to include the new Packet Delay for UE Performance IE for the collection configuration of DL/UL delay measurements at gNB-DU within the BEARER CONTEXT SETUP REQUEST and BEARER CONTEXT MODIFICATION REQUEST messages.

On how to configure the reporting over E1 of the overall packet delay DL/UL (per-DRB per-UE) to the gNB-CU-CP

Proposal 7: RAN3 to agree that the reporting periodicity of the overall DL/UL packet delay measurements over E1 is set by the gNB-CU-CP according to the value as received from the source gNB as part of the XnAP DATA COLLECTION REQUEST message.

Proposal 8: RAN3 to agree that a new UE Associated Info Result List IE is introduced in the E1AP DATA COLLECTION UPDATE message to report the overall DL/UL packet delay measurements on a per-DRB per-UE granularity.

Proposal 9: RAN3 to agree that the existing E1AP Bearer Context Setup and Bearer Context Modification (gNB-CU-CP initiated) procedures are enhanced to include a new Data Collection ID IE associated to the Data Collection Reporting Initiation over E1 used to configure the collection and reporting of the aggregated DL/UL delay measurements from gNB-CU-UP to gNB-CU-CP.

A TP for the E1AP BLCR reflecting proposals above in provided in [11].

Other aspects

Proposal 10: RAN3 to agree to include the Node Measurement Initiation Result List IE which lists the measurements failed to be reported by the gNB-CU-UP with an appropriate cause value.

Proposal 11: RAN3 to agree that it will be up to the gNB-CU-CP to derive the corresponding per-UE DL/UL Average Packet Delay measurements originating from per-DRB per-UE measurements sent over E1 by the gNB-CU-UP.

On gNB-DU EC reporting

Proposal 12: RAN3 to agree to turn the WA from RAN3#127bis into an agreement, hence the exchange of the gNB-DU’s EC is performed by enhancing the existing Resource Status Reporting (Initiation) procedures over F1.

The TP for the F1AP BLCR in [9] also reflects the above Proposal 12.

Proposal 13: RAN3 to agree to include the recommended time interval for averaging of the gNB-DU’s energy consumption measurements in the list of OAM requirements, along with the “energy consumption – to – energy cost” mapping rule previously agreed.

Proposal 14: RAN3 to agree to send a LS to SA5 (reply LS to R3-251529) to make SA5 aware of the need to introduce the recommended time interval for averaging of the gNB-DU’s energy consumption measurements in their relevant specifications.

A draft Reply LS to SA5 is provided in [19].

# 4. Reference

- RP-250326, “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#107

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- 3GPP RAN3#127bis Chairman’s notes (RAN3_127bis_agenda_20250415_EOM1.doc)

- 3GPP RAN3#127 Chairman’s notes (RAN3_127_agenda_20250221_1300_EOM.doc)

- R3-252436, LS on clarification on UE performance measurement at PDCP level, Rel-19, RAN3 (ZTE)

- 3GPP TS 38.473, NG-RAN; F1 Application Protocol (F1AP), Rel-19

- 3GPP TS 38.425, “NG-RAN; NR user plane protocol”, Rel-19

- R3-253498, (TP for AI/ML BLCR to TS 38.473) F1AP enhancements for split architecture support for Rel-18 “AI/ML for NG-RAN” use cases, Huawei, CMCC, China Unicom

- R3-252503, (BL CR to 37.483) Support of enhancements on AI/ML for NG-RAN, Samsung, ZTE Corporation

- R3-253499, (TP for AI/ML BLCR to TS 37.483) E1AP enhancements for split architecture support for Rel-18 “AI/ML for NG-RAN” use cases, Huawei, CMCC, China Unicom

- 3GPP RAN3#124 Chairman’s notes (RAN3_124_agenda_20240524_eom1.doc)

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- 3GPP RAN3#126 Chairman’s notes (RAN3_126_agenda_20241122_EOM1.doc)

- R3-247907, LS on energy saving for split gNB, RAN3 (CATT)

- R3-251529, LS reply to LS on energy saving for split gNB, Response to R3-247907 (SA5-250288), SA5 (Ericsson)

- 3GPP TS 38.300 “NR; NR and NG-RAN Overall description; Stage-2”, Rel-18

- R3-253500, (TP for AI/ML BLCR to TS 38.425) NR user plane protocol enhancements for split architecture support for Rel-18 “AI/ML for NG-RAN” use cases, Huawei, CMCC, China Unicom

- R3-253501, [DRAFT] LS reply to LS on energy saving for split gNB, Response to R3-252529 (S5-251087), Rel-19, Huawei, CMCC, China Unicom