3GPP TSG-RAN WG3 Meeting #128	R3-253650

Malta,MT, 19th – 23rd May 2025

Agenda item:	19.2

Source: 	Samsung

Title: 	Further discussion on information exchange for CLI handling

Document for:	Discussion and decision

# Introduction

There is quite some progress made on the information exchange for CLI handling during the meeting of RAN3#127-bis, especially on stage-3 details. In this paper, we tried to provide some further consideration on solving the remaining FFSes on the current BLCRs of XnAP and F1AP, as well as some opininos related to the Editor’s notes.

# Discussion

In the current stage-3 BLCRs, the definition of SBFD configuration is kept as a sub-IE for further study, pending RAN2 progress.

| >>>SBFD Configuration | O |  | FFS (pending on RAN2 progress) | FFS | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

It is noticed that, during the post-meeting email disscussion of RAN2#129-bis, a running CR for SBFD [1] has been created, to capture the agreements achieved in RAN2. When it comes to the SBFD configuration, according to the speific changes in the running CR, the time configuration parameters are defined separately within the TDD-UL-DL-ConfigCommon IE, as part of TDD-UL-DL-Pattern field, as shown below:

As for the frequency information, there is still not specific definition in RAN2 running CR. But what we could conclude from the progress of RAN2 is that, it is impossible to define a single container in network interface to cover all the SBFD configuration related parameters defined by RAN2, because these parameters are defined separately in the existing TDD configuration. Thus, we would propose to define a new separated IE over XnAP and F1AP to cover the time and frequency configuration of SBFD.

Proposal 1: Define a new separated IE in XnAP and F1AP to cover the time and frequency configuration of SBFD.

Regarding the remaining things in the stage-3 BLCRs, there is an Editor’s note for the name of the new procedure and procedure text:

Eitor’s Note: The name of the new procedure and details of procedure text can be further discussed.

For the procedure name, we don’t see any problem when calling this new procedure ‘CLI Measurement Update’. First of all, CLI is a terminology which have been mentioned in mant specifications and we believe there is no difficult for technique experts to understand the meaning of this word. As long as the full name of CLI, i.e., Cross-Link Interference, is captured in the abbreviation part of the spefication, it is totally fine to use CLI in the name of a procedure. Besides, it is reasonable to use a general procedure name to cover all the specific information to be exchanged for CLI handling, e.g., strongest DL beam, CLI mitigation request. Even if the CLI mitigation request is not exactly a kind of measurement, it should be based on the measurements of the victim gNB. So, we hold the view that the current name of the new procedure is suitable.

Proposal 2: The name of the new procedure, i.e., CLI Measurement Update, can be kept as it is.

When it comes to the procedure text, we admit some refinement is needed. The draft TPs for the BLCR of XnAP and F1AP are provided in [1]. After the confirmation of the new procedure name and refinement of procedure text, the corresponding Editor’s note could be removed.

Proposal 3: Remove the Editor’s Note on the name of new procedure and procedure text.

Text proposals for BLCRs of 38.423 and 38.473 are provided in [2].

# Conclusion

Proposal 1: Define a new separated IE in XnAP and F1AP to cover the time and frequency configuration of SBFD.

Proposal 2: The name of the new procedure, i.e., CLI Measurement Update, can be kept as it is.

Proposal 3: Remove the Editor’s Note on the name of new procedure and procedure text.

# References

[1] R2-25xxxxx RRC running CR for Evolution of NR duplex operation (SBFD)_v05_Rapp

[2] R3-253651 Text proposals to BLCRs of TS38.423 & TS38.473 on CLI handling