3GPP TSG-RAN WG3 Meeting #128	R3-253663

Malta, MT, May 19th – 23rd, 2025

Agenda item:	14.3

Source: 	LG Electronics Inc., Nokia, Nokia Shanghai Bell

Title: 	(TP for NR_NTN_Ph3 TS 38.300 BL CR) OAM for NG management

Document for:	Other

# Introduction

This contribution proposes text proposal for TS 38.300 BL CR [1] (rapporteur: E///), based on the discussions proposed in [2].

# Reference

[1] R3-253093, “(BL CR to 38.300) Support for Regenerative Payload and MBS broadcast in NR NTN” Ericsson, Thales, Deutsche Telekom, Nokia, ESA, CATT, ZTE, Sateliot, Huawei, Dish Networks, Echostar, Eutelsat Group, Xiaomi, Samsung, CMCC, LG Electronics, NEC, Lenovo, ETRI

[2] R3-253662, “OAM requirement for NG management”, LG Electronics Inc., Nokia, Nokia Shanghai Bell

# TP for NR_NTN_Ph3 TS 38.300 BL CR

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 16.14.7	O&M Requirements

The following NTN related parameters shall be provided by O&M to the gNB providing NTN access:

-	Ephemeris information describing the orbital trajectory information or coordinates for the NTN payload. This information is provided on a regular basis or upon demand to the gNB;

-	Two different sets of ephemeris format shall be supported:

-	Set 1: NTN payload position and velocity state vectors:

-	Position;

-	Velocity.

-	Set 2: At least the following parameters in orbital parameter ephemeris format, as specified in NIMA TR 8350.2 [51]:

-	Semi-major axis;

-	Eccentricity;

-	Argument of periapsis;

-	Longitude of ascending node;

-	Inclination;

-	Mean anomaly at epoch time.

-	The explicit epoch time associated to ephemeris data;

-	The location of the NTN Gateways;

NOTE 1:	The ephemeris of the NTN payloads and the location of the NTN Gateways, are used at least for the Uplink timing and frequency synchronization. It may also be used for the random access and the mobility management purposes.

-	Additional information to enable gNB operation for feeder/service link switch overs.

NOTE 2:	The NTN related parameters provided by O&M to the gNB may depend on the type of supported service links, i.e., Earth-fixed, quasi-Earth-fixed, or Earth-moving.

In addition, the following NTN related parameters shall be provided by O&M to the gNB hosted by the regenerative NTN payload:

-	The service areas of the AMFs for NG connection management.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////