3GPP TSG-RAN WG3 Meeting #128	R3-253677

Malta, MT, 19 – 23 May 2025

Agenda Item:	16.2

Source: 				LG Electronics Inc.

Title:			Discussion on remaining open issues in A-IoT

Document for:		Discussion and Decision

1.	Introduction

SA2 sent a LS in [1] to clarify the “supported Area” concept. Also, another LS from SA2 in [2] to check whether “Service Type” (i.e., Inventory or Command) information in “Assistance Information” is needed or not. In addition, it is still FFS on whether the NGAP signaling is enhanced to provide the A-IoT RAN Information (Reader List, Reader location, Supported A-IoT Areas) to the AIOTF. In this contribution, we will discuss the issues raised by SA2 and the remaining open issue in RAN3, and then provide our views on it.

2.	Discussion

In [3], the RAN3 agreements were provided to the SA2. Then, in [1], SA2 replied to RAN3 with the questions on the “supported Area” concept as follows:

S2-2504288 [1]: … SA2 would like to reply as following: - AIOTF obtains the A-IoT RAN information (supported Area, served reader ID list) via OAM configuration, FFS on RAN3 signalling for the A-IoT RAN information. [SA2 reply]: - SA2 would ask whether the ‘supported Area’ is Tracking Area or a new type of area. - SA2 would like to ask whether one reader is only part of one supported Area or can be part of multiple supported Areas. …

Regarding the first question, RAN3 already concluded to introduce a new “A-IoT Area” in RAN3#127bis meeting. The remaining issue is whether the new A-IoT Area is encoded as GNSS info or new A-IoT Area ID. We think that a new A-IoT Area ID should be defined, while the corresponding geographical area info (e.g., GNSS info) for each A-IoT Area ID is optionally used. A new A-IoT Area ID is a very similar concept with the legacy TAC and RANAC. This ID identifies the part of the service Area in a combination of a PLMN ID. The corresponding GNSS info for each A-IoT Area ID may be also used. For example, the mapping between the A-IoT Area ID and the corresponding GNSS info can be configured in CN by OAM. Therefore, if supported Area which is consisted of the A-IoT Area IDs is provided to the AIOTF by the NGAP signaling, the geographical area can be deduced based on the supported Area from the gNB and the mapping information by OAM. It is also possible that both the A-IoT Area ID and the corresponding GNSS info are provided to the AIOTF by the NGAP signaling (e.g., NG SETUP). In this case, the AIOTF can translate the service Area into the geographical area based on the information from the gNB.

Observation 1: RAN3 already agreed to introduce a new “A-IoT Area”.

Proposal 1: The new A-IoT Area is encoded as a new A-IoT Area ID.

Proposal 1a: The corresponding GNSS info is optionally used as the A-IoT Area together with the A-IoT Area ID.

RAN3 basically confirmed that the AIOTF obtains the A-IoT RAN information (Reader List, Reader location, Supported Areas) via OAM configuration. However, it is still FFS on whether the NGAP signaling is enhanced to provide the A-IoT RAN Information (Reader List, Reader location, Supported Areas) to the AIOTF. Regarding this issue, we think that this option (i.e., NGAP signaling for the A-IoT RAN Information) enables the operator to dynamically manage/update the Reader related information in the AIOTF, whereas it causes the Stage 3 specification impact. Specifically, if proposal 1 is agreed (i.e., a new “A-IoT Area ID” is defined), the Reader in the NG-RAN can dynamically control its supported Area and provide it to the AIOTF without the OAM configuration. Also, it seems that this option is very similar to the approach related to the TAC supported by the NG-RAN. Therefore, foreseen specification impacts are already well known.

Observation 2: The NGAP signaling for the A-IoT RAN Information enables the operator to dynamically manage/update the Reader related information in the AIoTF.

Observation 3: Foreseen specification impacts on NGAP signaling are already well known.

Based on the above observations, the following proposal is also suggested to RAN3:

Proposal 2: The NG-RAN provides the A-IoT RAN information (Reader List, Reader location, Supported Areas) to the A-IoT CN node during the NG setup procedure.

Another SA2 question in [1] is whether one reader is only part of one supported Area or can be part of multiple supported Areas. We think that one reader has a single A-IoT Area ID value. That is, the supported Area of the Reader is the geographical area represented by the corresponding A-IoT Area ID. As a result, the supported Area of the NG-RAN is consisted of the supported Areas of the Readers belonging to the NG-RAN. Therefore, if the Requested Service Area Information IE is consisted of the Reader list, this is eventually translated into the corresponding A-IoT Area ID list.

Proposal 3: One reader is only part of one supported Area.

Another LS [2] from SA2 is related to whether “Service Type” (i.e., Inventory or Command) information in “Assistance Information” provided by the AIOTF to the NG-RAN is still needed. However, since RAN3 already agreed to use separate NGAP procedures for Inventory and Command, the NG-RAN can differentiate between the Inventory service and the Command service upon receiving the NGAP message from the AIOTF. It was also agreed to introduce a follow-on command indication in the Inventory Request Transfer IE. Therefore, upon the reception of the follow-on command indication, the NG-RAN becomes aware of the “Command after inventory” scenario. As a result, there is no need to define “Service Type” in “Assistance Information”.

Proposal 4: There is no need to define “Service Type” in “Assistance Information”.

Based on the Proposals, we propose to agree the Relay LSes to SA2 in Appendices.

Proposal 5: It is proposed to agree the Relay LSes to SA2 in Appendices.

3.	Conclusion

In this contribution, we focused on the issues raised by SA2 and the remaining open issue in RAN3, and then provided our views on it. The following proposal is kindly suggested to RAN3:

Proposal 1: The new A-IoT Area is encoded as a new A-IoT Area ID.

Proposal 1a: The corresponding GNSS info is optionally used as the A-IoT Area together with the A-IoT Area ID.

Proposal 2: The NG-RAN provides the A-IoT RAN information (Reader List, Reader location, Supported Areas) to the A-IoT CN node during the NG setup procedure.

Proposal 3: One reader is only part of one supported Area.

Proposal 4: There is no need to define “Service Type” in “Assistance Information”.

Proposal 5: It is proposed to agree the Relay LSes to SA2 in Appendices.

4.	Reference

- S2-2504288 (R3-253015), “LS Reply on Ambient IoT progress of RAN3,” SA2 (NEC), April, 2025.

- S2-2504294 (R3-253016), “LS on the removal of service type information,” SA2 (InterDigital), April, 2025.

- R3-250905, “LS on Ambient IoT progress of RAN3”, RAN3 (Huawei), February, 2025.

5.	Appendix#1: Reply LS for S2-2504288 [1]

This appendix provides the draft reply LS to SA2 based on the proposals of this contribution.

----------------Start of the First Change---------------

3GPP TSG-RAN WG3 #128 R3-25xxxx Malta, MT, 19th – 23rd May 2025 # Title: [Draft] Reply LS on Ambient IoT progress of RAN3 # Response to: R3-253015/S2-2504288 # Release: Rel-19 # Work Item: AmbientIoT-ARC Source: RAN3 To: SA2 Cc: SA5 Contact Person: #### Name: Seokjung Kim #### E-mail Address: seokjung.kim@lge.com Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org # Attachments: - # 1 Overall description RAN3 thanks SA2 for the LS Reply on Ambient IoT progress of RAN3. RAN3 would like to provide the following feedback to SA2 regarding questions on ‘supported Area’ in the A-IoT RAN information. Q1) SA2 would ask whether the ‘supported Area’ is Tracking Area or a new type of area. RAN3’s Answer for Q1): RAN3 has agreed to introduce a new “A-IoT Area” in RAN3#127bis meeting. In addition, RAN3 has discussed the encoding of the new A-IoT Area, and concluded that the new A-IoT Area is encoded as a new A-IoT Area ID, while the corresponding GNSS info for the A-IoT Area ID is optionally included in the A-IoT Area. Q2) SA2 would like to ask whether one reader is only part of one supported Area or can be part of multiple supported Areas. RAN3’s Answer for Q2): RAN3 has concluded that one reader has a single A-IoT Area ID value. That is, the supported Area of the Reader is the geographical area represented by the corresponding A-IoT Area ID. As a result, the supported Area of the NG-RAN is consisted of the supported Areas of the Readers belonging to the NG-RAN. In addition, RAN3 also agreed that the NG-RAN provides the A-IoT RAN information (Reader List, Reader location, Supported Areas) to the A-IoT CN node during the NG setup and RAN Configuration Update procedure. 2. Actions: To SA2: ACTION: RAN3 kindly asks SA2 to take the above into account. 3. Date of Next RAN3 Meetings: 3GPP TSG RAN3#129 25 – 29 August 2025 Bangalore, India 3GPP TSG RAN3#129bis 13 – 17 October 2025 Prague, CZ

-----------------End of the First Change---------------

6.	Appendix#2: Reply LS for S2-2504294 [2]

This appendix provides the draft reply LS to SA2 based on the proposals of this contribution.

----------------Start of the First Change---------------

3GPP TSG-RAN WG3 #128 R3-25xxxx Malta, MT, 19th – 23rd May 2025 # Title: [Draft] LS on the removal of service type information # Response to: R3-253016/S2-2504294 # Release: Rel-19 # Work Item: AmbientIoT-ARC Source: RAN3 To: SA2, RAN2 Cc: RAN1 Contact Person: #### Name: Seokjung Kim #### E-mail Address: seokjung.kim@lge.com Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org # Attachments: - # 1 Overall description RAN3 thanks SA2 for the LS on the removal of service type information. RAN3 would like to further provide the following feedback to SA2. With respect to “Service Type”, RAN3 agreed to use separate NGAP procedures for Inventory and Command, respectively. Therefore, the NG-RAN can differentiate between the Inventory service and the Command service upon receiving the NGAP message from the AIOTF. RAN3 also confirmed to introduce a follow-on command indication in the Inventory Request Transfer IE. Upon the reception of the follow-on command indication, the NG-RAN becomes aware of the “Command after inventory” scenario. As a result, there is no need to define “Service Type” in “Assistance Information”. 2. Actions: To SA2 and RAN2: ACTION: RAN3 kindly asks SA2 and RAN2 to take the above into account. 3. Date of Next RAN3 Meetings: 3GPP TSG RAN3#129 25 – 29 August 2025 Bangalore, India 3GPP TSG RAN3#129bis 13 – 17 October 2025 Prague, CZ

-----------------End of the First Change---------------