3GPP TSG-RAN WG3 Meeting #128														R3-253684

Malta, MT, May 19 - 23, 2025

Agenda item:			11.3

Source:					CMCC

Title:						(TP to BLCR for 38.401 and 38.473) AI/ML based CCO

Document Type:		Other

Introduction

During RAN3#127bis, the following agreements have been captured for AI/ML enabled Coverage and Capacity Optimization.

The maximum value of the Time interval for predicted CCO issue and future CCO state is 60s.

Legacy UE performance measurement metrics can be reused for CCO.

For the receiving side, whether gNB-CU can also provide to gNB-DU a future CCO state as assistance information to derive a matching future CCO state?

- Option1: no need

- Opton2: gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU.

- Option3: gNB-CU generates a recommend further CCO state and send to gNB-DU.

Whether/what additional UE performance measurement metrics is needed?

Timing information for predicted CCO issue is NOT needed to exchange over Xn? Whether the predicted CCO issue and /or future CCO state can be updated over Xn?

In this paper, we provide our consideration on the FFS of AI/ML based Coverage and Capacity Optimization.

Discussion

Enhancement over XnAP

Timing information for predicted CCO issue is NOT needed to exchange over Xn?

In RAN3#127bis, RAN3 has left an FFS on whether to exchange time information of predicted CCO issue over Xn. From our understanding, whether the time information of the coverage modification cause needs to be transferred over Xn depends on whether this time information is needed in the target node. For the target node, what it needs to do is to modify its own coverage at the time the coverage of source node changes. So, the target node needs to know when to make what kind of adjustment. Since the target node has received the future CCO state and the corresponding time information, it can be used by the target node to determine when to make the corresponding modification. Therefore, the time information of the predicted CCO issue seems to be less essential.

Proposal 1: No need to send time information related to coverage modification cause over Xn.

Whether the predicted CCO issue and /or future CCO state can be updated over Xn?

Then, for whether the predicted CCO issue and/or future CCO state can be updated over Xn, we think that it’s feasible to update the predicted CCO issue and/or future CCO state before the time for future CCO state. As the predicted CCO issue and future CCO state are the predictions for the future time, the local node and neighbour node will not applied the future CCO state until the time for future CCO state. If the coverage of the local node or neighbour node changed after the future CCO state generated time and before the future CCO state applied time, the original predicted CCO issue may not occur anymore at the time for predicted CCO issue and the future CCO state may invalid, or the predicted CCO issue still exists but the future CCO state may no longer be applicable to the predicted CCO issue. Therefore, before the future CCO state applied, any updates to the predicted CCO issue and/or future CCO state can be transferred over Xn.

Observation 1: The coverage of the local node and neighbour node may change after the future CCO state generated time and before the time for future CCO state. The predicted CCO issue/future CCO state may invalid after the coverage changes before the time for future coverage state, which needs to be informed to its neighbour nodes.

Proposal 2: RAN3 agrees that the predicted CCO issue and/or future CCO state can be updated over Xn before the time for future coverage state.

Suggested CCO state

For the receiving side, whether gNB-CU can also provide to gNB-DU a future CCO state as assistance information to derive a matching future CCO state?

Option1: no need

Opton2: gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU.

Option3: gNB-CU generates a recommend further CCO state and send to gNB-DU.

During the RAN3#127bis offline discussion, whether the receiving node gNB-CU can provide future CCO state as assistance information to gNB-DU is captured with above options.

In legacy CCO mechanism, the target DU first obtain the CCO state of source DU by itself, e.g. by means of detection/measurement, then the target DU can modify its CCO state correspondingly to match that of the source node. However, for AI/ML-enabled CCO, target DU currently cannot obtain the future CCO state of the source DU as the future CCO state will not applied until time for future CCO state. If the target CU only sends the CCO issue to the target DU, the future CCO state inferred by the target DU may not match the future CCO state of source DU. One straightforward way to resolve this issue is that the target CU directly forwards the received future CCO state of source DU to target DU, then the target DU can generate a future CCO state to match the source DU future CCO state. So, Option 2 seems reasonable.

Observation 2: In Legacy CCO mechanism, the target DU can derive the CCO state, which match the modified CCO state of source DU, as the target DU can obtain the CCO state of source DU by itself, e.g. by means of detection/measurement.

Observation 3: For AI/ML-enabled CCO, as the target DU cannot obtain the future CCO state of the source DU currently, the future CCO state generated by the target DU may not match the future CCO state of source DU.

For Option 3, from our understanding, the CCO state information is only configured to gNB-DU by OAM, the gNB-CU do not have such information, which is not possible to generate recommended future CCO state. If gNB-CU provides the recommended future CCO state to gNB-DU, the OAM should also provide the CCO state configuration to gNB-CU, which may have impact of OAM. Also, when the target CU forwards the received future CCO state of source DU to target DU, the target DU has the ability to derive the corresponding future CCO state to match that of source DU. Therefore, we think it’s no need for the target CU to generate recommended future CCO state. So, we prefer option 2 in above.

Observation 4: The CCO state information is only configured to gNB-DU by OAM, it is not possible for the target gNB-CU to generate recommended future CCO state.

Proposal 3: In the receiving side, the gNB-CU directly forwards the received future CCO state to gNB-DU.

Feedback information

Whether/what additional UE performance measurement metrics is needed?

In RAN3#127bis, RAN3 has agreed that the legacy UE performance measurements can be reused for CCO.

But how to evaluate predicted CCO issue or future CCO state need to be discussed.

From our understanding, for the future CCO state, it’s feasible to be evaluated, such as checking whether the UE experience severe performance degradation after coverage modification, and whether CCO issue still be detected after coverage modification. For example, as the UE(s) may locate in the gNB1 before coverage configuration changes, and locate in gNB2 after coverage configuration changes, if the UE experience severe performance degradation after coverage modification, it seems that the generated future CCO state is not very reasonable. Also, if the CCO issue still be detected after coverage modification, the future CCO state might not resolved the predicted CCO issue, and the selection of the CCO state is not very appropriate.

Observation 5: It’s feasible to validate whether the future CCO state is reasonable by checking whether the UE experience severe performance degradation and whether CCO issue still be detected after coverage modification.

But for the predicted CCO issue, it’s not clear how to evaluate because the predicted CCO issue is actually a CCO issue that has not occurred yet. Although the future CCO state is generated based on the predicted CCO issue, it cannot determine that this CCO issue will actually occur. If this predicted CCO issue will not occur in the future, then no matter how the future CCO state is modified, it cannot prove whether the prediction of CCO issue is accurate.

Observation 6: The predicted CCO issue is difficult to be evaluated because even if no CCO issue occurs after the applying the future CCO state, it cannot be determined whether the predicted CCO issue has been resolved or does not actually exist.

Proposal 4: The legacy UE performance can be used to validate the future CCO state.

Then, for the UE performance, we also propose to collect the coverage/cell edge UE(s)’ performance, i.e. the UE(s) serving gNB is changed after CCO, which is valuable to be checked as these UE(s) may mainly affected by CCO actions.

Proposal 5: It’s proposed to collect the performance of coverage/cell edge UE(s) (e.g. the UE(s) serving gNB is changed after CCO) as feedback information.

For the additional metrics, we also think the cell/node level gNB performance needs to be feedback from the target nodes. When the coverage configuration of local node changes, the coverage of neighbor nodes also need to be changed. Whether the coverage configuration modification caused by the local node results in severe performance degradation of neighbor node needs to be considered. For example, after the coverage configuration modification, the number of RRC connections of neighbor node increases, which may cause the neighbor node overload, or the PRB usage decrease, which waste the neighbor node resources. The neighbor nodes need to inform the local node these information and the local node can update the AI/ML model with these information.

Proposal 6: It’s proposed to collect the cell/node level performance of neighbor node(s) as feedback information, such as the number of RRC connections and PRB usage.

Conclusion

In this contribution, we propose the following proposals:

Proposal 1: No need to send time information related to coverage modification cause over Xn.

Observation 1: The coverage of the local node and neighbour node may change after the future CCO state generated time and before the time for future CCO state. The predicted CCO issue/future CCO state may invalid after the coverage changes before the time for future coverage state, which needs to be informed to its neighbour nodes.

Proposal 2: RAN3 agrees that the predicted CCO issue and/or future CCO state can be updated over Xn before the time for future coverage state.

Observation 2: In Legacy CCO mechanism, the target DU can derive the CCO state, which match the modified CCO state of source DU, as the target DU can obtain the CCO state of source DU by itself, e.g. by means of detection/measurement.

Observation 3: For AI/ML-enabled CCO, as the target DU cannot obtain the future CCO state of the source DU currently, the future CCO state generated by the target DU may not match the future CCO state of source DU.

Observation 4: The CCO state information is only configured to gNB-DU by OAM, it is not possible for the target gNB-CU to generate recommended future CCO state.

Proposal 3: In the receiving side, the gNB-CU directly forwards the received future CCO state to gNB-DU.

Observation 5: It’s feasible to validate whether the future CCO state is reasonable by checking whether the UE experience severe performance degradation and whether CCO issue still be detected after coverage modification.

Observation 6: The predicted CCO issue is difficult to be evaluated because even if no CCO issue occurs after the applying the future CCO state, it cannot be determined whether the predicted CCO issue has been resolved or does not actually exist.

Proposal 4: The legacy UE performance can be used to validate the future CCO state.

Proposal 5: It’s proposed to collect the performance of coverage/cell edge UE(s) (e.g. the UE(s) serving gNB is changed after CCO) as feedback information.

Proposal 6: It’s proposed to collect the cell/node level performance of neighbor node(s) as feedback information, such as the number of RRC connections and PRB usage.

- References

[1] RP-242385, “New WID: Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, ZTE, NEC

- Annex: TP for 38.473

    - 9.3.1.A	Predicted CCO Assistance Information

This IE provides predicted assistance information for the future Coverage and Capacity Optimisation (CCO) actions for predicted CCO issues.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Predicted CCO issue | O |  | ENUMERATED (coverage, cell edge capacity, ...) | Indicates the type of predicted CCO issue. |
| Predicted Affected Cells and Beams | O |  | Affected Cells and Beams 9.3.1.212 |  |
| Time for Predicted CCO Issue | O | INTEGER (1..60, ...) |  | Indicates the time when the predicted CCO issue will happen from the time of receiving this information, in seconds. |
| Future Coverage Modification List |  | 0..1 |  | List of cells whose coverage will be modified in neighbour node. This IE is received from neighbour node. |
| >Future Coverage Modification Item |  | 1..<maxnoofCellsinNG-RAN node> |  |  |
| >>Global NG-RAN Cell Identity | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN Cell Global Identifier of the cell whose coverage will be modified. |
| >>Future Cell Coverage State | M |  | INTEGER (0..63, ...) | Value ‘0’ indicates that the cell will be inactive. Other values indicates that the cell will be active and also indicates the future coverage configuration of the concerned cell. |
| >>Future SSB Coverage Modification List |  | 0..1 |  | List of SSB beams whose coverage will be modified. |
| >>>Future SSB Coverage Modification Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>SSB index | M |  | INTEGER (0..63) | Identifier of the SSB beam whose coverage will be modified. |
| >>>>Future SSB Coverage State | M |  | INTEGER (0..15, ...) | Value ‘0’ indicates that the SSB beam will be inactive. Other values indicate that the SSB beams will be active and also Indicates the future coverage configuration of the concerned SSB beams. |
| >>Time for Future Coverage State | O |  | INTEGER (1..60, …) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the NG-RAN node1 relative to the time of receiving this information, in seconds. |

- Annex: TP for 38.401

  - 7.9	Support for CCO

    - 7.9.1	General

The NR Capacity and Coverage Optimization (CCO) Function in non-split gNB case is specified in TS 38.300 [2]. The objective of this function is to detect and mitigate coverage and cell edge interference issues.

    - 7.9.2	OAM requirements

Each gNB-DU may be configured with alternative coverage configurations by OAM. The alternative coverage configurations contain relevant radio parameters and may also include a range for how each parameter is allowed to be adjusted.

    - 7.9.3	Dynamic coverage configuration changes

In case of split gNB architecture, CCO detection function is located at the gNB-CU. The gNB-CU signals to the gNB-DU the CCO issue and the affected cells and beams. The gNB-DU resolves the CCO issue concerning own served cell by local action within the OAM configured limits. The gNB-DU may also take into account information received for other cells when adopting the CCO configuration. The gNB-DU informs the gNB-CU of the new coverage states adopted.

    - 7.9.X	AI/ML enabled CCO

In case of split gNB architecture, CCO issue is predicted at the gNB-CU. The gNB-CU sends to the gNB-DU the predicted CCO issue with the predicted affected cells and beams and the time for predicted CCO issue. The gNB-DU generates the future coverage state(s) and the time when the future coverage state(s) will be applied based on the predicted CCO issue and other local information. The gNB-DU may also take into account information received for other cells when generates the future CCO configuration. The gNB-DU informs the gNB-CU of the future coverage state and the time when the future coverage state will be applied.