3GPP TSG-RAN WG3 #128	              R3-253690

MT, Malta, 19th – 23th May 2025

Agenda Item:	18.2

Source:	CMCC

Title:	Discussion on supporting LP-WUS Indicating Paging Monitoring

Document for:	Discussion

- Introduction

Rel-19 NR LP-WUS/WUR WI was approved in [1], which states that in RRC IDLE/INACTIVE modes, it's observed that significant UE power saving gain (up to more than 90%) is obtained by using LP-WUS/WUR to trigger UE MR paging monitoring compared with existing I-DRX operation (with and without PEI). We will discuss on the following objectives in this contribution.

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

In RAN2#129bis, RAN2 had the following agreement on subgrouping:

- Use 5G-S-TMSI to determine the UE_ID in the formula of UE_ID based subgrouping for LP-WUS, i.e., UE_ID=5G-S-TMSI mod X.

- X is based on 32 subgrouping number. Details can be discussed in the running CR.

- LP-WUS is supported with eDRX, FFS on exact impact if any

In RAN2#129, RAN2 had the following agreement on subgrouping:

- For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e.,

SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where

-	UE_ID is related to 5G-S-TMSI,

-	N is the number of total paging frames in one DRX cycle,

-	Ns is the number of the PO for a PF,

-	Np is the number of subgroupNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1,

-	subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

- Discussion

2.1 UE_ID based subgrouping

RAN1 made a working assumption that the number of LP-WUS subgroups is 31, which is different from 8 in PEI.

According to the agreement in RAN2#129 meeting, option 1 in fig.1 has been approved. According to RAN2#129bis meeting, eDRX and 5G-S-TMSI are supported. See the following LS for details.

LS R3-253026 RAN2 has discussed LP-WUS subgrouping and made the following agreements: LP-WUS UE-ID based subgrouping formula (RAN2#129): For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e., SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where - UE_ID is related to 5G-S-TMSI, - N is the number of total paging frames in one DRX cycle, - Ns is the number of the PO for a PF, - Np is the number of subgroupsNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1, - subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively. UE-ID calculation (RAN2#129bis): Use 5G-S-TMSI to determine the UE_ID in the formula of UE_ID based subgrouping for LP-WUS, i.e., UE_ID=5G-S-TMSI mod X. X is based on 32 subgrouping number. Details can be discussed in the running CR. LP-WUS with eDRX case (RAN2#129bis): LP-WUS is supported with eDRX, FFS on exact impact if any RAN2 assumes that the above information is necessary for RAN3 specification work (e.g., providing additional bits for the UE identity index value) and would like to request RAN3 to take them into account.

According to TS 38.473, the Extended UE Identity Index is 16 bits. The following diagram illustrates the UE_ID required for paging, PEI, and LP-WUS:

Fig 1 the UE_ID required for paging, PEI, and LP-WUS

As shown in the diagram, with option 1, when the UE uses eDRX and PEI is configured together with LP-WUS, a maximum of 20 bits is required for the UE_ID. Therefore, a new IE should be introduced into F1 paging to include extra bits for UE_ID based LP-WUS subgrouping.

Proposal 1: A new IE should be introduced into F1 paging to include extra bits for UE_ID based LP-WUS subgrouping, with at least 20 bits.

2.2 LP-WUS capability issue

In SA2#166 Meeting, SA2 send a reply LS to RAN2/3 to ask the following issue:

SA2 S2-2412876 2) Initial Registration may be performed on a pre-Rel 19 gNB (or a later gNB that does not support LP-WUS) that analyses the UE’s UE Radio Access Capabilities to create the UE Radio Capability for Paging Information and send it to the AMF, see clause 5.4.4.3 of TS23.501. It will be important that LP-WUS related aspects of UE Radio Capability for Paging Information are not lost when Initial Registration is performed on a pre-Rel-19 gNB. If the LP-WUS aspects are lost, then there can be paging problems, when for example, paging is performed by a gNB that supports LP-WUS. SA2 invites RAN2 and RAN3 to investigate this and provides solution if any issue is identified. SA2 was not sure whether this issue also impacts RedCap, eRedCap and other features that are signalled within the UE Radio Capability for Paging Information. SA2 invites RAN2 and RAN3 to investigate this.

In RAN2#129 meeting, following agreement has been approved:

RAN2#129 Agreement - Regarding the SA2 raised issue on UE Radio Capability for Paging Information, R2 understand that there is no issue for NW after Release 17 (in which case the LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container). Whether there is issue for the other cases (for the features mentioned by SA2 LS R2-2500050) can be further discussed in the main session.

In RAN2#129bis, the following LS is sent to RAN3:

LS R3-253012 RAN2 further discussed the issue of UE radio paging capability loss mentioned in the LS S2-2412876 and acknowledged that based on the legacy signalling, a gNB implementing Release X may not be able to forward the radio paging capabilities for a feature introduced after Release X, which results in UE missing the paging message. For UE Radio Paging Capabilities introduced up to Rel-18, RAN2 agreed not to introduce any standard solution, i.e., rely on proper network implementation, as follows. We will not change legacy UE capabilities and rely on network implementation To address the issue for UE Radio Paging Capabilities introduced in Rel-19 onwards, RAN2 made the following agreement: Use a “container” using OCTET STRING with content generated by UE when new UE Radio Paging Capabilities are introduced. This is applicable only to new Rel-19 and future capabilities added to paging capabilities. This will only work for gNB that support this Rel-19 “container” In particular for Rel-19 LP-WUS feature, if it is deployed, all the gNBs belonging to a certain area need to support the Rel-19 “container”.

So, RAN3 should confirm the gNB can always copies and pastes the entire “UE-RadioPagingInfo-r17” into the “UERadioPagingInformation”, including every ASN.1 extension within “UE-RadioPagingInfo-r17”, regardless of whether the gNB can decode the extension or not.

Proposal 2: RAN3 confirm the rel-17 gNB or later can always copies and pastes the entire Rel-19 “container”, including every ASN.1 extension within “UE-RadioPagingInfo-r17”, regardless of whether the gNB can decode the extension or not.

Proposal 3: No enhancement is required for RAN3, and the use of LP-WUS feature depends on network implementation. That is, OAM ensures that all gNBs within a specific area support this Rel-19 “container”.

- Conclusions

In this contribution, we discussed supporting LP-WUS indicating paging monitoring in RRC_IDLE/INACTIVE mode, and following are the proposals.

Proposals:

Proposal 1: A new IE should be introduced into F1 paging to include extra bits for UE_ID based LP-WUS subgrouping, with at least 20 bits.

Proposal 2: RAN3 confirm the rel-17 gNB or later can always copies and pastes the entire Rel-19 “container”, including every ASN.1 extension within “UE-RadioPagingInfo-r17”, regardless of whether the gNB can decode the extension or not.

Proposal 3: No enhancement is needed for RAN3, and the use of LP-WUS feature depends on network implementation. That is, OAM ensures that all gNBs within a specific area support this Rel-19 “container”.

- References

- RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)