3GPP TSG-RAN WG3 Meeting #128	R3-253710

St. Julian - Malta, 19th – 23rd May 2025

Agenda Item:	13.2

Source:	Ericsson, Nokia, LG Electronics, Google, Jio Platforms

Title:	Support for Semi-persistent CSI-RS transmission

Document for:	Discussion, Agreement

# Introduction

At the last RAN3 meeting, in relation to the network coordination for semi-persistent CSI-RS transmission in a candidate cell, the following progress was made:

Introduce two new Class-1 UE-associated procedures over F1AP to support the source gNB-DU to trigger the activation/deactivation of the SP CSI-RS transmission in the candidate cell(s).

Introduce a new class-1 UE-associated procedures over XnAP for the SP CSI-RS activation/deactivation.

Open issues on the granularity of the CSI-RS resource/resource set, and need to be checked with RAN2 progress.

In this paper, we further discuss the open issues and the signaling details of the solution.

# Discussion

The more complex scenario where the network entities need to coordinate for semi-persistent CSI-RS transmission in a candidate cell, is the inter-CU case, where the source cell belongs to one gNB, the candidate cell to another gNB and both source and candidate gNB are split gNBs.

According to the agreement reached at the last RAN3 meeting, the network coordination can be achieved with three rounds of signaling, also shown in the figure below:

- a first class 1 procedure, between the source DU and the source CU

- a second class 1 procedure, between the source CU and the candidate CU

- a third class 1 procedure, from the candidate CU to the candidate DU

Figure 1. Inter-CU inter-DU LTM - Coordination for semi-persistent CSI-RS transmission.

At the source gNB, after the request from source DU (step 1) the source CU sends the response message to source DU (in step 6) only after it received the response of the candidate CU (in step 5). The source CU sends the request for activation/deactivation (in step 2) and it will receive the response (in step 5). At the candidate gNB, the candidate CU sends its response to the source CU (in step 5) only after it received the response of the candidate DU (in step 4).

The next two figures show the high-level flow charts of how the coordination can be achieved for intra-CU LTM and for inter-CU LTM. The detailed F1 and Xn signaling are captured in the TPs in R3-253712 and R3-253711.

Figure 2. Intra-CU inter-DU LTM - Coordination for semi-persistent CSI-RS transmission.

Figure 3. Inter-CU LTM - Coordination between source DU and candidate DU for semi-persistent CSI-RS transmission.

Open issues on the granularity of the CSI-RS resource/resource set, and need to be checked with RAN2 progress.

In the current running RRC CR, the following CSI-RS Resource Set is defined, including the associated LTM Candidate ID list and CSI-RS Resource list details. Accordingly, the granularity of CSI-RS should be based on the resource ID.

#### –	LTM-CSI-ResourceConfig

The IE LTM-CSI-ResourceConfig defines a group of one or more CSI resources for one or more LTM candidate configurations

LTM-CSI-ResourceConfig information element

-- ASN1START

-- TAG-LTM-CSI-RESOURCECONFIG-START

LTM-CSI-ResourceConfig-r18 ::=      SEQUENCE {

ltm-CSI-ResourceConfigId-r18        LTM-CSI-ResourceConfigId-r18,

ltm-SSB-ResourceSet-r18         LTM-SSB-ResourceSet-r18,

... ,

[[

ltm-NZP-CSI-RS-ResourceSet-r19          LTM-NZP-CSI-RS-ResourceSet-r19                     OPTIONAL, -- Need R

resourceType-r19                     ENUMERATED {periodic, semi-persistent}                OPTIONAL  -- Cond CSI-RS

]]

}

LTM-SSB-ResourceSet-r18 ::=     SEQUENCE {

ltm-SSB-ResourceList-r18        SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF SSB-Index,

ltm-CandidateIdList-r18             SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

LTM-NZP-CSI-RS-ResourceSet-r19 ::=     SEQUENCE {

ltm-CSI-RS-ResourceList-r19         SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r19)) OF NZP-CSI-RS-ResourceId,

ltm-CandidateIdList-r19             SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r19)) OF LTM-CandidateId-r18,

...

}

-- TAG-LTM-CSI-RESOURCECONFIG-STOP

-- ASN1STOP

LTM-NZP-CSI-RS-ResourceSet field descriptions

ltm-CandidateIdList This field indicates the LTM candidate configuration IDs related to the NZP-CSI-RSs in the ltm-CSI-RS-ResourceList. The list has the same number of entries as ltm-CSI-RS-ResourceList. The first entry in this list shall be associated to the first entry in ltm-CSI-RS-ResourceList, the second entry of this list shall be associated to the second entry in ltm-CSI-RS-ResourceList, and so on.

ltm-NZP-CSI-RS-ResourceList This field is used to indicate on NZP CSI-RS resources from one or more LTM candidate cells. If the resource type of the NZP-CSI-RS resource is semi-persistent, the ltm-ReportConfigType within LTM-CSI-ReportConfig that this resource set is associated with cannot be configured as eventTriggered.

- RAN3 agrees that, for both F1AP and XnAP, the activation and deactivation of CSI-RS transmission in LTM candidate cells are performed at the level of individual CSI-RS Resource IDs.

- Agree on the TPs for 38.423 and 38.473 respectively in R3-253711 and R3-253712 that capture the above proposals.

# Conclusions and Proposals

In this contribution we made observations and proposals for different LTM related topics, as listed below.

Proposal 1	RAN3 agrees that, for both F1AP and XnAP, the activation and deactivation of CSI-RS transmission in LTM candidate cells are performed at the level of individual CSI-RS Resource IDs.

Proposal 2	Agree on the TPs for 38.423 and 38.473 respectively in R3-253711 and R3-253712 that capture the above proposals.