3GPP TSG-RAN3 Meeting #128	R3-253727

Malta, MT, 19-23 April 2025

# Title:	Discussion on information exchange for SBFD

# Source:	ZTE Corporation

# Agenda Item:	19.2

# Document for:	Discussion, Decision

Introduction

WID for SBFD is agreed in [1], following aspects are related to RAN3.

- Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling - Notes: Following are assumed for information exchange among gNBs - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3 - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact

In this paper, we discus RAN3 related SBFD topics.

Discussion

High level issue discussion

Procedure for CLI handling

During the discussion in RAN1, following two alternatives are supported:

(a) Alt.1 beam pairing				(b) Alt. 2 beam nulling

Figure 1. Measurement & information exchange procedure.

Alt.1 is beam pairing based CLI solution(victim measures channel). Victim gNB measures the CLI-RS transmitted by aggressor gNB and feedback the channel information, e.g. strongest beam information. Based on the channel information provided by victim gNB, the aggressor gNB can make CLI mitigation based on it’s implementation.

Alt.2 is beam nulling based CLI mitigation solution(aggressor measures channel). Aggressor gNB performs measurement on the RS transmitted by victim gNB, and then apply CLI mitigation.

Before transmitting CLI-RS and measuring the channel state, the CLI-RS resource, e.g. SSB or CSI-RS can be exchanged. CLI-RS Resource configuration can be exchanged at following stage:

CLI mitigation signaling. At this stage, the CLI-RS resource can be exchanged after CLI is detected.

- Capture the high level procedure for inter-gNB CLI measurement into stage2 specification.

TX centric or RX centric configuration

Regarding the measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS, it is not clear who determine the measurement resource configuration. We have two following alternatives:

Alt1: TX centric manner, i.e. gNB transmitting the RS determines the measurement resource configuration

Alt2: RX centric manner, i.e. gNB receiving the RS determines the measurement resource configuration.

In Uu interface, gNB is a controller of UE, the UL RS and DL RS resource are both configured by gNB. With regards to Xn interface, there is no central controller between the gNBs, who determines the measurement resource configuration needs to be discussed.

From our view, TX centric manner is a straightforward way to configure measurement resource if there is no central controller. However, the measurement resource configured by RS TX gNB may not be a suitable resource for RS RX gNB, e.g. Transmitting/receiving resource of DL/UL data on RS RX gNB may overlap with the measurement resource configured by RS TX gNB. Additionally, due to it is possible that there are more than one aggressor gNBs, if TX centric manner is adopted, different aggressor gNBs may configure overlapping measurement resource configuration, which may cause measurement failure. Furthermore, in beam pairing scenario, only victim gNB knows which resource is the victim resource, allow victim gNB to configure measurement resource configuration is more reasonable.

- It is suggested to discuss TX centric manner(i.e. gNB transmitting RS) or RX centric manner(i.e. gNB receiving RS) is adopted for determining measurement resource configuration.

Information exchange

SBFD location configuration

From our view, depending on OAM configuration and channel condition, one gNB may turn on/off the SBFD feature and corresponding SBFD resource configuration will be released/added. SBFD frequency/time configuration may contains many parameters, “release/add” signaling is not efficient to exchange the enable or disable information. So, we suggest to introduce an on/off indication to indicate the state of SBFD resource configuration.

- An on/off indication is introduced to indicate the state of SBFD location configuration.

Measurement resource/result exchange

As agreed in the RAN3#127bis meeting, a New class-2 CLI procedure is used to deliver CLI mitigation request/strongest beam information. However, in the beam nulling situation, the aggressor gNB doesn't know when to stop the measurement or mitigation request procedure. Therefore, we suggest introducing an optional IE to indicate start/stop in the mitigation request.

- An option IE is introduced to indicate start/stop in the mitigation request.

SBFD RACH Configuration Exchange

Currently, F1 and NR interfaces exchange PRACH configuration of cells. Based on RAN1 and RAN2 discussions, for SBFD cells, dedicated SBFD RACH configurations will be introduced. This means that an SBFD cell may simultaneously configure two different types of RACH: legacy RACH and SBFD RACH.

However, in the current specification, only one type of RACH configuration is exchanged, making it impossible for the receiving node to determine whether the received configuration is for legacy RACH or SBFD RACH. Furthermore, without exchanging SBFD RACH configuration, the receiving gNB may transmit/receive data on resources allocated for SBFD RACH by the transmitting gNB, potentially causing interference to the transmitting gNB.We have two following alternatives:

-Alt 1: Confirm that the RACH transmitted over existing Xn interface is only for legacy RACH, and specify that SBFD cells should not populate SBFD RACH information in related IE

- Alt 2: Enhance Xn and F1 interfaces to support exchange of SBFD RACH configuration

While Alt 1 requires no signaling enhancement, it may lead to resource conflicts and interference issues. Alt 2 requires signaling enhancement but provides a complete solution with better resource coordination.

Proposal 5: It is suggested to discuss SBFD RACH Configuration Exchange manner or enhancement of Xn/F1 interfaces.

Conclusion

- Capture the high level procedure for inter-gNB CLI measurement into stage2 specification.

- It is suggested to discuss TX centric manner(i.e. gNB transmitting RS) or RX centric manner(i.e. gNB receiving RS) is adopted for determining measurement resource configuration.

- An on/off indication is introduced to indicate the state of SBFD location configuration.

- An option IE is introduced to indicate start/stop in the mitigation request

- It is suggested to discuss SBFD RACH Configuration Exchange manner or enhancement of Xn/F1 interfaces.

Reference

- RP-241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)