3GPP TSG-RAN WG3 Meeting #128	R3-253732

Malta, MT, May 19th – 23rd May, 2025

Agenda Item:	14.3

Source:	CSCN

Title:	Discussion on support of regenerative payload for NR NTN

Document for:	Discussion

1	Introduction

In the RAN3#127-bis meeting, there are some conclusions as below about discussions on NG interface management of supporting of Regenerative Payload [1].

NG Connection interruption due to hard feeder link switch over Proposal 3bis, Further check the functionality: in case of hard feeder link switch over, the 5GC suspends or resumes the DL signalling/data transmission(s) to the gNB based on the new indicator in RAN configuration update, FFS on the relationship with respect to NG suspend/resume (if any). NG suspend/resume No new NGAP suspend/resume procedures in R19.

In this contribution, we will further discuss the left open issue on hard feeder link switch over and NG suspend/resume.

2	Discussion

In the last meeting, it was proposed that in case of hard feeder link switch over, the 5GC suspends or resumes the DL signalling/data transmission(s) to the gNB based on the new indicator in RAN configuration update and there are still some issues on the relationship between hard FLSO and NG suspend/resume.

Scenario

For hard FLSO without AMF relocation, the on-board gNB disconnects from the current serving NTN Gateway, resulting in a break in NG connection between AMF/UPF and gNB. However, the on-board gNB is still in the AMF/UPF service area to which the current serving NTN Gateway belongs. The NG connection may be quickly resumed when the on-board gNB establishes connection with a new serving NTN Gateway belonging to the same AMF service area in short time.

For NG suspend/resume mechanism, when the on-board gNB moves out of the AMF service area, the NG connection with AMF/UPF may be suspended. This involves notifying the AMF/UPF that the current NG connection is temporarily unavailable while maintaining the logical NG interface. And when the on-board gNB re-enters the AMF service area, the NG connection can be resumed quickly. It is possible to resume the connection between on-board gNB and AMF after a complete circle (approximately 90-110 minutes) or after a regression period (approximately 12 hours).

CP impact

For both hard FLSO and NG suspend/resume mechanism, the AMF and gNB remain unchanged while only NTN gateway may change. For hard FLSO, the on-board gNB doesn’t need to additionally store NG related information since it doesn’t move out of the AMF service area. The AMF TNL information remains unchanged and the gNB could notify the AMF about the change of DL NG-U tunnel if needed. For NG suspend/resume mechanism, the on-board gNB may need to store the NG related information of each possible connected AMF. When the gNB on board reconnects to the AMF, the original AMF TNL information is valid only if the information remains unchanged.

UP impact

For hard FLSO, gNB stores the UL data and delivers it to UPF after FLSO. UPF stores the DL data and delivers it to gNB after receiving the resume indication. For NG suspend/resume mechanism, gNB and UPF may discard the UL data and DL data separately due to the long interruption.

The comparison between NG suspend/resume mechanism and hard FLSO summarized as follows:

|  | NG suspend/resume | Hard FLSO |
| --- | --- | --- |
| Scenario | On-board gNB moves out of the AMF service area | On-board gNB is still in the AMF service area |
| Interruption time | Long (may be several hours) | Short (may be several seconds) |
| Stored information | NG interface related information for each AMF gNB may connected | NG interface related information for the current AMF |
| CP impact | AMF and gNB TNL information may change | AMF TNL information unchanged gNB TNL information may change |
| UP impact (UL) | gNB may discard the UL data due to the long interruption | gNB stores the UL data, and delivers it to UPF after FLSO |
| UP impact (DL) | UPF may discard the DL data due to the long interruption | UPF stores the DL data, and delivers it to gNB after receiving the resume indication |

According to the above analysis, the key distinction between the NG suspend/resume and hard FLSO is whether the AMF TNL information changes. If the AMF/UPF TNL information remains unchanged, hard FLSO and NG suspend/resume have many similarities in the management of the NG interface. The new indicator can be extended to the NG suspend/resume mechanism, and whether initiate NG suspend/resume mechanism is up to network implementation.

Proposal 1: If the AMF/UPF TNL information remains unchanged, the new indicator can be extended to the NG suspend/resume mechanism, and whether initiate NG suspend/resume mechanism is up to network implementation.

3	Conclusion

In this contribution, we provide considerations on the NTN regenerative payload in RAN3, following observations and proposals are made:

Proposal 1: If the AMF/UPF TNL information remains unchanged, the new indicator can be extended to the NG suspend/resume mechanism, and whether initiate NG suspend/resume mechanism is up to network implementation.

4	Reference

[1] RAN3_127bis_agenda_20250411_1400_EOM