3GPP TSG-RAN WG3 Meeting #128	R3-254013

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1212 | rev | 8 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Regenerative Payload and MBS broadcast in NR NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Thales, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, ZTE, Qualcomm, Samsung, Xiaomi, CMCC, China Telecom, Jio Platforms, LG Electronics, NEC, ETRI, SES, ESA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2024-10-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-19, following enhancement are introduced: - Regenerative Payload with full gNB on board the satellite. - Broadcast service within intended small area. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Support of Regenerative payload with gNB on board the satellite, including: - introduce NG Removal procedure. - Introduce Downlink NG Transmission Control IE to RAN CONFIGURATION UPDATE. - Support of MBS broadcast, including: - introduce Geographical description(i.e. MBS Intended Service Area List) for MBS service area. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Regenerative payload and MBS broadcast service could not be supported for NR NTN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.2, 8.7.x(new), 8.17.1, 8.17.2, 9.2.6.4, 9.2.6.x1(new), 9.2.6.x2(new), 9.2.6.x3(new), 9.3.1.209, 9.3.1.x(new), 9.4.3, 9.4.4, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.300 CR ... TS 38.410 CR 0051 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev -. New BL CR to capture the agreed TPs R3-245799,R3-245843. Rev 1. Resubmit for RAN3#126. Rev 2. Implement the agreed TP: R3-247899. Rev 3. Resubmit for RAN3#127. Rev 4. Implement the agreed TP: R3-250893. Rev 5. Rebase and resubmit for RAN3#127bis. Rev 6. Implement the agreed TPs: R3-252343, R3-252409. Rev 7. Resubmission for RAN3#128. Rev 8. Implement the agreed TP: R3-253945. |  |  |  |  |  |  |  |  |

-----------------Start of the Changes-------------------

8.7.2	RAN Configuration Update

8.7.2.1	General

The purpose of the RAN Configuration Update procedure is to update application level configuration data needed for the NG-RAN node and the AMF to interoperate correctly on the NG-C interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non UE-associated signalling.

8.7.2.2	Successful Operation

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

If the TAI NSAG Support List IE is included in the Broadcast PLMN Item IE in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, use this information as specified in TS 23.501 [9].

If the Downlink NG Transmission Control IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, suspend or resume the downlink NG transmission accordingly.

-----------------Next Changes-------------------

### 8.7.x	NG Removal

#### 8.7.x.1	General

The purpose of the NG Removal procedure is to remove the interface instance between the NG-RAN node and the AMF in a controlled manner. If successful, this procedure erases any existing application level configuration data in the two nodes.

The procedure uses non-UE associated signaling.

#### 8.7.x.2	Successful Operation

Figure 8.7.x.2-1: NG Removal, successful operation

The NG-RAN node initiates the procedure by sending the NG REMOVAL REQUEST message to the AMF. Upon reception of the NG REMOVAL REQUEST message, the AMF shall reply with the NG REMOVAL RESPONSE message. After receiving the NG REMOVAL RESPONSE message, the NG-RAN node shall initiate removal of the TNL association towards the AMF and may remove all resources associated with that interface instance. The AMF may then remove all resources associated with the NG-RAN node.

#### 8.7.x.3	Unsuccessful Operation

Figure 8.7.x.3-1: NG Removal, unsuccessful operation

If the AMF cannot accept to remove the interface instance with NG-RAN node, it shall respond with an NG REMOVAL FAILURE message with an appropriate cause value.

#### 8.7.x.4	Abnormal Conditions

Void.

-----------------Next Change-------------------

8.17.1	Broadcast Session Setup

8.17.1.1	General

The purpose of the Broadcast Session Setup procedure is to request the NG-RAN node to setup MBS session resources for a broadcast MBS session. The procedure uses non-UE associated signalling.

8.17.1.2	Successful Operation

Figure 8.17.1.2-1: Broadcast Session Setup, successful operation.

The AMF initiates the procedure by sending a BROADCAST SESSION SETUP REQUEST message to the NG-RAN node. If the NG-RAN node accepts all the MBS QoS flows in the MBS session at least in one of its cells, the NG-RAN node responds with the BROADCAST SESSION SETUP RESPONSE message.

If the MBS Service Area IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account as specified in TS 23.247 [44], and TS 38.300 [8] for NTN operation.

If the MBS Session FSA ID List IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account to determine cells/frequencies within the MBS service area to broadcast MBS session data as specified in TS 23.247 [44].

If the Supported UE Type List IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall, if supported, take it into account for configuring MBS session resources.

If the Associated Session ID IE is included in the BROADCAST SESSION SETUP REQUEST message the NG-RAN node shall, if supported, take this information into account to determine whether MBS session resource sharing is possible, as specified in TS 38.300 [8].

-----------------Next Change-------------------

8.17.2	Broadcast Session Modification

8.17.2.1	General

The purpose of the Broadcast Session Modification procedure is to request the NG-RAN node to update the MBS session resources or the area related to a previously established broadcast MBS session. The procedure uses non-UE associated signalling.

8.17.2.2	Successful Operation

Figure 8.17.2.2-1: Broadcast Session Modification, successful operation.

The AMF initiates the procedure by sending a BROADCAST SESSION MODIFICATION REQUEST message to the NG-RAN node.

If the MBS Service Area IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall update the MBS service area as needed and send the BROADCAST SESSION MODIFICATION RESPONSE message. For NTN operation, the NG-RAN node shall, if supported, update the intended service area information as specified in TS 38.300 [8].

If the MBS Session Modification Request Transfer IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall replace the previously provided information by the newly received one and update the MBS session resources and area as requested and send the BROADCAST SESSION MODIFICATION RESPONSE message.

If the Supported UE Type List IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall, if supported, take it into account for configuring MBS session resources.

If the MBS NG-U Failure Indication IE is included in the BROADCAST SESSION MODIFICATION REQUEST message within the MBS Session Setup or Modification Request Transfer IE and set to "N3mb path failure", the NG-RAN node may provide new NG-U transport layer information to replace the failed one(s) or switch the data delivery to another 5GC as per the Broadcast MBS session restoration procedure for N3mb Path Failure specified in TS 23.527 [57].

-----------------Next Change-------------------

9.2.6.4	RAN CONFIGURATION UPDATE

This message is sent by the NG-RAN node to transfer updated application layer information for an NG-C interface instance.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 0..1 |  | Supported TAs in the NG-RAN node. | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| >>>>NPN Support | O |  | 9.3.3.44 | If the NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>>>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.191 | Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN. | YES | reject |
| >>>>TAI NSAG Support List | O |  | 9.3.1.238 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| >>Configured TAC Indication | O |  | 9.3.3.50 |  | YES | ignore |
| >>RAT Information | O |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | O |  | Paging DRX 9.3.1.90 |  | YES | ignore |
| Global RAN Node ID | O |  | 9.3.1.5 |  | YES | ignore |
| NG-RAN TNL Association to Remove List |  | 0..1 |  |  | YES | reject |
| >NG-RAN TNL Association to Remove Item |  | 1..<maxnoofTNLAssociations> |  |  | - |  |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the NG-RAN node. | - |  |
| >>TNL Association Transport Layer Address at AMF | O |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the AMF. | - |  |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |
| Downlink NG Transmission Control | O |  | ENUMERATED (suspend, resume, …) | Indicates the CN to suspend or resume the downlink NG Transmission | YES | ignore |

Editor’s Note: FFS whether this IE is needed.

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 12. |
| maxnoofTNLAssociations | Maximum no. of TNL Associations between the NG-RAN node and the AMF. Value is 32. |

-----------------Next Change-------------------

#### 9.2.6.x1	NG REMOVAL REQUEST

This message is sent by the NG-RAN node to initiate the removal of the interface instance.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |

#### 9.2.6.x2	NG REMOVAL RESPONSE

This message is sent by the AMF to acknowledge the initiation of removal of the interface instance.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF Name | M |  | 9.3.3.21 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.6.x3	NG REMOVAL FAILURE

This message is sent by the AMF to indicate that removing the interface instance cannot be accepted.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

-----------------Next Change-------------------

#### 9.3.1.209	MBS Service Area Information

This IE contains MBS service area information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Service Area Cell List |  | 0..<maxnoofCellsforMBS> |  |  |
| >NR CGI | M |  | 9.3.1.7 |  |
| MBS Service Area TAI List |  | 0..<maxnoofTAIforMBS> |  |  |
| >TAI | M |  | 9.3.3.11 |  |
| MBS Intended Service Area List |  | 0..<maxnoofIntendedAreasforMBS> |  |  |
| >Intended Service Area Coordinates | M |  | 9.3.1.x |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsforMBS | Maximum no. of cells allowed within one MBS Service Area. Value is 8192. |
| maxnoofTAIforMBS | Maximum no. of TAs allowed within one MBS Service Area. Value is 1024. |
| maxnoofIntendedAreasforMBS | Maximum no. of intended areas allowed within one MBS Service Area. Value is 65536. |

-----------------Next Change-------------------

9.3.1.x	Intended Service Area Coordinates

This IE provides information on the Intended Service Area Coordinates for MBS Broadcast Service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Intended Service Area Coordinates | M |  | OCTET STRING | Includes the areaCoordinates contained in the IntendedServiceAreaInfo IE as defined in TS 38.331 [18] |

-----------------Next Change-------------------

9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

NGAP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM NGAP-CommonDataTypes

UNCHANGED PART OMITTED

NASNonDeliveryIndication,

NGReset,

NGResetAcknowledge,

NGRemovalFailure,

NGRemovalRequest,

NGRemovalResponse,

NGSetupFailure,

NGSetupRequest,

NGSetupResponse,

UNCHANGED PART OMITTED

id-NASNonDeliveryIndication,

id-NGReset,

id-NGRemoval,

id-NGSetup,

id-OverloadStart,

id-OverloadStop,

id-Paging,

id-PathSwitchRequest,

UNCHANGED PART OMITTED

NGAP-ELEMENTARY-PROCEDURES-CLASS-1 NGAP-ELEMENTARY-PROCEDURE ::= {

aMFConfigurationUpdate			|

broadcastSessionModification	|

broadcastSessionRelease			|

broadcastSessionSetup			|

broadcastSessionTransport					|

distributionSetup				|

distributionRelease				|

handoverCancel					|

handoverPreparation				|

handoverResourceAllocation		|

initialContextSetup				|

mTCommunicationHandling			|

multicastSessionActivation					|

multicastSessionDeactivation				|

multicastSessionUpdate			|

nGReset							|

nGSetup							|

pathSwitchRequest				|

pDUSessionResourceModify		|

pDUSessionResourceModifyIndication			|

pDUSessionResourceRelease		|

pDUSessionResourceSetup			|

pWSCancel						|

rANConfigurationUpdate			|

timingSynchronisationStatus		|

uEContextModification			|

uEContextRelease				|

uEContextResume								|

uEContextSuspend							|

uERadioCapabilityCheck			|

uERadioCapabilityIDMapping		|

writeReplaceWarning							|

nGRemoval									,

...

}

UNCHANGED PART OMITTED

nGReset NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NGReset

SUCCESSFUL OUTCOME		NGResetAcknowledge

PROCEDURE CODE			id-NGReset

CRITICALITY				reject

}

nGSetup NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NGSetupRequest

SUCCESSFUL OUTCOME		NGSetupResponse

UNSUCCESSFUL OUTCOME	NGSetupFailure

PROCEDURE CODE			id-NGSetup

CRITICALITY				reject

}

nGRemoval NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NGRemovalRequest

SUCCESSFUL OUTCOME		NGRemovalResponse

UNSUCCESSFUL OUTCOME	NGRemovalFailure

PROCEDURE CODE			id-NGRemoval

CRITICALITY				reject

}

overloadStart NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		OverloadStart

PROCEDURE CODE			id-OverloadStart

CRITICALITY				ignore

}

UNCHANGED PART OMITTED

9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

UNCHANGED PART OMITTED

UNCHANGED PART OMITTED

WUS-Assistance-Information,

XrDeviceWith2Rx,

DownlinkNGTransmissionControl

UNCHANGED PART OMITTED

id-XrDeviceWith2Rx,

id-SLPositioningRangingServiceInfo,

id-DownlinkNGTransmissionControl

UNCHANGED PART OMITTED

-- **************************************************************

--

#### -- INTERFACE MANAGEMENT ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- NG Removal Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- NG REMOVAL REQUEST

--

-- **************************************************************

NGRemovalRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGRemovalRequestIEs} },

...

}

NGRemovalRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-GlobalRANNodeID				CRITICALITY reject	TYPE GlobalRANNodeID				PRESENCE mandatory	},

...

}

-- **************************************************************

--

###### -- NG REMOVAL RESPONSE

--

-- **************************************************************

NGRemovalResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGRemovalResponseIEs} },

...

}

NGRemovalResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMFName 					CRITICALITY reject	TYPE AMFName					PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- NG REMOVAL FAILURE

--

-- **************************************************************

NGRemovalFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGRemovalFailureIEs} },

...

}

NGRemovalFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

UNCHANGED PART OMITTED

-- **************************************************************

--

##### -- RAN Configuration Update Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- RAN CONFIGURATION UPDATE

--

-- **************************************************************

RANConfigurationUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {RANConfigurationUpdateIEs} },

...

}

RANConfigurationUpdateIEs NGAP-PROTOCOL-IES ::= {

{ ID id-RANNodeName								CRITICALITY ignore	TYPE RANNodeName									PRESENCE optional	}|

{ ID id-SupportedTAList							CRITICALITY reject	TYPE SupportedTAList								PRESENCE optional	}|

{ ID id-DefaultPagingDRX						CRITICALITY ignore	TYPE PagingDRX										PRESENCE optional	}|

{ ID id-GlobalRANNodeID							CRITICALITY ignore	TYPE GlobalRANNodeID								PRESENCE optional	}|

{ ID id-NGRAN-TNLAssociationToRemoveList		CRITICALITY reject	TYPE NGRAN-TNLAssociationToRemoveList			PRESENCE optional	}|

{ ID id-NB-IoT-DefaultPagingDRX					CRITICALITY ignore	TYPE NB-IoT-DefaultPagingDRX						PRESENCE optional	}|

{ ID id-Extended-RANNodeName					CRITICALITY ignore	TYPE Extended-RANNodeName							PRESENCE optional	}|

{ ID id-DownlinkNGTransmissionControl			CRITICALITY ignore	TYPE DownlinkNGTransmissionControl				PRESENCE optional	},

...

}

UNCHANGED PART OMITTED

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

UNCHANGED PART OMITTED

id-MBS-ActiveSessionInformation-TargettoSourceList,

id-MBS-AssistanceInformation,

id-MBS-IntendedServiceAreaList,

id-MBS-SessionTNLInfo5GC,

id-MBS-SupportIndicator,

id-MBSSessionFailedtoSetupList,

id-MBSSessionFailedtoSetuporModifyList,

id-MBSSessionSetupResponseList,

id-MBSSessionSetuporModifyResponseList,

id-MBSSessionToReleaseList,

id-MBSSessionSetupRequestList,

id-MBSSessionSetuporModifyRequestList,

UNCHANGED PART OMITTED

maxnoofCandidateRelayUEs,

maxnoofSuccessfulPSCellChangeReports,

maxnoofCellsTSS,

maxnoofPeriodicities,

maxnoofPartiallyAllowedS-NSSAIs,

maxnoofRSPPQoSFlows,

maxnoofIntendedAreasforMBS

UNCHANGED PART OMITTED

DL-NGU-TNLInformationReused ::= ENUMERATED {

true,

...

}

DirectForwardingPathAvailability ::= ENUMERATED {

direct-path-available,

...

}

DownlinkNGTransmissionControl ::= ENUMERATED {

suspend,

resume,

...

}

UNCHANGED PART OMITTED

MBS-ServiceArea ::= CHOICE {

locationindependent		MBS-ServiceAreaInformation,

locationdependent		MBS-ServiceAreaInformationList,

choice-Extensions		ProtocolIE-SingleContainer { {MBS-ServiceArea-ExtIEs} }

}

MBS-ServiceArea-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

MBS-ServiceAreaInformationList ::= SEQUENCE (SIZE(1..maxnoofMBSServiceAreaInformation)) OF MBS-ServiceAreaInformationItem

MBS-ServiceAreaInformationItem ::= SEQUENCE {

mBS-AreaSessionID				MBS-AreaSessionID,

mBS-ServiceAreaInformation		MBS-ServiceAreaInformation,

iE-Extensions					ProtocolExtensionContainer { {MBS-ServiceAreaInformationItem-ExtIEs} }	OPTIONAL,

...

}

MBS-ServiceAreaInformationItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-ServiceAreaInformation ::= SEQUENCE {

mBS-ServiceAreaCellList		MBS-ServiceAreaCellList																OPTIONAL,

mBS-ServiceAreaTAIList		MBS-ServiceAreaTAIList																OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBS-ServiceAreaInformation-ExtIEs} }	OPTIONAL,

...

}

MBS-ServiceAreaInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MBS-IntendedServiceAreaList		CRITICALITY ignore	EXTENSION MBS-IntendedServiceAreaList	PRESENCE optional },

...

}

MBS-ServiceAreaCellList ::= SEQUENCE (SIZE(1.. maxnoofCellsforMBS)) OF NR-CGI

MBS-ServiceAreaTAIList ::= SEQUENCE (SIZE(1.. maxnoofTAIforMBS)) OF TAI

MBS-IntendedServiceAreaList ::= SEQUENCE (SIZE(1..maxnoofIntendedAreasforMBS)) OF IntendedServiceAreaCoordinates

IntendedServiceAreaCoordinates ::= OCTET STRING

MBS-SessionID ::= SEQUENCE {

tMGI								TMGI,

nID								NID																			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBS-SessionID-ExtIEs} } 		OPTIONAL,

...

}

MBS-SessionID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UNCHANGED PART OMITTED

9.4.7	Constant Definitions

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

UNCHANGED PART OMITTED

id-ConnectionEstablishmentIndication		ProcedureCode ::= 65

id-BroadcastSessionModification				ProcedureCode ::= 66

id-BroadcastSessionRelease					ProcedureCode ::= 67

id-BroadcastSessionSetup					ProcedureCode ::= 68

id-DistributionSetup 						ProcedureCode ::= 69

id-DistributionRelease 						ProcedureCode ::= 70

id-MulticastSessionActivation 				ProcedureCode ::= 71

id-MulticastSessionDeactivation 			ProcedureCode ::= 72

id-MulticastSessionUpdate 					ProcedureCode ::= 73

id-MulticastGroupPaging						ProcedureCode ::= 74

id-BroadcastSessionReleaseRequired			ProcedureCode ::= 75

id-TimingSynchronisationStatus				ProcedureCode ::= 76

id-TimingSynchronisationStatusReport		ProcedureCode ::= 77

id-MTCommunicationHandling					ProcedureCode ::= 78

id-RANPagingRequest							ProcedureCode ::= 79

id-BroadcastSessionTransport				ProcedureCode ::= 80

id-NGRemoval								ProcedureCode ::= x

UNCHANGED PART OMITTED

maxnoofSNSSAIforQMC							INTEGER ::= 16

maxnoofTAforQMC								INTEGER ::= 8

maxnoofThresholdsForExcessPacketDelay		INTEGER ::= 255

maxnoofESNPNs								INTEGER ::= 15

maxnoofCandidateRelayUEs					INTEGER ::= 32

maxnoofSuccessfulPSCellChangeReports       	INTEGER ::= 64

maxnoofCellsTSS								INTEGER ::= 16384

maxnoofPeriodicities						INTEGER ::= 8

maxnoofCAGforMDT							INTEGER ::= 256

maxnoofMDTSNPNs								INTEGER ::= 16

maxnoofPartiallyAllowedS-NSSAIs				INTEGER ::= 8

maxnoofRSPPQoSFlows							INTEGER ::= 2048

maxnoofIntendedAreasforMBS					INTEGER ::= 65536

UNCHANGED PART OMITTED

id-PDUSessionListMTCommHReq								ProtocolIE-ID ::= 431

id-MaximumDataBurstVolume 								ProtocolIE-ID ::= 432

id-MN-only-MDT-collection								ProtocolIE-ID ::= 433

id-MBS-NGUFailureIndication								ProtocolIE-ID ::= 434

id-UserPlaneFailureIndication							ProtocolIE-ID ::= 435

id-UserPlaneFailureIndicationReport						ProtocolIE-ID ::= 436

id-SourceSN-to-TargetSN-QMCInfo							ProtocolIE-ID ::= 437

id-QoERVQoEReportingPaths								ProtocolIE-ID ::= 438

id-UserLocationInformationN3IWF-without-PortNumber		ProtocolIE-ID ::= 439

id-AUN3DeviceAccessInfo									ProtocolIE-ID ::= 440

id-MBS-IntendedServiceAreaList							ProtocolIE-ID ::= x

id-DownlinkNGTransmissionControl						ProtocolIE-ID ::= a

-----------------End of the Change-------------------