3GPP TSG-RAN WG3 Meeting #128	R3-253770

MT, MT, May 19 –23, 2025

Agenda Item:	9.2

Source:	ZTE Corporation(moderator)

Title:	SoD of UE context retrieval

Document for:	Approval

# For the Chairman’s Notes

Propose to capture the following:

R3-253273 can be noted and R3-253738 can be agreed

# Discussion

CB: # 8_UEContextPropogation

- The second change in R3-253738 is agreeable

- Check the Resume case on PLMN selection and when the MDT configuration/MRL needs to be transferred to the new gNB?

(moderator - ZTE)

Summary of offline disc R3-253770

## How the UE informs the gNB selected PLMN during RRC Resume procedure:

UE triggered transition from RRC_INACTIVE to RRC_CONNECTED
(UE context retrieval success)

- PLMN selection is performed by the UE. The UE has the capability to support PLMN selection while in RRC_INACTIVE state. More importantly, when performing the RRC connection resume procedure, the UE obtains the list of available PLMNs (via the plmn-IdentityInfoList or npn-IdentityInfoList fields) from the SIB1 broadcasted by the current cell.

- The UE notifies the new gNB of the selected PLMN's index via the RRCResumeComplete message[ANNEX 1]. In a successful RRC connection resume procedure, the UE sends an RRCResumeComplete message to complete the process. This message includes an optional Information Element (IE) called selectedPLMN-Identity.

- Meaning of the selectedPLMN-Identity field: The value of this field is the index of the PLMN that the UE selected from the list of PLMNs broadcasted in the SIB1 of the current cell (plmn-IdentityInfoList or npn-IdentityInfoList).

- During UE transmit from RRC-INACTIVE to RRC-CONNECTED procedure:

  - The UE first initiates the procedure by sending an RRCResumeRequest message. This message contains the UE's identity (like I-RNTI) and the resume cause.

  - Upon receiving the RRCResumeRequest, the new gNB attempts to retrieve the UE context. This context contains the PLMN information the UE was served by before entering the RRC_INACTIVE state.

  - The new send the RRCResume message to the UE.

  - When the UE sends the RRCResumeComplete message, it includes the selectedPLMN-Identity IE, sending the index of the PLMN it selected from the current cell's SIB1 list to the base station. This is the UE indicating to the new gNB its selected PLMN within the current cell's context, in addition to the network already knowing the UE's previous serving PLMN via context retrieval.The new gNB may update previous serving PLMN.

Observation 1:  During the RRC connection resume process, the PLMN selection action is performed by the UE, and the UE informs the new gNB of the index of the PLMN it selected from the SIB1 broadcast list by including the selectedPLMN-Identity Information Element in the RRCResumeComplete message.

Observation 2:  During the UE context retrieval procedure, neither new gNB nor old gNB know the PLMN selected by the UE.

Question: Can we agree the following agreement?

It is UE itself select UE’s PLMN during RRC_INACTIVE to RRC_CONNECTED (Resume procedure).

| Company name | View |
| --- | --- |
| ZTE | Agree |

If Question can be agreed , then the following proposal can be agreedable.

Proposal : R3-253273 can be noted and R3-253738 can be agreed.

# Conclusion, Recommendations [if needed]

If needed.

# Refrence

[1] R3-253738 Clarification for propagation of MDT Configuration in stage2 (ZTE Corporation,China Unicom,China Telecom,CMCC, Huawei)

[2] R3-253273 Clarification for propagation of roaming and access restrictions (Samsung)

# Annex 1 RRCResumeComplete message

#### –	RRCResumeComplete

The RRCResumeComplete message is used to confirm the successful completion of an RRC connection resumption.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

RRCResumeComplete message

-- ASN1START

-- TAG-RRCRESUMECOMPLETE-START

RRCResumeComplete ::=                   SEQUENCE {

rrc-TransactionIdentifier               RRC-TransactionIdentifier,

criticalExtensions                      CHOICE {

rrcResumeComplete                       RRCResumeComplete-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

RRCResumeComplete-IEs ::=               SEQUENCE {

dedicatedNAS-Message                    DedicatedNAS-Message                                                    OPTIONAL,

selectedPLMN-Identity                   INTEGER (1..maxPLMN)                                                    OPTIONAL,

uplinkTxDirectCurrentList               UplinkTxDirectCurrentList                                               OPTIONAL,

lateNonCriticalExtension                OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                    RRCResumeComplete-v1610-IEs                                             OPTIONAL

}

RRCResumeComplete-v1610-IEs ::=         SEQUENCE {

idleMeasAvailable-r16                   ENUMERATED {true}                                                       OPTIONAL,

measResultIdleEUTRA-r16                 MeasResultIdleEUTRA-r16                                                 OPTIONAL,

measResultIdleNR-r16                    MeasResultIdleNR-r16                                                    OPTIONAL,

scg-Response-r16                        CHOICE {

nr-SCG-Response                         OCTET STRING (CONTAINING RRCReconfigurationComplete),

eutra-SCG-Response                      OCTET STRING

}                                                                                                               OPTIONAL,

ue-MeasurementsAvailable-r16            UE-MeasurementsAvailable-r16                                            OPTIONAL,

mobilityHistoryAvail-r16                ENUMERATED {true}                                                       OPTIONAL,

mobilityState-r16                       ENUMERATED {normal, medium, high, spare}                                OPTIONAL,

needForGapsInfoNR-r16                   NeedForGapsInfoNR-r16                                                   OPTIONAL,

nonCriticalExtension                    RRCResumeComplete-v1640-IEs                                             OPTIONAL

}

RRCResumeComplete-v1640-IEs ::=         SEQUENCE {

uplinkTxDirectCurrentTwoCarrierList-r16 UplinkTxDirectCurrentTwoCarrierList-r16                                 OPTIONAL,

nonCriticalExtension                    RRCResumeComplete-v1700-IEs                                             OPTIONAL

}

RRCResumeComplete-v1700-IEs ::=         SEQUENCE {

needForGapNCSG-InfoNR-r17               NeedForGapNCSG-InfoNR-r17                                               OPTIONAL,

needForGapNCSG-InfoEUTRA-r17            NeedForGapNCSG-InfoEUTRA-r17                                            OPTIONAL,

nonCriticalExtension                    RRCResumeComplete-v1720-IEs                                             OPTIONAL

}

RRCResumeComplete-v1720-IEs ::=         SEQUENCE {

uplinkTxDirectCurrentMoreCarrierList-r17 UplinkTxDirectCurrentMoreCarrierList-r17                               OPTIONAL,

nonCriticalExtension                     RRCResumeComplete-v1800-IEs                                            OPTIONAL

}

RRCResumeComplete-v1800-IEs ::=         SEQUENCE {

needForInterruptionInfoNR-r18           NeedForInterruptionInfoNR-r18                                           OPTIONAL,

musim-CapRestrictionInd-r18             ENUMERATED {true}                                                       OPTIONAL,

flightPathInfoAvailable-r18             ENUMERATED {true}                                                       OPTIONAL,

measConfigReportAppLayerAvailable-r18   ENUMERATED {true}                                                       OPTIONAL,

nonCriticalExtension                    SEQUENCE {}                                                             OPTIONAL

}

-- TAG-RRCRESUMECOMPLETE-STOP

-- ASN1STOP

RRCResumeComplete-IEs field descriptions

idleMeasAvailable Indication that the UE has idle/inactive measurement report available.

measConfigReportAppLayerAvailable Indication that the UE has stored one or more application layer measurement reports while the UE was in RRC_IDLE/RRC_INACTIVE state and/or that the UE is configured with at least one application layer measurement configuration with configforRRC-IdleInactive set to true.

measResultIdleEUTRA EUTRA measurement results performed during RRC_INACTIVE.

measResultIdleNR NR measurement results performed during RRC_INACTIVE.

musim-CapRestrictionInd This field indicates the UE temporary capability restriction due to MUSIM operation.

needForGapsInfoNR This field is used to indicate the measurement gap requirement information of the UE for NR target bands.

needForGapNCSG-InfoEUTRA This field is used to indicate the measurement gap and NCSG requirement information of the UE for E-UTRA target bands

needForGapNCSG-InfoNR This field is used to indicate the measurement gap and NCSG requirement information of the UE for NR target bands

needForInterruptionInfoNR This field indicates whether interruption is needed while performing measurement on NR target bands without measurement gap.

selectedPLMN-Identity Index of the PLMN selected by the UE from the plmn-IdentityInfoList or npn-IdentityInfoList fields included in SIB1.

uplinkTxDirectCurrentList The Tx Direct Current locations for the configured serving cells and BWPs if requested by the NW (see reportUplinkTxDirectCurrent in CellGroupConfig).

uplinkTxDirectCurrentMoreCarrierList The Tx Direct Current locations for the configured intra-band CA requested by reportUplinkTxDirectCurrentMoreCarrier-r17.

uplinkTxDirectCurrentTwoCarrierList The Tx Direct Current locations for the configured uplink intra-band CA with two carriers if requested by the NW (see reportUplinkTxDirectCurrentTwoCarrier-r16 in CellGroupConfig).