3GPP TSG-RAN WG3 Meeting #128	R3-253877

Malta, MT, 19th-23th May, 2025

Title:	(TP to BL CR for 38.420) introduction of Evolution of NR duplex operation Sub-band full duplex (SBFD)

Agenda Item:	19.2

Source:	CMCC, Ericsson, Samsung, ZTE

Document for:	Approval

# Introduction

This contribution provides the TP to TS38.420 Introduction of Evolution of NR duplex operation Sub-band full duplex, which reflects the agreements achieved in RAN3#128.

# TP for TS 38.420

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

# 3	Definitions and abbreviations

<<<Unchanged part omitted>>>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CLI	Cross Link Interference

IAB	Integrated Access and Backhaul

MBS	Multicast Broadcast Service

QMC	QoE Measurement Collection

QoE	Quality of Experience

SCTP	Stream Control Transmission Protocol

Xn-C	Xn Control plane

Xn-U	Xn User plane

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

# 5	Functions of the Xn interface

## 5.1	General

The following clauses describe the functions supported in Xn interface.

## 5.2	Functions of Xn-C

<<<Unchanged part omitted>>>

### 5.2.x	CLI indication function

This function allows the transfer of CLI measurements and CLI mitigation request between NG-RAN nodes.

# 6	Xn interface procedures

## 6.1	General

The Xn interface supports procedures over the control plane (Xn-C) and user plane (Xn-U).

## 6.2	Control plane protocol procedures

<<<Unchanged part omitted>>>

### 6.2.x	CLI indication procedure

The CLI indication procedure is used to report the result of CLI measurements, and to request the CLI mitigation.

-	CLI INDICATION

<<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>