3GPP TSG-RAN WG3 #128	R3-253855

St Julian, Malta, May 19th – 23rd 2025

Agenda Item:	10.4

Source:	Ericsson, Huawei, Lenovo

Title:	(TP for BL CR to 38.413 for SON) UHI for SCG activation and deactivation

Document for:	Discussion, Decision

# 1	Introduction

The following TP is meant to capture the following agreements reached during RAN3#128.

NW collects the absolute time (i.e. accumulated time) spent in the PSCell with SCG activated state.

Add the absolute time with SCG activated in the Last Visited PSCell Information IE.

Additional information is not needed for UHI Enhancement for SCG Deactivation/Activation.

# Appendix A - Text Proposal to 38.413

#### 9.3.1.235	Last Visited PSCell Information

The Last Visited PSCell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PSCell ID | O |  | NG-RAN CGI 9.3.1.73 | This IE is present when the SCG resources are configured for the UE. | - |  |
| Time Stay | M |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. Or the duration of the time when no SCG resources are configured for the UE. | - |  |
| SCG Activation Time | O |  | INTEGER (0..40950) | The total duration of the time the UE stayed in the cell while the SCG was activated, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. | YES | ignore |

[Unchanged parts omitted]

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

[Unchanged parts omitted]

id-UserLocationInformationN3IWF-without-PortNumber,

id-SCGActivationTime,

maxnoofAllowedAreas,

[Unchanged parts omitted]

LastVisitedPSCellList ::= SEQUENCE (SIZE(1..maxnoofPSCellsPerPrimaryCellinUEHistoryInfo)) OF LastVisitedPSCellInformation

LastVisitedPSCellInformation ::= SEQUENCE {

pSCellID			NGRAN-CGI	 			OPTIONAL,

timeStay			INTEGER (0..40950),

iE-Extensions		ProtocolExtensionContainer { {LastVisitedPSCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedPSCellInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-SCGActivationTime	CRITICALITY ignore	EXTENSION SCGActivationTime	PRESENCE optional },

...

}

[Unchanged parts omitted]

SCGActivationTime ::= INTEGER (0..40950)

[Unchanged parts omitted]

id-UserLocationInformationN3IWF-without-PortNumber		ProtocolIE-ID ::= 439

id-AUN3DeviceAccessInfo									ProtocolIE-ID ::= 440

id-SCGActivationTime									ProtocolIE-ID ::= xxx

[End of changes]