3GPP TSG-RAN WG3 Meeting #129bis	R3-256544

Prague, Czech Republic, 13th – 17th October, 2025

Title:	Discussion on Feasibility of RAN Service Based Architecture

Source:	FiberCop, Jio Platforms, KT Corp., Qualcomm Inc., Telstra

Agenda item:	10.2.1

Document Type:	Discussion and Decision

# 1	Introduction

In 5G system, the Service-Based Architecture (SBA) was natively introduced with success in the Core Network (CN) since Rel-15, and the same approach is used in OAM domains. This paradigm has proven to enable modularity, loosely coupled design, easier integration of new functionalities, smoother lifecycle management of services, and enhanced interaction across domains and efficient usage of cloud native implementations enabling operational efficiency.

In contrast, the Radio Access Network (RAN) still relies on a Protocol-Based Architecture (PBA), where interactions between Network Functions (NFs) are implemented through rigid protocol interfaces between specific pair of nodes. This approach limits flexibility, scalability, specification complexity, duplicate functions to be specified in multiple interfaces and complex for the seamless introduction of new functionalities or services.

Given the experience from other domains, extending SBA principles to the RAN could open up similar advantages, such as simplified integration of AI/ML algorithms, service reusability, improved architectural agility, and consistency across the overall end to end network design.

It follows that a study should be addressed aiming at evaluating the feasibility, architectural impacts, and potential benefits of adopting SBA in the RAN as part of the 6G system design, paving the way for more adaptable, scalable, and service-oriented mobile networks.

Of course, extending SBA to the RAN must also carefully consider performance constraints of each function. For example, the stringent requirements of fronthaul traffic could make this approach infeasible for the functionalities of DU–RU interaction (which however is not the scope of 3GPP).

This contribution is grounded on ongoing activities within the research project ORIGAMI [1]. The ORIGAMI project brings together leading industry and academic partners across the European mobile telecommunications ecosystem and is funded by the European Commission under the 6G Smart Networks and Services Joint Undertaking (SNS JU). Running from 2024 to 2026, ORIGAMI focuses on advancing architectural models for next-generation mobile networks and addressing key barriers to the deployment of successful 6G technologies. Specifically, ORIGAMI aims to study 6G architectural innovations, like the Global Service-Based Architecture (GSBA), that enables solutions of Network Intelligence (NI) designed to enhance network performance, security, and flexibility. Beyond technological advances, ORIGAMI is committed to establishing global unified standards, supporting the green transition, improving affordability and accessibility, and fostering the development of groundbreaking applications to create strong ecosystem, novel business models for operators to generate new revenue opportunities and allows operator networks to evolve into future generation with advanced capabilities. This holistic approach positions ORIGAMI at the forefront of shaping SBA that will underpin future 6G networks, as addressed in [2].

In this context, the present contribution elaborates on the general principles of the SBA approach and, building upon prior analysis, identifies potential benefits, constraints, and key requirements for its adoption in the RAN domain.

# 2	Discussion

## Service Based Architecture principles

SBA, also known as Service-Oriented Architectures (SOAs), is a distributed approach based on the service concept, which implements a specific functionality, and where each network function is treated as an independent and autonomous entity [3], as depicted in Figure 1.

Figure 1 – High level schema of SBA approach

In an SBA system, services are designed as “black boxes”, meaning their internal workings are irrelevant to other components of the system, performing tasks that satisfy a specific set of requirements. Each service is created by a Service Producer (SP), which is responsible for registering the service in one or more Service Directories (SD). Service Consumers (SC), other entities in the network, can discover available services and start using them without prior knowledge of the specific details or access points of those services.

This distributed approach increases the modularity of the system, allowing components to interact without needing direct knowledge of each other, enables producer and consumer to be deployed anywhere in cloud network, thus promoting agility, efficient resource utilization, scalability and efficient operation of networks. The SBA model aligns well with technologies like Network Function Virtualization (NFV) and Software Defined Networking (SDN), which are foundational to network cloudification and provide more flexible, dynamic network control. Additionally, the SBA design facilitates the integration of advanced AI/ML solutions in cloud native manner, which can be easily added and managed as services, with producer-consumer interactions based on service requests.

## General Context

The possibility of introducing SBA principles in the RAN has already been considered in the past. During the work for 5G specification, a study on applying SBA to the interaction between RAN and CN for the control plane (i.e., N2 interface) was proposed within SA2 [4]. However, due to the lack of agreement during the discussion, the study item phase was not pursued. The main reasons raised were:

- the introduction of an architectural paradigm shift in the middle of a mature generation would have risked fragmentation of ongoing deployments. This point suggests that it could be fine to consider it for the next generation

- the proposed approach was limited to N2 interface only, without a comprehensive RAN-CN framework. This point suggests that a general framework also including UE-CN interaction (i.e., N1 interface) and RAN-CN for the user plane (i.e., N3 interface) could be considered for the next generation

- the introduction of the SBA at that stage (i.e., in the middle of a 5G generation) would have required maintaining both protocol-based and service-based solutions in parallel, adding complexity. This point suggests that addressing the SBA approach in a new generation will avoid the issue

As a result, the consensus was that such a change of paradigm could only be realistically addressed at the beginning of a new generation. Introducing SBA in RAN as part of 6G system design ensures that the architecture can be conceived holistically, avoiding mid-generation disruptions and guaranteeing alignment with CN evolution. Hence, it follows that the advent of 6G represents the appropriate timing to study the feasibility of introducing SBA into the RAN. If not addressed from the outset, such a change would likely remain excluded, just as it was during the 5G cycle.

Observation 1: The introduction of the new generation (i.e., 6G) is the best timing for addressing a feasibility study on a RAN architecture paradigm change towards the SBA.

Moreover, any future work will require coordination with SA working groups (e.g., SA1 for use cases and requirements, SA2 for architecture, SA3 for security, SA5 for management) to ensure that SBA in RAN is designed consistently with the broader 6G system framework.

Of course, study the feasibility to introduce SBA approach to the RAN must also carefully consider performance constraints of each function. For example, the stringent requirements of fronthaul traffic could make this approach infeasible for the functionalities related to DU–RU interaction, which however will be discussed outside 3GPP (e.g., in O-RAN Alliance).

## Benefits of introducing the SBA approach in the RAN

The 5G network design established a clear separation between the RAN and the CN, with only a few control-plane and user-plane interfaces crossing the two traditionally distinct domains. While this separation enabled standalone operation and decoupling of the two domains, the increasing trend of cloudification has led to limitations in addressing new use cases and requirements, particularly when it comes to tight integration of resources with radio access network functions. In contrast, SBA offers several significant advantages, particularly for the RAN. These advantages align well with the ongoing shift towards software-driven, modular network designs, but they also highlight some of the shortcomings of the traditional protocol-based approach (Table 1).

| Aspect | Service-Based Architecture (SBA) | Protocol-Based Architecture (PBA) |
| --- | --- | --- |
| Modularity & Flexibility | ✓ Highly modular: functions exposed as independent, reusable services via APIs | ✗ Rigid layering with tightly coupled protocol stacks |
| Interoperability | ✓ Enables vendor-agnostic integration through open, standardized interfaces (e.g., REST) | ✗ Interfaces often proprietary with specific communication patterns |
| Scalability | ✓ Supports dynamic scaling of individual services based on load or demand | ✗ Scaling typically applies to entire network functions or elements |
| Upgradability | ✓ Individual services can be upgraded or replaced independently | ✗ Software updates often require revalidation of entire function stack |
| Observability & Monitoring | ✓ Fine-grained monitoring per service, enabling better analytics and closed-loop automation | ✗ Monitoring is often coarse-grained and tied to protocol endpoints |
| Automation & AI Integration | ✓ Natively supports closed-loop automation and AI/ML-based optimization | ✗ Automation is limited to predefined rule sets or external tools |
| Latency | ✗ May introduce some additional overhead/latency due to service granularity, API calls, and message translation | ✓ Efficient runtime performance with tightly integrated protocol layers |
| Legacy Compatibility | ✗ Requires adaptation layers to interoperate with protocol-based systems | ✓ Direct compatibility with existing legacy systems |

Table 1 – Comparison between architecture approaches

In particular, the following points can be identified:

- Modularity & Flexibility: SBA provides a higher level of modularity with independent services that can be easily integrated and reused via APIs, whereas PBA’s tightly coupled protocol stacks limit flexibility and complicate upgrades

- Interoperability: SBA uses open, standardized interfaces (like Representational State Transfer – REST), making it vendor-agnostic and easy to integrate. In contrast, PBA often relies on proprietary interfaces that may limit vendor diversity

- Scalability: SBA allows for dynamic scaling of individual services, while PBA typically scales entire network functions or elements as a whole, which is less efficient and more rigid

- Upgradability: In SBA, services can be independently upgraded or replaced without affecting other network functions. PBA requires revalidating the entire function stack when updates are made

- Observability: SBA offers fine-grained monitoring, enabling better analytics and closed-loop automation. PBA typically provides more coarse-grained monitoring tied to protocol endpoints

- Automation & AI Integration: SBA is designed to support closed-loop automation and AI/ML-based optimization natively, whereas PBA’s automation is limited to predefined rule sets or external tools

- Latency & Performance: While SBA might introduce overhead due to its service granularity, API calls, and message translation, PBA ensures better performance due to tightly integrated protocol layers

- Legacy Compatibility: SBA requires adaptation layers to ensure interoperability with legacy systems, while PBA is directly compatible with existing legacy infrastructure

Limitations highlighted for current architecture underscore the need for a more flexible, modular, and scalable architecture, and the SBA approach seems to address them while enabling a more dynamic and future-proof RAN design.

Moreover, adopting an SBA in the design and deployment of the 6G RAN enables several architectural and operational benefits:

- Architectural Flexibility – Network functions can be dynamically composed and reconfigured to adapt to variations in user density, mobility patterns, service types, and traffic load

- Reduced Vendor Lock-in – The clear separation between service producers and consumers promotes the use of open interfaces, minimizing dependency on proprietary implementations

- Network Lifecycle Cost Efficiency – The modular nature of SBA simplifies the deployment, upgrade, and decommissioning of individual services, contributing to reductions in both operational (OPEX) and capital expenditures (CAPEX)

- Seamless AI Integration – The architecture natively supports the integration of AI-driven functionalities, such as mobility prediction and anomaly detection, including their training, deployment, and lifecycle management

- Cloud-Native Alignment – SBA aligns closely with cloud-native principles, leveraging container-based deployments and microservice orchestration to enable on-demand scalability and improved system resilience

- Enhanced Observability and Automation – Built-in service exposure and API-based approach facilitate advanced monitoring, policy enforcement, and support for intent-based automation mechanisms

- Cross-Domain Coordination – The use of SBA supports orchestration across RAN, CN and edge domains enabling end-to-end network agility and optimization centered on user experience.

Observation 2: The introduction of SBA approach will avoid current limitation of protocol-based non-cloud native RAN architecture, while enabling a more dynamic and future-proof RAN design, architectural and operational efficiency benefits.

Considering the prior analysis conducted during the 5G standardization and the potential expected benefits over the current protocol-based architecture model, it is proposed RAN3 to initiate a study to evaluate the feasibility of introducing an SBA approach in the RAN.

Proposal 1: Based on past studies and expected benefits, it is proposed that RAN3 study the feasibility of introducing an SBA in the RAN.

## Requirements for introducing the SBA approach in the RAN

The introduction of an SBA into the RAN requires a careful evaluation of specific criteria to ensure the suitability of NFs for transformation. These criteria are designed to identify which RAN functions can benefit from a shift towards SBA, while considering the technical and operational constraints of the RAN environment. Replacing protocol-based network functions with services in an SBA requires evaluating specific criteria to ensure suitability.

- Functional Decoupling – The first step in migrating from PBA to SBA is evaluating whether the RAN function is logically self-contained and not tightly dependent on specific message sequences or implicit shared state. For a network function to be exposed as a service, it must be decoupled from the underlying protocol layers and operate independently. Functions that require deeply interdependent operations across multiple layers or maintain implicit shared state should be redesigned or re-engineered to facilitate functional decoupling

Observation 3: If the functionalities can be exposed as a clear and autonomous API, it’s a strong candidate for service transformation.

- Statelessness or Explicit State Handling – SBA favours services that are stateless or those where the state is explicitly managed through external repositories. Stateless services can scale more easily and remain flexible, whereas services relying on implicit or deeply distributed state require significant changes

Observation 4: RAN functions that rely on complex, dynamic, or implicit state management may need substantial redesign efforts to meet SBA principles.

- Modularity and Reusability – Functions intended for SBA should be modular and reusable across multiple consumers or components. Hence, functions that serve a broad set of use cases or are common across multiple network elements are strong candidates for service-based architecture

Observation 5: If a function is designed for a single, specific purpose on a single node, its transformation into a service may not offer sufficient value.

- Temporal Independence and Asynchronicity – SBA promotes asynchronous interactions and on-demand service invocations (e.g., REST, HTTP/2, HTTP/3), making it ideal for use cases where real-time, tightly coupled communication is not required. Functions that rely on synchronous, time-sensitive interactions, such as those found in the lower layers of the MAC and physical layer, may not be well suited for SBA and should maintain PBA-based mechanisms to meet latency and performance requirements

Observation 6: Functions that require real-time, tightly coupled communication (e.g., hard real-time constraints) may not translate well into service-based models.

- Well-defined Interfaces – For successful SBA integration, the RAN function must expose clear, contract-first APIs with well-defined input/output parameters, specified semantic behaviours, and robust error handling mechanisms. Legacy functions with implicit behaviour can present significant challenges when attempting to expose them as services in SBA.

Observation 7: Legacy protocols often embed implicit behaviors that are hard to expose as clean APIs.

- Observability and Manageability – One of the core principles of SBA is the ability to observe and manage services independently. Services in the RAN should be instrumented to expose metrics and events that allow for closed-loop automation, fault detection, and performance optimization

Observation 8: Functions that cannot be monitored or orchestrated independently of the broader network may not be suitable for a service-based implementation.

Taking into account previous criteria, the approach to designing a SBA for the 6G RAN can be inspired by the principles already established in the 5G Core and extending them to meet the unique challenges of the RAN domain. The following requirements and principles outline the key factors for a successful SBA integration in the RAN.

- Service Discovery and Registration – Each RAN function is exposed as an independent service, uniquely identifiable and dynamically registered in a service repository. SCs can query this repository at runtime to discover available service producers and their capabilities

- Network-Level Connectivity – The architecture ensures end-to-end reachability between service consumers and producers, both within the RAN and across domains. This is enabled through a service bus or equivalent message routing layer that supports seamless communication

- Security and Access Control – Robust mechanisms for Authentication, Authorization, and Accounting (AAA) are embedded in the architecture. These ensure trust enforcement and policy compliance across service interactions, even in dynamic, multi-stakeholder environments

- Service Invocation and Routing – The architecture supports dynamic service invocation and intelligent routing mechanisms. This abstracts consumers from the underlying physical topology and enables flexible orchestration of RAN functions

- Monitoring – Each service is instrumented to expose standardized metrics and events. This enables observability, closed-loop automation, fault detection, and performance optimization at the service level

- Interoperability and Extensibility – The architecture promotes vendor-neutral, standardized interfaces to minimize proprietary dependencies. This facilitates the integration of new services, including AI-driven modules and Digital Twin instances, as the network evolves

Based on the analysis of the main characteristics of the SBA paradigm, a set of key architectural requirements has been identified. These requirements should be carefully considered when evaluating the feasibility and scope of applicability of introducing the SBA approach in the RAN.

Proposal 2: RAN3 to take into account the identified SBA architectural requirements when evaluating the feasibility of introducing SBA in the RAN.

## Constraints for introducing the SBA approach in the RAN

While the SBA approach aligns well with the ongoing softwarization trends in network design, its adoption also exposes some additional challenges, that can also guide to identify in which part of the RAN this approach might be applied.

- Performance and Latency – Not all functions in the RAN are suitable for being directly integrated into the SBA. Some control loops (such as the ones between the lower parts of the MAC and the physical layer) should not be integrated in the SBA and should keep current PBA approach

- Operational Complexity – Discovery protocols shall be enabled also in the RAN, integrating current efforts already taking place in the 5G Core, as well as specific RAN orchestration components

- Security Surface – Security on RAN interfaces has attracted attention in the past few years. These efforts are instrumental for the SB transformation of the RAN

- Legacy Compatibility – The transition towards SBA necessarily involves the utilization of gateways and translation to the new paradigm, to guarantee a smooth transition towards the new generation

Therefore, network functions implemented within an SBA framework must carefully address various aspects to ensure reliable and secure operation without compromising interoperability or efficiency.

Observation 9: The applicability of the SBA approach in the RAN (or part of it) depends on the ability to meet the constraints outlined above.

Based on the aspects outlined above, the feasibility study for introducing the SBA in the RAN shall take into account the identified constraints, in order to determine the most appropriate architectural evolution and assess in which parts of the RAN the SBA approach can be effectively applied.

Proposal 3: RAN3 to take into account the identified SBA architectural constraints when evaluating the feasibility of introducing SBA in the RAN.

# 3	Conclusion

In this paper we discuss the possibility to study the introduction of an SBA approach in the RAN architecture highlighting several observations leading to the following proposals:

Proposal 1: Based on past studies and expected benefits, it is proposed RAN3 to study the feasibility of introducing a Service Based approach in the RAN architecture.

Proposal 2: RAN3 to take into account the identified SBA architectural requirements when evaluating the feasibility of introducing SBA in the RAN.

Proposal 3: RAN3 to take into account the identified SBA architectural constraints when evaluating the feasibility of introducing SBA in the RAN.

# 4	References

[1]	https://sns-origami.eu/

[2]	S. Bizzarri, M. Fodrini, G. Garcia-Aviles, F. Spinelli, M. Fiore, M. Gramaglia, “Rethinking Network Architecture: Enabling Service-Based Design Beyond 5G”, Proceedings of the 2025 IEEE Conference on Standards for Communications and Networking (CSCN 2025), Bologna, Italy, Sep. 2025

[3]	3GPP TS 28.533, “Management and orchestration; Architecture framework” (Rel.19)

[4]	3GPP S2-2105493, “Study on Service-based N2” (Rel.18)