3GPP TSG-RAN3 Meeting #129-bis	R3-256575

Prague, Czech Republic, 13 – 17 Oct 2025

Agenda Item:	10.3.1

Source: 	Qualcomm Inc, NTT Docomo Inc, Fibercop, Tejas Network Limited, KT Corp, Charter Communications Inc.

Title: 	6G RAN-CN interface: General principles and requirements

Document for:	Discussion

# Background

6G RAN WG SID approved in [1], RP-252912, has the below objectives on 6G RAN architecture:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In this contribution, we discuss the details on General principles and requirements of 6G RAN-CN functional split and interface.

# Discussion

## 6G RAN-CN functional split

During 4G to 5G transition, RAN-CN functional split was preserved in the 5G system. This functional split helped the operators support a multi-vendor ecosystem in the networks and enable flexibility in network evolution. It may be beneficial to retain similar RAN-CN functional split in 6G. We can start discussing 6G RAN-CN functional split by taking TS 38.300, Clause 4.2, 5G RAN-CN functional split as reference and identify necessary changes to functional split by analysing existing RAN-CN functional split.

Proposal 1: RAN3 to discuss the 6G RAN-CN functional split based on 5G RAN-CN functional split defined in TS38.300, clause 4.2 as starting point. FFS new 6G RAN functions.

In 5G systems, key functions such as paging, UE mobility management, UE radio capability storage, and UE context storage are distributed between the RAN and CN. This functional distribution is designed to support enabling efficient resource management and signaling behaviour for the various RRC states of the UE (IDLE, INACTIVE, and CONNECTED).

However, for 6G, it is too early to determine whether the same set of RRC states will be supported, and how the associated functionalities and procedures will be handled in each state. These aspects require detailed study by SA2 and RAN2. Based on the outcome of this study, RAN3 can then evaluate the implications on the RAN-CN functional split.

Proposal 2: RAN3 to discuss in conjunction with RAN2/SA2 whether the functionality of paging, UE mobility management, UE radio capabilities storage, UE context storage are supported on 6G RAN, 6G CN or both.

## 6G RAN-CN interface

## General principles and requirements

In 5G, many RAN functions are deployed as appliances. Some operators have started using cloud-based RAN deployments. As 6G networks evolve, more operators are expected to shift from appliance-like RAN deployments to cloud-native RAN deployments. This transition will bring greater flexibility, scalability, efficiency to network operations and allows to use cloud native services for both RAN and CN operations. This will also avoid operators from maintaining 3GPP specific protocols and interfaces which are not cloud friendly (example: SCTP is not used in any cloud deployments). To support this change, the 6G RAN-CN interface should be open, multi-vendor inter-operable, and designed to work in cloudified RAN deployments that leverage cloud-native commercial solutions, such as containerization, message brokers, etc. At the same time, 6G RAN-CN interface should still be able to support appliance like deployments.

The RAN-CN interface needs to be security protected. RAN3 should coordinate with SA3 to understand if there are any implications in the security mechanisms used for the RAN-CN interface protection when deploying the RAN in a cloud-native environment.

Observation 1: Over the 6G timeline, many operators may start supporting cloud-native RAN function deployments.

Proposal 3: 6G RAN-CN interface shall be open, and multi-vendor inter-operable and shall support both user plane and control plane.

Proposal 4: The 6G RAN-CN interface should support cloudified RAN deployments, which leverage commercial cloud-native solutions such as containerization and message brokers.

Proposal 5: RAN3 and SA3 coordinate on the security support for cloud-native RAN deployments.

5G RAN communicates with 5G core NFs (e.g. SMF, LMF, NWDAF) through the AMF, which acts as the main control point for mobility, registration, and connection setup. However, for some use cases such as sensing and positioning, ambient IoT, there may be both functional and performance benefits if the RAN communicates directly with service-specific core NFs without routing data though the AMF. Therefore, RAN3 and SA2 should jointly evaluate the 6G architectural options and decide whether the 6G RAN-CN interface should support a single or multiple direct connections to 6G core NFs for the different 6G use cases.

Observation 2: 5G RAN communicates with 5G core NFs via the AMF only. However, for some use cases and features (e.g., A-IoT, potentially sensing), there are functional and performance advantages for 5G RAN to interact directly with new 5G core NFs.

Proposal 6: RAN3 and SA2 to decide on whether only one or multiple core NFs have a direct RAN-CN interface to 6G RAN.

## TNL requirements

The Transport Network Layer (TNL) in 5G supports appliance-like deployments, but it shows limitations when used in cloud-native deployments, as discussed below. Moving forward to 6G, the TNL should be designed to properly support cloud-native RAN deployments.

Proposal 7: The transport layer of the 6G RAN-CN interface should allow for cloud-native RAN deployments.

In the 5G RAN-CN interface, interface end point locations on the TNL are signalled as TNL addresses. In cloud-native deployments, the TNL endpoint location can be signalled either in form of TNL addresses or in form of FQDNs, which are then resolved to TNL addresses via a discovery service (e.g., DNS). To provide more flexibility to 6G RAN deployments, both mechanisms need to be e supported in 6G.

Observation 3: In 5G, interface endpoint locations are signalled in form of a TNL addresses. In cloud-native deployments, interface endpoint locations can be signalled in form of TNL addresses or in form of FQDNs, which are resolvable to TNL addresses via a discovery service (e.g., DNS).

Proposal 8: The 6G RAN-CN interface should support signalling of interface endpoint locations in form of TNL addresses as well as in form of FQDNs, which are resolvable to TNL address via a discovery service (e.g., DNS).

In traditional 5G RAN-CN deployments, it is typically assumed that the application layer endpoint is co-located with the underlying transport network layer (TNL) endpoint. However, in cloud-native environments, this assumption may not hold, especially when TNL bridges like message brokers or service meshes are used. Similar to cloud services, where the HTTP layer terminates at an edge location, such as an API Gateway or Load Balancer, while the actual application logic is present deeper in the network, cloud-native deployments benefit from separation of the application layer and transport layer endpoints. This separation improves scalability, flexibility, and fault tolerance, especially in environments where functions are frequently scaled, moved, or restarted.

Observation 4: For the 5G RAN-CN interface, it is assumed that the application layer end point is co-located with underlying TNL end point. In cloud-native deployments, this assumption may not apply, e.g., in case TNL bridges such as message brokers are used.

Proposal 9: For the 6G RAN-CN interface, the application layer end point may not be co-located with the underlying TNL end point

In some 5G architectures, NTN with regenerative payload, Femto nodes, and wireless access/backhaul (WAB), the connection between the 5G RAN and CN can occasionally be unstable or interrupted. These interruptions may happen due to changing network conditions or movement of the RAN node. In some cases, the TNL may need to change its address to keep the connection working. Also, during failover situations, when RAN functions switch to backup RAN nodes, the TNL address might change. To handle these situations smoothly, the 6G RAN-CN interface should be designed to be robust and allow dynamic changes to TNL addresses on either side of the connection.

Observation 5: 5G efforts on NTN regenerative payload, Femto node and WAB have shown that the RAN-CN interface may be subject to interruption and that it may require dynamic change of TNL addresses.

Observation 6: Robustness to RAN-CN interface interruption and TNL address change may also be needed during fail over of RAN functions.

Proposal 10: The 6G RAN-CN interface shall be designed to provide robustness to potentially unpredictable interruptions of the TNL and to allow for dynamic TNL address changes on either end point.

# 3 Conclusion

Below are the observations and proposals discussed in this document.

6G RAN-CN functional split

Proposal 1: RAN3 to discuss the 6G RAN-CN functional split based on 5G RAN-CN functional split defined in TS38.300, clause 4.2 as starting point. FFS new 6G RAN functions.

Proposal 2: RAN3 to discuss in conjunction with RAN2/SA2 whether the functionality of paging, UE mobility management, UE radio capabilities storage, UE context storage are supported on 6G RAN, 6G CN or both.

6G RAN-CN interface – General principles and requirements

Observation 1: Over the 6G timeline, many operators may start supporting cloud-native RAN function deployments.

Proposal 3: 6G RAN-CN interface shall be open, and multi-vendor inter-operable and shall support both user plane and control plane.

Proposal 4: The 6G RAN-CN interface should support cloudified RAN deployments, which leverage commercial cloud-native solutions such as containerization and message brokers.

Proposal 5: RAN3 and SA3 coordinate on the security support for cloud-native RAN deployments.

Observation 2: 5G RAN communicates with 5G core NFs via the AMF only. However, for some use cases and features (e.g., A-IoT, potentially sensing), there are functional and performance advantages for 5G RAN to interact directly with new 5G core NFs.

Proposal 6: RAN3 and SA2 to decide on whether only one or multiple core NFs have a direct RAN-CN interface to 6G RAN.

6G RAN-CN interface – TNL requirements

Proposal 7: The transport layer of the 6G RAN-CN interface should allow for cloud-native RAN deployments.

Observation 3: In 5G, interface endpoint locations are signalled in form of a TNL addresses. In cloud-native deployments, interface endpoint locations can be signalled in form of TNL addresses or in form of FQDNs, which are resolvable to TNL addresses via a discovery service (e.g., DNS).

Proposal 8: The 6G RAN-CN interface should support signalling of interface endpoint locations in form of TNL addresses as well as in form of FQDNs, which are resolvable to TNL address via a discovery service (e.g., DNS).

Observation 4: For the 5G RAN-CN interface, it is assumed that the application layer end point is co-located with underlying TNL end point. In cloud-native deployments, this assumption may not apply, e.g., in case TNL bridges such as message brokers are used.

Proposal 9: For the 6G RAN-CN interface, the application layer end point may not be co-located with the underlying TNL end point

Observation 5: 5G efforts on NTN regenerative payload, Femto node and WAB have shown that the RAN-CN interface may be subject to interruption and that it may require dynamic change of TNL addresses.

Observation 6: Robustness to RAN-CN interface interruption and TNL address change may also be needed during fail over of RAN functions.

Proposal 10: The 6G RAN-CN interface shall be designed to provide robustness to potentially unpredictable interruptions of the TNL and to allow for dynamic TNL address changes on either end point.

# 4 References

[1] RP-252912, Revised SID: Study on 6G Radio