3GPP TSG-RAN WG3 Meeting #129bis	R3-256581

Prague, Czech Republic,  13th-17th October 2025

Title:	RAN-CN interface considerations

Source:				China Telecom

Agenda item:		10.3.2

Document for:		Discussion and Decision

1. Introduction

In RAN#109 meeting, the 6G requirements for architecture and migration were discussed. The following requirements were captured in TR 38.914:

-	The 6G RAN architecture shall support standalone RAN architecture.

-	The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

-	The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

-	The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

-	The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

-	3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.

-	The 6G RAN architecture shall allow for control plane and user plane separation.

-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.

-	The 6G RAN architecture shall allow for the operation of network slicing.

-	The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

-	The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

-	The 6G RAN architecture shall allow non-public networks.

In this contribution, we further discuss the RAN-CN interface.

2. Discussion

In 5G networks, the NG interface connects the RAN to the CN. The NG interface splits into NG-C for control plane and NG-U for user plane. It uses point-to-point protocols like NG-AP (for NG-C) and GTP-U (for NG-U). The 5G CN has largely adopted a Service-Based Architecture (SBA), where Network Functions (NFs) expose services via HTTP/2-based Service-Based Interfaces (SBIs) for modularity and cloud-native deployment. But the RAN side remains more traditional and integrated, limiting full end-to-end SBA benefits. In 6G, there is discussion on whether to inherit and evolve the 5G NG interface for continuity or define a fully new SBI for innovation.

From our side, the NG interface, with its point-to-point design, offers deterministic latency and proven stability, making it well suited for critical signalling and user-plane procedures. Inheriting and envolving the NG interface can reduce costs and accelerate deployment. Therefore, we propse to inherit and envolve NG interface between 6G RAN and 6G CN for legacy services.

Proposal 1: The 6G RAN architecture could support NG like interface between 6G RAN and 6G CN.

As outlined in the 6G SID [1], a key objective is the investigation of 6G RAN architecture, interface protocols and procedures, to natively support various new services and functionalities, such as AI/ML and integrated sensing and communication (ISAC).

From our side, the existing NG interface may pose limitations for the efficient transfer of data generated for new 6G services. For instance, transferring AI/ML training data or high-volume sensing information from the RAN to the CN might suffer from suboptimal latency and overhead due to the interface's rigid data models and procedures. In contrast, SBI could enable the RAN to interact directly with new 6G service functions within the CN through flexible APIs, potentially enhancing interaction efficiency and enabling more dynamic data exchange. However, adopting a full SBI approach for the RAN-CN connection introduces challenges, including increased system complexity, potential backward compatibility issues with previous generations, and a risk of prolonging the standardization timeline towards Releases 21/22. Therfore, as for 6G new services, we propose to discuss the pros and cons of NG like interface and SBI. Then, determine whether to define a fully new SBI for new 6G services or not.

Proposal 2: Discuss the trade-off between NG like interface and SBI to determine whether the 6G RAN architecture shall support SBI between 6G RAN and 6G CN.

3. Conclusion

In this contribution, we discuss RAN-CN interface. We have the following proposals:

Proposal 1: The 6G RAN architecture could support NG like interface between 6G RAN and 6G CN.

Proposal 2:  Discuss the trade-off between NG like interface and SBI to determine whether the 6G RAN architecture shall support SBI between 6G RAN and 6G CN.

4. Reference

- RP-251881 New SID: Study on 6G Radio.