3GPP TSG-RAN WG3 #129bis                                    R3-256666

# Prague, Czech, Oct. 13~17, 2025

Agenda item:	11.2.1

Source: 	Samsung

Title: 	Discussion on MRO enhancement on inter-CU LTM

Document for:	Discussion and decision

Introduction

Rel-20 SON WI has been approved [1], and the work needed to be done in RAN3 is as follows,

- MRO enhancement for R19 mobility mechanisms, including inter-CU LTM and intra-CU conditional LTM [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements on the existing interfaces [RAN3]

In this contribution, we discuss the support of MRO on inter-CU LTM.

Discussion

As indicated in the WID, the MRO enhancement on inter-CU LTM is in the scope.

Recall that in R19 SON, the MRO is enhanced for intra-CU LTM. In our understanding, the main difference between intra-CU LTM and inter-CU LTM is that more CU(s) takes part in the LTM procedure, so there is chance to forward e.g. the RLF report from one gNB-CU to another over XnAP because of inter-CU LTM cell switch failure.

Currently in TS 38.300, the following text is captured for the detection mechanism,

Detection mechanism

A failure indication may be initiated after a UE attempts to re-establish the radio link connection at NG-RAN node B after a failure at NG-RAN node A. NG-RAN node B may initiate the Failure Indication procedure towards multiple NG-RAN nodes if they control cells which use the PCI signalled by the UE during the re-establishment procedure. The NG-RAN node receiving this selects the UE context that matches the received Failure Cell ID and C-RNTI, and, if available, uses the shortMAC-I to confirm this identification, by calculating the shortMAC-I and comparing it to the received IE.

A failure indication may also be sent to the node last serving the UE when the NG-RAN node fetches the RLF REPORT from UE by triggering:

-	The Failure Indication procedure over Xn;

- The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

…

In case of Too Early Handover or Handover to Wrong Cell, the NG-RAN node receiving the failure indication may inform the NG-RAN node controlling the cell where the mobility configuration caused the failure by means of the Handover Report procedure over Xn or the Uplink RAN Configuration Transfer procedure over NG. This may include the RLF report.

It should be noted that the above text also covers the connection failure cases for intra-CU LTM cell switch; in our opinion, the above text can also cover connection failure cases for inter-CU LTM cell switch in principle. More specifically, we think the following principle should be reused,

- The Failure Indication procedure over Xn is used to forward the failure indication to the last serving gNB/gNB-CU.

- After receiving the failure indication, the last serving gNB/gNB-CU performs initial analysis.

- In case of Too early LTM cell switch and LTM cell switch to wrong cell, the last serving gNB/gNB-CU may forward the failure indication to the node causing the failure by means of the Handover Report procedure over Xn.

Proposal 1: The following principles on connection failure detection mechanism should be reused for inter-CU LTM cell switch failure cases.

- The Failure Indication procedure over Xn is used to forward the failure indication to the last serving gNB/gNB-CU.

- After receiving the failure indication, the last serving gNB/gNB-CU performs initial analysis.

- In case of Too early LTM cell switch and LTM cell switch to wrong cell, the last serving gNB/gNB-CU may forward the failure indication to the node causing the failure by means of the Handover Report procedure over Xn.

Another aspect that should be discussed is the transfer of LTM information over XnAP for additional failure cases.

Recall that in R19 MRO enhancement on intra-CU LTM, three additional failure cases are identified, and the recovery beam information (for Case1), the re-established or recovery beam information (for Case 2 or 3), and the TA value used at successful RA (for Case 3) are introduced to be transferred over F1AP from target gNB-DU to source/last serving gNB-DU via gNB-CU in ACCESS AND MOBILITY INDICATION message and DU-CU ACCESS AND MOBILITY INDICATION message, which is basically described in TS 38.401 as follows,

The target gNB-DU identifies that a Beam Failure Recovery (BFR) has happened in the UE shortly after a successful LTM cell switch by detecting that a time gap between the successful LTM cell switch and the BFR in the same cell is smaller than the configured threshold (e.g. Tstore_UE_cntxt). The target gNB-DU performs initial analysis and sends the recovery beam information to the source gNB-DU via the gNB-CU.

The target gNB-DU identifies that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell. The target gNB-DU sends the re-established or recovery beam information, the TA value used at successful RACH-based access, or both to the source gNB-DU via the gNB-CU.

For inter-CU LTM case, such information (i.e. the recovery beam information (for Case1), the re-established or recovery beam information (for Case 2 or 3), and the TA value used at successful RA (for Case 3)) is also needed to be transferred over XnAP.

Proposal 2: RAN3 continues to consider the following failure cases for inter-CU LTM,

- Case 1: BFR to the same cell shortly after successful LTM cell switch.

- Case 2: RACH-less LTM cell switch to a wrong beam under the same cell.

- Case 3: The TA is outdated.

Proposal 3: The following information needs to be transferred over XnAP in order to support Cases mentioned in P2,

- For Case 1, the recovery beam information.

- For Case 2 and 3, the re-established or recovery beam information.

- For Case 3, the TA value used at successful RA.

In addition, transferring the information mentioned above in which XnAP message is needed to be further discussed. Currently, there are several options,

- Option 1: Using XnAP ACCESS AND MOBILITY INDICATION message.

- Option 2: Using XnAP FAILURE INDICATION message.

- Option 3: Using XnAP HANDOVER REPORT message.

In our understanding, Option 1 is the most suitable approach, similar to the approach introduced over F1AP for intra-CU LTM.

Proposal 4: RAN3 is kindly asked to discuss on which XnAP message to transfer the information provided in P3 based on the following options,

- Option 1: Using XnAP ACCESS AND MOBILITY INDICATION message.

- Option 2: Using XnAP FAILURE INDICATION message.

- Option 3: Using XnAP HANDOVER REPORT message.

Conclusion

In this paper, we provide discussion on MRO enhancement on inter-CU LTM. The following proposals are provided,

Proposal 1: The following principles on connection failure detection mechanism should be reused for inter-CU LTM cell switch failure cases.

- The Failure Indication procedure over Xn is used to forward the failure indication to the last serving gNB/gNB-CU.

- After receiving the failure indication, the last serving gNB/gNB-CU performs initial analysis.

- In case of Too early LTM cell switch and LTM cell switch to wrong cell, the last serving gNB/gNB-CU may forward the failure indication to the node causing the failure by means of the Handover Report procedure over Xn.

Proposal 2: RAN3 continues to consider the following failure cases for inter-CU LTM,

- Case 1: BFR to the same cell shortly after successful LTM cell switch.

- Case 2: RACH-less LTM cell switch to a wrong beam under the same cell.

- Case 3: The TA is outdated.

Proposal 3: The following information needs to be transferred over XnAP in order to support Cases mentioned in P2,

- For Case 1, the recovery beam information.

- For Case 2 and 3, the re-established or recovery beam information.

- For Case 3, the TA value used at successful RA.

Proposal 4: RAN3 is kindly asked to discuss on which XnAP message to transfer the information provided in P3 based on the following options,

- Option 1: Using XnAP ACCESS AND MOBILITY INDICATION message.

- Option 2: Using XnAP FAILURE INDICATION message.

- Option 3: Using XnAP HANDOVER REPORT message.

Reference

[1] RP-252560