3GPP TSG-RAN WG3 Meeting #129bis                                                                          R3-256767

Prague, Czech Republic, 13 – 17 October, 2025

Source:	CATT

Title:	Discussion on Paging capability missing issue

Agenda Item:	9.2.14

Document for:	Discussion and Decision

1 Introduction

In the last meeting[1], RAN3 has some issues leftover for paging capability missing during the LP-WUS session, as captured by chair’s notes:

The solution for missing UE capability for paging features will be introduced from rel 17 for initial context setup. The following may be captured in the reply LS sent in the next meeting: RAN3 note that the possibility of paging loss due to paging capability mismatch may still exist in case that paging message is sent to the gNB before paging capability mismatch resolution. - RAN3 will agree corresponding CRs and reply LS in the next meeting based on the above, considering following aspects: - Whether the extension of the solution for Path Switch Request procedure and Handover Resource Allocation procedure is needed or not. - Whether the optimization for the solution proposed by Vodafone in R3-255529/R3-255530/R3-255531 is supported or not.

In this contribution, we discuss these issues.

2 Discussion

2.1 Handling missing UE capability for paging features in initial context setup

For the paging capability missing issue, SA2 assumed that the way recommended by RAN2 cannot address the issue effectively because it potentially requires upgrading gNBs in whole NR coverage of the serving PLMN. Meanwhile, SA2 has studied another way of implementation which is inspired by the solution captured in TS 23.501 clause 5.4.4.3 with respect to handling the inter-RAT paging loss, and SA2 sent a LS (R3-255026) to RAN3/RAN2. The LS [2] and clause 5.4.4.3 in TS 23.501[3] are copied as follows.

R3-255026: The 5GS design means that the UE Radio Capability for Paging is loaded into the AMF only in the cell where the UE attaches (does “initial registration”) or first moves from LTE to NR. After that the UE can move throughout the whole PLMN and normally there is no event that triggers any update of the UE Radio Capability for Paging information stored in the AMF. Hence the “certain area” mentioned by RAN 2 is the whole NR coverage of that PLMN. It is essential that operators are able to use RAN equipment from different vendors and that competition between those RAN vendors is not inhibited by RAN 2 requiring all RAN vendors to upgrade their equipment before new UE functionality can be used with the first RAN vendor to develop a solution. Hence this RAN2 solution is highly undesirable compared to what was proposed in RAN2’s earlier LS in R2-2501388=S2-2504576. SA2 are investigating whether a solution that builds on the current process in TS 23.501 clause 5.4.4.3 for handling UEs that support both “LTE connected to 5GC” and NR can be used to solve the problem. For example, when (as already specified) the UE Radio Capability for Paging is sent in the NGAP Initial UE Context Setup Request to the gNB, the gNB could use the UE Radio Access Capabilities (received in the same message) to check that the UE Radio Capability for Paging contains the full information on paging related features. If the gNB detects an omission, then the gNB should ensure that an update of the UE Radio Capability for Paging Information is sent to the AMF. Clause 5.4.4.3 in TS 23.501: a) UE radio capability for paging information: - The UE Radio Capability for Paging Information contains information derived by the NG-RAN node (e.g. band support information) from the UE Radio Capability information. The AMF stores this information without needing to understand its contents. As the AMF only infrequently (e.g. at Initial Registration) prompts the NG-RAN to retrieve and upload the UE radio capabilities i.e. UE Radio Capability information to the AMF and the AMF may be connected to more than one NG-RAN RAT, it is the responsibility of the NG-RAN to ensure that UE Radio Capability for Paging Information (which is derived by the NG-RAN node) contains information on all NG-RAN RATs that the UE supports in that PLMN. To assist the NG-RAN in this task, as specified in TS 38.413 [34], the AMF provides its stored UE Radio Capability for Paging Information in every NG-AP INITIAL CONTEXT SETUP REQUEST message sent to the NG-RAN. - The UE Radio Capability for Paging Information is maintained in the core network, even during AMF reselection and is stored in the UCMF alongside the UE Radio Capability information associated to a UE Radio Capability ID.

It proposed that gNB could use the UE Radio Access Capabilities (received in INITIAL CONTEXT SETUP REQUEST message) to check that the UE Radio Capability for Paging (URCP) contains the full information on paging related features, if not, it sends the updated URCP to AMF. In the last meeting, RAN3 agreed to capture this solution in NGAP spec from Rel-17.

However, corrections[4] on the similar solution which handles the UE paging capability missing in inter-RAT scenario have already been approved from Rel-16. Some view is that the two implementations, i.e., NG-RAN nodes check whether the URCP contains UE radio capability for paging of all RAT(s) and NG-RAN nodes check whether the URCP contains full paging information, can be performed by the same behavior. Indeed, the two can be implemented together. But we consider the requirements essentially different, because one would only need to check the presence of the paging capabilities for all RAT(s) while another one needs to look into the content of the RAT container.

9.3.1.68 UE Radio Capability for Paging This IE contains paging specific UE Radio Capability information. IE/Group Name; Presence; Range; IE type and reference; Semantics description. UE Radio Capability for Paging of NR; O; OCTET STRING; Includes the UERadioPagingInformation message as defined in TS 38.331 [18].. UE Radio Capability for Paging of E-UTRA; O; OCTET STRING; Includes the UERadioPagingInformation message as defined in TS 36.331 [21].. UE Radio Capability for Paging of NB-IoT; O; OCTET STRING; Includes the UERadioPagingInformation-NB message as defined in TS 36.331 [21]..

Therefore, some clarification for the NGAP initial context setup procedure is necessary to support the NG-RAN node checking whether the URCP of each RAT contains all paging features the UE supports.

Proposal 1: Clarification to NGAP UE context setup procedure is necessary to support NG-RAN node checking whether the URCP of each RAT contains all paging features UE supports.

It is also beneficial to capture the solution for handling the UE paging loss issue in stage-2 spec. The draftCRs to TS 38.300 R17/R18/R19 are provided in other contributions [5][6][7].

2.2 Whether to extend the solution to other scenarios

In the last meeting, companies assumed that the issue of paging loss may still exist in heterogeneous network, based on the solution that the gNB checks URCP in the NGAP UE CONTEXT SETUP message.

In our understanding, this solution can address the paging loss issue in the scenario where the UE keeps in RRC IDLE mode within the area deployed with early release gNBs and turns to RRC CONNECTED mode (i.e., UE context setup is performed) upon entering the area deployed with late release gNBs, relying on the behavior of the late release gNB to check the URCP at receiving the UE Context Setup message. However, sometimes it does not work:

- Case 1: The idle UE coming from the area of early release gNBs does not turn to RRC CONNECTED mode after entering the area of late release gNBs

- Case 2: The connected mode UE moves from an early release gNB to a late release gNB

In both cases, the late release gNBs have no chance to receive the Initial Context Setup Request message from AMF, and therefore the gNB cannot perform the URCP checking nor update the URCP to AMF.

We understand Case 1 can be prevented by specific deployment. For example, the network can ensure that all gNBs within each registration area of UE are the same release. If the idle UE moves from one registration area of early release gNBs to another area of late release gNBs, MRU procedure can be triggered in the UE, which makes it possible for the AMF to initiate the initial context setup procedure to the late release gNB. However, Case 2 cannot be resolved by appropriate deployment. So, it’s observed that the solution can work in case the idle UE moves from area of early release gNB to area of late release gNB, but it cannot work for connected UE.

It's easy to understand that the paging loss issue due to inter-RAT mobility also exist in case the connected UE moves between different RATs of NG-RAN nodes, if the solution only applies to the initial context setup procedure.

Observation 1: The solution cannot work in case a connected UE moves from an early release gNB to a later release gNB, or from one RAT to another RAT.

It’s necessary to extend the solution to connected scenarios, i.e., the gNB needs to check the URCP during the Handover Resource Allocation procedure and Path Switch Request procedure and send the updated URCP if needed. Since the solution for initial context setup scenario will be introduced from Rel-17, we propose to support the HO scenarios for Rel-17 as well.

Proposal 2: RAN3 agree to extend the solution to connected mobility scenarios from Rel-17.

As the URCP IE is already included in Core Network Assistance Information for RRC INACTIVE IE in the HANDOVER REQUEST and PATH SWITCH ACK messages, gNB can check the URCP with paging features of all RAT(s) the UE supports and update it (if needed) for subsequent RAN paging, which is similar as the handling for URCP in the CN assistance information IE in INITIAL CONTEXT SETUP REQUEST.

Proposal 3: GNB can check the URCP included in in Core Network Assistance Information for RRC INACTIVE IE in HANDOVER REQUEST and PATH SWITCH ACK messages with all paging features of all RAT(s) UE supports and update it for subsequent RAN paging.

However, considering the gNBs not supporting RRC Inactive state may not be able to decode the Core Network Assistance Information for RRC INACTIVE IE, a URCP IE under the HANDOVER REQUEST and PATH SWITCH REQUEST ACK messages should be introduced.

AMF can include the URCP in the HANDOVER REQUEST and PATH SWITCH REQUEST ACK messages during the handover resource allocation procedure and the path switch request procedure respectively. With this URCP, gNBs beyond R17 can check whether all paging features of all RAT(s) are included, if not, the gNB sends the updated URCP to AMF via the UE Radio Capability Info Indication procedure.

Proposal 4: Introduce a URCP IE under the HANDOVER REQUEST and PATH SWITCH REQUEST ACK message. AMF can include the URCP in NGAP messages during the handover resource allocation procedure and the path switch request procedure respectively. With this URCP, gNBs can check whether all paging features of all RAT(s) are included and send the updated URCP to AMF.

RAN3 can send the response LS to SA2 on NG-RAN handling the paging capability mismatch in initial context setup, handover resource allocation and path switch request, so that SA2 can update their spec accordingly.

2.3 Whether to enhance the URCP

In the last meeting, an open issue is left that whether the optimization proposed by the paper (R3-255527) is necessary. The paper suggested that the URCP introduces a new IE indicating the release of gNB which constructs the URCP. When a gNB receives the URCP from AMF, this field allows the gNB to identify the release of the originating gNB. In this way, the gNB can skip the paging capability mismatch checking if this field indicates a compatible release, and thus the processing load on gNB can be alleviated.

#### 9.3.1.68 UE Radio Capability for Paging This IE contains paging specific UE Radio Capability information. IE/Group Name; Presence; Range; IE type and reference; Semantics description. UE Radio Capability for Paging of NR; O; OCTET STRING; Includes the UERadioPagingInformation message as defined in TS 38.331 [18].. UE Radio Capability for Paging of E-UTRA; O; OCTET STRING; Includes the UERadioPagingInformation message as defined in TS 36.331 [21].. UE Radio Capability for Paging of NB-IoT; O; OCTET STRING; Includes the UERadioPagingInformation-NB message as defined in TS 36.331 [21].. UE Radio Capability for Paging check status; O; ENUMERATED (CheckedbygNBofRelease17, CheckedbygNBofRelease18, CheckedbygNBofRelease19...); When received in the Initial Context Setup Request message, this informs the gNB whether or not a gNB that understands Release 17 or Rel 18 or Rel 19 TS 38.413 ASN.1 has used the UE’s Radio Capability IE to check the correctness of the UE Radio Capability for paging fields within this IE..

We understand that processing the URCP check during each initial context setup procedure or handover procedure would not impose much to NG-RAN nodes, because they anyway have to process and store all the information of the URCP received from AMF.

On the other hand, if companies see any necessity to reduce the processing load on NG-RAN nodes, the implementation based on core network can be applied. For example, AMF does not provide the URCP to the subsequent gNB in NGAP messages in case the URCP was collected from a compatible release gNB. Note that present TS 23.501 specifies that AMF will provide the URCP in each INITIAL CONTEXT SETUP REQUEST message. If the AMF based optimization is confirmed, RAN3 need to inform SA2 to update their specification .

Observation 4: No critical necessity to enhance the URCP.

3 Conclusion

According to the analysis in section 2, we have following observations and proposals:

Proposal 1: Clarification to NGAP UE context setup procedure is necessary to support NG-RAN node checking whether the URCP of each RAT contains all paging features UE supports.

Observation 1: The solution cannot work in case a connected UE moves from an early release gNB to a later release gNB, or from one RAT to another RAT.

Proposal 2: RAN3 agree to extend the solution to connected mobility scenarios from Rel-17.

Proposal 3: GNB can check the URCP included in in Core Network Assistance Information for RRC INACTIVE IE in HANDOVER REQUEST and PATH SWITCH ACK messages with all paging features of all RAT(s) UE supports and update it for subsequent RAN paging.

Proposal 4: Introduce a URCP IE under the HANDOVER REQUEST and PATH SWITCH REQUEST ACK message. AMF can include the URCP in NGAP messages during the handover resource allocation procedure and the path switch request procedure respectively. With this URCP, gNBs can check whether all paging features of all RAT(s) are included and send the updated URCP to AMF.

Observation 4: No critical necessity to enhance the URCP.

4 Reference

- RAN3#129 Minutes, RAN3 chair

- R3-255026, Response to Reply LS on paging capability loss issue, SA2

- TS 23.501, System architecture for the 5G System (5GS), Stage-2, Rel-19, 2025-06

- R3-255885, Correction on UE radio capability for paging information, CMCC, RAN3#129

- R3-256824, Avoiding UE paging loss-r17, CATT, RAN3#129bis

- R3-256823, Avoiding UE paging loss-r18, CATT, RAN3#129bis

- R3-256822, Avoiding UE paging loss-r19, CATT, RAN3#129bis