3GPP TSG-RAN WG3 Meeting #129bis	R3-256780

Prague, Czech Republic, 13th – 17th October 2025

Agenda item:	9.2.11

Title:	Remaining Issue on AIML for Positioning

Source:	NEC

Document for:	Discussion and Decision

- Introduction

This contribution presents our views on the remaining issues for AI/ML-based positioning accuracy enhancement.

- Discussion

  - Lack of signaling for pairing Part A and Part B

It was previously agreed in RAN1 that the training data comprises Part A and Part B, where:

- Part A is used for measurement extraction

- Part B is used for ground truth extraction

RAN1 also agreed that Part A and Part B may be generated by different entities. For Case 3a, the possible entity combinations for generating Part A and Part B are specified in [1].

Agreement RAN1#116b For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Agreement RAN1#116b For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope.

To enable the pairing of Part A and Part B when they originate from different entities, RAN1 reached a consensus to use their timestamps as a key value element within the training data set [2].

Agreement RAN1#120 - For training data collection of AI/ML based positioning 3a/3b, if Part A and Part B are generated by different entities, for pairing between a Part A entry and a Part B entry, the following is needed: - The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted). - FFS: other information is not precluded (e.g., if Part B is valid for a duration) Note: Purpose such as “training data collection” will not necessarily be specified in RAN1 specifications. RAN1 send an LS to RAN3 and RAN2 to inform the above.

However, this agreement has not yet been reflected in the relevant specifications. Since this operation involves the interaction between the gNB and the LMF (via NRPPa signaling), it is necessary for RAN3 to incorporate it into TS 38.455.

Capture the following RAN1 agreement in TS 38.455.

Agreement RAN1#120 - For training data collection of AI/ML based positioning 3a/3b, if Part A and Part B are generated by different entities, for pairing between a Part A entry and a Part B entry, the following is needed: - The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted). - FFS: other information is not precluded (e.g., if Part B is valid for a duration) Note: Purpose such as “training data collection” will not necessarily be specified in RAN1 specifications. RAN1 send an LS to RAN3 and RAN2 to inform the above.

A TP for the AI/ML BLCR to TS 38.455 is provided in Annex 1.

- Conclusion

In this contribution, we discussed the issues of AI/ML for positioning accuracy enhancement. Proposals are summarized as following:

Proposal 1:	Capture following agreement in TS38.455.

Agreement RAN1#120 - For training data collection of AI/ML based positioning 3a/3b, if Part A and Part B are generated by different entities, for pairing between a Part A entry and a Part B entry, the following is needed: - The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted). - FFS: other information is not precluded (e.g., if Part B is valid for a duration) Note: Purpose such as “training data collection” will not necessarily be specified in RAN1 specifications. RAN1 send an LS to RAN3 and RAN2 to inform the above.

# References

- RAN1 Chairman’s notes, 3GPP RAN1 #116b

- RAN1 Chairman’s notes, 3GPP RAN1 #120

# Annex 1 – TP for the AI/ML BLCR to TS 38.455

<<<<<<<<<<<<<<<<<< Start of the Changes >>>>>>>>>>>>>>>>>>

### 9.2.107	Positioning Data Information

This information element contains positioning information for data collection.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| TRP Measurement List |  | 0..1 |  | Indicates the area of interest. |
| >TRP Measurement Item |  | 1..<maxnoofMeasTRPs> |  |  |
| >>TRP ID | M |  | 9.2.24 |  |
| >>UL RTOA | O |  | 9.2.39 |  |
| >>gNB Rx-Tx Time Difference | O |  | 9.2.40 |  |
| >>LoS/NLoS Information | O |  | 9.2.77 |  |
| >>Time Stamp | M |  | 9.2.42 | Used to assist in the pairing with the TRP's measurement. |
| >>Measurement Quality | O |  | 9.2.43 |  |

<<<<<<<<<<<<<<<<<< End of the Change >>>>>>>>>>>>>>>>>>