3GPP TSG-RAN WG3 Meeting #129bis	                               R3-256836

Prague, Czech Republic, October 13 - October 17, 2025

Agenda item:	14.2

Source:	CMCC

Title:	Discussion on Topology 2 for A-IoT

Document for:	Discussion

# Introduction

In revised WID for A-IoT, the following objectives are listed [1]:

- Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]: - According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769. - No specification support for CW control - According to D2T2-C for active Device(s). - Start after March 2026 - Traffic types DO-DTT and DT, and DO-A (only for active Device(s)). - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026 - Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command). - FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum. - Specify RRC-based architecture, in coordination with SA2. - The following are not supported: - Coordination between UEs operating as UE readers - New Uu protocol layer(s) - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band This objective begins after RAN#109 (September 2025), and after RAN1#111 (March 2026) for RAN1. The work is subject to the following constraints: RAN3 - Specify signaling for Topology 2 - The objective includes specification of UE reader authorization, including F1AP support, and UE reader selection. - NOTE: F1AP/XnAP/NGAP impact is expected to be minimized. - Inter-gNB resource coordination is not specified. - Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility

In this contribution, we would like to provide our views on the issues related to A-IoT Topology 2.

# Discussion

## 2.1 Relationship with R19 Topology 1

In Release 19, A-IoT topology 1 has been specified to support A-IoT. When it starts specifying topology 2 in R20, some clarification is necessary on the relationship between topology 1 and topology 2. To be specific, the objective of R19 A-IoT is towards D1T1, where indoor A-IoT devices communicate with an indoor base station. In R20, D2T2-B scenario, where UE-reader assists the indoor A-IoT device to communicate with outdoor base station, is going to be specified. Therefore, from RAN3 perspective, it is obvious that the A-IoT enabled gNBs for R19 and those for R20 are not co-located. In other words, we do not need to consider the scenario where the same gNB supports both Topology 1 and Topology 2 for device 1.

Observation 1: The objective of R19 A-IoT is towards D1T1, where indoor A-IoT devices communicate with an indoor base station. In R20, D2T2-B scenario, where UE-reader assists the indoor A-IoT device to communicate with outdoor base station, is going to be specified.

Proposal 1: The scenario where the same gNB supports both Topology 1 and Topology 2 for device 1 is not considered in R20.

## 2.2 UE-reader authorization

In order to identity UEs which support A-IoT, UE-reader authorization information need to be made available in NG-RAN. The authorization provided to gNB can assistant gNB to select UE-reader and control how to manage the resource allocation.

The UE-reader authorization procedure is performed in the registration, service request, HO, etc. AMF should include the UE-reader authorization information for A-IoT to NG-RAN during initial context setup, UE context modification, handover preparation and path switch procedures. Source/Old gNB should include the UE-reader authorization information for A-IoT to NG-RAN to the target/new gNB during Handover preparation or Retrieve UE Context procedures. In case of split gNB architecture, gNB-CU should include the UE-reader authorization information for A-IoT to gNB-DU during UE Context setup and UE context setup modification procedures.

Observation 2: UE-reader authorization information for A-IoT need to be made available in NG-RAN in order to support UE-reader selection and A-IoT radio resource control in NG-RAN.

Proposal 2: AMF should include the UE-reader authorization information for A-IoT to NG-RAN during initial context setup, UE context modification, handover preparation and path switch procedures.

Proposal 3: Source/Old gNB should include the UE-reader authorization information for A-IoT to NG-RAN to the target/new gNB during Handover preparation or Retrieve UE Context procedures.

Proposal 4: In case of split gNB architecture, gNB-CU should include the UE-reader authorization information for A-IoT to gNB-DU during UE Context setup and UE context setup modification procedures.

Proposal 5: Define a new IE “5G UE-reader Authorized” to indicate whether the UE is authorized to act as an UE-reader.

Proposal 6: Introduce the “5G UE-reader Authorized” IE in the following messages:

- NG: INITIAL CONTEXT SETUP REQUEST

- NG: UE CONTEXT MODIFICATION REQUEST

- NG: HANDOVER REQUEST

- NG: PATH SWITCH REQUEST ACKNOWLEDGE

- Xn: HANDOVER REQUEST

- Xn: RETRIEVE UE CONTEXT RESPONSE

- F1: UE CONTEXT SETUP REQUEST

- F1: UE CONTEXT MODIFICATION REQUEST

## 2.3 Selection of UE-reader

According to the final TS 38.300 CR, the reader selection related information within inventory procedure is described as following [2]:

The Inventory Request message also includes Requested Service Area Information (list of A-IoT Area IDs and/or reader list). If the Requested Service Area Information contains: - neither the Requested AIoT Area List nor the Requested Reader List, the gNB shall select all readers it serves - the Requested AIoT Area List, the gNB shall select readers within the indicated AIoT Area(s) - the Requested Reader List, the gNB shall take the reader list into account to perform inventory.

It is clear that both CN and gNB are involved in the reader selection in R19. As for the selection of UE-reader, it is assumed by RAN2 in TR 38.769 that [3]:

For the UE reader selection, it is up to RAN3 and SA2 on whether BS or CN selects the UE reader(s) and what information is shared between BS and CN. From RAN2 perspective, it is assumed, at least for RRC based solution, there may be radio related reasons for the BS to be involved in the UE reader selection.

From our perspective, since gNB is aware of the resource usage of the A-IoT radio interface and approximate position of the UE-reader (e.g., under which beam direction), core network can provide some assistance information to the gNB to assist UE-reader selection. RAN3 can discuss the details of UE-reader selection under several potential cases: no assistance information, or assistance information with area information, or assistance information with UE reader information. For example, we think that the CN sends the list of UEs that have been authorized and permitted to act as readers to the gNB; then, the gNB selects a suitable UE-reader from the UE list provided by the CN based on the UEs' approximate locations to perform inventory or command procedure.

Observation 3: Due to the mobility of UE-reader(s), interference coordination between UE-readers and coordination of A-IoT radio interface resources become even more important than that in topology 1.

Proposal 7: RAN3 assumes that gNB is responsible for UE-reader selection.

Proposal 8: In terms of the scheme for UE-reader selection, core network can provide some assistance information to the gNB.

Proposal 9: RAN3 can further discuss the details of UE-reader selection under several potential cases: no assistance information, or assistance information with area information, or assistance information with UE reader information.

## 2.4 Mobility of UE-reader

### 2.4.1 Handover

In legacy NR, the handover procedure includes the preparation and execution phase. The handover preparation phase is used to establish necessary resources in an NG-RAN node for an incoming handover, where the UE context transfer, admission control and resources preparation at the target gNB are performed. In handover execution phase, the RAN node assists the UE in performing the handover, and a path switch is performed after the UE successfully handover to the target gNB.

For the handover procedure of UE-reader, the UE-reader can continue to perform the A-IoT procedure under the control of the network after a successful handover.

In handover preparation phase, for UE-reader which performs A-IoT inventory/command procedure, the A-IoT-related capability/information of the UE-reader (e.g., UE-reader Authorization Status, A-IoT radio interface resource, etc.) should also be transmitted to target gNB, so that the target gNB can perform admission control based on this information. Therefore, we think that the A-IoT-related capability/information of the UE-reader can be defined as UE context and can be transmitted within Handover Request message during handover. Similarly, in terms of split architecture (i.e. F1AP interface), some UE Context Management messages (e.g. UE CONTEXT SETUP REQUEST, UE CONTEXT SETUP RESPONSE, UE CONTEXT RELEASE COMMAND, UE CONTEXT MODIFICATION REQUEST, etc.) may need to be enhanced. In terms of HANDOVER REQUEST ACKNOWLEDGE message, we think that RAN3 is also asked to discuss the potential enhancement(s), such as the A-IoT radio resource configuration and/or other necessary information for A-IoTfrom target gNB.

Proposal 10: From RAN3 perspective, the UE-reader can continue to perform the A-IoT procedure under the control of the network after a successful handover.

Proposal 11: In handover preparation phase, the A-IoT-related capability/information of the UE-reader can be defined as UE context and can be transmitted in Handover Request message. The potential enhancement(s) of HANDOVER REQUEST ACKNOWLEDGE message also need to be discussed.

Proposal 12: In F1AP, RAN3 is asked to discuss which UE Context Management message(s) need to be enhanced to add A-IoT related information.

### 2.4.2 Re-restablishment procedure

In legacy NR, a UE in RRC_CONNECTED may initiate the re-establishment procedure to continue the RRC connection when a failure condition occurs (e.g. radio link failure, reconfiguration failure, integrity check failure, etc.). In the re-establishment procedure, UE context retrieval may be performed between gNBs, and path switch may be performed between the gNB and the CN node.

In A-IoT, the UE-reader in RRC_CONNECTED may also encounter problems such as RLF, reconfiguration failure, etc., thus the legacy re-establishment procedure may need to be enhanced. To be specific, the request for A-IoT-related information can be added to RETRIEVE UE CONTEXT REQUEST message and A-IoT related capability/information of the UE-reader can be added to RETRIEVE UE CONTEXT RESPONSE message.

Observation 4: In A-IoT, the UE-reader in RRC_CONNECTED may encounter problems such as RLF, reconfiguration failure, etc.

Proposal 13: The request for A-IoT related information can be added to RETRIEVE UE CONTEXT REQUEST message and A-IoT-related capability/information of the UE-reader can be added to RETRIEVE UE CONTEXT RESPONSE message.

### 2.4.3 Conditional handover

For CHO, the source gNB may conduct the handover preparation phase with multiple potential target gNBs, and during the handover execution phase, the UE evaluates the CHO criteria by itself and handover to a new cell.

From RAN3 perspective, we do not find much additional work required for supporting UE-reader to perform the Conditional handover, compared to the regular handover procedure. The main work will likely remain with RAN2, such as: whether to support the configuration of A-IoT radio interface resources for UE-reader in the RRCReconfiguration message. So, RAN3 can wait for RAN2 decision on whether UE-reader can support CHO.

Observation 5: From RAN3 perspective, there is not much additional work required for supporting UE-reader to perform the Conditional handover, compared to the regular handover procedure.

Proposal 14: RAN3 can wait for RAN2 decision on whether UE-reader can support CHO.

### 2.4.4 LTM/CLTM

Similar to CHO, if UE-reader supports intra-gNB LTM and/or inter-gNB LTM and/or conditional LTM, the main work lies with RAN2. Therefore, we can wait for RAN2 decision on whether UE-reader can support intra-gNB LTM and/or inter-gNB LTM and/or conditional LTM.

Proposal 15: RAN3 can wait for RAN2 decision on whether UE-reader can support intra-gNB LTM and/or inter-gNB LTM and/or conditional LTM.

# Conclusions

Based on the discussions mentioned above, in this contribution we provide some discussions on A-IoT Topology 2:

Observation 1: The objective of R19 A-IoT is towards D1T1, where indoor A-IoT devices communicate with an indoor base station. In R20, D2T2-B scenario, where UE-reader assists the indoor A-IoT device to communicate with outdoor base station, is going to be specified.

Observation 2: UE-reader authorization information for A-IoT need to be made available in NG-RAN in order to support UE-reader selection and A-IoT radio resource control in NG-RAN.

Observation 3: Due to the mobility of UE-reader(s), interference coordination between UE-readers and coordination of A-IoT radio interface resources become even more important than that in topology 1.

Observation 4: In A-IoT, the UE-reader in RRC_CONNECTED may encounter problems such as RLF, reconfiguration failure, etc.

Observation 5: From RAN3 perspective, there is not much additional work required for supporting UE-reader to perform the Conditional handover, compared to the regular handover procedure.

Proposal 1: The scenario where the same gNB supports both Topology 1 and Topology 2 for device 1 is not considered in R20.

Proposal 2: AMF should include the UE-reader authorization information for A-IoT to NG-RAN during initial context setup, UE context modification, handover preparation and path switch procedures.

Proposal 3: Source/Old gNB should include the UE-reader authorization information for A-IoT to NG-RAN to the target/new gNB during Handover preparation or Retrieve UE Context procedures.

Proposal 4: In case of split gNB architecture, gNB-CU should include the UE-reader authorization information for A-IoT to gNB-DU during UE Context setup and UE context setup modification procedures.

Proposal 5: Define a new IE “5G UE-reader Authorized” to indicate whether the UE is authorized to act as an UE-reader.

Proposal 6: Introduce the “5G UE-reader Authorized” IE in the following messages:

- NG: INITIAL CONTEXT SETUP REQUEST

- NG: UE CONTEXT MODIFICATION REQUEST

- NG: HANDOVER REQUEST

- NG: PATH SWITCH REQUEST ACKNOWLEDGE

- Xn: HANDOVER REQUEST

- Xn: RETRIEVE UE CONTEXT RESPONSE

- F1: UE CONTEXT SETUP REQUEST

- F1: UE CONTEXT MODIFICATION REQUEST

Proposal 7: RAN3 assumes that gNB is responsible for UE-reader selection.

Proposal 8: In terms of the scheme for UE-reader selection, core network can provide some assistance information to the gNB.

Proposal 9: RAN3 can further discuss the details of UE-reader selection under several potential cases: no assistance information, or assistance information with area information, or assistance information with UE reader information.

Proposal 10: From RAN3 perspective, the UE-reader can continue to perform the A-IoT procedure under the control of the network after a successful handover.

Proposal 11: In handover preparation phase, the A-IoT-related capability/information of the UE-reader can be defined as UE context and can be transmitted in Handover Request message. The potential enhancement(s) of HANDOVER REQUEST ACKNOWLEDGE message also need to be discussed.

Proposal 12: In F1AP, RAN3 is asked to discuss which UE Context Management message(s) need to be enhanced to add A-IoT related information.

Proposal 13: The request for A-IoT related information can be added to RETRIEVE UE CONTEXT REQUEST message and A-IoT-related capability/information of the UE-reader can be added to RETRIEVE UE CONTEXT RESPONSE message.

Proposal 14: RAN3 can wait for RAN2 decision on whether UE-reader can support CHO.

Proposal 15: RAN3 can wait for RAN2 decision on whether UE-reader can support intra-gNB LTM and/or inter-gNB LTM and/or conditional LTM.

# References

- RP-252894, Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2, RAN#109.

- R3-255964, Introduction of Ambient IoT, RAN3#129.

- TR 38.769, Study on solutions for Ambient IoT (Internet of Things) in NR (Release 19), V19.0.0.