3GPP TSG-RAN WG3 #129bis                                                 R3-256862

Prague, CZ, 13th – 17th Oct, 2025

Agenda Item:	12.2.2

Source:	CATT

Title:	Discussion on support of AI/ML enabled intra-CU LTM

Document for:	Discussion

Introduction

According to the WI [1], the following objective is captured:

The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows:

-	Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios:

-	multi-hop UE trajectory across gNBs;

-	intra-CU LTM;

-	handover enhancements, e.g. inter-CU LTM.

Here we provide some analysis on AI/ML based intra-CU LTM.

Discussion

  - Description and objectives

In Release 18, AI/ML-based Mobility Optimization has been specified to improve network performance and user experience for legacy handover. For L1/L2 Triggered Mobility, handover decision is based on L1/L2 measurement configuration and reporting, additional information is necessary for AI-enabled RAN to optimize LTM.

-	Efficient early TA acquisition;

LTM support early TA acquisition procedure, aiming to enable the UE performs a RACH-less LTM, and decrease handover interruption. Currently early TA acquisition procedure is triggered by source DU by implementation. However, each time UE execute early TA, a short interruption will happen at source node. To avoid unnecessary interruption, when to trigger the TA acquisition and the candidate cells to be triggered early TA shall be considered carefully. Besides, when source DU received the TA, it responsible for maintaining TA validity, how to effective maintain it is not discussed in LTM part. AI/ML model could be introduced to optimize TA acquisition procedure and guarantee provides valid TA value for LTM.

-	Beam level UE Location prediction;

UE Location Prediction has been supported in R18 mobility optimization. While for LTM, TCI state is included in cell switch command which impacts LTM performance. AI/ML model can make prediction on beam level to facilitate better beam selection.

-	Optimization on candidate cells selection;

LTM candidate cell is limited at 8. Unsuitable candidate selection may lead to complex interaction and failure HO. For example, if one of the requested candidate cell reject the request at the preparation phase, the CU shall update CSI configuration to all the other candidate cells to update the configured CSI report configuration. What’s more, once a configured candidate cell needs to be released, the CU also need to trigger a modification procedure to all other candidate cells, and resend RRCReConfiguration to UE, which should be avoided in LTM procedure. Currently, the candidate cell selection is only based on L3 measurement, suggest to using AI/ML model for better candidate cell selection.

-	Optimization on execution conditions;

Similar as R18 mobility optimization case, mobility aspects of SON can be enhanced by the use of AI/ML in case of LTM.

Proposal 1: Based on R18 AI/ML-based Mobility Optimization, following objective for LTM may be achieved by AI/ML model:

-	Efficient early TA acquisition;

-	Beam level UE Location prediction;

-	Optimization on candidate cells selection;

-	Optimization on execution conditions;

  - Input/output/feedback of AI/ML Model Inference

To complete the above objectives, besides legacy input in R18 AI/ML-based Mobility Optimization, gNB may need following new information as input.

From the local node:

-	beam level UE trajectory prediction;

-	Configuration on triggering TA acquisition procedure (e.g., target cell ID, triggering time or period, RACH information);

Beam level UE trajectory prediction can assist gNB to select suitable TCI state included in cell switch command.

Information on TA acquisition procedure is used to optimize TA configuration to provide valid TA for LTM.

From the UE:

-	UE beam level location information

-	UE Mobility History Information with beam level

-	UE L1 and/or L3 measurement report

From neighbouring NG-RAN Nodes:

-	beam level UE’s history information from neighbour

-	TA information from neighbour after triggering TA acquisition procedure.

-	UE LTM result in past that were successful and unsuccessful

Beam level UE’s history information assists UE trajectory (beam) prediction.

TA information is used to optimize TA acquisition procedure.

AI/ML-based LTM model can generate following information as output:

-	beam level UE trajectory prediction

-	suggested candidate cells list

-	Predicted L1 execution condition, i.e. handover trigger points.

-	TA acquisition configuration (e.g.,target cell ID, triggering time or period, RACH information)

-	TA validity results

Following feedback can be considered to be collected from NG-RAN nodes:

-	beam level UE trajectory information;

-	RACH-less or RACH based access in target cell.

-	TA value used during RACH to target cell.

Feedback information from target NG-RAN is used by AI/ML Model to retrain or revise the Inference for mobility UE.

Proposal 2: It is proposed for RAN3 to discuss the above new Input/output/feedback of AI/ML Model Inference on the bases of R18 AI/ML-based Mobility Optimization.

  - Location of AI/ML Model Training and Model Inference

Here we discuss the location of AI/ML Model Training and Model Inference.

Handover decision and TA acquisition is the function of NG-RAN, similar as R18 AI/ML-based Mobility Optimization, so, AI/ML Model Inference could be located in gNB.

As for AI/ML Model training, either OAM or gNB is feasible which is aligned with legacy solution.

Proposal 3: As for the location of AI/ML Model Training and Model Inference, the following solutions can be considered:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, AI/ML Model Inference could be located in either gNB-CU-CP or gNB-DU.

Following is the benefit for AI/ML Model Inference deployed in gNB-DU:

- gNB-DU is responsible for triggering LTM execution and TA acquisition procedure. AI/ML Model Inference deployed in gNB-DU can predict LTM trigger point, TCI state, TA acquisition configuration.

- L1/L2 measure report is transferred to gNB-DU directly. Since the date volume of L1/L2 measure report may be very large, it is not proposed to transfer L1/L2 measure report from DU to CU.

While AI/ML Model Inference deployed in gNB-CU-CP can receive resource status information from neighbour gNB.

So, AI/ML Model Inference should be located in DU to support optimization on LTM triggering point and TA acquisition. And AI/ML Model Inference deployed in CU is to support Legacy candidate cell selection which is the same as legacy CHO.

As for AI/ML Model training, it could be located in OAM or gNB as discussed before. In case of deployed in gNB, AI/ML Model Training should be located in gNB-CU-CP. After AI/ML Model Training at gNB-CU-CP or OAM, it can be deployed to gNB-DU. It may be not suitable for each gNB-DU to make its own AI/ML Model Training because of low efficiency.

Proposal 4: In case of CU-DU split architecture, the following solutions can be considered for supporting AI/ML-based mobility:

-	AI/ML Model Inference is located in gNB-DU to support optimization on LTM configuration (handover trigger point, TCI state, etc.) and TA acquisition configuration.

-	AI/ML Model Inference is located in gNB-CU to support candidate cell selection.

-	AI/ML Model Training is located in the gNB-CU or OAM.

Conclusions

Proposal 1: Based on R18 AI/ML-based Mobility Optimization, following objective for LTM may be achieved by AI/ML model:

-	Efficient early TA acquisition;

-	Beam level UE Location prediction;

-	Optimization on candidate cells selection;

-	Optimization on execution conditions;

Proposal 2: It is proposed for RAN3 to discuss the above new Input/output/feedback of AI/ML Model Inference on the bases of R18 AI/ML-based Mobility Optimization.

Proposal 3: As for the location of AI/ML Model Training and Model Inference, the following solutions can be considered:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

Proposal 4: In case of CU-DU split architecture, the following solutions can be considered for supporting AI/ML-based mobility:

-	AI/ML Model Inference is located in gNB-DU to support optimization on LTM configuration (handover trigger point, TCI state, etc.) and TA acquisition configuration.

-	AI/ML Model Inference is located in gNB-CU to support candidate cell selection.

-	AI/ML Model Training is located in the gNB-CU or OAM.

Reference

[1] RP-252867 “Revised SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3” 3GPP TSG RAN Meeting #109