3GPP TSG-RAN WG3 Meeting #129bis	R3-256895

# Prague, CZ, 13 – 17 Oct 2025

Agenda item:	13.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on architecture for ISAC

Document for:	Discussion and Decision

# 1	Introduction

The ISAC SID ([1]) agrees following RAN3 related objectives:

Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

This contribution discusses the architecture for ISAC.

# 2	Design principles

This is the 1st release to support the ISAC. When RAN3 design the architecture in Rel-20, it is necessary to consider the future evolvement. Current SID only considers gNB-based mono-static sensing. The architecture needs to consider the future release that UE can be used as Sensing Entity, and bi/multi-static sensing using UE/gNB as sensing entity. So we propose the architecture should be future proof.

Same as other interfaces developed by RAN3, the interface with the gNB for sensing operation shall be Open.

Current gNB uses NGAP/SCTP for communication with CN CP nodes. No matter whether the direct or indirect interface is used between SF and SE, the NGAP/SCTP shall be reused for the control plane signalling to/from the gNB.

Proposal 1: Capture the following principles in the TR

- the ISAC RAN architecture should be future proof.

- The ISAC interface with the gNB shall be open.

- The interface with the gNB shall be based on NGAP/SCTP.

# 3	Overall ISAC architecture

SA2 is discussing the ISAC architecture, and will have the conclusion before the RAN3 complete the analysis. The RAN3 discussion shall align with SA2 architecture decision.

To some extent, the sensing operation is similar to positioning operation, i.e. the CN node request the selected gNB/TRP to perform measurement, then the gNB report the result to the CN node. So it is straightforward that the overall architecture/interface for sensing is similar to the architecture/interface for positioning in the RAN side. More specifically,

- The Sensing procedure is designed as an Application Protocol, similar to NRPPa that is transferred between the CN node and gNB.

- The sensing AP message can be encapsulated in a NGAP message communicated between the gNB and the AMF.

- For sensing configuration/control, the Sensing Function initiate a request to the AMF, which is further routed to the related gNB.

- For sensing measurement report, the gNB report the sensing measurement to AMF, which is further routed to the Sensing Function.

In addition, this architecture is more future proof, when the further release supports the UE as Sensing Entity and the AMF involvement is unavoidable. This architecture can also avoid the SF to setup/maintain the many SCTP/TNL connections with the gNBs. An overall architecture for ISAC is described below.

Figure 1: Overall Sensing Architecture applicable to gNB

Proposal 2-1: Agree the overall architecture in the TR.

## 3.1	Interface between SF and SE

For a typical sensing operation, the SF request the SE to start a sensing operation. The request includes the related configuration, e.g. the target sensing area, the type of sensing service (e.g. Object Detection, Object Tracking etc.), the type of event reporting (e.g. One time, Periodic, Event Triggered, etc.), object definition/description, etc. Using an indirect interface via the AMF (i.e. SF send the sensing request to AMF, then the AMF send the sensing request to the selected SE) can avoid the SF to setup/maintain the many SCTP/TNL connections with the SEs/gNBs. The AMF route the Sensing message received from the SF to the related SE/gNB. The SF performs the AMF discovery and selection as other CN nodes. An example call flow is shown below.

Figure 2: Example for a Sensing operation

Proposal 2-2: Agree the above sensing operation figure in the TR.

## 3.2	How to report sensing data to SF

In Positioning operation, the gNB reports the measurement to the CN node via Control Plane. We see no reason why Sensing cannot use similar way that gNB reports the sensing measurement result via Control Plane. Using the Control Plane to report the sensing measurement result is better than using User Plane.

- Control Plane Traffic is usually treated with higher priority than the User Plane Traffic. This is beneficial when sensing operation is performed in certain mission critical scenario, e.g. to detect an intruder UAV.

- Control Plane transport ensures lossless transfer. User Plane does not guarantee lossless transfer.

Proposal 2-3: using Control Plane, e.g. a Sensing Application Protocol, to report the sensing result to SF.

## 3.3	gNB capability awareness in the SF

The SF need to be aware of the sensing capability (including the serving area) of the gNB. There are two options that SF can be aware of the sensing capability:

- Via signaling

This has been used in positioning. The TRP Information Exchange procedure is to allow the LMF to request the NG-RAN node to provide detailed information for TRPs hosted by the NG-RAN node.

- Via OAM

This has been used in the recent Ambient IoT, where the AIOTF is aware of the serving area of the gNB, the reader’s location, the mapping relationship among gNBs, readers and A-IoT areas by means of OAM configuration.

Both options are feasible. The final solution may depend on the detailed capability information of the gNB that needs to be known in the SF. In case the required information is limited and static, the OAM based solution can be used. Otherwise, the signaling-based solution may be better. So it may be better that RAN3 first discuss the detailed capability information of the gNB that needs to be known in the SF.

Observation: both signaling-based solution and OAM-based solution are feasible. The final solution depends on the list of gNB capability information that needs to be known by the SF.

Proposal 2-4: RAN3 need to first discuss the list of gNB capability information that needs to be known by the SF.

# 4	Conclusion

In this contribution, we discussed the high-level issues for ISAC. Our proposals are:

Proposal 1: Capture the following principles in the TR

- the ISAC RAN architecture should be future proof.

- The ISAC interface with the gNB shall be open.

- The interface with the gNB shall be based on NGAP/SCTP.

Proposal 2-1: Agree the overall architecture in the TR.

Proposal 2-2: Agree the above sensing operation figure in the TR.

Proposal 2-3: using Control Plane, e.g. a Sensing Application Protocol, to report the sensing result to SF.

Proposal 2-4: RAN3 need to first discuss the list of gNB capability information that needs to be known by the SF.

# References

- RP-251861, New SID: Study on Integrated Sensing And Communication (ISAC) for NR

- TR 23.700-14