3GPP TSG-RAN WG3 Meeting #129-bis	R3-256907

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	10.3.2

Source: 	Lenovo

Title: 	6G RAN-CN Interface Options

Document for:	Discussion and Approval

1. Introduction

A new study on 6G radio was approved at the RAN#108 meeting and then revised at the RAN#109 meeting with the following objectives [1].

The study aims to develop one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements as defined during the RAN requirements study [TR38.914]. The study should strive at dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, focusing on practical user experience. The study should identify principles to ensure extensibility and deliver superior performance. NOTE: Section 4.1.1 shows interim milestones to reach certain agreements, including early consensus items. From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz). NOTE: TR 38.921, TR 38.922 will be taken into account. NOTE: Frequency ranges beyond 52.6 GHz are not in scope of the work. The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion. - Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1] - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1] - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4] - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2] - MIMO operation [RAN1, RAN4] - Duplexing [RAN1, RAN4] - Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] - 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4] - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1]. - RAN4 can be involved, if necessary, based on the LS from RAN1 - Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3] - Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3] - 6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles - UE RF requirement aspects including band and band combination [RAN4] - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination - Study UE RF capabilities considering different device types and implementations - RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles - Demodulation and performance aspects [RAN4] - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS - Aspects related to the testability [RAN4] - Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity - Other aspects - Handling irregular channel bandwidths including the definition [RAN4, RAN1] - Definition of ‘frequency range(s)’[RAN4] - Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2] - Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4] RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary - AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML - Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914] - PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4] - Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1] - Aspects of integration with communication services [RAN1] - higher layer procedures and protocol aspects [RAN2] - RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4] Note: RAN1 identify detailed requirements, if triggered by TSG RAN NOTE: Coordination with TSG-SA and SA WGs will be done, as necessary. 4.1.1 Interim Milestone Interim results shall be delivered as per the milestones below, in coordination with the Rel-20 RAN Plenary 6G Study (FS_6G_RAN_Scen_Req). TSG#112 (June/2026): RAN1 to provide interim assessment on the following areas: - Waveform, modulation, channel coding: scope of enhancements beyond NR baseline ((2) a, c) - Channel bandwidth (min and max), frame structure, numerology ((2) b, d) - Basic sync signal structure and associated periodicity(ies) ((2) h) For objectives where RAN4 may be impacted, RAN1 shall coordinate with RAN4 early to enable the above assessment by June 2026. RAN3 to provide interim study results to allow TSGs to make a decision on: - RAN-CN interface: P2P vs SBI - RAN internal interfaces: CU-DU split, CP-UP split. NOTE: It is planned to decide on Release-21 timeline in June/2026. TSG#113 (September/2026): RAN plenary to make a decision on additional migration option(s) (other than standalone, MRSS, and inter-RAT mobility between NR-6G). This includes decision on additional 6G-6G aggregation beyond 6G CA: 6G-6G DC. RAN plenary will task relevant RAN WGs for any specific technical analysis, as needed.

In this contribution, we discuss the 6G RAN-CN interface options.

2. Discussion

The 5G RAN-CN interface is designed as a point-to-point interface, where all UE related signalling, e.g., registration, connection, mobility, etc., are handled in a single entity (AMF in 5GC) between the RAN and CN. This approach guarantees a clear separation of concerns between the RAN and CN and reduces the security complexity.

The point-to-point interface between the NR and 5GC showed its flexibility and multi-vendor support. Motivated by the relatively recent uptake by many operators of 5GC, it is reasonable to assume that the 6GC is an evolution of the 5GC to reduce the investments. In line with the assumption, for fundamental functions that is relevant to at least eMBB services, e.g., interface management, UE context management, UE mobility management, and data transmission, the 6G RAN-CN interface can be the evolution of the interfaces related to the 5G RAN, and there is no strong requirement to change the functionality assignment between RAN and CN. Therefore, for the control plane, the 6GR communicates directly with the 6GC (e.g., AMF like function in the 6GC) over a point-to-point interface.

With respect to the user plane, GTP/UDP has provided to be sufficient and efficient in previous generations. Therefore, it should be adopted in the user plane interface between the 6G RAN and 6GC.

In summary, for fundamental functions that is relevant to at least eMBB services, e.g., interface management, UE context management, UE mobility management, and PDU packet transmission,

-	The 6G RAN is connected to the 6GC via an NG-C like interface for control plane, e.g., by means of the evolution of SCTP based NG-C interface.

-	The 6G RAN is connected to the 6GC via an NG-U like interface for user plane, e.g., by means of the evolution of GTP-U based NG-U interface.

Proposal 1: To support the fundamental eMBB services, point-to-point interface is considered as the option for signaling/messaging between 6G RAN and 6GC.

-	The 6G RAN is connected to the 6GC via an NG-C like interface for control plane, e.g., by means of the evolution of SCTP based NG-C interface.

-	The 6G RAN is connected to the 6GC via an NG-U like interface for user plane, e.g., by means of the evolution of GTP-U based NG-U interface.

The 6G network are expected to include new service types beyond traditional communication, such as data collection, sensing and AI. The characteristics of those new services are quite different from legacy functions considering:

- Less time/robustness critical compared to regular UL/DL UE data traffic, e.g., the data collection for AI model training can be conducted in a best effort manner.

- Decoupled with the regular UL/DL UE data traffic (e.g., audio/video), e.g., support of the new services shall not impact the legacy UL/DL communication.

- The consumer of the service could be variant, i.e., more than 6G AMF, e.g., NWDAF and SMF could benefit from the RAN side performance status.

- Extensible and cloud-native, e.g., the implementation of those services has to deal with various of sub use cases and transfer/handling of large amount of data

As a result, more demands on network capability exposure will be required, while the complexity for configuration and internal function interaction should be reduced. A potential approach is to adopt a HTTPs-based service-based interface (SBI) between the RAN and CN for the new functions supporting the new service types, which can avoid too many P2P interface definitions and unnecessary transparent forwarding among network entities, thus enable efficient coordination between the RAN and CN. In addition, the RAN can more directly expose information to the CN.

Therefore, we propose to consider the service-based interface between the RAN and CN to facilitate the exchange of RAN related information for 6G new services, e.g., AI, sensing.

Proposal 2: To support new services (e.g., data collection, AI and sensing) with the following characteristics, RAN3 considers HTPPs-based SBI as a feasible option for signalling/messaging between 6G RAN and 6GC:

- Less time/robustness critical compared to regular UL/DL UE data traffic

- Decoupled from the regular UL/DL UE data traffic (e.g., audio/video)

- The consumer of the service could be variant

- Extensible and cloud-native

An example of the RAN-CN interface is shown as below, where the new function communicates with the 6GC via the service-based interface while the 6G BS communicates with the 6GC via the point-to-point interface for legacy service.

NOTE: the new function may be an internal function provided by the 6G BS.

NOTE: the new function may be provided by a RAN node different from the 6G BS.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals.

Proposal 1: To support the fundamental eMBB services, point-to-point interface is considered as the option for signaling/messaging between 6G RAN and 6GC.

-	The 6G RAN is connected to the 6GC via an NG-C like interface for control plane, e.g., by means of the evolution of SCTP based NG-C interface.

-	The 6G RAN is connected to the 6GC via an NG-U like interface for user plane, e.g., by means of the evolution of GTP-U based NG-U interface.

Proposal 2: To support new services (e.g., data collection, AI and sensing) with the following characteristics, RAN3 considers HTPPs-based SBI as a feasible option for signalling/messaging between 6G RAN and 6GC:

- Less time/robustness critical compared to regular UL/DL UE data traffic

- Decoupled from the regular UL/DL UE data traffic (e.g., audio/video)

- The consumer of the service could be variant

- Extensible and cloud-native

4. Reference

- RP-252912, Revised SID: Study on 6G Radio.

- TR 38.801: Study on new radio access technology: radio access architecture and interfaces.