3GPP TSG-RAN WG3 Meeting #129bis	R3-256911

Prague, CZ, 13 - 17 October 2025

Agenda item:	 11.2.2

Source:		Lenovo

Title:		(TP for TS38.300) Discussion on MRO for intra-CU CLTM

Document for:		Discussion and Decision

1	Introduction

It was agreed to support MRO in R20 [1]:

The objective of this work item is to specify data collection enhancement in NR standalone for SON/MDT purpose. The specific objectives of this work item are:

- MRO enhancement for R19 mobility mechanisms, including inter-CU LTM and intra-CU conditional LTM [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements on the existing interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

In this paper, we would discuss MRO for intra-CU CLTM.

2	Discussion

Intra-CU CLTM meachnism is supported in R19, wherenin source gNB sends an RRCReconfiguration message to the UE including the CLTM configurations of candidate cells as well as the execution condition for the CLTM, and the UE starts evaluating the execution conditions based on the provided configuration, when the CLTM execution condition for the candidate LTM cell is satisfied, the UE performs the CLTM switch by applying the configuration of the satisfied LTM candidate cell.

Similar as intra-CU/inter-CU LTM, early UL synchronization is also supported in CLTM. Since there is no LTM cell switch command MAC CE for CLTM, the source gNB can provide the TA value for each candidate cells the UE has performed UL synchronization with via an LTM Candidate Timing Advance Command MAC CE. CLTM execution can be RACH-based or RACH-less, e.g. if the UE has valid TA when CLTM execution is triggered (e.g. CLTM execution condition is satisfied), the UE skips RACH, otherwise, RACH is performed as part of the CLTM switch.

Both L1 and L3 based intra-CU CLTM are supported in R19,

- For L1 based intra-CU CLTM, events e.g. CondEvent LTM2/LTM3/LTM4/LTM5 based on beam specific quality of serving cell and/or candidate cells are supported, DU generates the L1 based intra-CU CLTM execution conditions based on the request from CU;

- For L3 based intra-CU CLTM, events e.g. CondEvent A3/A5 based on cell specific quality of serving cell and/or candidate cells are supported similar as legacy CHO, CU generates the L3 based intra-CU CLTM execution conditions.

In R20, we should support MRO for both L1 and L3 based intra-CU CLTM.

Proposal 1: MRO for both L1 and L3 based intra-CU CLTM should be supported.

Similar as MRO for intra-CU MCG LTM in R19, failure and near failure are also two normal and important cases in intra-CU CLTM, MRO for both failure and near failure cases in intra-CU CLTM should be supported in R20.

Proposal 2: MRO for both failure and near failure cases in intra-CU CLTM should be supported.

  - 2.1 MRO for connection failure case in intra-CU CLTM

In a CLTM procedure, a CLTM execution failure (i.e. HOF) or RLF may happen. For MRO for intra-CU LTM in R19, three connection failures including Too Late LTM, Too Early LTM and LTM to Wrong Cell are supported, similarily in R20, Too Late CLTM, Too Early CLTM and CLTM to Wrong Cell should also be supported for intra-CU CLTM as following:

- For too late CLTM, the following sub-cases are considered:

  - Case 1a: the UE detects RLF in source cell after receiving CLTM related configurations and prior to CLTM execution, and then performs re-establishment procedure.

  - Case 1b: the UE detects RLF in source cell after receiving CLTM related configurations and prior to CLTM execution, and then selects an CLTM candidate cell, detects HOF with the selected CLTM candidate cell during CLTM recovery.

  - Case 1c: the UE detects RLF in source cell after receiving CLTM related configurations and prior to CLTM execution, and then successfully completes CLTM execution with the selected CLTM candidate cell during CLTM recovery.

- For too early CLTM, the following sub-cases are considered:

  - Case 2a: the UE detects HOF/RLF in the CLTM target cell and performs re-establishment procedure with the source cell.

  - Case 2b: the UE detects HOF/RLF in the CLTM target cell, selects the source cell which is also an CLTM candidate cell, detects HOF with the source cell, and performs re-establishment procedure.

  - Case 2c: the UE detects HOF/RLF in the CLTM target cell, and successfully completes CLTM execution with the selected source cell which is also an CLTM candidate cell.

- For CLTM to wrong cell, the following sub-cases are considered:

  - Case 3a: the UE detects HOF/RLF in the CLTM target cell and performs re-establishment procedure in a cell other than the source cell and the target cell.

  - Case 3b: the UE detects HOF/RLF in the CLTM target cell, selects an CLTM candidate cell which is different from the source or target one, detects HOF with the selected CLTM candidate cell, and performs re-establishment procedure.

  - Case 3c: the UE detects HOF/RLF in the CLTM target cell, and successfully completes CLTM execution with the selected CLTM candidate cell which is different from the source or target one.

Proposal 3: Too Late CLTM, Too Early CLTM and CLTM to Wrong Cell should be supported for intra-CU CLTM as following:

For too late CLTM, the following sub-cases are considered:

  - Case 1a: the UE detects RLF in source cell after receiving CLTM related configurations and prior to CLTM execution, and then performs re-establishment procedure.

  - Case 1b: the UE detects RLF in source cell after receiving CLTM related configurations and prior to CLTM execution, and then selects an CLTM candidate cell, detects HOF with the selected CLTM candidate cell during CLTM recovery.

  - Case 1c: the UE detects RLF in source cell after receiving CLTM related configurations and prior to CLTM execution, and then successfully completes CLTM execution with the selected CLTM candidate cell during CLTM recovery.

- For too early CLTM, the following sub-cases are considered:

  - Case 2a: the UE detects HOF/RLF in the CLTM target cell and performs re-establishment procedure with the source cell.

  - Case 2b: the UE detects HOF/RLF in the CLTM target cell, selects the source cell which is also an CLTM candidate cell, detects HOF with the source cell, and performs re-establishment procedure.

  - Case 2c: the UE detects HOF/RLF in the CLTM target cell, and successfully completes CLTM execution with the selected source cell which is also an CLTM candidate cell.

- For CLTM to wrong cell, the following sub-cases are considered:

  - Case 3a: the UE detects HOF/RLF in the CLTM target cell and performs re-establishment procedure in a cell other than the source cell and the target cell.

  - Case 3b: the UE detects HOF/RLF in the CLTM target cell, selects an CLTM candidate cell which is different from the source or target one, detects HOF with the selected CLTM candidate cell, and performs re-establishment procedure.

  - Case 3c: the UE detects HOF/RLF in the CLTM target cell, and successfully completes CLTM execution with the selected CLTM candidate cell which is different from the source or target one.

Additionally in stage 2, the existing connection failure definition and failure detection mechanism of Too Late LTM, Too Early LTM and LTM to Wrong Cell captured in TS38.300 can be reused for intra-CU CLTM with necessary updates as shown in the Annex.

Proposal 4: The existing connection failure definition and failure detection mechanism of Too Late LTM, Too Early LTM and LTM to Wrong Cell captured in TS38.300 can be reused for intra-CU CLTM with necessary updates.

Similar as intra-CU LTM, the failure occuring in intra-CU CLTM may be due to wrong selection of CLTM candidate cell, e.g. the cell for re-establishment is not one of at least one CLTM candidate cell selected by CU for CLTM preparation. For failures due to wrong selection of CLTM candidate cell, CU performs analysis and MRO optimization.

Proposal 5: The connection failure due to wrong selection of CLTM candidate cell should be considered for MRO for intra-CU CLTM, e.g. CU performs analysis and MRO optimization.

On the other hand, the failure occuring in intra-CU CLTM may be due to inappropriate CLTM execution condition, e.g. the cell for re-establishment or CLTM failure recovery is one of the at least one CLTM candidate cell selected by CU. Concerning failure detection and optimization,

- for failures due to inappropriate CLTM execution condition in L3 based intra-CU CLTM, CU performs analysis and MRO optimization;

- for failures due to inappropriate CLTM execution condition in L1 based intra-CU CLTM, CU initiates analysis and forwards the RLF report to the DU responsible for the failure (e.g. target DU or candidate DU which generates the inappropriate CLTM execution condition), then, the relevant DU performs MRO optimization.

Proposal 6: The connection failure due to inappropriate CLTM execution condition should be considered for MRO for intra-CU CLTM, e.g.

- for failures due to inappropriate CLTM execution condition in L3 based intra-CU CLTM, CU performs analysis and MRO optimization;

- for failures due to inappropriate CLTM execution condition in L1 based intra-CU CLTM, CU initiates analysis and forwards the RLF report to the DU responsible for the failure (e.g. target DU or candidate DU which generates the inappropriate CLTM execution condition), and the relevant DU then performs MRO optimization.

  - 2.2 MRO for near failure successful case in intra-CU CLTM

In R18 intra-CU LTM, fallback from RACH-less to RACH based LTM is not supported. For example, when receiving the LTM cell switch command MAC CE which includes TA value for early UL synchronization, UE starts the LTM TA validity timer and T304 timer, and performs RACH-less LTM as the TA value is valid.  If RACH-less LTM execution fails (e.g. LTM TA validity timer expires) but T304 is still running, the UE will wait T304 to expiry and then perform RRC re-establishment or RACH-based LTM failure recovery procedure.

However, in intra-CU CLTM, since the TA value for early UL synchronization is provided to the UE via an LTM Candidate Timing Advance Command MAC CE, it is possible that CLTM TAT timer starts before T304 timer starts. Fallback from RACH-less to RACH based CLTM is supported, e.g. UE has valid TA value of target cell at CLTM execution, UE performs RACH-less CLTM execution with target cell, but CLTM TAT timer associated with target cell expires during RACH-less CLTM execution, while T304 is still running, then UE will fallback to RACH based CLTM execution. This fallback happens due to the early TA value is outdated. Since CLTM aims at reducing mobility interruption time, fallback from RACH-less to RACH based CLTM is against this purpose. Even fallback from RACH-less to RACH based CLTM is successful, this may be considered as a near failure successful case.

Proposal 7: Consider successful fallback from RACH-less to RACH based CLTM as a near failure successful case for MRO.

Regarding SHR for intra-CU CLTM, similar as legacy, it may be forwarded to the node(s) which configured the SHR trigger causing the SHR to be generated, by using the ACCESS AND MOBILITY INDICATION message over Xn or by means of the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG. Upon receiving of an SHR, the node may analyse whether its mobility configuration needs adjustment.

Proposal 8: The legacy SHR forwarding mechanism in Xn/NG interface can be taken as baseline for intra-CU CLTM.

Similarly, the legacy SHR forwarding mechanism in F1 interface can be taken as baseline for intra-CU CLTM, e.g. SHR may be forwarded by gNB-CU to gNB-DU using the ACCESS AND MOBILITY INDICATION message, then the gNB-DU may take it into account for optimisation of mobility parameters.

Proposal 9: The legacy SHR forwarding mechanism in F1 interface can be taken as baseline for inter-CU MCG LTM.

3	Conclusion

In this paper, MRO for intra-CU CLTM is discussed. We have the following proposals:

Proposal 1: MRO for both L1 and L3 based intra-CU CLTM should be supported.

Proposal 2: I MRO for both failure and near failure cases in intra-CU CLTM should be supported.

Proposal 3: Too Late CLTM, Too Early CLTM and CLTM to Wrong Cell should be supported for intra-CU CLTM as following:

- For too late CLTM, the following sub-cases are considered:

  - Case 1a: the UE detects RLF in source cell after receiving CLTM related configurations and prior to CLTM execution, and then performs re-establishment procedure.

  - Case 1b: the UE detects RLF in source cell after receiving CLTM related configurations and prior to CLTM execution, and then selects an CLTM candidate cell, detects HOF with the selected CLTM candidate cell during CLTM recovery.

  - Case 1c: the UE detects RLF in source cell after receiving CLTM related configurations and prior to CLTM execution, and then successfully completes CLTM execution with the selected CLTM candidate cell during CLTM recovery.

- For too early CLTM, the following sub-cases are considered:

  - Case 2a: the UE detects HOF/RLF in the CLTM target cell and performs re-establishment procedure with the source cell.

  - Case 2b: the UE detects HOF/RLF in the CLTM target cell, selects the source cell which is also an CLTM candidate cell, detects HOF with the source cell, and performs re-establishment procedure.

  - Case 2c: the UE detects HOF/RLF in the CLTM target cell, and successfully completes CLTM execution with the selected source cell which is also an CLTM candidate cell.

- For CLTM to wrong cell, the following sub-cases are considered:

  - Case 3a: the UE detects HOF/RLF in the CLTM target cell and performs re-establishment procedure in a cell other than the source cell and the target cell.

  - Case 3b: the UE detects HOF/RLF in the CLTM target cell, selects an CLTM candidate cell which is different from the source or target one, detects HOF with the selected CLTM candidate cell, and performs re-establishment procedure.

  - Case 3c: the UE detects HOF/RLF in the CLTM target cell, and successfully completes CLTM execution with the selected CLTM candidate cell which is different from the source or target one.

Proposal 4: The existing connection failure definition and failure detection mechanism of Too Late LTM, Too Early LTM and LTM to Wrong Cell captured in TS38.300 can be reused for intra-CU CLTM with necessary updates.

Proposal 5: The connection failure due to wrong selection of CLTM candidate cell should be considered for MRO for intra-CU CLTM, e.g. CU performs analysis and MRO optimization.

Proposal 6: The connection failure due to inappropriate CLTM execution condition should be considered for MRO for intra-CU CLTM, e.g.

- for failures due to inappropriate CLTM execution condition in L3 based intra-CU CLTM, CU performs analysis and MRO optimization;

- for failures due to inappropriate CLTM execution condition in L1 based intra-CU CLTM, CU initiates analysis and forwards the RLF report to the DU responsible for the failure (e.g. target DU or candidate DU which generates the inappropriate CLTM execution condition), and the relevant DU then performs MRO optimization.

Proposal 7: Consider successful fallback from RACH-less to RACH based CLTM as a near failure successful case for MRO.

Proposal 8: The legacy SHR forwarding mechanism in Xn/NG interface can be taken as baseline for intra-CU CLTM.

Proposal 9: The legacy SHR forwarding mechanism in F1 interface can be taken as baseline for inter-CU MCG LTM.

4 Reference

[1] RP-252560, Revised WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR Phase 5, China Unicom

Annex: TP for TS38.300

##### 15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of LTM cell switch, the "successful handover" in the definitions above refers to a successful completion of the RA procedure in case of RACH-based LTM or the successful reception of RRCReconfigurationComplete in case of RACH-less LTM.

In case of CLTM, the "successful handover" in the definitions above refers to a successful completion of the RA procedure in case of RACH-based CLTM or the successful reception of RRCReconfigurationComplete in case of RACH-less CLTM.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell.

In case of CHO with candidate SCG(s), the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO with candidate SCG(s), Too Early CHO with candidate SCG and CHO Execution to Wrong Cell.

In case of CLTM, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CLTM, Too Early CLTM and CLTM to Wrong Cell.

Detection mechanism

A failure indication may be initiated after a UE attempts to re-establish the radio link connection at NG-RAN node B after a failure at NG-RAN node A. NG-RAN node B may initiate the Failure Indication procedure towards multiple NG-RAN nodes if they control cells which use the PCI signalled by the UE during the re-establishment procedure. The NG-RAN node receiving this selects the UE context that matches the received Failure Cell ID and C-RNTI, and, if available, uses the shortMAC-I to confirm this identification, by calculating the shortMAC-I and comparing it to the received IE.

A failure indication may also be sent to the node last serving the UE when the NG-RAN node fetches the RLF REPORT from UE by triggering:

-	The Failure Indication procedure over Xn;

-	The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-	Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO/LTM/CLTM or CHO with candidate SCG(s) is configured but the CHO/LTM/CLTM execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).

-	Intra-system Too Early Handover: there is a recent handover/LTM cell switch/CLTM cell switch for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell/the cell UE attempts LTM/CLTM recovery is the cell that served the UE at the last handover/LTM/CLTM initialisation or fall back to the source cell configuration in case of DAPS HO.

-	Intra-system Handover to Wrong Cell: there is a recent handover/LTM cell switch/CLTM cell switch for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery/the cell UE attempts LTM/CLTM recovery is neither the cell that served the UE at the last handover/LTM/CLTM initialisation nor the cell that served the UE where the RLF happened or the cell that the handover/LTM/CLTM was initialized toward.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO/LTM/CLTM execution until connection failure.