3GPP TSG-RAN WG3 Meeting #129bis	R3-256973

Prague, Czech Republic, 13th – 17th October, 2025

Title: 	Further discussion on per-UE UE performance in AI/ML for NG-RAN

Source: 	Huawei, FiberCop, BT, Jio Platforms

Agenda item:	8.1

Document Type:	Discussion and Decision

# 1. Introduction

In the context of the Rel-18 “Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” RAN3 introduced in TS 38.423 [1] a set of metrics representing the per-UE UE performance that could be requested by the source gNB to be reported by the target gNB via the Data Collection Reporting (Initiation) procedures after an AI/ML-driven mobility decision.

UE performance metrics, i.e., Average UE Throughput DL/UL, Average Packet Delay DL/UL and Average Packet Loss DL, are intended to be collected by the target gNB on a per-UE basis.

However, there is no clear understanding on how a gNB could derive such per-UE metrics, and for that reason RAN3 initiated an LS exchange with SA5 in order to check whether existing measurements in TS 28.558 [2] could accommodate the RAN3 requirements, or new measurements need to be specified.

This contribution discusses the impacts on RAN3 specifications pertaining the latest reply LS from SA5 in [3] and provides corresponding Rel-18 and Rel-19 CRs to impacted specifications.

# 2. Discussion

In the latest version of Rel-18 TS 38.423 (V18.6.0 – June 2025) the UE Performance IE is defined as follows:

#### 9.2.3.179 UE Performance This IE indicates the UE performance measurements metrics. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Average UE Throughput DL; O; Bit Rate 9.2.3.4; Corresponds to Average UE Throughput DL per-UE as specified in TS 28.558 [58] clause 6.3.1.4.1.. Average UE Throughput UL; O; Bit Rate 9.2.3.4; Corresponds to Average UE Throughput UL per-UE as specified in TS 28.558 [58] clause 6.3.1.4.2.. Average Packet Delay; O; 9.2.3.187. Average Packet Loss DL; O; Packet Loss Rate 9.2.3.11. a

Back in past RAN3 meetings it was brought up to the attention of the companies that there is the need to ensure that, for a UE subject to an AI/ML-driven mobility decision, the UE performance metrics are derived consistently in both the source gNB and target gNB, so to enable the possibility to actually compare UE performance before and after the mobility action. Therefore, RAN3 started discussing how to fulfil such requirement, and the outcome of such discussion was essentially the investigation of UE-level measurements defined by SA5 in TS 28.558 which, in turn, resulted in requesting clarifications to SA5 on existing measurements in TS 28.558 as well as asking for the introduction of new measurements.

In the last LS sent by RAN3 in [4], RAN3 asked clarifications specifically for the Average Packet Delay DL/UL and Average Packet Loss DL (and UL) measurements, in particular:

- whether per-DRB per-UE aggregation of packet delay measurements is already supported or needs to be specified (if feasible);

- whether packet loss measurements could be specified with a per-DRB per-UE granularity in the UL and DL at PDCP layer, that could also be aggregated to reflect a per-UE metric.

In [3] SA5 replied as follows:

- Overall description SA5 would like to thank RAN3 for the reply LS on per-UE UE performance metrics. SA5 has been asked to check the feasibility of the principles agreed by RAN3, to specify a solution that reflects such principles (if feasible) and to provide feedback to RAN3. RAN3: For Average packet delay measurements: RAN3 would like to ask SA5 whether the measurements are aggregated per-DRB per-UE to reflect corresponding per-UE metrics. If such per UE aggregation is not supported and if such UE aggregation is feasible, RAN3 would like to ask SA5 to specify these measurements. SA5’s answer: SA5 confirms that RAN3’s assumption is correct and the measurements defined in TS 28.558 clause 6.3.1.1 are aggregated per-DRB per-UE to reflect corresponding per-UE metrics. For Average packet loss measurements: RAN3 would like to further ask whether it is feasible to specify the above Packet Loss Rate metrics with a per-DRB per-UE granularity in the UL and DL at PDCP layer, which may also be aggregated to reflect a per-UE Packet Loss Rate metric. SA5’s answer: SA5 confirms that it is feasible to specify the packet loss/drop measurements with a per-DRB per-UE granularity. SA5 has specified the corresponding packet loss and drop rate measurements in TS 28.558.

We now discuss the impact on RAN3 specifications according to the reply LS from SA5 in [3].

Rel-18 and Rel-19 XnAP

For the Average Packet Delay, following the same approach agreed in RAN3 for the Average UE Throughput DL/UL – whose semantics description was enhanced by adding references to existing measurements in SA5’s TS 28.558 (clauses 6.3.1.4.1 and 6.3.1.4.2) based on the SA5 confirmation that such measurements can be provided per-UE as explained in by SA5 in [5]), it is proposed to add a semantics description with a reference to the Packet delay measurement as in clause 6.3.1.1 of TS 28.558. Note that a similar semantics description has been already agreed for the Rel-19 XnAP BLCR for the Slice Based UE Performance IE.

Proposal 1: RAN3 to agree to add a reference to clause 6.3.1.1 of TS 28.558 (Packet delay) for the Average Packet Delay IE within the UE Performance IE.

For the average packet loss measurements, instead, SA5 agreed in their last meeting to introduce the following measurements in TS 28.558, starting from Rel-18:

- for non-split gNB deployment scenario: DL PDCP SDU Drop Rate in gNB and UL PDCP SDU Loss Rate

- for split gNB deployment scenario: DL PDCP SDU Drop Rate in gNB-CU-UP (note that the UL PDCP SDU Loss Rate for the split gNB scenario has already been specified in TS 28.558 clause 6.3.1.3.1)

Based on the specification work in SA5, it should be straightforward to also enhance the semantics description of the Average Packet Loss DL IE by adding a reference to the DL PDCP SDU Drop Rate in gNB as specified in TS 28.558 clause 6.3.1.6.1.1.

Proposal 2: RAN3 to agree to add a reference to clause 6.3.1.6.1.1 of TS 28.558 (DL PDCP SDU Drop Rate in gNB) for the Average Packet Loss DL IE within the UE Performance IE.

Still on the Average Packet Loss DL IE, there were previous discussions in RAN3 concerning the current encoding of the IE itself. Different contributions in the past, e.g., [6] detailed the shortcomings of the Rel-18 encoding of the Average Packet Loss DL IE in XnAP, and RAN3 agreed in RAN3#127bis the following [7]:

A Rel18 correction to the Average Packet Loss DL IE may be made when agreements on whether and how to specify Average Packet Loss in Rel19 are taken.

In Rel-19, for the Slice Based UE Performance IE in XnAP (as well as for the UE Performance IE in E1AP), the encoding of the Slice Average Packet Loss DL/UL was agreed to be an INTEGER type, ranging from 0 to 106, with extension mark.

Therefore, we propose to change the encoding of the Rel-18 Average Packet Loss DL IE, aligning it with the encoding used for the Slice Based UE Performance IE in XnAP (as well as for the UE Performance IE in E1AP).

Proposal 3: RAN3 to agree to change the encoding of the Rel-18 Average Packet Loss DL IE within the UE Performance IE, aligning it with the encoding used in Rel-19 for the Slice Based UE Performance IE in XnAP (as well as for the UE Performance IE in E1AP).

Proposal 3 can be implemented by considering either a non-backwards compatible (NBC) change, or in a backwards compatible (BC) manner. The most straightforward approach is a NBC change, which should still be acceptable. We are anyway open to discuss introducing the change in Proposal 3 as a BC change.

Proposal 4: If Proposal 3 is agreed, RAN3 to discuss whether the change of the encoding of the Rel-18 Average Packet Loss DL IE should be NBC or BC.

The above proposals are reflected in the Rel-18 CRs to TS 38.423 in [8] (NBC) and [9] (BC), as well as in the corresponding Rel-19 mirror CRs towards the same specification in [10] (mirror of [8]) and [11] (mirror of [9]).

As part of the discussions on the AI/ML-based Network Slicing use case in Rel-19, RAN3 agreed to introduce the Slice Based UE Performance IE which includes metrics for UE performance to be reported with a per-UE per-slice granularity:

#### 9.2.3.238 Slice Based UE Performance This IE represents UE performance metrics per S-NSSAI. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Slice Average UE Throughput DL; O; Bit Rate 9.2.3.4; Corresponds to Average UE Throughput DL per S-NSSAI as specified in TS 28.558 [58] clause 6.3.1.4.1.. Slice Average UE Throughput UL; O; Bit Rate 9.2.3.4; Corresponds to Average UE Throughput UL per S-NSSAI as specified in TS 28.558 [58] clause 6.3.1.4.2.. Slice Average Packet Delay; O; Average Packet Delay 9.2.3.187; Corresponds to the Average Packet Delay per S-NSSAI as specified in TS 28.558 [58], clause 6.3.1.1.. Slice Average Packet Loss DL; O; INTEGER (0..1000000, ...); Corresponds to DL PDCP SDU Drop Rate per S-NSSAI.. Slice Average Packet Loss UL; O; INTEGER (0..1000000, ...); Corresponds to UL PDCP SDU Loss Rate per S-NSSAI.. a

In our understanding, the feedback from SA5 in their reply LS in [3] on the PDCP-level measurements for the average packet loss DL/UL also applies to the per-UE per-slice granularity of these measurements: in fact, in the description of the new measurements for the packet loss DL/UL agreed to be introduced in TS 28.558 since Rel-18, it is stated that such measurements can also be reported per supported S-NSSAI – refer for instance to the DL PDCP SDU Drop Rate in gNB measurement agreed by SA5 reflected in the latest Rel-18 version of TS 28.558 (V18.4.0 – 2025-09):

#### 6.3.1.6 UE Packet Drop Rate ##### 6.3.1.6.1 Measurements valid for non-split gNB deployment scenario ###### 6.3.1.6.1.1 DL PDCP SDU Drop Rate in gNB *** skip non-relevant parts *** c) This measurement is obtained as: 1000000*Number of dropped DL PDCP SDU packets whose contexts are removed from the gNB without any part of it having been transmitted on the Xn-U or X2-U interface, of a data radio bearer, divided by Number of DL PDCP SDU packets for data radio bearers that have entered PDCP-SAP after being decoded from GTP-U packets. Separate counters are optionally maintained for mapped 5QI (or QCI for EN-DC) and per supported S-NSSAI.

Based on the above, we propose to enhance the semantics description of the Slice Average Packet Loss DL and Slice Average Packet Loss UL IEs included in the Slice Based UE Performance IE by adding a reference to the DL PDCP SDU Drop Rate in gNB and UL PDCP SDU Loss Rate measurements as specified in TS 28.558 clause 6.3.1.6.1.1 and 6.3.1.7.1, respectively.

Proposal 5: RAN3 to agree to add a reference to clauses 6.3.1.6.1.1 and 6.3.1.7.1 of TS 28.558 (DL PDCP SDU Drop Rate in gNB and UL PDCP SDU Loss Rate, respectively) for the Slice Average Packet Loss DL and Slice Average Packet Loss UL IEs included in the Slice Based UE Performance IE.

The above proposal is reflected in the Rel-19 CR to TS 38.423 in [12].

SA5 also agreed to introduce in TS 28.558, since Rel-18, the UL PDCP SDU Loss Rate measurements for both split and non-split gNB scenarios. Therefore, also considering that in Rel-19 E1AP’s UE Performance IE the Average Packet Loss UL measurement has been agreed to be part of UE performance metrics exchanged over the E1 interface, we think that also in Rel-18 such measurement should be added to ensure feature parity between split and non-split architectures.

Proposal 6: RAN3 to agree to introduce, since Rel-18, the Average Packet Loss UL IE in the UE Performance IE, by adding a reference to clause 6.3.1.7.1 of TS 28.558 (UL PDCP SDU Loss Rate) in the corresponding semantics description.

The above Proposal 6 is reflected in the Rel-18 CR to TS 38.423 in [13] and in the corresponding Rel-19 mirror CR towards the same specification in [14].

Rel-19 E1AP

For the split architecture support of AI/ML for NG-RAN discussed in Rel-19 RAN3 agreed to exchange the Average Packet Loss DL/UL and the Average Packet Delay over the E1 interface, from the gNB-CU-UP to the gNB-CU-CP upon request. In Rel-19 E1AP the following has been specified:

#### 9.3.1.149 UE Performance This IE indicates the UE performance measurements metrics. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Average Packet Delay; O; 9.3.1.150. Average Packet Loss DL; O; INTEGER(0..1000000, …); Corresponds to DL PDCP SDU Drop Rate.. Average Packet Loss UL; O; INTEGER(0..1000000, …); Corresponds to UL PDCP SDU Loss Rate.. a

Following the same approach previously illustrated for the XnAP specification, we think that the semantics description of the Average Packet Delay, Average Packet Loss DL and Average Packet Loss UL IEs should be enhanced with proper references to measurements specified by SA5 in the Rel-19 version of TS 28.558. Specifically:

- Average Packet Delay: it should refer to the Packet delay measurement in clause 6.3.1.1 of TS 28.558;

- Average Packet Loss DL: it should refer to the DL PDCP SDU Drop Rate in gNB-CU-UP measurement in clause 6.3.1.6.2.1 of TS 28.558;

- Average Packet Loss UL: it should refer to the UL PDCP SDU Loss Rate measurement in clause 6.3.1.3.1	of TS 28.558.

Proposal 7: RAN3 to add references to clauses 6.3.1.6.2.1 and 6.3.1.3.1 of TS 28.558 for the Average Packet Loss DL and Average Packet Loss UL IEs, respectively, included in the E1AP’s UE Performance IE.

The above Proposal 7 is reflected in the Rel-19 CR to TS 37.483 in [15].

# 3. Conclusion

Based on the above discussion, we have the following observation and proposals:

Proposal 1: RAN3 to agree to add a reference to clause 6.3.1.1 of TS 28.558 (Packet delay) for the Average Packet Delay IE within the UE Performance IE.

Proposal 2: RAN3 to agree to add a reference to clause 6.3.1.6.1.1 of TS 28.558 (DL PDCP SDU Drop Rate in gNB) for the Average Packet Loss DL IE within the UE Performance IE.

Proposal 3: RAN3 to agree to change the encoding of the Rel-18 Average Packet Loss DL IE within the UE Performance IE, aligning it with the encoding used in Rel-19 for the Slice Based UE Performance IE in XnAP (as well as for the UE Performance IE in E1AP).

Proposal 4: If Proposal 3 is agreed, RAN3 to discuss whether the change of the encoding of the Rel-18 Average Packet Loss DL IE should be NBC or BC.

The above proposals are reflected in the Rel-18 CRs to TS 38.423 in R3-256974 (NBC) and R3-256975 (BC), as well as in the corresponding Rel-19 mirror CRs towards the same specification in R3-256976 (mirror of R3-256974) and R3-256977 (mirror of R3-256975).

Proposal 5: RAN3 to agree to add a reference to clauses 6.3.1.6.1.1 and 6.3.1.7.1 of TS 28.558 (DL PDCP SDU Drop Rate in gNB and UL PDCP SDU Loss Rate, respectively) for the Slice Average Packet Loss DL and Slice Average Packet Loss UL IEs included in the Slice Based UE Performance IE.

The above proposal is reflected in the Rel-19 CR to TS 38.423 in R3-256978.

Proposal 6: RAN3 to agree to introduce, since Rel-18, the Average Packet Loss UL IE in the UE Performance IE, by adding a reference to clause 6.3.1.7.1 of TS 28.558 (UL PDCP SDU Loss Rate) in the corresponding semantics description.

The above Proposal 6 is reflected in the Rel-18 CR to TS 38.423 in R3-256979 and in the corresponding Rel-19 mirror CR towards the same specification in R3-256980.

Proposal 7: RAN3 to add references to clauses 6.3.1.6.2.1 and 6.3.1.3.1 of TS 28.558 for the Average Packet Loss DL and Average Packet Loss UL IEs, respectively, included in the E1AP’s UE Performance IE.

The above Proposal 7 is reflected in the Rel-19 CR to TS 37.483 in R3-256981.

# 4. Reference

- TS 38.423, NG-RAN; Xn Application Protocol (XnAP)

- TS 28.558, Management and orchestration; UE level measurements for 5G system

- R3-256518 (S5-253854), Reply LS on per-UE UE performance metrics (Response to R3-253816/S5-253295), Rel-18, SA5 (Huawei)

- R3-253816, Reply LS on per-UE UE performance metrics (Response to R3-253023 / S5-252056), Rel-18, RAN3 (Nokia)

- R3-253023 (S5-252056), Reply LS on per-UE UE performance metrics (Response to R3-247775), Rel-19, SA5 (NEC, Intel, Ericsson)

- R3-255460, On packet loss metric in UE performance, Ericsson, Deutsche Telekom, Jio Platforms

- 3GPP RAN3#127bis Chairman’s notes (RAN3_127bis_agenda_20250415_EOM1.doc)

- R3-256974, Correction on Average Packet Delay and Average Packet Loss DL – Option 1, Rel-18 CR to TS 38.423, Cat. F, Huawei, FiberCop, BT, Jio Platforms

- R3-256975, Correction on Average Packet Delay and Average Packet Loss DL – Option 2, Rel-18 CR to TS 38.423, Cat. F, Huawei, FiberCop, BT, Jio Platforms

- R3-256976, Correction on Average Packet Delay and Average Packet Loss DL – Option 1, Rel-19 CR to TS 38.423, Cat. A, Huawei, FiberCop, BT, Jio Platforms

- R3-256977, Correction on Average Packet Delay and Average Packet Loss DL – Option 2, Rel-19 CR to TS 38.423, Cat. A, Huawei, FiberCop, BT, Jio Platforms

- R3-256978, Correction on Slice UE Performance, Rel-19 CR to TS 38.423, Cat. F, Huawei, FiberCop, BT, Jio Platforms

- R3-256979, Addition of the Average Packet Loss UL measurement as UE Performance, Rel-18 CR to TS 38.423, Cat. C, Huawei, FiberCop, BT

- R3-256980, Addition of the Average Packet Loss UL measurement as UE Performance, Rel-19 CR to TS 38.423, Cat. A, Huawei, FiberCop, BT

- R3-256981, Correction on Average Packet Delay, Average Packet Loss DL and Average Packet Loss UL, Rel-19 CR to TS 37.483, Cat. F, Huawei, FiberCop, BT, Jio Platforms