3GPP TSG-RAN WG3 #129-bis																	R3-257031

Prague, Czech Republic, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0498 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on SL Multi-hop |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay_multihop-Core |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For SL multi-hop, the preparation of Uu relay RLC channels and PC5 relay RLC channels may take place before the interaction between the gNB and the AMF. However, the current procedural text does not specify the exact step at which this occurs, which may lead to mistake regarding the timing of the establishing the channels. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 8.19.5, add “i.e., after step 27” for NOTE 2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity remains for SL multi-hop. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.19.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

### 8.19.5	Remote UE initial access for Multi-hop Layer-2 UE-to-Network Relay

The signalling flow for Remote UE Initial access is shown in Figure 8.19.5-1.

Figure 8.19.5-1: Overall procedure for Remote UE’s initial access via multi-hop relay

1.	The U2N Remote UE, the First U2N Relay UE, the Intermediate U2N Relay UE, and the Last U2N Relay UE perform discovery procedure, and establish PC5 connection using NR ProSe procedure.

2.	The U2N Remote UE sends an RRCSetupRequest message to the First U2N Relay UE via PC5 Relay RLC channel.

3.	The First U2N Relay UE withholds the received RRC message. If the First U2N Relay UE is in RRC_IDLE/RRC_INACTIVE state, it should send its own RRCSetupRequest message to the Intermediate U2N Relay UE via PC5 Relay RLC channel in order to trigger the RRC establishment/resume procedure to enter RRC_CONNECTED state upon reception of the RRC message from the U2N Remote UE. If the Intermediate U2N Relay UE is in RRC_IDLE/RRC_INACTIVE state, it should trigger the RRC establishment/resume procedure in clause 8.19.1 to enter RRC_CONNECTED state upon reception of the RRC message from the First U2N Relay UE. If all Relay UEs are in RRC_CONNECTED state, this step could be skipped.

4.	The First U2N Relay UE in RRC_CONNECTED state sends the SidelinkUEInformationNR message to the gNB-DU via the Intermediate U2N Relay UE and Last U2N Relay UE.

5.	The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the First U2N Relay UE by encapsulating the SidelinkUEInformationNR message to gNB-CU, and gNB-CU allocates the local ID of U2N Remote UE to uniquely identify the U2N Remote UE within the Last U2N Relay UE.

6.	The gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message of the Last U2N Relay UE to gNB-DU. Such message may request the establishment of Uu Relay RLC channel(s) and PC5 Relay RLC channel(s) for the transmission of U2N Remote UE’s SRB0.

7.	The gNB-DU sends the UE CONTEXT MODIFICATION RESPONSE message of the Last U2N Relay UE to gNB-CU.

8.	The gNB-CU sends the DL RRC MESSAGE TRANSFER message of the Last U2N Relay UE to gNB-DU by encapsulating the RRCReconfiguration message, which contains the local ID allocated to the U2N Remote UE. The RRCReconfiguration message shall also contain the Uu Relay RLC channel(s) configuration and PC5 Relay RLC channel(s) configuration if not configured and bearer mapping for relaying of U2N Remote UE’s SRB0.

9.	The gNB-DU sends the RRCReconfiguration message to the Last U2N Relay UE to configure the local ID of the U2N Remote UE, the Uu Relay RLC channel(s) configuration, PC5 Relay RLC channel(s) configuration and bearer mapping for relaying of U2N Remote UE’s SRB0.

10.	The Last U2N Relay UE sends the RRCReconfigurationComplete message to gNB-DU.

11.	The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the Last U2N Relay UE by encapsulating the RRCReconfigurationComplete message to gNB-CU.

12.	The gNB-CU configures the Intermediate U2N Relay UE with the local ID allocated to the U2N Remote UE, PC5 Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB0. According to the configuration from gNB-CU, the Intermediate U2N Relay UE may establish a PC5 Relay RLC channel for relaying of U2N Remote UE’s SRB0 over PC5. This step follows the same signaling flow as described in steps 6-11.

13.	The gNB-CU configures the First U2N Relay UE with the local ID allocated to the U2N Remote UE, PC5 Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB0. According to the configuration from gNB-CU, the First U2N Relay UE may establish a PC5 Relay RLC channel for relaying of U2N Remote UE’s SRB0 over PC5. This step follows the same signaling flow as described in steps 6-11.

14.	After receiving the local ID of the U2N Remote UE and the PC5 Relay RLC channel(s) configuration and bearer mapping for relaying of U2N Remote UE’s SRB0, the First U2N Relay UE sends the RRCSetupRequest message of the U2N Remote UE to gNB-DU via the Intermediate U2N Relay UE and the Last U2N Relay UE. The local ID of the U2N Remote UE and RB ID for SRB0 are conveyed in the SRAP header.

15.	The gNB-DU allocates a C-RNTI and a gNB-DU UE F1AP ID for the U2N Remote UE and sends the INITIAL UL RRC MESSAGE TRANSFER message to gNB-CU by encapsulating the RRCSetupRequest message of the U2N Remote UE. In addition, the local ID of the U2N Remote UE, the gNB-DU UE F1AP ID of the Last U2N Relay UE and the sidelink configuration container for the PC5 Relay RLC channel configuration for relaying of U2N Remote UE’s SRB1 are included in the INITIAL UL RRC MESSAGE TRANSFER message.

16.	The gNB-CU allocates a gNB-CU UE F1AP ID for the U2N Remote UE and generates a RRCSetup message towards the U2N Remote UE. The RRC message is encapsulated in the DL RRC MESSAGE TRANSFER message, and includes the configurations of PC5 Relay RLC channel and bearer mapping at least for the transmission of U2N Remote UE’s SRB1.

17.	The gNB-DU sends the RRCSetup message to the U2N Remote UE via the First U2N Relay UE, the Intermediate U2N Relay UE and Last U2N Relay UE.

18.	The gNB-CU configures the Last U2N Relay UE with PC5 Relay RLC channel, Uu Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB1. According to the configuration from gNB-CU, the Last U2N Relay UE establishes a PC5 Relay RLC channel for relaying of U2N Remote UE’s SRB1 over PC5 and establishes a Uu Relay RLC channel for relaying of U2N Remote UE’s SRB1 towards gNB-DU if not configured yet.

The gNB-CU configures the First U2N Relay UE and the Intermediate U2N Relay UE with PC5 Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB1. According to the configuration from gNB-CU, the First U2N Relay UE and the Intermediate U2N Relay UE establish the PC5 Relay RLC channels for relaying of U2N Remote UE’s SRB1 over PC5 if not configured yet.

NOTE 1: Step 18 can be performed earlier, e.g., via Steps 6-13.

19.	The U2N Remote UE sends the RRCSetupComplete message to the gNB-DU via the First U2N Relay UE, the Intermediate U2N Relay UE and Last U2N Relay UE.

20.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

21.	Upon receiving the RRCSetupComplete message of U2N Remote UE, the gNB-CU sends the INITIAL UE MESSAGE message to the AMF.

22.	The AMF sends the INITIAL CONTEXT SETUP REQUEST message to the gNB-CU.

23.	The gNB-CU sends the UE CONTEXT SETUP REQUEST message to establish the U2N Remote UE context in the gNB-DU. Such message may request the configuration of PC5 Relay RLC channels for the transmission of U2N Remote UE’s SRB2 and DRBs, and may also encapsulate the SecurityModeCommand message.

24.	The gNB-DU sends the SecurityModeCommand message to the U2N Remote UE via the First U2N Relay UE, the Intermediate U2N Relay UE and Last U2N Relay UE.

25.	The gNB-DU sends the UE CONTEXT SETUP RESPONSE message of the U2N Remote UE to the gNB-CU, which contains the configuration of PC5 Relay RLC channels for the transmission of U2N Remote UE’s SRB2 and DRBs.

26.	The U2N Remote UE responds with the SecurityModeComplete message.

27.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

28.	The gNB-CU generates the RRCReconfiguration message for U2N Remote UE and encapsulates it in the DL RRC MESSAGE TRANSFER message. The RRCReconfiguration message contains the configuration of PC5 Relay RLC channels and bearer mapping for the transmission of U2N Remote UE’s SRB2 and DRBs.

29.	The gNB-DU sends RRCReconfiguration message to the U2N Remote UE via the First U2N Relay UE, the Intermediate U2N Relay UE and Last U2N Relay UE.

30.	The U2N Remote UE sends RRCReconfigurationComplete message to the gNB-DU via the First U2N Relay UE, the Intermediate U2N Relay UE and Last U2N Relay UE.

31.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and send it to the gNB-CU.

32.	The gNB-CU sends the INITIAL CONTEXT SETUP RESPONSE message to the AMF.

33.	The gNB-CU configures additional Uu Relay RLC channels between the gNB-DU and the Last U2N Relay UE, and additional PC5 Relay RLC channels for the First U2N Relay UE, the Intermediate U2N Relay UE, and the Last U2N Relay UE for relaying of U2N Remote UE’s DRBs and SRBs. Also, such step may configure the bearer mapping between U2N Remote UE’s DRB/SRB and PC5/Uu Relay RLC channel at the First U2N Relay UE, the Intermediate U2N Relay UE, and the Last U2N Relay UE.

NOTE 2: This step can be performed earlier, i.e., after step 27.

<<<<<<<<<<<<<<<<<<<< End of changes >>>>>>>>>>>>>>>>>>>>