3GPP TSG-RAN WG3 Meeting #129bis	R3-257032

Prague, Czech Republic, 13 – 17 October 2025

Agenda Item:		14.2

Source:		Ofinno

Title:	Consideration on the Impacts of A-IoT Topology 2

Document for:		Discussion

# Introduction

In RAN #108 meeting, the following objectives were approved for on A-IoT Phase 2 WID [1]:

RAN2

- The objective includes specification of necessary Uu procedures to enable the A-IoT operations.

- RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios.

- A-IoT radio resource of the UE reader is only valid within the serving cell.

- Resource allocation to the UE Reader shall be via Uu RRC signaling.

RAN3

- Specify signaling for Topology 2

- The objective includes specification of UE reader authorization, including F1AP support, and UE reader selection.

  - NOTE: F1AP/XnAP/NGAP impact is expected to be minimized.

- Inter-gNB resource coordination is not specified.

- Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility

This contribution discusses the potential impacts of Topology 2 for A-IoT.

# Discussion

Fig. 1. Logical system architecture for topology 2

2.1 UE reader authorization

On this issue, no matter whether RRC based solution or UP based solution is adopted, the A-IoT enbled UE reader should be an authorized UE. Therefore, it’s straightforward that a new IE, shown as follows, should be defined to indicate to A-IoT enabled gNB if an UE is an authorized A-IoT UE Reader. The A-IoT enabled gNB gNB may perform UE reader selection based on the new IE and other assistant information.

#### 9.3.1.xxx	A-IoT UE Reader Authorized

This IE provides information about the authorization status of the A-IoT UE reader.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT UE Reader Authorized | M |  | ENUMERATED (authorized, not authorized, ...) | Indicates the A-IoT UE reader authorization status. |

To support this function in the A-IoT enabled gNB, the following NGAP messages should be enhanced to include the new A-IoT UE Reader Authorized IE.

NGAP:

- INITIAL CONTEXT SETUP REQUEST

- UE CONTEXT MODIFICATION REQUEST

- HANDOVER REQUEST

- PATH SWITCH REQUEST ACKNOWLEDGE

In case of split RAN architecture, gNB-DU should also know if the A-IoT enbled UE reader is authorized so that it determine and perform radio resource allocation based on the indication. Thus, the following F1AP messages should be enhanced to include the new A-IoT UE Reader Authorized IE.

F1AP:

- UE CONTEXT SETUP REQUEST

- UE CONTEXT MODIFICATION REQUEST

Proposal 1: To define a new A-IoT UE Reader Authorized IE and include it in the NGAP, and F1AP messages as shown above.

2.2 Inter-gNB RRC-connected UE Reader Mobility

When a connected mode A-IoT UE reader performs handover, the target A-IoT enabled gNB should also be notified whether the A-IoT enbled UE reader is authorized or not. So, the source A-IoT enabled gNB should also notify the target A-IoT enabled gNB the new A-IoT UE Reader Authorized IE. Thus, the following XnAP messages should be enhanced to include the new A-IoT UE Reader Authorized IE.

XnAP:

- HANDOVER REQUEST

- RETRIEVE UE CONTEXT RESPONSE

Proposal 2: To support RRC-connected UE reader handover, include the new A-IoT UE Reader Authorized IE in the XnAP HANDOVER REQUEST message and RETRIEVE UE CONTEXT RESPONSE message.

An issue exists when a connected mode A-IoT UE reader performs handover, the source A-IoT enabled gNB may not receive the inventory report yet.

On the other hand, a connected mode A-IoT UE reader may suffer from RLF or HOF, how to get the inventory report should also be investigated.

Proposal 3: To investigate the issues on how to receive inventory report when A-IoT UE reader performs handover or suffers from RLF or HOF.

2.3 On how to send inventory report to A-IoTF

In Rel-19, a time interval was defined for A-IoT enabled gNB to refer to and send the inventory report to A-IoTF, as shown below, defined in TS 23.369.

- Time interval: the fixed time interval for which NG-RAN collects multiple AIoT Devices’ operation responses before reporting the aggregated AIoT response to the AIOTF. The reporting based on time interval may potentially happen multiple times until the NG-RAN completes the request operation.

A fixed time interval may be fine for Topology 1. A-IoT enabled gNB may send the inventory report via NG interface, which is wired interface.

However, it’s not enough for Topology 2. In Topology 2, a UE reader is involved for sending the inventory report. A short time interval may force the UE reader to send the inventory report very frequently, which may cause radio resource waste from the UE reader to A-IoT enabled gNB. Within a short time interval, the UE reader may not identify enough number of A-IoT devices. On the other hand, a long time interval may cause infrequent inventory report from UE reader to A-IoTF, which may delay the network process.

Furthermore, in order to send the inventory report by the UE reader, the UE reader needs to receive a UL grant, which may triggers a random access procedure and/or a scheduling report. Thus, the time to send the inventory report may not be aligned with the time interval defined in the core network.

Therefore, an enhancement on how to send inventory report to A-IoTF should be investigated in Rel-20. For example, sending the inventory report to A-IoTF may be based on the inventory status that the UE reader completes the inventory procedure (e.g., the number of A-IoT devices identified by UE reader, a number of times the UE reader sends the paging message/access triggers, etc).

Proposal 4: To investigate an enhancement on how to send inventory report to A-IoTF, e.g., based on the inventory status that the UE reader completes the inventory procedure.

# Proposals

Proposal 1: To define a new A-IoT UE Reader Authorized IE, as shown above, and include it in the following NGAP and F1AP messages:

NGAP:

- INITIAL CONTEXT SETUP REQUEST

- UE CONTEXT MODIFICATION REQUEST

- HANDOVER REQUEST

- PATH SWITCH REQUEST ACKNOWLEDGE

F1AP:

- UE CONTEXT SETUP REQUEST

- UE CONTEXT MODIFICATION REQUEST

Proposal 2: To support RRC-connected UE reader handover, include the new A-IoT UE Reader Authorized IE in the XnAP HANDOVER REQUEST message and RETRIEVE UE CONTEXT RESPONSE message.

Proposal 3: To investigate the issues on how to receive inventory report when A-IoT UE reader performs handover or suffers from RLF or HOF.

Proposal 4: To investigate an enhancement on how to send inventory report to A-IoTF, e.g., based on the inventory status that the UE reader completes the inventory procedure.

# Reference

[1] RP-252894, Solutions for Ambient IoT (Internet of Things) in NR Phase 2 (WID)

[2] R3-256501, RAN3#129-bis Meeting Agenda