3GPP TSG-RAN WG3 Meeting #129-bis	R3-257082

# Prague, Czech Republic, 13 – 17 October 2025

Agenda Item:	10.4

Source:	Ericsson

Title:	About 6G RAN-internal architecture topics

Document for:	Discussion and decision

# 1	Introduction

This paper aims at starting the RAN3 discussion on 6G RAN-internal architecture topics. The objectives of this work have been agreed during RAN#109 in RP-252912 [1]. For RAN3, the objectives which are relevant for this paper are listed as follows:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving: both for network and device.

[…]

- System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

[…]

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

[…]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

[…]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

- Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

[…]

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

[…]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

- Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR:

[…]

- Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

It is also important to note that the following requirements were captured in TS 38.914 [2]:

-	The 6G RAN architecture shall support standalone RAN architecture.

-	The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

-	The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

-	The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

-	The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

-	3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.

-	The 6G RAN architecture shall allow for control plane and user plane separation.

-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.

-	The 6G RAN architecture shall allow for the operation of network slicing.

-	The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

-	The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

-	The 6G RAN architecture shall allow non-public networks.

This paper will therefore consider the above objectives and requirements.

# 2	Discussion

The first objective of the discussion on RAN-internal functional split and interfaces should aim at defining general design principles and identifying the problems that the RAN architecture needs to solve. These principles should aim at supporting a modular design of RAN protocols and functions by means of ensuring interoperable communication between such modules. This would ensure a common understanding in RAN3 about what this new RAN-internal architecture needs to achieve in order to enable efficient interoperability. To do so, it would also be interesting to look at what was done for previous generations (e.g. for NG-RAN), acknowledge the rationale that led to this architecture and reason on the lessons learned along the different releases standardised with this RAN-internal architecture.

This first discussion should lead to a common understanding that will help in agreeing the first general principles for the RAN-internal functional split and the different RAN-internal interfaces.

This ordering is essential to conduct an efficient design of a new RAN-internal architecture that will likely be kept for most of this new generation life cycle, if not for all.

Proposal 1: RAN3 to focus first on general principles for studying standardisation aspects of the 6G RAN-internal architecture.

## 2.1 RAN-internal architecture in NG-RAN

### 2.1.1 Short description of RAN-internal architecture

As a reminder of the 5G RAN-internal fuctional split, the following figures from TS 38.401 can be shown.

Figure 6.1-1: Overall architecture

Figure 6.1.2-1. Overall architecture for separation of gNB-CU-CP and gNB-CU-UP

It shows the different RAN-internal interfaces that were defined in RAN3:

- Xn as a horizontal interface, used mainly for mobility and Dual-Connectivity.

- F1 as the interface for Higher Layer Split.

- E1 as the interface for CP/UP separation.

Release after release, different functions were added, such as NPN, Positioning, CCO, support for XR, AI/ML, etc…

While the introduction of such functionalities was quite straightforward for a non split RAN architecture, extra complexity was incurred when the features had to be adapted to a split architecture.

The intentions of specifying an HLS architecture for 5G were obviously not those of increasing complexity. The HLS was specified to enable an architecture option that would adapt to cloud environments, where virtualisation of compute platforms can be exploited. For this reason, the HLS architecture assumes that the protocols and functionalities in the gNB-CU would be centralised and virtualised, while those in the gNB-DU would be distributed and, likely, not be virtualised.

Observation 1: The objective of specifying an HLS architecture was to enable a deployment scenario for the support of cloud RAN and virtualisation.

### 2.1.2. Lessons learned

#### 2.1.2.1 General

A high level of architecture flexibility is needed for deployment and efficient functionality design. However, with the introduction of a fixed split architecture in 5G, constraints that reduced flexibility were automatically generated and created complex dependencies. Standardising RAN-internal interfaces supporting this architecture flexibility is very challenging due to, among other things, the high level of dependency between different RAN functions.

Many deployment options involving Dual-Connectivity were defined in the first releases of 5G. The majority of them are not implemented, while some of them hold a very limited market share. However, these options (acknowledged in the standard) needed to be taken into account in standardisation during the course of 5G, increasing standard complexity and efficiency. The same options needed to be taken into account for implementation, monopolising unnecessary resources. Such outcome should be avoided for 6G, and this is the reason why the 6G SID mentions the following:

Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting

Observation 2: Standardisation of the 5G RAN introduced architecture options that brought complexity without enabling real benefits and without addressing real multi-vendor interoperability requirements.

3GPP spent entire releases to specify features options for a split RAN architecture, without a much evidence of such features being deployed in a multi-vendor interoperable scenario. Indeed, often, the features specified would consist of compromise designs, which do not ensure interoperability and hinder performance. Some of the challenges encountered when standardising features support for split architectures are:

- Lack of knowledge of RAN nodes capabilities, e.g. a gNB-CU does not know the capabilities of a gNB-DU and vice versa.

- Dependencies on the nodes forming the split architecture on processes that are implementation specific. For example, beam management/shaping, radio resource management, mobility decisions, scheduling. To make a split architecture work optimally, all nodes in such architecture should be fully aware of such processes designs.

- Split of functionalities to handle the UE across multiple logical entities. This implies sub-optimal configurations and increase in latency.

At the beginning of 5G standardisation these drawbacks were not appreciated. They were only discovered during the course of the releases.

Observation 3: Adapting features to a split RAN architecture has resulted in increased standardisation complexity without achieving interoperability.

#### 2.1.2.2 Dual-Connectivity

For Dual-Connectivity, issues were related to dependencies and split configuration selection between MN and SN. It is not possible to determine the overall best MCG/SCG configuration. Scheduling, link adaptation, buffering, and power handling are done per link. To enable a lean, efficient, and fast control plane for 6G, each UE should be controlled by a single RAN control function. From a standardisation point-of-view, DC can be also quite time consuming because 3GPP needs to define coordination procedures between MN and SN for nearly every feature e.g. QoE, IAB, etc... However, this coordination is rarely justified by better performance. The time spent handling the complexity in 3GPP and implementation could be better spent on features with clear benefits for operators.

Also, flow control is too slow to utilise both links fully and, in reality, DC resulted in providing a “switch” on how portions of traffic are handled by each RAN node. Namely, DC does not provide a dynamic resource aggregation functionality aimed at exploiting in a dynamic way resources at different transmission points. Instead, it partitions traffic between different transmission points in a static way, implying that traffic served by each node is still subject to the resource limitations of the node itself..

Observation 4: Dual-connectivity brought complexity by splitting UE control and resources between MN and SN, without delivering a dynamic resource aggregation solution with maximum exploitation of available resources at different transmission points.

#### 2.1.2.3 HLS

With HLS, the UE context and state handling is split between CU and DU which complicates the implementation of optimised radio features and therefore lead to suboptimal performance and increased configuration delays. However, performance should be regarded as a key requirement considering the cost and scarcity of radio spectrum. To emphasise a bit more on this issue, here are examples of the difficulties of separating UE state between different logical entities:

- Selection of overall best cell-group;

- CU-C lacking information on overlapping bands chosen by DU(s);

- latency spikes due to DU drops/ PDPC re-ordering as well as due to F1 propagation delays;

- DU has to send content to the UE all the way via the CU-C, which terminates the RRC;

- Slow adaptation of flow control between CU-U and DU, impliying that bursts of available resources cannot be properly exploited.

The split complicated the decision logic and overall complexity in CU-CP, CU-UP and DU substantially, while leading to worse decisions, delayed actions and increased signalling and processing load. Similar to DC issues listed above, the following also apply to HLS. The scheduling, link adaptation, buffering, and power handling is done per link. To enable a lean, efficient, and fast control plane for 6G, each UE should be controlled by a single RAN control function.

Observation 5: HLS brought complexity by splitting UE Control and traffic handling between gNB-CU-CP, gNB-CU-UP and gNB-DU.

#### 2.1.2.4 Conclusions

For speedy deployment of new features, the internal RAN functional architecture shall remain as simple as possible, as this is the framework within which the most difficult and complex solutions for resource management will have to be deployed, as well as the framework where upgrades to increase performance are expected to be delivered in the fastest and most efficient way. This is why a highly flexible RAN-internal architecture is desired. Therefore, and considering the complexity that a functional split can bring, it is necessary to first define the problems to be solved in order to justify the need, or not, for a given functional split.

Proposal 2: RAN3 to first discuss the problems to be solved by the standard in order to justify the need of a functional split.

One of the key requirements agreed in the 6G RAN study for RAN interfaces is interoperability (“3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability"). The work in RAN3 shall focus on the design and definition of logical entities and interfaces that are essential for inter-vendor interoperability. It is widely acknowledged that the standard should provide design guidelines to enable interoperability between different parts of the 6G system, in the most efficient way. Any standardisation effort towards processes that are not required to be multi-vendor interoperable would simply come as a limitation to the fostering of a competitive market based on innovation and execution speed.

Proposal 3: RAN3 to focus standardisation effort on the design of interfaces and logical entities that are truly needed for inter-vendor interoperability.

## 2.2 General principles for RAN-internal architecture

As discussed at the beginning of this paper, general principles should be discussed at the RAN3 first meeting on 6G. Taking into account the requirements from 6G RAN Requirements TR, the SID objectives, principles that are already acknowledged in RAN, and the lessons learned from previous generations, a first list of principles should be defined.

The first requirement to consider is the following:

The 6G RAN architecture shall support standalone RAN architecture

One of the main design blocks that we think a standalone 6G RAN should support is the ability to maintain functionalities for the configuration and handling of a UE in a single logical entity. In previous sections about lessons learned, we emphasised how complexity increases, when the configuration of a UE has to be negotiated between multiple logical nodes. The result is often a sub-optimal configuration, which impacts the overall performance. Coordination between different logical entities implies delays in exchanging information, processing, and converging to a viable configuration option. Such delays slow down the activation of optimised data transfer options. As a consequence, the following general principle can be derived and should be agreed.

Proposal 4: UE-associated functionalities shall be controlled by a single logical RAN entity.

Another important requirement that was discussed in previous sections concerns flexibility and virtualisation. In order to achieve this goal, a modular design of RAN functions, protocols and interfaces is key. Virtualisation is an increasingly used tool to create additional instances of a function (or module) in a data centre to distribute load across additional CPUs or servers.

A logical node (e.g. neighbour 6G RAN node) that communicates with a 6G RAN node should not be required to know the internals of the 6G RAN node. It should be able to reach the 6G RAN node at its known IP address(es) and port number(s) and to have a guarantee that the AP message is successfully delivered at the instance of the function where its content is meant to be used. Assuming TNL to provide functions to the application layer creates problems which can only be solved on AP level - end to end (AP worker to AP worker).

Therefore, the standard should be based on a design of functions/protocols and interfaces that is modular, so that such “modules” can be virtualised, instantiated and distributed.

As a consequence, the following general principle can be derived and should be agreed.

Proposal 5: The RAN-internal architecture shall allow modularity at function, protocol and interface procedure level, with the objective to support flexible distribution of 6G RAN processes.

As for other general principles which can also be derived from the objectives and requirements for 6G and can be applied to the 6G RAN-internal architecture, we propose to discuss the following.

The RAN-internal architecture shall:

- Ensure implementation flexibility; -- see previous sections --

- Ensure robustness and resilience of RAN interfaces; -- see RAN TS 38.914 --

- Ensure scalability; -- see previous sections --

- Ensure independence between Application Protocal and transport; -- see previous sections --

- Allow for C-plane/U-plane separation; -- see RAN TS 38.914 --

Proposal 6: Agree on additional general principles for the RAN-internal architecture, taking the above principles as baseline.

### 2.2.1 6G RAN node to to 6G RAN node interface

A “horizontal” interface has always been part of 3GPP RAN generations. This is essential for interoperability between different vendors deployed in different neighboring areas, or deployed in different node sizes (e.g. macro and small cells).

Proposal 7: Agree to standardise a 6G RAN node to 6G RAN node interface

In order to achieve a common understanding of what functionalities should be supported by this new interface, we should initiate discussions on potential additional requirements that could warrant the inclusion of further functionalities that are truly needed for intervendor-interoperability.

We should therefore first focus on the functionalities which are essential for inter-vendor interoperability and needs to exist on the new interface from day one, such as:

- Mobility between 6G RAN nodes

- Interface Management and error handling

Proposal 8: Intra 6G-RAN Mobility and Interface Management are supported by the 6G RAN Node to 6G RAN node interface.

### 2.2.2 About Lower Layer Split

During RAN#109, the following Way Forward was agreed for the LLS in 6G:

3GPP shall capture the following text in italic in a normative Annex of TSxx.401 or TSxx.300 (RAN3):

6G NodeB architecture includes a distinct logical Radio Unit (RU).

The functions hosted by the RU as well as the interface between RU and other parts of 6G NodeB are not specified in 3GPP.

Note-1: This interface can be implemented according to O-RAN ALLIANCE specifications for multi-vendor interoperability.

Note-2: This interface can be implemented according to e.g. (e)CPRI-based or fully proprietary solution(s)

This Way Forward shall also be considered by RAN3 during the Study Item phase, and it should be reflected in the RAN3 TR from the start, in order to guide the discussions on RAN-internal functional split and interfaces.

Proposal 9: Capture the RAN#109 Way Forward on LLS in RAN3 TR.

# 3	Conclusion

The different 6G RAN-internal architecture topics were discussed and the following observations and proposals were made:

Proposal 1: RAN3 to focus first on general principles for studying standardisation aspects of the 6G RAN-internal architecture.

Observation 1: The objective of specifying an HLS architecture was to enable a deployment scenario for the support of cloud RAN and virtualisation.

Observation 2: Standardisation of the 5G RAN introduced architecture options that brought complexity without enabling real benefits and without addressing real multi-vendor interoperability requirements.

Observation 3: Adapting features to a split RAN architecture has resulted in increased standardisation complexity without achieving interoperability.

Observation 4: Dual-connectivity brought complexity by splitting UE control and resources between MN and SN, without delivering a dynamic resource aggregation solution with maximum exploitation of available resources at different transmission points.

Observation 5: HLS brought complexity by splitting UE Control and traffic handling between gNB-CU-CP, gNB-CU-UP and gNB-DU.

Proposal 2: RAN3 to first discuss the problems to be solved by the standard in order to justify the need of a functional split.

Proposal 3: RAN3 to focus standardisation effort on the design of interfaces and logical entities that are truly needed for inter-vendor interoperability.

Proposal 4: UE-associated functionalities shall be controlled by a single logical RAN entity.

Proposal 5: The RAN-internal architecture shall allow modularity at function, protocol and interface procedure level, with the objective to support flexible distribution of 6G RAN processes.

Proposal 6: Agree on additional general principles for the RAN-internal architecture, taking the above principles as baseline.

Proposal 7: Agree to standardise a 6G RAN node to 6G RAN node interface

Proposal 8: Intra 6G-RAN Mobility and Interface Management are supported by the 6G RAN Node to 6G RAN node interface.

Proposal 9: Capture the RAN#109 Way Forward on LLS in RAN3 TR.

# 4	References

[1]	RP-252912 - Revised SID: Study on 6G Radio - NTT DOCOMO, CMCC, AT&T, Vodafone

[2] TS 38.914 - Study on 6G Scenarios and Requirements; (Release 20)