3GPP TSG-RAN WG3 Meeting #129bis	R3-257124

Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	13.3

Source: 	LG Electronics Inc.

Title: 	Discussions on ISAC RAN-CN procedures and signalling

Document for:	Discussion and Decision

# Introduction

Rel-20 ISAC study (5G Advanced) for RAN WGs was agreed at RAN#108 and further revised in RAN#109 [1], with the following objectives:

This study item aims to study the following aspects for Integrated Sensing and Communication (ISAC): Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized. Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

In Rel-19, various ISAC-related use cases were studied by SA1 and captured into TR 22.837 [2], and their corresponding service requirements were specified in TS 22.137 [3]. In parallel, RAN has been conducting studies on channel modeling for ISAC (FS_Sensing_NR, RP-242348 [4]), and SA2 recently initiated a Rel-20 study (FS_Sensing_ARC, SP-250833) to enhance the system architecture and 5GC for sensing, targeting for completion in December 2025.

In line with the SA2 progress, for the network architecture, we have proposed in [5] as follows:

Proposal 1: RAN3 designs the ISAC network architecture and signalling interactions between gNB and 5GC in a unified way that is agnostic to sensing type (e.g. mono-static vs bi-static) or a specific use case. Proposal 2: RAN3 confirms to study that a Sensing Entity (SE) is performed only by a gNB. Proposal 3: RAN3 agrees that, in Rel-20, Sensing Function (SF) shall not be located at gNB, as it may require inter-gNB coordination with another gNB acting as a Sensing Entity (SE). Proposal 4: RAN3 to capture the entity in 5GC that SE (gNB) interacts with, by defining “Sensing CN node” with the following definition (similar to “A-IoT CN node” in Rel-19): - Sensing CN node: The core network node which the gNB connects to 5GC for sensing operation. More specifically, it is the SF (Sensing Function) in case the gNB directly connects with the SF, and it includes both AMF and SF in case the gNB connects with the SF via AMF. Proposal 5: RAN3 to capture the following figure as the baseline design for the sensing service cycle and interactions between gNB and Sensing CN node for Rel-20 ISAC network architecture study. Figure 1: Signalling flow focusing on RAN-CN sensing service cycle (request, execution, report)

In line with these proposals for network architecture, and also considering SA2 progress, in this contribution, we discuss several aspects and principles for designing RAN-CN procedures and signalling for Rel-20 ISAC.

# Discussion

## Support for SE (gNB) discovery and (re)selection

First, one important aspect is how a SE (gNB) is selected to perform sensing executions. This requires that the Sensing CN node be aware of which gNBs can act as SEs and which sensing services they support.

Such sensing capability from RAN is essential for SE discovery and (re)selection, and should be exposed to the Sensing CN node. Moreover, from a RAN-CN signalling standpoint, it may also be closely coupled with how a SE (gNB) is authorized for sensing, all of which are currently under SA2 study by key issue #2, #3 [6]:

## Key Issue #2: Authorization and Revocation to Support Sensing Service The key issue is to address the authorization and revocation for a particular Sensing Service, specifically: - authorization of Sensing Service requests and revocation of an on-going Sensing Service; and - authorization and revocation for the Sensing Entities if applicable. For the authorization of the Sensing Service request, and the revocation of an on-going Sensing Service, the study needs to address the following aspects: - whether and how to perform Sensing Service request authorization if required, and the criteria for the Sensing Service request authorization, based on the requirements from TS 22.137 [2]; - whether and how to perform Sensing Service revocation if required, and the criteria for the revocation, based on the requirements from TS 22.137 [2]; - whether, how and which entity(/entities) performs the Sensing Service request authorization and Sensing Service revocation if required; - whether and where to store the Sensing Service request authorization information and Sensing Service revocation information, and when and how to update the authorization and revocation information if required. For the authorization and revocation for the Sensing Entities if applicable, the study needs to address the following aspects: - whether, when and which entity performs the authorization and revocation of the Sensing Entity(/Entities); - if needed, the granularity of the Sensing Entity authorization, e.g. based on TS 22.137 [2]; - if needed, storage and management of the authorization information for the Sensing Entities. NOTE: The authorization and revocation for the Sensing Entities can be related to the key issue #3 on Sensing Entities discovery and selection. ## Key Issue #3: Sensing Entity and Sensing Function Discovery and (Re-)Selection This key issue aims to study how to perform the Sensing Entity(ies) and Sensing Function(s) discovery and (re-)selection, in particular: - When required, for the discovery and (re-)selection of the Sensing Entity(ies), the study needs to address the following aspects: - Which entity(ies) performs the discovery and (re-)selection of the Sensing Entity(ies); - What the information and/or criteria are taken into account for the discovery and (re-)selection of the Sensing Entity(ies); and - The procedure(s) for the discovery and (re-)selection of the Sensing Entity(ies). - When required, for the discovery and (re-)selection of the Sensing Function(s), the study needs to address the following aspects: - Which entity(ies) performs the discovery and (re-)selection of Sensing Function(s); - What the information and/or criteria are taken into account for the discovery and (re-)selection of Sensing Function(s); and - The procedure(s) for the discovery and (re-)selection of the Sensing Function(s).

Given that this is the first meeting in RAN3 and the SA2 study is scheduled to conclude later this year, there is no need to finalize detailed procedures to support discovery, (re)selection, or authorization (if needed) at this stage. For now, it would be sufficient for RAN3 to agree at a high level that a gNB acting as an SE shall provide the necessary information to the Sensing CN node so it can be discovered and selected for sensing executions. The detailed procedures can be discussed later once SA2 study has progressed further.

Proposal 1: RAN3 to agree that a gNB acting as SE shall provide the necessary information to the Sensing CN node to be discovered and selected for sensing executions, with detailed procedures and information FFS (pending SA2 progress).

## RAN-CN signalling design for sensing service cycle

Another important aspect for discussion is how sensing operations are managed across SE (gNB) and SF (residing at 5GC). According to the definitions from SA2 TR 23.700-14 [6], it is evident that a SF responsible for orchestrating sensing services needs to request sensing executions from the selected SE (gNB) and receives the corresponding reports.

To enable consistent life cycle management, it would be beneficial for RAN3 to introduce a sensing-task abstraction over the RAN-CN procedures for sensing service cycle (request, execution, report). A sensing task can represent a single job (requiring one or more sensing executions) and be uniquely identified by an associated ID (sensing task ID), so that this task ID can serve as the anchor for initiating, modifying, and terminating the task, and also for controlling the associated sensing parameters and reporting between them.

Proposal 2: RAN3 to introduce a sensing “task”, as a basic unit of managing sensing service cycle between SF (residing at 5GC) and SE (gNB), which can be uniquely identifiable by a sensing task ID and can serve as the anchor for initiating, modifying, and terminating the task, and for controlling the associated sensing parameter configuration and reporting between them.

Having the sensing task abstraction in mind, the other aspect is how we design the actual signalling for RAN-CN sensing service cycle. These are related to the step 2 and step 4 signalling designs of the below figure proposed to be captured as basis for RAN-CN sensing service cycle [5]:

Figure 1: Signalling flow focusing on RAN-CN sensing service cycle (request, execution, report)

It is evident that the request message must contain the necessary information required for SE (gNB) to execute a requested sensing task. Needless to say, “configuration” parameters associated with a sensing task requested shall be carried over the request message, and the outcome of sensing executions shall be reported back. Then, how a sensing is requested and what parameters needs to be conveyed, how the outcome should be generated (one time, periodic, or event-triggered, etc.), how the outcome is reported, should they be handled over control-plane or use-plane NG interface, etc. – there are many details that we need to discuss and decide to design those step 2 and step 4 signalling, but we again observe that they are all currently under the SA2 study by the key issues #4, #5 and #6 [6]:

## Key Issue #4: Sensing Data and the Associated Information Collection and Transport Integration of sensing and communication requires 3GPP network to collect and transfer sensing data and the associated information to a Sensing Function. This Key Issue aims to study: - How to collect sensing data (and the associated information, if any) for Sensing result generation? - Which Sensing Function performs Sensing result generation? - Whether and what the above associated information is required for Sensing result generation NOTE: Implications to RAN or RAN dependent aspects will be coordinated with RAN WGs. ## Key Issue #5: Sensing Result Exposure This key issue addresses the exposure of Sensing results to support Sensing Services. The aspects to be studied in this key issue include: - Study what types of Sensing results with or without the Sensing contextual information defined in TS 22.137 [2] could be exposed from the Core Network to the Sensing Service Consumer. - How the Core Network exposes Sensing results to the Sensing Service Consumer (e.g. one time, periodic or event trigger). ## Key Issue #6: Configuration of Parameters for Sensing Entities This Key Issue will study how parameter configuration for Sensing Entities is performed. In particular, this Key Issue will study: - How and what parameters need to be exchanged between the Sensing Entities and the Core Network. NOTE: This KI needs to be aligned with RAN WGs.

It would be premature to start detailing these signalling procedures. We can wait for SA2 to progress.

However, one aspect we can discuss now is the nature of those request and reporting signalling from high-level. We think that a request from the Sensing CN node should be handled as “class-1” nature, meaning that the result of the request (either accepted or rejected by the SE) is immediately notified back to the Sensing CN node. The reason is because, such deterministic and immediate feedback would allow the Sensing CN node to promptly select another SE (gNB) or leverage/modify the existing sensing task without waiting for the sensing request outcome.

On the other hand, the reporting signalling does not seem to require “class-1” handling, as it is essentially a one-way information transfer that does not demand an explicit acknowledgement or real-time feedback. Regardless of whether the reporting is carried over control-plane or user-plane interfaces, one-directional transfer (i.e. no acknowledgement of receipt) is expected to be sufficient.

Proposal 3: For RAN-CN sensing service cycle (request, execution, report) signalling, FFS details (pending SA2 progress). But from a high-level, RAN3 to agree that

- The request signalling is handled as “class-1” so that the result of request (accepted or rejected by gNB) can be notified immediately back to the Sensing CN node.

- The report signalling from gNB is handled as “class-2” (i.e. without acknowledgement).

# Conclusion

Based on the discussion in the present contribution and the observations above we propose:

Proposal 1: RAN3 to agree that a gNB acting as SE shall provide the necessary information to the Sensing CN node to be discovered and selected for sensing executions, with detailed procedures and information FFS (pending SA2 progress).

Proposal 2: RAN3 to introduce a sensing “task”, as a basic unit of managing sensing service cycle between SF (residing at 5GC) and SE (gNB), which can be uniquely identifiable by a sensing task ID and can serve as the anchor for initiating, modifying, and terminating the task, and for controlling the associated sensing parameter configuration and reporting between them.

Proposal 3: For RAN-CN sensing service cycle (request, execution, report) signalling, FFS details (pending SA2 progress). But from a high-level, RAN3 to agree that

- The request signalling is handled as “class-1” so that the result of request (accepted or rejected by gNB) can be notified immediately back to the Sensing CN node.

- The report signalling from gNB is handled as “class-2” (i.e. without acknowledgement).

# Reference

[1] RP-252819, New SI: Study on Integrated Sensing And Communication (ISAC) for NR (RAN1-led)

[2] TR 22.837, Study on Integrated Sensing and Communication (Rel-19), SA1

[3] TS 22.137, Integrated Sensing and Communication (Rel-19), SA1

[4] RP-242348, Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR

[5] R3-257123, “Discussions for ISAC network architecture in RAN3”, LG Electronics Inc.

[6] TR 23.700-14, Study on Stage 2 for Integrated Sensing and Communication (Rel-20), SA2