3GPP TSG-RAN WG3 Meeting #129bis	R3-257226

# Prague, Czech Republic, 13 – 17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.443 | CR | 0136 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the reference to TS 36.331 IEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-10-17 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The sementics description references to some TS 36.331 IEs with incorrect IE name, for example, the below IE names with -v14x0 or -v19xy. Encoded as the mch-SchedulingPeriod-v14x0 IE in TS 36.331 [11]. If this IE is present, the value signalled in the MCH Scheduling Period IE is ignored. Encoded as the mch-SchedulingPeriod-v19xy contained in the PMCH-TFI-Config-r19 IE in TS 36.331 [11]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the IE name, i.e. - Change “-v14x0” to “-v1430” - Change “-v19xy” to “-v1900” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error remains in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.8, 9.2.1.13, 9.2.1.31 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: add the changes to Section 9.2.1.31 |  |  |  |  |  |  |  |  |

-----------------Start of the Changes-------------------

#### 9.2.1.8	PMCH Configuration

This information element provided PMCH configuration related content for MCCH.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Allocated Subframes End | M |  | INTEGER (0..1535) | Encoded as the sf-AllocEnd IE in TS 36.331 [11]. | − | − |
| Modulation and Coding Scheme | M |  | INTEGER (0..28) | Encoded as the dataMCS IE in TS 36.331 [11]. | − | − |
| MCH Scheduling Period | M |  | ENUMERATED (rf8, rf16, rf32, rf64, rf128, rf256, rf512, rf1024) | Encoded as the mch-SchedulingPeriod IE in TS 36.331 [11]. | − | − |
| Modulation and Coding Scheme 2 | O |  | INTEGER (0..27) | Encoded as the t2 in the dataMCS IE in TS 36.331 [11]. If this IE is present, the value signalled in the Modulation and Coding Scheme IE is ignored. | YES | reject |
| MCH Scheduling Period Extended | O |  | ENUMERATED (rf4, …) | Encoded as the mch-SchedulingPeriod-r12 IE in TS 36.331 [11]. If this IE is present, the value signalled in the MCH Scheduling Period IE is ignored. | YES | reject |
| MCH Scheduling Period Extended 2 | O |  | ENUMERATED (rf1, rf2, …) | Encoded as the mch-SchedulingPeriod-v1430 IE in TS 36.331 [11]. If this IE is present, the value signalled in the MCH Scheduling Period IE is ignored. | YES | reject |
| MCH Scheduling Period Extended 3 | O |  | ENUMERATED (rf7, rf14, rf28, rf53, rf56, rf108, rf112, rf212, rf424, …) | Encoded as the mch-SchedulingPeriod-v1900 contained in the PMCH-TFI-Config-r19 IE in TS 36.331 [11]. If this IE is present, the value signalled in the MCH Scheduling Period IE is ignored. | YES | reject |
| Frequency Interleaving Indicator | O |  | 9.2.1.29 |  | YES | reject |
| Time Interleaving Parameters | O |  | 9.2.1.30 |  | YES | reject |

#### -----------------Next Change-------------------

#### 9.2.1.13	MCCH related BCCH Configuration Item

This information element provides MCCH related BCCH configuration information to the eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBSFN Area Id | M |  | 9.2.1.14 |  | − | − |
| PDCCH Length | M |  | ENUMERATED (s1, s2,…) | This IE is encoded along the number of OFDM symbols for PDCCH as of table 6.7-1. In TS 36.211 [12]. | − | − |
| Repetition Period | M |  | ENUMERATED (rf32, rf64, rf128, rf256) | The same encoding as the mcch-RepetitionPeriod IE in the mcch-Config IE as specified in TS 36.331 [11]. | − | − |
| Repetition Period Extended | O |  | ENUMERATED (rf1, rf2, rf4, rf8, rf16, …) | The same encoding as the mcch-RepetitionPeriod-v1430 IE in the mcch-Config IE as specified in TS 36.331 [11]. If this IE is present, the value signalled in the Repetition Period IE is ignored. | YES | reject |
| Offset | M |  | INTEGER (0..10) | The same encoding as the mcch-Offset in mcch-Config IE as specified in TS 36.331 [11]. | − | − |
| Modification Period | M |  | ENUMERATED (rf512, rf1024) | The same encoding as the mcch-ModificationPeriod IE in the mcch-Config IE as specified in TS 36.331 [11]. | − | − |
| Modification Period Extended | O |  | ENUMERATED (rf1, rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256, …) | The same encoding as the mcch-ModificationPeriod-v1430 IE in the mcch-Config IE as specified in TS 36.331 [11]. If this IE is present, the value signalled in the Modification Period IE is ignored. | YES | reject |
| Subframe Allocation Info | M |  | BIT STRING (SIZE(6)) | The same encoding as the sf-AllocInfo IE specified in TS 36.331 [11]. | − | − |
| Modulation and Coding Scheme | M |  | ENUMERATED (n2, n7, n13, n19) | The same encoding as the signallingMCS IE specified in TS 36.331 [11]. | − | − |
| Cell Information List |  | 0..1 |  |  |  |  |
| >Cell Information |  | 1 to <maxnoofCells> |  |  |  |  |
| >>E-UTRAN CGI | M |  | 9.2.1.11 |  | − | − |
| >>Cell Reservation Info | M |  | ENUMERATED (reservedCell, nonReservedCell, …) |  | − | − |
| Subcarrier Spacing MBMS | O |  | ENUMERATED (khz-7dot5, khz-1dot25, …) | Semantics along the definition of the subcarrierSpacingMBMS-r14 IE as specified in TS 36.331 [11]. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCells | Maximum no. of cells that may be served by an eNB. The value for maxnoofCells is 256. |

#### -----------------Next Change-------------------

#### 9.2.1.31	CAS Muting Parameters

This IE indicates the parameters of where the non-MBSFN subframes containing the cell acquisition signals in an MBMS-dedicated cell shall only be transmitted, as defined in TS 36.211 [12].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| K-CAS | M |  | INTEGER (4..63, ...) | The same encoding as the k-CAS-r19 in the cas-MutingConfig-r19 contained in the SystemInformationBlockType1-MBMS-v1900 IE as specified in TS 36.331 [11]. |
| N-CAS | M |  | ENUMERATED (n2, n4, n8, n16, ...) | The same encoding as the n-CAS-r19 in the cas-MutingConfig-r19 contained in the SystemInformationBlockType1-MBMS-v1900 IE as specified in TS 36.331 [11]. |

-----------------End of the Change-------------------