3GPP TSG-RAN WG3 Meeting #129                                                                    R3-255245

Bengaluru, India, 25th – 29th, August 2025

Source:	CATT

Title:	Discussion on remaining issue of NR Femto

Agenda Item:	12.3

Document for:	Discussion and Decision

1 Introduction

In the last meeting [1], the security verifications and the terminology alignment of NR Femto were discussed and the agreement was made. The modifications have been described in the BLCR of TS 38.300 [2].

In this contribution, we will discuss three remaining issues of NR Femto in the summary of offline discussion [3].

2 Discussion

In the last meeting, the security aspects were captured in the BLCR to TS 38.300 [2]. But several issues were summarized by the offline discussion [3].

2.1 Specific Femto Indication in Initial UE message

One issue regards to enabling control of sending Allowed PNI-NPN list to NR Femto node

To be further discussed: whether to send a specific Femto indication in the NG: Initial UE message, from NR Femto to enable control of sending Allowed PNI NPN List or not.

This issue considers whether to add a security indication in the Initial UE message to enable control of sending Allowed PNI-NPN List or not. The reason is that the NR Femto node may be untrusted, the Allowed PNI-NPN List should not be sent to the NR Femto node.

Firstly, if this IE is not sent from AMF to NR Femto nodes, there would be some issues. Allowed PNI-NPN List IE is the information with respect to access control for NPN. The Allowed PNI-NPN List IE can be sent from AMF to the NG-RAN node via the INITIAL CONTEXT SETUP REQUEST, HANDOVER REQUEST, DOWNLINK NAS TRANSPORT, PAGING messages over NG. If this IE is not sent to NR Femto node in HANDOVER REQUEST message during an inbound NG handover, the NR Femto node will understand that no CAG cell is allowed for the UE that the NR Femto node’s cell will always refuse UE’s handover if it serves a CAG. The Allowed PNI-NPN List IE is also delivered from source node to target node for access control over Xn in legacy. If the NR Femto node has not received Allowed PNI-NPN List IE from AMF, it will not send this IE to the target node during an outbound Xn handover that the target node will assume no CAG cell is allowed and the UE cannot be accessed by any CAG cell.

Secondly, the issue on security should be discussed by SA3, but SA3 has not pointed out this issue. So, there would be no concern on sending the Allowed PNI-NPN List to NR Femto nodes from SA3 point of view. From our view, given the AMF/NR Femto GW can verify whether the CAG ID is valid for the indicated cell of the NR Femto node during the NG setup, which means the NR Femto node is authorized by operator, the NR Femto node would be trustable after NG setup.

Therefore, no restriction is observed for sending Allowed PNI-NPN List to NR Femto nodes. It’s not necessary to add a specific Femto indication in the Initial UE message to enable control of sending Allowed PNI-NPN List or not.

Proposal 1: No need to introduce a specific Femto indication in the Initial UE message to enable control of sending Allowed PNI NPN List or not.

2.2 NG-based handover

The second issue is regarding NR Femto GW transferring the HANDOVER REQUEST message to the target NR Femto node during NG-based handover, which is raised by [3]. Because target node ID is not included in the HANDOVER REQUEST message received by NR Femto GW from the AMF, the NR Femto GW does not know to which NR Femto node to route this message.

[4] further points out that even if the NR Femto GW can read the Targt Cell ID IE in Source NG-RAN Node to Target NG-RAN Node Transparent Container IE which is included in the HANDOVER REQUEST message, the NR Femto GW may be still unable to find the target NR Femto node. Because the gNB-ID length can be variable from 22 bits to 32 bits that the target NCGI may be matched to multiple target NR Femto nodes with different gNB-ID length under the same NR Femto GW.

Some solutions address this issue by adding the target gNB ID IE or the gNB-ID length of the target node in the HANDOVER REQUEST message [3]. We understand this method would be the most straight forward. Nevertheless, if the NR Femto GW can read the Target Cell ID in the container of HANDOVER REQUEST message, this issue could also be solved by implementation ensuring the NR CGI of target cell can only map to one gNB ID of the NR Femto nodes connecting with the same NR Femto GW. For example,

Solution 1: Ensuring that the NR Femto nodes connecting to the same NR Femto GW have the same gNB-ID length.

Solution 2: If the NR Femto nodes connecting to the same NR Femto GW have different gNB-ID length, ensuring the overlapping bits of gNB IDs are different. In this way, the NCGI of one NR Femto node cannot be mapped to other NR Femto nodes. For example:

Fig. 1 Overlapping bits of gNB IDs are different

Solution 3: If the overlapping bits of gNB-IDs are same for the NR Femto nodes connecting to the same NR Femto GW, ensuring the NR CGI configured to the NR Femto node with shorter gNB-ID does not match the longer gNB-ID(s) of other NR Femto nodes. As shown in Fig. 2, the NCGI of NR Femto node 1 does not match the gNB ID of NR Femto node 2.

Fig. 2 NR CGI for shorter gNB-ID does not match longer gNB-ID

Solution 2 and solution 3 allow different gNB-ID length of the NR Femto nodes under the same GW and prevents the same target NCGI matching with gNB-IDs of multiple target NR Femto nodes through gNB-ID/NCGI configuration, whereas solution 1 requires the alignment of gNB-ID between different NR Femto nodes connecting with the same GW. From our perspective, the three implementations are all feasible. The NR Femto GW can read the Target Cell ID in the container of HANDOVER REQUEST message received from AMF and determine the target NR Femto node whose gNB-ID matches the Target Cell ID.

Thus, we prefer to handle this issue by implementation than spec enhancement.

Proposal 2: For selecting the target NR Femto node in NG-based handover, it should be ensured that the NCGI of one NR Femto node cannot match the gNB-ID of any other NR Femto node connecting with the same Femto GW.

Proposal 3: The NR Femto GW can read the Target Cell ID in the container of HANDOVER REQUEST message received from AMF and determine the target NR Femto node whose gNB-ID matches the Target Cell ID.

2.3 CAG time validity information

The third issue is the temporary usage of CAG ID by Femto, because the AMF may release the N2 connection of UE when the evaluation of the validity condition of the CAG ID in use changes from true to false. One solution proposed in [5] considers informing NG-RAN node the time validity for the allowed CAG IDs of UE over N2 to enable the NG-RAN node to perform a graceful handover before expiry of the validity time of CAG. However, we believe the solution is not necessary. According to TS 23.501, AMF can update the Allowed CAG List per PLMN IE towards NG-RAN if the validity of an entry in the Allowed CAG list changes between true and false:

The AMF shall update the Allowed CAG list in the Mobility Restrictions towards NG-RAN if the evaluation of the time validity information of an entry in the Allowed CAG list changes between true and false, unless the AMF releases the NAS signalling connection to the UE based on operator's policy if the evaluation of the validity condition changes from true to false.

At the reception of updated Allowed CAG list, if the CAG ID in use is absent in the updated Allowed CAG list, which means the validity condition of the CAG ID which UE uses changes from true to false, the NG-RAN node can initiate a handover to UE. So, there is no need to let AMF inform the CAG time validity to NG-RAN.

Proposal 4: No need to introduce the CAG time validity information in NGAP signal.

3 Conclusion

In the previous sections we made the following observations and proposals:

Proposal 1: No need to introduce a specific Femto indication in the Initial UE message to enable control of sending Allowed PNI NPN List or not.

Proposal 2: For selecting the target NR Femto node in NG-based handover, it should be ensured that the NCGI of one NR Femto node cannot match the gNB-ID of any other NR Femto node connecting with the same Femto GW.

Proposal 3: The NR Femto GW can read the Target Cell ID in the container of HANDOVER REQUEST message received from AMF and determine the target NR Femto node whose gNB-ID matches the Target Cell ID.

Proposal 4: No need to introduce the CAG time validity information in NGAP signal.

4 Reference

- RAN3#127bis Minutes, RAN3 Chair;

- R3-254017, (BL CR to 38.300) Introduction of NR Femto Architecture and Protocol Aspects, Ericsson, Nokia, TMO US, AT&T, Verizon Wireless, BT, Charter, Huawei, LG Electronics, Samsung, Lenovo, Baicells, ZTE, NEC, CATT, Qualcomm, RAN3#128;

- R3-253790, Summary of Offline Discussion on additional topological enhancement, NTTDOCOMO, RAN3#128;

- R3-253346, (TP for Femto BL CRs for TS 38.300/38.413) Discussion on remaining issues for NR Femto, Huwawei, RAN3#128;

- R3-253225, Completion of other open points of NR Femto, Nokia, RAN3#128;