3GPP TSG-RAN WG3 Meeting #129	R3-255261

Bengaluru, India, 25 - 29 August 2025

Agenda Item:	10.3.1

Source:	ZTE Corporation

Title:	(TPs to BL CR for 38.300, 38.413 and 38.423) Further discussion on SON/MDT enhancements for NTN

Document for:	Discussion

1  Introduction

In the previous meetings, some agreements on SON/MDT enhancements for NTN were reached as below.

Logged MDT enhancement based on scenarios to be worked first, work on immediate MDT if time allows. MRO mechanisms for intra-NTN should be addressed based on failure scenarios first, work on near failure scenarios later. Take the connection failure scenarios in §15.5.2.2.2 of TS 38.300 as starting point with necessary update, if needed, in MRO for intra-NTN mobility. MRO for intra-NTN mobility shall consider the following trigger conditions: - Measurement-based trigger condition; - A time-based trigger condition; - A location-based trigger condition; - A time-based and Measurement-based combined trigger condition; - A location-based and Measurement-based combined trigger condition; Do not consider MRO for feeder link switch. MRO for intra-NTN mobility shall consider both conditional and non-conditional intra-NTN handover. MRO for CHO with a time-based trigger condition: Case 1 Too Late CHO Execution: RLF happens in the source cell before UE executes CHO (before or after T1 threshold starts), and the UE re-connects the cell other than the source cell. - Distinguish if the RLF happens before or after T1 Case 2 Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell. - FFS on whether it matters if RLF happens within the duration or after the duration period. WA: down-select among below three alternatives. Alt 1: the RLF report includes for single time-based trigger condition: - Absolute time (measured UTC time) when RLF occurred Alt 2: a flag to indicate whether the RLF happens befor e T1 or after T1 Alt 3: based on existing timers in RLF report. Further check the granularity of the current timers. Existing timers can be reused to identify whether RLF happens before or after T1 with 100ms accuracy, FFS if the accuracy is a problem to be solved. Existing timers can be reused to identify when RLF happens with 100ms accuracy. FFS whether more accuracy is needed. No need to identify whether RLF happens before or after T1 for time-based CHO. No further discussion on Time-based CHO. MRO for CHO with a location-based trigger condition: For earth fixed case, there is no need to report the measured distance from the UE. FFS on the earth moving case. WA: There is no need to report any other info in the RLF report for the location based CHO. Revert the WA: There is no need to report any other info in the RLF report for the location based CHO. For earth moving case, the accuracy and complexity issue of the network-based solution is acknowledged. - Option 1: to introduce the absolute UTC time of when RLF/HOF happens in the RLF report. - Option 2: to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell (when RLF occurred). FFS on option 1 or option 2. MRO for CHO with joint trigger conditions: UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”, it is up to RAN2 to decide to reuse the existing IE or not. MRO for RACH-less Handover for NTN: Whether to consider the fallback issue from RACH-less HO to RACH-based HO for NTN? Divide the MRO for RACH-less handover case into two scenarios: - RACH-less handover was attempted but failed; - UE was configured RACH-less handover but did not attempt RACH-less handover because RSRP is below threshold MRO mechanism for intra-NTN mobility shall be enhanced to enable handling of local and temporary propagation changes (e.g. due to clouds, rain etc.)? New finer area scope for MDT: Is this for signaling based MDT or for management-based MDT? Introduce a geographical area (a reference location plus a radius) as the area scope for MDT or a mapped cell ID list? RAN3 assumes both geographical area and the mapped cell ID as Area Scope of logged MDT for NTN over NGAP are feasible and confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for MDT NTN. Distance-based event trigger for logged MDT? FFS on whether to add the geographical area scope inside or outside the existing area scope IE. FFS on whether the geographical area scope applies to both logged and immediate MDT or only logged MDT.

In this contribution, we provide our further views on this topic, including the MRO for intra-NTN failure scenarios and the MDT enhancement for NTN.

2  Discussion

  - MRO for NTN

2.1.1  MRO for RACH-less handover

In last RAN3 meeting, the MRO for RACH-less handover has been discussed, and this issue is divided into two scenarios:

- RACH-less handover was attempted but failed;

- UE was configured RACH-less handover b	ut did not attempt RACH-less handover because RSRP is below threshold

Regarding Scenario 1,  the RACH-less handover could be attempted by configured grant (CG) or dynamic grant (DG). The RACH-less HO is failed which means that the pre-allocated UL grant configured by the NW is not appropriate, and the RACH-less HO related configuration should be reported in the RLF report.

For the CG RACH-less handover, according to the TS 38.331,,the RSRP threshold configured for SSB selection for the CG configuration should be reported by the UE.

---------------------------------------------------------TS 38.331-------------------------------------------------------------------------

---------------------------------------------------------TS 38.331-------------------------------------------------------------------------

For DG RACH-less handover, according to the TS 38.331, the TCI State ID or the SSB index in the target cell for initial UL transmission could also be reported by the UE.

Regarding Scenario 2, since the RACH-less handover was not attempted because RSRP is below threshold, the configured RSRP threshold should be reported by the UE.

Proposal 1: Regarding the MRO for RACH-less handover, the RACH-less HO related configuration (e.g., RSRP threshold for CG configuration, TCI State ID or SSB index for DG configuration) should be included in the RLF report.

2.1.2  Stage 2 update

Since it has been agreed that “Take the connection failure scenarios in §15.5.2.2.2 of TS 38.300 as starting point with necessary update, if needed, in MRO for intra-NTN mobility.”, the corresponding stage 2 description could be update to capture the connection failure scenarios for intra-NTN mobility.

Proposal 2: Update the 15.5.2.2.2 of TS 38.300 to capture the connection failure scenarios for intra-NTN mobility.

  - MDT for NTN

In RAN2#129bis meeting, RAN2 has discussed the logged MDT for NTN and replied the LS on this issue[1], which is given as below.

RAN2 would like to thank RAN3 about the progress on SON/MDT for NTN and provide the reply for the following question. Q: On MDT for NTN, RAN3 assumes that the geographical area and/or the mapped cell ID can be used as the Area Scope for the logged MDT for NTN over NGAP. Further, RAN3 would like to confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for Area Scope of logged MDT over Uu for NTN, to help RAN3 decide whether to specify geographical area and/or mapped cell ID over NGAP. A: RAN2 confirms that the geographical area defined for MBS NTN can be used as a reference for Area Scope of logged MDT over Uu for NTN, including both reference location/radius and polygon-based area indication. RAN2 also agreed that only geographical area scope is used to indicate applicable logging area to the UE over Uu, i.e., no mapped cell ID(s).

Based on the RAN2’s feedback, only geographical area scope is used to indicate the applicable logging area to the UE over Uu, which means the mapped cell ID shall not be used.

Observation: Only geographical area is used as Area Scope of logged MDT for NTN over NGAP.

For the geographical area solution, according to the progress of MBS in NR NTN WI, for the encoding of the geographical service area of MBS, the following agreements were reached as below.

1. The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle. 2. The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon. 3. The IntendedServiceAreaInfo is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed)

In current Running RRC for NR NTN[2], the areaCoordinates-r19 within the IntendedServiceAreaInfo-r1 in SIBXX has been defined to represent the geographical area information of MBS broadcast service in a NTN cell. Accordingly, our RAN3 is able to refer to this IE to represent the geographical area information of logged MDT for NTN.

-----------------------------------------------------Running RRC CR for NR NTN---------------------------------------------------

-----------------------------------------------------Running RRC CR for NR NTN---------------------------------------------------

The introduction of geographical area info should be applied for both NGAP and XnAP, and it should be applied for both logged and immediate MDT.

Proposal 3: Agree the TPs of introducing geographical area info for Area Scope of logged MDT over NGAP and XnAP.

Proposal 4: The introduction of geographical area info for Area Scope should be applied to both logged MDT and immediate MDT.

3  Conclusion

MRO for intra-NTN mobility failure

Proposal 1: Regarding the MRO for RACH-less handover, the RACH-less HO related configuration (e.g., RSRP threshold for CG configuration, TCI State ID or SSB index for DG configuration) should be included in the RLF report.

Proposal 2: Update the 15.5.2.2.2 of TS 38.300 to capture the connection failure scenarios for intra-NTN mobility.

MDT enhancement

Proposal 3: Agree the TPs of introducing geographical area for Area Scope of logged MDT over NGAP and XnAP.

Proposal 4: The introduction of geographical area for Area Scope should be applied to both logged MDT and immediate MDT.

The corresponding TPs and draft reply LS are given in the Annex.

References

- R3-253033, Reply LS on MDT for NTN, RAN2

- R2-2504656, Running RRC CR for NR NTN phase 3, Ericsson

Annex

TP for TS 38.300 on connection failure scenarios for intra-NTN mobility

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of intra-NTN mobility, the Too Late Handover, Too Early Handover, Handover to Wrong Cell, Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell are all applied.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

TP for TS 38.413 on logged MDT for intra-NTN mobility

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

9.3.1.169	MDT Configuration-NR

This IE defines the MDT configuration parameters of NR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED (Immediate MDT only, Logged MDT only, Immediate MDT and Trace, …) |  | - |  |
| CHOICE Area Scope of MDT | M |  |  |  | - |  |
| >Cell based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  | - |  |
| >>>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >TA based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TA List for MDT |  | 1..<maxnoofTAforMDT> |  |  | - |  |
| >>>TAC | M |  | 9.3.3.10 | The TAI is derived using the current serving PLMN. | - |  |
| >PLMN wide |  |  | NULL |  |  |  |
| >TAI based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TAI List for MDT |  | 1..<maxnoofTAforMDT> |  |  | - |  |
| >>>TAI | M |  | 9.3.3.11 |  | - |  |
| >PNI-NPN Based MDT |  |  |  |  | YES | ignore |
| >>CAG List for MDT |  |  | 9.3.3.65 |  | - |  |
| >SNPN Cell Based MDT |  |  |  |  | YES | ignore |
| >>SNPN Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  | - |  |
| >>>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >>>NID | M |  | 9.3.3.42 | Identifies an SNPN together with the PLMN Identity in the NR CGI IE. | - |  |
| >SNPN TAI Based MDT |  |  |  |  | YES | ignore |
| >>SNPN TAI List for MDT |  | 1..<maxnoofTAforMDT> |  |  | - |  |
| >>>TAI | M |  | 9.3.3.11 |  | - |  |
| >>>NID | M |  | 9.3.3.42 | Identifies an SNPN together with the PLMN Identity in the TAI IE. | - |  |
| >SNPN Based MDT |  |  |  |  | YES | ignore |
| >>SNPN List for MDT |  | 1..<maxnoofMDTSNPNs> |  |  | - |  |
| >>>PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >>>NID | M |  | 9.3.3.42 | Identifies an SNPN together with the PLMN Identity IE. | - |  |
| >Geographical Area Based MDT |  |  |  |  |  |  |
| >>Geographical Area List for MDT |  | 1..<maxnoofGeographicalAreaforMDT> |  |  | - |  |
| >>>Geographical Area | M |  | OCTET STRING | Encoded in the same format as the areaCoordinates-r19 IE as defined in TS 38.331 [18]. | - |  |
| CHOICE MDT Mode | M |  |  |  | - |  |
| >Immediate MDT |  |  |  |  |  |  |
| >>Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [41]. First Bit = M1, Second Bit= M2, Third Bit = M4, Fourth Bit = M5, Fifth Bit = M6, Sixth Bit = M7, Seventh Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration, other bits reserved for future use. Value “1” indicates “activate” and value “0” indicates “do not activate”. | - |  |
| >>M1 Configuration | C-ifM1 |  | 9.3.1.171 |  | - |  |
| >>M4 Configuration | C-ifM4 |  | 9.3.1.172 |  | - |  |
| >>M5 Configuration | C-ifM5 |  | 9.3.1.173 |  | - |  |
| >>M6 Configuration | C-ifM6 |  | 9.3.1.174 |  | - |  |
| >>M7 Configuration | C-ifM7 |  | 9.3.1.175 |  | - |  |
| >>Bluetooth Measurement Configuration | O |  | 9.3.1.177 |  | - |  |
| >>WLAN Measurement Configuration | O |  | 9.3.1.178 |  | - |  |
| >>MDT Location Information | O |  | 9.3.1.176 |  | - |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.179 |  | - |  |
| >Logged MDT |  |  |  |  |  |  |
| >>Logging Interval | M |  | ENUMERATED (320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms, 20480ms, 30720ms, 40960ms, 61440ms, infinity, …) | Corresponds to the LoggingInterval IE as defined in TS 38.331 [18]. | - |  |
| >>Logging Duration | M |  | ENUMERATED (10, 20, 40, 60, 90,120, …) | Corresponds to the LoggingDuration IE as defined in TS 38.331 [18]. Unit: [minute]. | - |  |
| >>CHOICE Report Type | M |  |  |  | - |  |
| >>>Periodical |  |  | NULL |  |  |  |
| >>>Event Triggered |  |  |  |  |  |  |
| >>>>Event Trigger Logged MDT Configuration | M |  | 9.3.1.180 |  | - |  |
| >>Bluetooth Measurement Configuration | O |  | 9.3.1.177 |  | - |  |
| >>WLAN Measurement Configuration | O |  | 9.3.1.178 |  | - |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.179 |  | - |  |
| >>Area Scope of Neighbour Cells | O |  | 9.3.1.182 |  | - |  |
| >>Early Measurement | O |  | ENUMERATED (true, ...) | This IE indicates whether the UE is allowed to log measurements on early measurement related frequencies in logged MDT as specified in TS 38.331 [18]. | YES | ignore |
| Signalling Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.168 |  | - |  |
| PNI-NPN Area Scope of MDT |  | 0..1 |  | This IE is ignored if the Area Scope of MDT IE is set to "PLMN Wide" | YES | ignore |
| >CAG List for MDT | M |  | 9.3.3.65 |  | - |  |
| Network Slice Area Scope of MDT | O |  | 9.3.3.A |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforMDT | Maximum no. of Cell ID subject for MDT scope. Value is 32. |
| maxnoofTAforMDT | Maximum no. of TA subject for MDT scope. Value is 8. |
| maxnoofMDTSNPNs | Maximum no. of SNPNs in the MDT SNPN list. Value is 16. |
| maxnoofGeographicalAreaforMDT | Maximum no. of Geographical Area subject for MDT scope. Value is 32. |

| Condition | Explanation |
| --- | --- |
| C-ifM1 | This IE shall be present if the Measurements to Activate IE has the first bit set to “1”. |
| C-ifM4 | This IE shall be present if the Measurements to Activate IE has the third bit set to “1”. |
| C-ifM5 | This IE shall be present if the Measurements to Activate IE has the fourth bit set to “1”. |
| C-ifM6 | This IE shall be present if the Measurements to Activate IE has the fitth bit set to “1”. |
| C-ifM7 | This IE shall be present if the Measurements to Activate IE has the sixth bit set to “1”. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

>>Unchanged part skipped<<

id-EUTRA-PagingeDRXInformation,

id-EquivalentSNPNsList,

id-GeographicalArea,

id-GeographicalAreaBasedMDT,

id-GlobalCable-ID,

id-GlobalRANNodeID,

id-GlobalTNGF-ID,

id-GlobalTWIF-ID,

id-GlobalW-AGF-ID,

id-GUAMIType,

>>Unchanged part skipped<<

maxnoofUETypes,

maxnoofWLANName,

maxnoofXnExtTLAs,

maxnoofXnGTP-TLAs,

maxnoofXnTLAs,

maxnoofThresholdsForExcessPacketDelay,

maxnoofCandidateRelayUEs,

maxnoofSuccessfulPSCellChangeReports,

maxnoofCellsTSS,

maxnoofPeriodicities,

maxnoofPartiallyAllowedS-NSSAIs,

maxnoofRSPPQoSFlows,

maxnoofSliceItemsforMDT,

maxnoofGeographicalAreasforMDT

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

AreaScopeOfMDT-NR ::= CHOICE {

cellBased					CellBasedMDT-NR,

tABased						TABasedMDT,

pLMNWide					NULL,

tAIBased					TAIBasedMDT,

choice-Extensions		ProtocolIE-SingleContainer { {AreaScopeOfMDT-NR-ExtIEs} }

}

AreaScopeOfMDT-NR-ExtIEs NGAP-PROTOCOL-IES ::= {

{ ID id-PNI-NPNBasedMDT		CRITICALITY ignore	TYPE PNI-NPNBasedMDT		PRESENCE mandatory }|

{ ID id-SNPN-CellBasedMDT	CRITICALITY ignore	TYPE SNPN-CellBasedMDT		PRESENCE mandatory }|

{ ID id-SNPN-TAIBasedMDT	CRITICALITY ignore	TYPE SNPN-TAIBasedMDT		PRESENCE mandatory }|

{ ID id-SNPN-BasedMDT		CRITICALITY ignore	TYPE SNPN-BasedMDT			PRESENCE mandatory }|

ID id-GeographicalAreaBasedMDT		CRITICALITY ignore	TYPE GeographicalAreaBasedMDT			PRESENCE mandatory },

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

-- G

GBR-QosInformation ::= SEQUENCE {

maximumFlowBitRateDL		BitRate,

maximumFlowBitRateUL		BitRate,

guaranteedFlowBitRateDL		BitRate,

guaranteedFlowBitRateUL		BitRate,

notificationControl			NotificationControl									OPTIONAL,

maximumPacketLossRateDL		PacketLossRate										OPTIONAL,

maximumPacketLossRateUL		PacketLossRate										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {GBR-QosInformation-ExtIEs} }	OPTIONAL,

...

}

GBR-QosInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-AlternativeQoSParaSetList	CRITICALITY ignore	EXTENSION AlternativeQoSParaSetList	PRESENCE optional	},

...

}

GeographicalArea ::= OCTET STRING

GeographicalAreaBasedMDT::= SEQUENCE {

geographicalAreaListforMDT		GeographicalAreaListforMDT,

iE-Extensions		ProtocolExtensionContainer { {GeographicalAreaBasedMDT-ExtIEs} } OPTIONAL,

...

}

GeographicalAreaBasedMDT-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

GeographicalAreaListforMDT ::= SEQUENCE (SIZE(1.. maxnoofGeographicalAreaforMDT)) OF GeographicalAreaListforMDTItem

GeographicalAreaListforMDTItem ::= SEQUENCE {

geographicalArea				GeographicalArea,

iE-Extensions		ProtocolExtensionContainer {{ GeographicalAreaListforMDTItem-ExtIEs}}		OPTIONAL,

...

}

GeographicalAreaListforMDTItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs							INTEGER ::= 65535

maxProtocolExtensions					INTEGER ::= 65535

maxProtocolIEs							INTEGER ::= 65535

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxnoofAllowedAreas							INTEGER ::= 16

maxnoofAllowedCAGsperPLMN					INTEGER ::= 256

maxnoofAllowedS-NSSAIs						INTEGER ::= 8

>>Unchanged part skipped<<

maxnoofRSPPQoSFlows							INTEGER ::= 2048

maxnoofSliceItemsforMDT						INTEGER ::= 1024

maxnoofGeographicalAreaforMDT				INTEGER ::= 32

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-AllowedNSSAI											ProtocolIE-ID ::= 0

id-AMFName												ProtocolIE-ID ::= 1

>>Unchanged part skipped<<

id-UserLocationInformationN3IWF-without-PortNumber		ProtocolIE-ID ::= 439

id-AUN3DeviceAccessInfo									ProtocolIE-ID ::= 440

id-NetworkSliceAreaScopeofMDT							ProtocolIE-ID ::= XX1

id-GeographicalArea				        			    ProtocolIE-ID ::= XX2

Id-GeographicalAreaBasedMDT                             ProtocolIE-ID ::= XX3

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

TP for TS 38.423 on MDT for intra-NTN mobility

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

- 9.2.3.126	MDT Configuration-NR

The IE defines the MDT configuration parameters of NR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED (Immediate MDT only, Immediate MDT and Trace, Logged MDT only, ...) |  | – |  |
| CHOICE Area Scope of MDT-NR | O |  |  |  | – |  |
| >Cell based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>Cell ID List for MDT-NR |  | 1 .. <maxnoofCellIDforMDT> |  |  | – |  |
| >>>NR CGI | M |  | 9.2.2.7 |  | – |  |
| >TA based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TA List for MDT |  | 1 .. <maxnoofTAforMDT> |  |  | – |  |
| >>>TAC | M |  | 9.2.2.5 | The TAI is derived using the current serving PLMN. | – |  |
| >TAI based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, it covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TAI List for MDT |  | 1 |  |  | – |  |
| >>>TAI List for MDT Item |  | 1 .. <maxnoofTAforMDT> |  |  | – |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 |  | – |  |
| >>>>TAC | M |  | 9.2.2.5 |  | – |  |
| >PNI-NPN Based MDT |  |  |  |  | YES | ignore |
| >>CAG List for MDT |  |  | 9.2.3.191 |  | – |  |
| >SNPN Cell Based MDT |  |  |  |  | YES | ignore |
| >>SNPN Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  | – |  |
| >>>NR CGI | M |  | 9.2.2.7 |  | – | - |
| >>>NID | M |  | 9.2.2.65 | Identifies an SNPN together with the PLMN Identity in the NR CGI IE. | – | - |
| >SNPN TAI Based MDT |  |  |  |  | YES | ignore |
| >>SNPN TAI List for MDT |  | 1..<maxnoofTAforMDT> |  |  | – | - |
| >>>PLMN Identity | M |  | 9.2.2.4 |  | – | - |
| >>>TAC | M |  | 9.2.2.5 |  | – | - |
| >>>NID | M |  | 9.2.2.65 | Identifies an SNPN together with the PLMN Identity IE. | – | - |
| >SNPN Based MDT |  |  |  |  | YES | ignore |
| >> SNPN List for MDT |  | 1..<maxnoofMDTSNPNs> |  |  | – | - |
| >>>PLMN Identity | M |  | 9.2.2.4 |  | – | - |
| >>>NID | M |  | 9.2.2.65 | Identifies an SNPN together with the PLMN Identity IE. | – | - |
| >Geographical Area Based MDT |  |  |  |  |  |  |
| >>Geographical Area List for MDT |  | 1..<maxnoofGeographicalAreaforMDT> |  |  | - |  |
| >>>Geographical Area | M |  | OCTET STRING | Encoded in the same format as the areaCoordinates-r19 IE as defined in TS 38.331 [10]. | - |  |
| CHOICE MDT Mode | M |  |  |  | – |  |
| >Immediate MDT-NR |  |  |  |  |  |  |
| >>Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [43]. First Bit = M1, Second Bit= M2, Fourth Bit = M4, Fifth Bit = M5, Sixth Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration, Seventh Bit = M6, Eighth Bit = M7. Value "1" indicates "activate" and value "0" indicates "do not activate". This version of the specification does not use bits 3. | – |  |
| >>M1 Configuration | C-ifM1 |  | 9.2.3.128 |  | – |  |
| >>M4 Configuration | C-ifM4 |  | 9.2.3.129 |  | – |  |
| >>M5 Configuration | C-ifM5 |  | 9.2.3.130 |  | – |  |
| >>MDT Location Information | O |  | BITSTRING(SIZE(8)) | Each position in the bitmap represents requested location information as defined in TS 37.320 [43]. First Bit = GNSS Other bits are reserved for future use and are ignored if received. Value "1" indicates "activate" and value "0" indicates "do not activate". The eNB shall ignore the first bit unless the Measurements to Activate IE has the first bit or the sixth bit set to "1". | – |  |
| >>M6 Configuration | C-ifM6 |  | 9.2.3.131 |  | – |  |
| >>M7 Configuration | C-ifM7 |  | 9.2.3.132 |  | – |  |
| >>Bluetooth Measurement Configuration | O |  | 9.2.3.134 |  | – |  |
| >>WLAN Measurement Configuration | O |  | 9.2.3.135 |  | – |  |
| >>Sensor Measurement Configuration | O |  | 9.2.3.136 |  | – |  |
| >Logged MDT-NR |  |  |  |  |  |  |
| >>Logging interval | M |  | ENUMERATED (ms320, ms640, ms1280, ms2560, ms5120, ms10240, ms20480, ms30720, ms40960 , ms61440, infinity, ...) | Corresponds to information provided in the LoggingInterval IE as defined in TS 38.331 [10]. The value "infinity" represents one shot logging, i.e., only one log per event in the logged MDT report. | – |  |
| >>Logging duration | M |  | ENUMERATED (10, 20, 40, 60, 90, 120) | Corresponds to information provided in the LoggingDuration IE as defined in TS 38.331 [10]. Unit: [minute]. | – |  |
| >>CHOICE Report Type | M |  |  |  | – |  |
| >>>Periodical |  |  |  |  |  |  |
| >>>Event Triggered |  |  |  |  |  |  |
| >>>>Logged Event Trigger Config | M |  | 9.2.3.137 |  | – |  |
| >>Bluetooth Measurement Configuration | O |  | 9.2.3.134 |  | – |  |
| >>WLAN Measurement Configuration | O |  | 9.2.3.135 |  | – |  |
| >>Sensor Measurement Configuration | O |  | 9.2.3.136 |  | – |  |
| >>Area Scope of Neighbour Cells | O |  | 9.2.3.140 |  | – |  |
| >>Early Measurement | O |  | ENUMERATED (true, ...) | This IE indicates whether the UE is allowed to log measurements on early measurement related frequencies in logged MDT as specified in TS 38.331 [10]. | – |  |
| Signalling based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | – |  |
| PNI-NPN Area Scope of MDT |  | 0..1 |  |  | YES | Ignore |
| >CAG List for MDT | M |  | 9.2.3.191 |  | – |  |
| Network Slice Area Scope of MDT | O |  | 9.2.3.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforMDT | Maximum no. of Cell ID subject for MDT scope. Value is 32. |
| maxnoofTAforMDT | Maximum no. of TA subject for MDT scope. Value is 8. |
| maxnoofMDTSNPNs | Maximum no. of SNPNs in the MDT SNPN list. Value is 16. |
| maxnoofGeographicalAreaforMDT | Maximum no. of Geographical Area for MDT scope. Value is 32. |

| Condition | Explanation |
| --- | --- |
| ifM1 | This IE shall be present if the Measurements to Activate IE has the first bit set to "1". |
| ifM4 | This IE shall be present if the Measurements to Activate IE has the fourth bit set to "1". |
| ifM5 | This IE shall be present if the Measurements to Activate IE has the fifth bit set to "1". |
| ifM6 | This IE shall be present if the Measurements to Activate IE has the seventh bit set to "1". |
| ifM7 | This IE shall be present if the Measurements to Activate IE has the eighth bit set to "1". |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<unchanged is omitted>>>

id-MN-only-MDT-collection,

id-BarringExemptionforEmerCallInfo,

id-Transmission-Bandwidth-asymmetric,

id-NRPPaPositioningInformation,

id-NetworkSliceAreaScopeofMDT,

id-GeographicalArea,

id-GeographicalAreaBasedMDT

maxEARFCN,

maxnoofAllowedAreas,

<<<unchanged is omitted>>>

maxnoofMDTSNPNs,

maxnoofSecurityConfigurations,

maxnoofRSPPQoSFlows,

maxnoofSliceItemsforMDT,

maxnoofGeographicalAreasforMDT

<<<unchanged is omitted>>>

AreaScopeOfMDT-NR ::= CHOICE {

cellBased					CellBasedMDT-NR,

tABased						TABasedMDT,

tAIBased					TAIBasedMDT,

...,

choice-extension		ProtocolIE-Single-Container { {AreaScopeOfMDT-NR-ExtIEs} }

}

AreaScopeOfMDT-NR-ExtIEs XNAP-PROTOCOL-IES ::= {

{ ID id-PNI-NPNBasedMDT			CRITICALITY ignore	TYPE PNI-NPNBasedMDT			PRESENCE mandatory}|

{ ID id-SNPN-CellBasedMDT		CRITICALITY ignore	TYPE SNPN-CellBasedMDT			PRESENCE mandatory}|

{ ID id-SNPN-TAIBasedMDT		CRITICALITY ignore	TYPE SNPN-TAIBasedMDT			PRESENCE mandatory}|

{ ID id-SNPN-BasedMDT			CRITICALITY ignore	TYPE SNPN-BasedMDT			PRESENCE mandatory}|

ID id-GeographicalAreaBasedMDT CRITICALITY ignore	 TYPE GeographicalAreaBasedMDT			PRESENCE mandatory},

...

}

<<<unchanged is omitted>>>

GBRQoSFlowInfo ::= SEQUENCE {

maxFlowBitRateDL			BitRate,

maxFlowBitRateUL			BitRate,

guaranteedFlowBitRateDL		BitRate,

guaranteedFlowBitRateUL		BitRate,

notificationControl			ENUMERATED {notification-requested, ...}				OPTIONAL,

maxPacketLossRateDL			PacketLossRate											OPTIONAL,

maxPacketLossRateUL			PacketLossRate											OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {GBRQoSFlowInfo-ExtIEs} }	OPTIONAL,

...

}

GBRQoSFlowInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-AlternativeQoSParaSetList	CRITICALITY ignore	EXTENSION AlternativeQoSParaSetList	PRESENCE optional	},

...

}

GeographicalArea ::= OCTET STRING

GeographicalAreaBasedMDT::= SEQUENCE {

geographicalAreaListforMDT		GeographicalAreaListforMDT,

iE-Extensions		ProtocolExtensionContainer { {GeographicalAreaBasedMDT-ExtIEs} } OPTIONAL,

...

}

GeographicalAreaBasedMDT-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

GeographicalAreaListforMDT ::= SEQUENCE (SIZE(1.. maxnoofGeographicalAreaforMDT)) OF GeographicalAreaListforMDTItem

GeographicalAreaListforMDTItem ::= SEQUENCE {

geographicalArea				GeographicalArea,

iE-Extensions		ProtocolExtensionContainer {{ GeographicalAreaListorMDTItem-ExtIEs}}		OPTIONAL,

...

}

GeographicalAreaListforMDTItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<unchanged is omitted>>>

9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

<<<unchanged is omitted>>>

maxnoofSecurityConfigurations				INTEGER ::= 8

maxnoofRSPPQoSFlows							INTEGER ::= 2048

maxnoofSliceItemsforMDT						INTEGER ::= 1024

maxnoofGeographicalAreaforMDT				INTEGER ::= 32

<<<unchanged is omitted>>>

id-SRSPositioningConfigOrActivationRequest															ProtocolIE-ID ::= 473

id-NRPPaPositioningInformation																		ProtocolIE-ID ::= 474

id-NetworkSliceAreaScopeofMDT																		ProtocolIE-ID ::= XX1

id-GeographicalArea																		  ProtocolIE-ID ::= XX2

id-GeographicalAreaBasedMDT																		ProtocolIE-ID ::= XX3

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

Draft Reply LS on MDT for NTN

3GPP TSG-RAN WG3 Meeting #129	R3-25xxxx

Bengaluru, India, 25 - 29 August 2025

Title:	[Draft] Reply LS on MDT for NTN

Response to:	R2-2503203/ R3-253033

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	ZTE Corporation (will be RAN3)

To:	RAN2

Cc:	N/A

Contact person:	Jiren Han

han.jiren@zte.com.cn

Send any reply LS to:  3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: FFS

- Overall description

RAN3 thanks RAN2 for the reply LS on NTN for MDT.

RAN3 has agreed to only use the geographical area as the Area Scope for the logged MDT for NTN over both NGAP and XnAP.And the corresponding agreed TPs are given in the attachment.

- Actions

To RAN2:

ACTION: 	RAN3 kindly asks RAN2 to take the above information into consideration.

3	Dates of next RAN3 meetings

RAN3#129bis	    13th -17th October   		     Prague, Czech Republic

RAN3#130	         17th – 21st November   		Dallas, USA